| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | PRO#23 |
| Source:* | Huawei Technologies Co.,Ltd. |
| Date:* | 2016-05-06 |
| Contact:* | Jiaxin Yin, yinjiaxin@huawei.com |
| Reason for Change/s:* | The CR is to clarify the usage of times in oneM2M |
| CR against: Release* | Release One |
| CR against: WI* | Active MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 v1.6.0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR is a mirror CR? YES NO if YES, please indicate the document number of the original CR: <Document Number)<CR Number of the original CR to the current Release> |
| Template Version:23 February 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

### -----------------------Start of change 1-------------------------------------------

#### Forwarding

If the To parameter in the request does not start with the CSEBase URI of the receiver, the receiver CSE shall forward the request or shall serve the request locally (see below).

If the To parameter in the request starts with the CSEBase URI of the receiver, then the receiver CSE shall handle the request locally.

If the Request Expiration Timestamp, Result Expiration Timestamp or Operation Execution Time is set in the request, the receiver CSE shall forward the request before any of the time.

Acting as an originator the CSE shall perform the following procedures:

- "Send a Request to the receiver CSE".

- "Wait for Response primitive".

When the Response is received the receiver CSE shall:

- Primitive specific procedure: Forward the Response to the original CSE.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

#### Check existence of the addressed resource

If the Request Expiration Timestamp is given in the request and expired, the Hosting CSE shall reject the request with an "REQUEST_TIMEOUT" Response Status Code parameter value. Otherwise, the Hosting CSE shall handle the request before the Request Expiration Timestamp.

The Hosting CSE shall check if the resource addressed by the To parameter exists in the repository. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

The Hosting CSE shall also check if the conditions specified in the Filter Criteria parameter in the Retrieve/Update/Delete operation are met. If the condition check fails, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

If the Filter Criteria parameter is included in the Create request, the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

If the Hosting CSE does not support the content format (i.e. type of serialization) requested by the originator, the request shall be rejected with a Response Status Code indicating "NOT_ACCEPTABLE" error.

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

#### Create <request> resource locally

Creation of a <request> resource can only be done on a Receiver CSE implicitly. When the Receiver CSE receives a request for targeting any other resource type or requesting a notification in non-blocking mode, i.e. the Response Type parameter of the request is set to either 'nonBlockingRequestSynch' or 'nonBlockingRequestAsynch', and if the Receiver CSE supports the <request> resource type as indicated by the 'supportedResourceType' attribute of the <CSEBase> resource, the Receiver CSE shall create an instance of <request> resource based on the following steps. If the Receiver CSE does not support the <request> resource type, the 'nonBlockingRequestSynch' request shall be rejected with a Response Status Code indicating "NON_BLOCKING_REQUEST_NOT_SUPPORTED" error. For the 'nonBlockingRequestAsynch'request, a Receiver CSE that does not support the <request> resource type shall be able to respond to an acceptable request with a response containing an Acknowledgement without a reference to a resource containing the context of the request.

The Receiver CSE of a non-blocking request is the Hosting CSE for the <request> resource that shall be associated with the non-blocking request.

- Assign a value to the common attributes of <request> resource according to the following table:

Table 7.3.2.2-1: Common attributes settings for <request> resource

| Attribute Name | Value |
| --- | --- |
| resourceType | Request |
| resourceID | Hosting CSE shall assign a value to this attribute. |
| expirationTime | The value of the expirationTime shall be chosen dependent on the Request Expiration Timestamp, Result Expiration Timestamp, Operation Execution Time and Result Persistence parameters associated with the original request. If the value consistent with the Request Expiration Timestamp, Result Expiration Timestamp, Operation Execution Time and Result Persistence parameters is too long, the Hosting CSE shall reject the request. The expirationTime of the <request> resource shall last longer than the specified Result Persistence if provided in the request. Within the time period, the <request> resource may be updated for result sharing and tracking purpose. |
| parentID | The parent resource of a <request> resource shall be the <CSEBase> resource of the Hosting CSE. |
| creationTime | Date/time of creation of this resource. |
| lastModifiedTime | Date/time which is equal to the creationTime. |
| accessControlPolicyIDs | Populate with one ID of an <accessControlPolicy> that contains the following: In the 'privileges' attribute - Allow RUD operations to the Hosting CSE - Allow RD operations to the Originator, i.e. the value of the parameter From in the associated non-blocking request In the 'selfPrivileges' attribute - Allow U operations the parent <accessControlPolicy> resource to the Originator, i.e. the value of the parameter From in the associated non-blocking request |
| Labels | Originator ID |
| stateTag | 0 |
| resourceName | Hosting CSE shall assign a value to this attribute. |

- Assign a value to the resource-specific attributes of <request> resource according to the following table:

Table 7.3.2.2-2: Resource-specific attributes settings for <request> resource

| Attribute Name | Value |
| --- | --- |
| operation | The value of the parameter Operation in the associated non-blocking request. |
| Target | The value of the parameter To in the associated non-blocking request. |
| originator | The value of the parameter From in the associated non-blocking request. |
| requestID | The value of the parameter Request Identifier in the associated non-blocking request. |
| metaInformation | The content of this attribute is set to information in optional parameters described in clause 8.1.2 of [Error: Reference source not found] given in the associated non-blocking request. |
| Content | The value of the parameter Content, if any, in the associated non-blocking request. |
| requestStatus | The Receiver CSE shall set this to "PENDING". |
| operationResult | Empty |

### -----------------------End of change 3---------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?