| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 34 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2018-03-04 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active < WI-0058> MNT maintenance Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / |
| CR against: TS/TR* | TS-0026-V0.7.0 |
| Clauses * | ### 5.2 Functional mapping between 3GPP and oneM2M |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0001 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The Figure 5.2-1 shows Functional mapping between 3GPP and oneM2M.

It is mixed reference point and functions between elements. For example, (S)Gi is a reference point between P-GW and SCS. However API is not reference point between SCEF and SCS. The reference point is T8 which is to use between SCEF and SCS specified at clause 4.2 in 3GPP TS 23.682. Other reference points are omitted i.e. SCEF, MME, S-GW, P-GW and so on.

Another point is SMS, NAS and IP. This is a kinds of communication methods or protocol stack between UE and 3GPP domain. This is not a reference point. In terms of reference point, Uu reference point uses between UE and UTRAN/E-UTRAN

This contribution proposes to right reference point for oneM2M interworking with 3GPP network. After that, we propose three kinds of interworking scheme: (i) IP, (ii) NAS, (iii) SMS

- 5.2	Functional mapping between 3GPP and oneM2M

Figure 5.2-1 shows an architecture for interworking 3GPP domain which may provide the features and services that are exposed by 3GPP.  An IN-CSE , oneM2M entities, may provide SCS functionalities to 3GPP domain. An IN-AE may provide Application Service to 3GPP domain. An UE may host oneM2M based application and/or CSE.

The “MTC Applications” that are hosted on the UE correspond to oneM2M entities in case:

- Application only: UE may be ADN as of oneM2M entity

- Application and CSE: UE may be ASN or MN as of oneM2M entity

- CSE only: UE may be MN as of oneM2M entity

- Neither application nor CSE: UE may host a NoDN.

The SCS may be an IN-CSE, and the “MTC-Applications” or ASs that are hosted in an external network may be an IN-AE.

The “3GPP Trust Domain” in Figures 5.1-1 and Figure 5.2-1 captures the functional entities that MUST be part of the 3GPP domain (the network). Although Figure 5.2-1 show that the IN-CSE and IN-AE are outside of the 3GPP Domain, it should be noted that the IN-CSE may be part of the operator domain.

This figure also shows the oneM2M reference points Mcn and Mca. The Mcc reference point is not shown explicitly as it would have rendered the picture less legible, but as is defined in [1], clause 5, the Mcc reference point is per definition between the two CSEs on respectively the left-hand side and the right-hand side of the figure. It is transported over any available data path between the two CSEs.

Figure 5.2-1: Architecture for interworking 3GPP domain

Several implementation options for the placement of the oneM2M (IN) CSE relative to the SCEF and the 3GPP network are envisioned. In all implementations, the SCEF always resides within 3GPP domain.

In some options the IN-CSE and the SCEF are deployed by a mobile network provider and are both part of the operator domain.  In other options the SCEF is part of the 3GPP domain and the IN-CSE is not part of the operator domain.

In all options, services within the IN-CSE may access the network services that are exposed by the SCEF via the API in T8 reference point.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?