| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 34 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2018-03-04 |
| Reason for Change/s:* | See the introduction. Several attributes are needed for 3GPP interworking. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active < WI-0035> MNT maintenance Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / |
| CR against: TS/TR* | TS-0001-V3.10.0 |
| Clauses * | ### 9.6.49 Resource Type triggerRequest |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0026 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The 3GPP TS 29.337 specifies Diameter based T4 interface for device triggering which consists of Device Trigger Request (DTR), Device Trigger Answer (DTA), Delivery Report Request (DRR) and Delivery Report Asnswer (DRA).

First, this contribution proposes to add triggerDeliveryReport value to triggerPurpose attribute to support DRR and DRA.

The information element for DRR message is specified at clause 5.2.2 in 3GPP TS 29.337 as follows .

| Information Element Name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identifier (See 3GPP TS 29.336 [12]) | User-Identifier | M | This information element shall contain the IMSI of the UE the device trigger was delivered, formatted according to 3GPP TS 23.003 [8], clause 2.2. This Information Element may contain the international ISDN number of the UE the device trigger was delivered, formatted according to 3GPP TS 23.003 [8], clause 3.3. The ISDN number shall be present if it is available to the SMS-SC. This Information Element may contain the external identifier of the UE the device trigger was delivered, formatted according to 3GPP TS 23.003 [8], clause 19.7.2. The external identifier shall be present if it is available to the SMS-SC. |
| SM RP SMEA (See 3GPP TS 29.338 [13]) | SM-RP-SMEA | M | This Information Element shall contain the SME address of the Service Capability Server that is requesting a device trigger to the UE as received from the MTC-IWF. |
| SM Delivery Outcome (See 6.3.1) | SM-Delivery-Outcome | M | This information element shall be present and indicate one of the following outcomes of the device trigger delivery: - Absent subscriber; - UE memory capacity exceeded; - Successful transfer. |
| Absent Subscriber Diagnostic SM (See 6.3.2) | Absent-Subscriber-Diagnostic-SM | C | This information element shall indicate the reason why the subscriber is absent. It shall be present if the device trigger failed to be delivered due to absent subscriber. |
| Trigger Reference Number (See 3GPP TS 29.368 [15]) | Reference-Number | C | This information element shall contain the Trigger Reference Number as received from the MTC-IWF for the device trigger. This information element shall be present if it is available in the SMS-SC. |
| Supported Features (See 3GPP TS 29.229 [6]) | Supported-Features | O | If present, this information element shall contain the list of features supported by the origin host. |

The specific value for SM-Delivery-Outcome is described at clause 6.3.1 in 3GPP TS 29.337 as follows.

ABSENT_SUBSCRIBER (0)

UE_MEMORY_CAPACITY_EXCEEDED (1)

SUCCESSFUL_TRANSFER (2)

VALIDITY_TIME_EXPIRED (3)

Second, this contribution proposes to add SM-Delivery-Outcome value to triggerStatus attribute to support DRR and DRA.

Third, the Trigger Reference Number is an information element for both DTR and DRR. A triggerReference attribute is introduced for Trigger Reference Number.

The information element for DTR message is specified at clause 5.2.1 in 3GPP TS 29.337 as follows .

| Information Element Name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identifier (See 3GPP TS 29.336 [12]) | User-Identifier | M | This information element shall contain the IMSI of the UE the trigger is to be applied, formatted according to 3GPP TS 23.003 [8], clause 2.2. This Information Element may contain the external identifier of the UE the device trigger was delivered, formatted according to 3GPP TS 23.003 [8], clause 19.7.2. The external identifier shall be present if it is available to the MTC-IWF. This Information Element may contain the international ISDN number of the UE the device trigger was delivered, formatted according to 3GPP TS 23.003 [8], clause 3.3. The ISDN number shall be present if it is available to the MTC-IWF. |
| SM RP SMEA (See 3GPP TS 29.338 [13]) | SM-RP-SMEA | M | This Information Element shall contain the SME address of the Service Capability Server that is requesting a device trigger to the UE. It shall be encoded according to the formatting rules of the address fields described in 3GPP TS 23.040 [9]. |
| Payload (See 3GPP TS 29.368 [15]) | Payload | M | This information element shall contain the device triggering payload as received over the Tsp. For device trigger recall, this IE shall be empty. |
| Serving Node Identity (See 3GPP TS 29.173 [14]) | Serving-Node | C | This information element shall contain the serving node identity, i.e. SGSN/MME/MSC identity serving the UE. It shall be present if it is available to the MTC-IWF, e.g. the MTC-IWF retrieved this information from the HSS. |
| Additional Serving Node Identity (See 3GPP TS 29.173 [14]) | Additional-Serving-Node | C | This information element shall contain another serving node identity, e.g. SGSN/MME/MSC identity, if there is any serving the UE. There may be multiple instances of this information elements. |
| Trigger Reference Number (See 3GPP TS 29.368 [15]) | Reference-Number | C | This information element shall contain the Reference Number related to the device trigger request. It shall be present if it is available to the MTC-IWF, e.g. the MTC-IWF received this information over Tsp. |
| Old Trigger Reference Number (See 3GPP TS 29.368 [15]) | Old-Reference-Number | C | This information element shall contain the Reference Number of the device trigger to be replaced. It shall be present if it is available to the MTC-IWF, e.g. the MTC-IWF received this information over Tsp. |
| Validity Time (See 3GPP TS 29.368 [15]) | Validity-Time | C | This information element shall contain the validity time of the device trigger request. It shall be present if it is available to the MTC-IWF, e.g. the MTC-IWF received this information over Tsp. |
| Priority Indication (See 3GPP TS 29.368 [15]) | Priority-Indication | C | This information element shall contain the priority of the device trigger request.. It shall be present if it is available to the MTC-IWF, e.g. the MTC-IWF received this information over Tsp. |
| SMS Application Port ID (See 3GPP TS 29.368 [15]) | SMS-Application-Port-ID | C | This information element shall contain the Application Port ID of the triggering application for the device trigger request. It shall be present if it is available to the MTC-IWF, e.g. the MTC-IWF received this information over Tsp. |
| Supported Features (See 3GPP TS 29.229 [6]) | Supported-Features | O | If present, this information element shall contain the list of features supported by the origin host. |
| Trigger-Action (See 6.3.6) | Trigger-Action | C | This information element shall be present for device trigger action replace or recall. This information element may be present for a normal device. |

Fourth, there are information element for DTR such as Priority Indication,  SMS Application Port ID which is not specified resource at architecture.

So, A triggerPriority attribute is introduced for Priority Indication.

A triggerPortID attribute is for SMS Application Port ID.

Fifth, there is information element for DRR Absent Subscriber Diagnostic SM that is missed resource at architecture.

A triggerDeviceDiagnostic attribute is for Absent Subscriber Diagnostic SM information.

Finally, the T8 Transaction Reference ID is described for one of the common parameters for T8 interfaces which is between SCEF and SCS (IN-CSE) as following at clause 4.9.2 in 3GPP TS 23.682.

T8 Transaction Reference ID (TTRI) is a parameter which refers to transactions (e.g. Set Chargeable Party Request followed by Set Chargeable Party Response, NIDD Submit, etc.) between the SCEF and the SCS/AS when using T8 interface. The transactions consist of one request message followed by one or more response messages. It is created by the originator of the transaction, and is unique through the duration of the transaction. It is stored on both the SCEF and the SCS/AS for the duration of the transaction.

### So, a t8TransactionRef attribute is introduced to resource for T8 Transaction Reference ID (TTRI).

### -----------------------Start of change 1-------------------------------------------

### 9.6.49	Resource Type triggerRequest

The <triggerRequest> resource is used to initiate a device trigger request.  This resource type shall only be instantiated on an IN-CSE.

The successful creation of a <triggerRequesst> resource results in the IN-CSE initiating a trigger request to a targeted device (e.g. 3GPP UE).   A pending trigger request can be replaced with a new trigger request by updating the <triggerRequesst> resource.  A pending trigger request can be cancelled by deleting the <triggerRequesst> resource.

The <triggerRequest> resource shall contain the child resource specified in table 9.6.49-1.

Table 9.6.49-1: Child resources of <triggerReqeust> resource

| Child Resources of <trafficPattern> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 of TS-0001 |

The <triggerRequest> resource shall contain the attributes specified in table 9.6.49-2.

Table 9.6.49-1: Attributes of <triggerRequest> resource

| Attributes of <triggerRequest> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| Labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| M2M-Ext-ID | 1 | WO | M2M External Identifier of the device being triggered. See clause 7.1.8. This attribute shall be configured by the Originator when the resource is created. |
| Trigger-Recipient-ID | 1 | RW | Trigger-Recipient-ID of the ASN/MN-CSE or ADN-AE that is hosted on the device being triggered. See clause 7.1.10. This attribute shall be configured by the Originator when the resource is created and may also be updated when performing a trigger replace procedure. See clause 8.3.3.2.2. |
| triggerPurpose | 1 | RW | The purpose of the trigger. See clause 8.3.3.2.1. This attribute may be configured by the Originator when the resource is created and may also be updated when performing a trigger replace procedure. The allowed values are: - establishConnection - enrolmentRequest - registrationRequest - executeCRUD - triggerDeliveryReport If not specified by the Originator, the default is "establishConnection". |
| triggerPayloadSerialization | 1 | RW | The type of serialization used to encode the trigger payload (i.e. XML, JSON or CBOR). This attribute may be configured by the Originator when the resource is created and may also be updated when performing a trigger replace procedure. See clause 8.3.3.2.2. If not specificied by the Originator, the default is "JSON". |
| triggerStatus | 1 | RO | The status of the trigger request. The Hosting CSE shall control the value of this attribute. The following values are valid values. - PROCESSING - ERROR-NSE-NOT-FOUND - TRIGGER-SUBMITTED - TRIGGER-DELIVERED - TRIGGER-FAILED - ABSENT-SUBSCRIBER - UE-MEMORY-CAPACITY-EXCEEDED - SUCCESSFUL-TRANSFER - VALIDITY-TIME-EXPIRED |
| triggerValidityTime | 0..1 | RW | The time duration for which the trigger request is valid. After this time expires, the trigger shall be recalled (i.e. cancelled) by the Hosting CSE. This attribute may be configured by the Originator when the resource is created and may also be updated when performing a trigger replace procedure. See clause 8.3.3.2.2. |
| triggerInfoAE-ID | 0..1 | RW | When the triggerPurpose is “executeCRUD”, this attribute is mandatory otherwise it is not applicable. This attribute is configured with the AE-ID of the ASN/MN-AE that should perform the CRUD operation. When this attribute is configured, the trigger originator shall also configure the triggerInfoAddress, triggerInfoOperation and targetedResouceType attributes. |
| triggerInfoAddress | 0..1 | RW | When the triggerPurpose is “executeCRUD”, this attribute is mandatory otherwise it is not applicable. This attribute is configured with an unstructured CSE-Relative-Resource-ID of the resource that the ASN/MN-AE should perform the CRUD operation on. When this attribute is configured, the trigger originator shall also configure the triggerInfoAE-ID, triggerInfoOperation and targetedResouceType attributes. |
| triggerInfoOperation | 0..1 | RW | When the triggerPurpose is “executeCRUD”, this attribute is mandatory otherwise it is not applicable. This attribute is configured with the CRUD operation that the ASN/MN-AE should perform on the targeted resource specified by triggerInfoAddress. When this attribute is configured, the trigger originator shall also configure the triggerInfoAE-ID, triggerInfoAddress and targetedResouceType attributes. |
| targetedResourceType | 0..1 | RW | When the triggerPurpose is “executeCRUD”, this attribute is mandatory otherwise it is not applicable. This attribute is configured with the resource type of the targeted resource specified by triggerInfoAddress. When this attribute is configured, the trigger originator shall also configure the triggerInfoAE-ID, triggerInfoAddress and triggerInfoOperation attributes. |
| triggerReference | 0..1 | WO | This attribute is reference number to trace Device Trigger Request and Delivery Report Request to support 3GPP 29.337 |
| triggerPriority | 0..1 | RW | This attribute is priority for Device Trigger Request either low priority or high priority to support 3GPP TS 23.272. |
| triggerDeviceDiagnostic | 0..1 | RW | This attribute may use Delivery Report Request to check the reason for error. |
| triggerPortID | 0..1 | RW | This attribute is application port ID for device trigger to support 3GPP 29.337. |
| t8TransactionRef | 0..1 | WO | This attribute is transaction reference identifier between IN-CSE for SCS and SCEF to support 3GPP TS 23.682. |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?