| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 34 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2018-03-04 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active < WI-0058> MNT maintenance Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / |
| CR against: TS/TR* | TS-0026-V0.7.0 |
| Clauses * | ### 6. Connectivity Establishment |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Figure 6.1-1 is difficult to understand and ambiguous 3GPP Underlying Network

This document proposes to clarify Figure 6.1-1 which has GTP-tunnel, IP connection and Mcc interface in hierarchical aspects. i.e. protocol stack for user plane of each element.

Based on this concept, it proposes re-organize a table of contents with IP connection and Mcc connection.

Current clause 6.2, 6.3 and 6.4 is about IP connection

- 6.2 ASN/MN-CSE Pre-configuration

- 6.3 ASN/MN-CSE or ADN-AE Initiated Connectivity Establishment Procedure

- 6.4 IN-CSE initiated connectivity establishment procedure

Therefore, it proposes to change table of contents without any change of their contents

- 6.2 IP connectivity

- 6.2.1 ASN/MN-CSE Pre-configuration

- 6.2.2 ASN/MN-CSE or ADN-AE Initiated Connectivity Establishment Procedure

- 6.2.3 IN-CSE initiated connectivity establishment procedure

The clause 6.5 is about Mcc connection. So, the numbering for table of contents is proposed to change 6.3.

### -----------------------Start of change 1-------------------------------------------

- 6 Connectivity Establishment

## 6.1 Overview

ASN/MN-CSE and the serving IN-CSE communicate after completion of the Underlying Network bearer establishment and discovery of the serving IN-CSE. Data can then traverse between CSEs over the IP connection in the Underling Network over 3GPP Gi/SGi interface. In addition, the signalling connectivity between the two CSEs is also realized. Figure 6.1-1 depicts the connectivity between the ASN/MN-CSE and the IN-CSE.

Figure 6.1-1: Connectivity Establishment between ASN/MN-CSE or ADN-AE and IN-CSE

For ASN/MN initiated connectivity establishment, it is assumed that there is no connectivity previously established, i.e. no association between the ASN/MN-CSE or ADN-AE and the serving IN-CSE exists. When the ASN/MN-CSE or ADN-AE needs to send data to the serving IN-CSE it first discovers the serving IN-CSE, which is located in a packet data network, and establishes connection. Two methods can be used, as follows:

- Use of DHCP and DNS.

- Pre-configuration.

For serving IN-CSE initiated connectivity establishment, it is assumed that there is no connectivity previously established between the ASN/MN-CSE and the serving IN-CSE. When the serving IN-CSE needs to contact the ASN/MN-CSE to send data or request data, connectivity between them is established. This connectivity is triggered by the serving IN-CSE.

The ASN/MN-CSE or ADN-AE requests the DNS server address from the DHCP server followed by requesting the serving IN-CSE IP address from the DNS server.

## 6.2	IP connectivity

### 6.2.1	ASN/MN-CSE Pre-configuration

The ASN/MN-CSE or ADN-AE is preconfigured with the fully qualified domain name (FQDN) of the serving IN-CSE or the IP address of the serving IN-CSE. If the FQDN is known, DNS resolution is used to obtain the IP address.

### 6.2.2 ASN/MN-CSE or ADN-AE Initiated Connectivity Establishment Procedure

Figure 6.3-1: ASN/MN-CSE or ADN-AE initiated connectivity establishment

Step-0: Trigger

Subsequent procedures are triggered either when the ASN/MN-CSE or ADN-AE powers on or resulting from Device Triggering

Step-1: Bearer Setup Procedure

Establish a 3GPP bearer(s) if not already available by using the procedures available in the 3GPP  network.

Step-2: DHCP Query & Response

The ASN/MN-CSE or ADN-AE sends a query to a DHCP server to find a particular DNS server IP address. The DHCP server responds with the IP address of a corresponding DNS server. Additionally, it is also possible to include one or a list of domain names, i.e. FQDNs of target IN-CSEs.

Step-3: DNS Query & Response

The ASN/MN-CSE or ADN-AE performs a DNS query to retrieve the IN-CSE(s) IP addresses from which one is selected. If the response does not contain the IP addresses, an additional DNS query is needed to resolve a Fully Qualified Domain Name (FQDN) of the serving IN-CSE to an IP address.

Step-4: Connection Establishment

After reception of domain name and IP address of an IN-CSE, the ASN/MN-CSE or ADN-AE can initiate communication towards the IN-CSE via the IP connection. The IN-CSE at this time shall be informed which Trigger Recipient ID of the ASN/MN-CSE or ADN-AE or the AE-PoA of the ADN-AE to use for establishing communication.

Step-5: CSE-PoA Update

Once the M2M Service Connection (Mcc) is established, in the IN-CSE the CSE-PoA of the ASN-CSE/MN-CSE or the AE-PoA of the ADN-AE shall be updated with the new established IP address.

The IN-CSE holds the state information and needs to be informed when the connection is closed.

### 6.2.3 IN-CSE initiated connectivity establishment procedure

See clause 7.1 for NIDD based connection establishment and clause 7.5 SMS Based Device Triggering.

## 6.3	UE Attach with oneM2M Registration Procedure

After a UE attaches to 3GPP network and establishes a data (i.e. PDN/PDP) connection, assuming it has not been registered previously with the oneM2M platform, it will initiate a oneM2M registration procedure. The oneM2M part of the procedure corresponds to the generic one described in clause 10.2.2 of TS-0001[1], where the Registrar CSE is the IN-CSE.

If the UE hosts an ADN, the registration will result in the IN-CSE hosting one or more corresponding <AE> resources for the UE. If the UE hosts an ASN or a MN, the registration of the ASN/MN-CSE will result in the IN-CSE hosting a corresponding <remoteCSE> resource for the ASN/MN-CSE.  The flow is depicted in figure 6.5-1.

Figure 6.4-1 UE Attach Procedure with oneM2M Registration

Steps 1a and 1b: UE Attach Request and Response

The UE determines that it needs to perform the 3GPP Attach procedure.

The 3GPP Network responds to the UE Attach request.

Steps 2a and 2b: oneM2M Registration Request and Response

The UE Hosting an ADN or an ASN/MN determines that it needs oneM2M services and decides to register to the oneM2M infrastructure, so it performs the registration procedure detailed in clause 10.2.2 of TS-0001[1].

- If the UE hosts an ADN, the registration procedure of the ADN-AE(s) will include the creation of corresponding <AE> resource(s) being hosted at the IN-CSE.

- If the UE hosts an ASN or MN, the registration procedure of the ASN/MN-CSE will include the establishment of a security association and the creation of a <remoteCSE> resource, corresponding to the registree CSE, to be hosted at the IN-CSE.

The IN-CSE responds to the UE hosted ADN-AE(s) or ASN/MN-CSE request for registration.

These steps allow the UE hosted ADN-AE(s) or ASN/MN-CSE to provide the IN-CSE with information needed for communication and providing services, e.g. pointOfAccess. In return the IN-CSE provides the UE with oneM2M specific information, e.g. by assigning AE-ID(s) or CSE-ID to the registree(s), respectively

Information provided by the ASN/MN-CSE or ADN-AE(s) to the IN-CSE at this time shall include:

- M2M-Ext-ID, Trigger-Recipient-ID as attributes of <AE> or <remoteCSE>, if available.

- externalGroupID as attribute of <remoteCSE>, if available.

- nodeLink as an attribute of the <AE> or <remoteCSE> resources, providing the resource identifier of a corresponding <node> resource, if pre-provisioned at the UE.

Step 3: Handling of association with a <node> resource

If in Step 2, the IN-CSE receives a valid nodeLink (as an attribute of the <AE> or <remoteCSE> resources) it means that the <node> resource storing the node specific information for this UE exists and has been pre-provisioned at the UE. For a UE hosted ASN/MN, the IN-CSE shall ensure that the hostedCSELink attribute of the corresponding <node> resource contains the resource identifier of the <remoteCSE > resource. For a UE hosted ADN, the IN-CSE shall ensure that the hostedAELinks attribute of the corresponding <node> resource includes the resource identifier of all the corresponding <AE > resources.

If in Step 2, the IN-CSE does not receive a valid nodeLink (as an attribute of the <AE> or <remoteCSE> resources) or the attribute is not present, the IN-CSE shall create a <node> resource for this UE. The IN-CSE shall populate the nodeLink attribute of the <AE> or <remoteCSE> resources with the resource identifier of the newly created <node> resource. For a UE hosted ASN/MN, the IN-CSE shall also populate the hostedCSELink attribute of the <node> resource with the resource identifier of the <remoteCSE > resource. For a UE hosted ADN, the IN-CSE shall ensure that the hostedAELinks attribute of the <node> resource includes the resource identifiers of all the corresponding <AE > resources.

If the following resources do not exist for this Underlying Network, the IN-CSE shall:

- create a <schedule> resource as a child of the <node> representing the UE, which represents the communication schedule for the 3GPP Underlying Network where the Attach procedure was completed. While no specific scheduling information is available, the <schedule> may reflect that communications are available continuously. The IN-CSE may set the networkCoordinated attribute, based on the UE pre-provisioned information or local policies, to indicate if the schedule should be coordinated with the Underlying Network.

create the [activeCmdhPolicy] resource as a child of the <node> resource representing the UE, providing the active communication policies for the 3GPP Underlying Network where the Attach procedure was completed. The IN-CSE shall populate the mgmtLink of the active [cmdhNwAccessRule] with a link to the <schedule> resource created for the communication schedule with this Underlying Network.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?