| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #34 |
| Source:* | Miguel Angel Reina Ortega, miguelangel.reinaortega@etsi.org Bob Flynn, Bob.Flynn@InterDigital.com |
| Date:* | 2018-03-08 |
| Reason for Change/s:* | timeSeries procedure correction |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: ARC-2018-0077R01-TS-0001_timeSeries_procedure_correction_R3 STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v2.18.0 |
| Clauses * | Clause 10.2.39 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0004 v2.16.0 Clauses 7.5.1.2.9 (CR PRO-2018-0100-TS-0004_timeSeries_procedure_correction_R2) |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to correct the timeSeries procedure defined in TS-0001 Section 10.2.4.29, regarding the rules of the reporting policy.

The first rule in the list indicates that “The missing data points counter is reset back to 0” which will make that the missing data points never be “greater than the minimum specified missing number of the Times Series Data specified in missingData condition”.

In addition, it’s not aligned with the diagram depicted in Figure 10.2.4.29-1 regarding item T3. According to the diagram at T3 a NOTIFY request is sent. However, if we follow the rules, after T2 the missing data points counter is reset, so that missing data counter at T3 is not equal to or greater than the minimum specified missing number.

The solution proposed is to follow the procedure defined in the diagram by not reseting the missing data counter but keeping on counting instead.

Also text for all rules is improved for more clarification

### -----------------------Start of change 1-------------------------------------------

#### 10.2.39	Procedure for Time Series Data Detecting and Reporting

In the case that the periodicInterval is set and the missingDataDetect is TRUE, the Hosting CSE shall monitor the Time Series Data based on its periodicalInterval. When the Hosting CSE detects a missing data point, the dataGenerationTime of the missing data point is inserted into the missingDataList attribute and the missingDataCurrentNr shall be increased by one. When the missingDataCurrentNr reaches the missingDataMaxNr, the oldest dataGenerationTime shall be removed from missingDataList to enable the insertion of the new missing data point information.

When an AE wants to be informed of  the number of missing data points in a given renewable time duration, the AE should request the creation of  a <subscription> resource and set the missingData in the eventNotificationCriteria conditions to specify  the reporting policy. This enables the AE  to keep track of the number of  missing data points and the corresponding time-stamps over a predefined but renewable duration (i.e. the “window durarion” of the missingData condition).

When the Hosting CSE reports missing data points, it shall check the missingData condition in the applicable subscription resource created by the AE for that purpose.

When the first missing data point is detected(i.e. a detection of the first discontinuous time-stamp), following the creation of the subscription, the Hosting CSE shall start a timer, and keep counting the number of the missing data points. The timer is set according to the “window duration” in the missingData  condition. The reporting policy is governed by the rules below:

- If the total number of missing data points become equal to or greater than the “number” specified in missingData condition before the timer expires, a NOTIFY request shall be sent with the missingDataList and currentMissingDataNr included in the NOTIFY request. The missing data points counter shall continue counting while the timer continues to run (since it did not expire). Initiating NOTIFY request to report missing data points shall follow the same logic described above until the timer expires (see next bullet for behavior when the timer expires).

- If the timer expires, the timer is restarted and the missing data points counter is reset back to 0.

- The renewal of the timer and the missing data points counter upon timer expiry shall continue until such time as the subscription is cancelled or terminated. Once a subscription is terminated, a final NOTIFY request is sent out with the current number of missing data points and the timer is stopped.

- If no missing data points have been detected at all during the lifetime of a subscription, then no timer shall be started at all. But once a timer is started triggered by the first missing data point, then the above rules in the previous bullets shall apply.

Figure 10.2.39-1 depicts the above rules.

Figure 10.2.39-1: Time Series Data Detecting and Reporting Mechanism

T1: the timer is started and the number of the missing data points is counted.

T2: the NOTIFY Request is sent because the total number of missing data points becomes equal to or greater than the “minimum specified missing number of the Time Series Data” in missingData condition.

T3: the NOTIFY Request is sent.

T4: the timer is restarted and the missing data points counter is reset back to 0.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?