| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#34 |
| Source:* | Bob Flynn, Convida Wireless; Flynn.Bob@convidawireless.com |
| Date:* | 2018-03-15 |
| Reason for Change/s:* | See Introduction |
| CR against: Release* | Release 3 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 V0_7_0-Clean-Up |
| Clauses * | 7.5.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0001, TS-0004 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

During review of TS-0026 section 7.5.1 for the purpose of generating conformance test purposes exception cases were identified and are contributed in this document.

- A companion is needed for TS-0001 and TS-0004 to add a new value for the triggerStatus attribute of the <triggerRequest> - TRIGGER_EXPIRED

- The subscription/notification part of the notification process is removed from the figure.  The step is converted to a comment to provide the intended information.

- Combined step 9 and 10.

- A remaining points for this document

- Step 3 - the procedure is aligned to the “explicit” trigger procedure. However, it is not clear where the information needed should come from in the case that this is an “implicit” trigger request.

Step 9 :If the trigger has no payload, the ASN/MN-CSE or ADN-AE shall re-establish connectivity with the IN-CSE. If the address of the IN-CSE is not known to the ASN/MN-CSE or ADN.

- It was stated that the 3GPP communication module is defined by 3GPP. Additional detail such as the name of the 3GPP message should be provided in this step so that we can define behaviour for the receiving CSE/AE.

What 3GPP messages arrive at the AE/CSE.  How should oneM2M entity handle these?

Also, can this be ADN AE, or will we support only UE with CSE?  AE does not have Mcn interface

### -----------------------Start of change 1-------------------------------------------

### General Procedure for 3GPP Based Device Triggering

An IN-CSE may initiate a Device Trigger Request to an ASN/MN-CSE or ADN-AE to have it establish a connection to the IN-CSE, enroll to a MEF, register to the IN-CSE, update its PoA, or perform a CRUD operation on a specified resource.  The IN-CSE may initiate the Device Trigger Request itself or it may be initiated by a request that the IN-CSE receives from an AE.

Whenever the IN-CSE sends a Device Trigger Request to an ASN/MN-CSE or ADN-AE hosted on a 3GPP UE, the device triggering procedure as described in 3GPP TS 23.682 [2] is used as the basis for the procedures defined below.

This procedure supports an ASN/MN-CSE or ADN-AE that is hosted on a 3GPP UE that is directly connected to a 3GPP access network.

Figure 7.5.1-1: General Procedure for Device Triggering

Pre-conditions

The UE that hosts the ASN/MN-CSE or ADN-AE is available to receive the Device Trigger Request using one of the connectivity establishment methods described in clause 6.

Step 1 (Optional): Request targeted to ASN/MN-CSE or ADN-AE

An AE may initiate a Device Trigger Request to an ASN/MN-CSE or ADN-AE explicitly by creating or updating a <triggerRequest> resource [reference to TS-0001] hosted on an IN-CSE. Alternatively, an AE may initiate a Device Trigger Request to an ASN/MN-CSE or ADN-AE implicitly by issuing a request to an IN-CSE that requires device triggering. For example, if an IN-CSE receives an AE request to perform a CRUD operation targeting an ASN/MN-CSE or ADN-AE that is not reachable by the IN-CSE, the IN-CSE may generate a trigger request.

Step 2: Determine if Device Triggering is required

The IN-CSE determines whether to send a Device Trigger Request to the targeted ASN/MN-CSE or ADN-AE.  Further details are provided in clause 8.3.3.2.1 of oneM2M TS-0001 [1].

Step 3: Device Trigger Request

The IN-CSE sends the Device Trigger Request that contains information as specified in 3GPP TS 29.122 [5]Error: Reference source not found. Such information includes:

- externalId shall be set to the M2M-Ext-ID of the targeted UE hosting an ASN/MN-CSE or ADN-AE.

- scsAsId shall be set to a value that is prearranged between the Service Provider and MNO.

- tltrId is used to identify the trigger request.  It shall be assigned based on internal IN-CSE polices and shall be different from other tltrIds issued by this IN-CSE during the lifetime of this request.  Later, if the request needs to be recalled, this value will be used to reference the request that needs to be recalled.

- triggerReferenceNumber shall be set to a value assigned based on internal IN-CSE polices and shall be different from other triggers issued by this IN-CSE during the lifetime of this request.

- validityPeriod shall be set to either the triggerValidityTime attribute of the <triggerRequest> resource if the trigger request is initiated by an AE or otherwise by IN-CSE internal policies.

- priority may be set to either PRIORITY or NO_PRIORITY per internal IN-CSE policies and/or agreements between the Service Provider and MNO.

- applicationPortID shall be set to Trigger-Recipient-ID of the targeted ASN/MN-CSE or ADN-AE.

- triggerPayload (if present) shall be configured as described in clause 8.3.3.2.1 of oneM2M TS-0001 [1] and clause 9.2.1 of TS-0004 [4]. An empty payload indicates that the targeted ASN/MN-CSE or ADN-AE shall re-establish connectivity with the IN-CSE.

- notificationDestination shall be configured with a URI that the SCEF can target Device Trigger notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

- General Exceptions:

- The SCEF is not reachable when IN-CSE tries to send Device Trigger Request. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to ERROR_NSE_NOT_FOUND after a prior timeout period (cse local policy)

Step 4: Device Trigger Response

The SCEF handles the Device Trigger Request together with the Mobile Core Network, as described in 3GPP TS23.682[2]. The SCEF sends a Device Trigger Response to the IN-CSE with a response code of 201 CREATED to acknowledge the Device Trigger Request has been accepted for delivery to the UE.  This message is defined in 3GPP TS 29.122 [5] and includes the following information.

- id is a SCEF-chosen identifier for this Device Trigger Request.

- tltrId (same as in the request)

- self is configured with a link to the resource created by the SCEF for the request

- deliveryResult is configured with the delivery status of the Device Trigger Request as defined in 3GPP TS 29.122 [5]

- The SCEF sends Device trigger response with deliveryResult attribute set to SUCCESS or CONFIRMED. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_SUBMITTED.

- General Exceptions:

- The SCEF does not response to the Device Trigger Request sent in step 3 prior to a timeout period (cse local policy). In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to ERROR_NSE_NOT_FOUND

- The SCEF sends failure statusCode in the Device Trigger Response. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_FAILED

- The SCEF sends Device trigger response with deliveryResult attribute set to error status (UNKNOWN, FAILURE, UNCONFIRMED, TERMINATE). In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_FAILED

- The SCEF sends Device trigger response with deliveryResult attribute set to “EXPIRED”. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_EXPIRED

Step 5: Device Triggering delivery procedure

The trigger is delivered to the UE hosting the ASN/MN-CSE or ADN-AE.

Step 6: Device Trigger Notification

The SCEF sends a Device Trigger Notification containing a Delivery Report message to the IN-CSE to indicate the trigger delivery outcome. The SCEF indicates if delivery was successful, unknown, or failed.  This message is defined in 3GPP TS 29.122 [5] and includes the following:

- transactionId configured with the identifier of the device triggering transaction resource

- transaction configured with a link to the related device triggering transaction resource to which this notification is related

- tltrId (same as in the request)

- result is configured with the delivery status of the Device Trigger Request

Step 7: Device Trigger Notification Response

After receiving a UE Reachability Monitoring Event Notification, the IN-CSE shall return a HTTP response having a response code of 204 NO CONTENT and no payload.

Step 8: IN-CSE Updates <triggerRequest>

If Device Trigger was initiated by an AE via a <triggerRequest> resource, then IN-CSE shall update the <triggerRequest> resource with trigger status information as given below:

- IN-CSE receives Device Trigger Notification with result attribute set to “SUCCESS” or “CONFIRMED”. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_DELIVERED.

- IN-CSE receives Device Trigger Notification with result attribute set to error status (UNKNOWN, FAILURE, UNCONFIRMED, TERMINATE, EXPIRED). In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_FAILED.

Step 9: Perform Trigger Actions

Based on the type of trigger request received, the ASN/MN-CSE or ADN-AE performs the corresponding trigger actions such as establish connectivity with the IN-CSE, enrol with the MEF, register to the IN-CSE, update its PoA, or execute a CRUD request on a specified resource.

If the trigger has no payload, the ASN/MN-CSE or ADN-AE shall re-establish connectivity with the IN-CSE. If the address of the IN-CSE is not known to the ASN/MN-CSE or ADN.

Further details are described in clause 8.3.3.2.1 of oneM2M TS-0001 [1] and clause 9.2.1 of TS-0004 [4]

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?