| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 35 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com Bob Flynn, Convida Wireless; Flynn.Bob@convidawireless.com Dale Seed, Convida, Seed.Dale@convidawireless.com |
| Date:* | 2018-05-23 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active < WI-0058> MNT maintenance Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / |
| CR against: TS/TR* | TS-0026-V0.8.1 |
| Clauses * | ### 7.5.1 General Procedure for 3GPP Based Device Triggering |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0001 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The general procedure for device triggering is described at claue 8.3.3.2 in TS-0001.

An interworking architecture for 3GPP and oneM2M can be considered as following figure.

A device triggering for UE that runs oneM2M entities may perform either T8 or Tsp. It is two different interface toward IN-CSE. However, information element for triggering procedure may identical between T8 and Tsp.

The difference between T8 and Tsp is protocol which is communication methods.

T8 support Application Programming Interface and Tsp support Diameter over SCTP as specified at clause 6 in 3GPP TS 29.368.

R01:

- R01 is based on ARC-2018-0060R02 which is WG agreed clean up for TS-0026.

- Procedure is discussed with Bob and agreed  with Bob based on 3GPP TS 29.122 which define two API for device triggering.

- DeviceTriggering

- DeviceTriggeringDeliveryReportNotification

- Description of procedure is revised.

Figure has been changed with 3GPP Network Elements instead of detailed network elements such as SGSN or C-SGN, SMS-SC/GMSC/IWMSC. Based on the figure, procedure is revised.

3GPP TS 29.122 describes procedures for device triggering as following.

### 4.4.6	Procedures for Device Triggering

The procedures are used by the SCS/AS to deliver the device trigger via T8 interface.

In order to create a new device trigger, the SCS/AS shall send an HTTP POST message to the SCEF. The body of the HTTP POST message shall include the External Identifier or MSISDN, SCS/AS Identity, trigger reference number, validity period, priority, Application Port ID and trigger payload, and may include TLTRI.

Upon receipt of the corresponding HTTP POST message, the SCEF shall check if the SCS/AS is authorised to send a trigger request and if the SCS/AS has exceeded its quota or rate of trigger submission. The SCEF shall also resolve the External Identifier or MSISDN to IMSI and retrieve the "Routing Information" from HSS for the triggering delivery. If the authorisation check fails, or if the quota or rate of trigger submission was exceeded, or if there is no valid subscription information or if the "Routing Information" cannot be found, then the SCEF shall reject the request with an error message to the SCS/AS. Otherwise, the SCEF shall perform the device trigger procedure over Tsp as defined in 3GPP TS 29.368 [24] and T4 as defined in 3GPP TS 29.337 [25]. Upon completion of this procedure, the SCEF shall create a resource which represents the triggering transaction, addressed by a URI that contains the SCS/AS identity and an SCEF-created transaction identifier, and shall respond to the SCS/AS with a 201 Created message, including the trigger and a Location header field containing the URI for the created resource. The SCS/AS shall use the URI received in the Location header in subsequent requests to the SCEF to refer to this device triggering transaction.

In order to replace an existing device trigger, the SCS/AS shall send an HTTP PUT message to the SCEF, using the URI received in the response to the request that has created the device triggering transaction resource. The body of the HTTP PUT message shall include SCS/AS Identity, trigger reference number, validity period, priority, Application Port ID and trigger payload, and may include TLTRI. The SCS/AS Identity and TLTRI (if available) shall remain unchanged from previously provided values.

After receiving the corresponding HTTP PUT message from the SCS/AS, the SCEF shall check if the SCS/AS is authorised to replace an existing device trigger and if the SCS/AS has not exceeded its quota or rate of trigger submission. If any of these checks fail, then the SCEF shall reject the message with an error. Otherwise, the SCEF shall replace the device triggering with the SMS-SC by performing the device trigger replace procedure over Tsp as defined in 3GPP TS 29.368 [24] and T4 as defined in 3GPP TS 29.337 [25]. Upon completion of this procedure, the SCEF shall send an HTTP response to the SCS/AS to indicate trigger replace success or failure.

In order to recall an existing device trigger, the SCS/AS shall send an HTTP DELETE message to the SCEF, using the URI received in the response to the request that has created the device triggering transaction resource.

After receiving the corresponding HTTP DELETE message from the SCS/AS, the SCEF shall check if the SCS/AS is authorised to send a recall trigger request and if the SCS/AS has not exceeded its quota or rate of trigger submission. The SCEF shall also check if the device triggering transaction resource referenced by the URI exists. If any of these checks fail, then the SCEF shall reject the message with an error. Otherwise, the SCEF shall recall the device triggering with the SMS-SC by performing the device trigger replace procedure over Tsp as defined in 3GPP TS 29.368 [24] and T4 as defined in 3GPP TS 29.337 [25]. Upon completion of this procedure, the SCEF shall send an HTTP response to the SCS/AS to indicate trigger recall success or failure.

When it receives the Message Delivery Report from the SMS/SC, the SCEF shall send an HTTP POST message to the SCS/AS to report the trigger delivery result. The body of the HTTP POST message shall include the identifier if the transaction, TLTRI (if available), and cause. If available, the TLTRI shall be the same as the one assigned by the SCS/AS when the correlated device triggering transaction resource was created. The SCS/AS shall respond with an HTTP 200 OK or 204 No Content response.

A data model for device triggering is describes in 3GPP TS 29.122 as following.

##### 5.7.2.1.2	Type: DeviceTriggering

This type represents device triggering request. The same structure is used in the request and response.

Table 5.7.2.1.2-1: Definition of type DeviceTriggering

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| tltrId | TltrId | 0..1 | SCS/AS-chosen correlator. May be provided by the SCS/AS in the request to create a resource. Shall be provided in the body of all subsequent request and response if it was present in the body of the resource creation request. |  |
| self | Id | 0..1 | Link to this resource. This parameter shall be supplied by the SCEF in HTTP responses that include an object of DeviceTriggering type |  |
| triggerReferenceNumber | integer | 1 | The reference number is allocated by the initiator of a transaction and is used in all subsequent messages related to that transaction. |  |
| validityPeriod | DurationSec | 1 | The validity time in seconds for the specific action requested. |  |
| priority | Priority | 0..1 | Identifies the priority of the device trigger. |  |
| applicationPortId | Port | 0..1 | This is used to uniquely identify the triggering application addressed in the device. |  |
| triggerPayload | binary | 1 | The device triggering payload. |  |
| notificationDestination | Link | 0..1 | A URI indicating the notification destination for T8 notifications. |  |
| requestTestNotification | boolean | 0..1 | Set to true by the SCS/AS to request the SCEF to send a test notification as defined in subclause 5.2.5.3. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in subclause 5.2.5.4. | Notification_websocket |
| deliveryResult | DeliveryResult | 0..1 | The delivery result shall be included in the HTTP responses that indicate the delivery status of the device triggering. |  |
| NOTE: Properties marked with a feature as defined in subclause 5.7.4 are applicable as described in subclause 5.2.7. If no feature are indicated, the related property applies for all the features. |  |  |  |  |

Editor´s note: It is FFS if trigger reference number is made redundant by the tltrId

##### 5.7.2.1.3	Type: DeviceTriggeringDeliveryReportNotification

This type represents device triggering delivery report notification.

Table 5.7.2.1.4-1: Definition of type DeviceTriggeringDeliveryReportNotification

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| transactionId | string | 1 | Identifier of the related device triggering transaction resource, as defined by the "id" attribute of the "DeviceTriggering" data type. |  |
| transaction | Link | 1 | Link to the related device triggering transaction resource to which this notification is related. |  |
| tltrId | TltrId | 0..1 | SCS/AS-chosen correlator for the related device triggering transaction resource. Shall be present if it was provided when the related device triggering transaction resource was created. |  |
| result | DeliveryResult | 1 | OK, unknown or diverse failures |  |
| NOTE: Properties marked with a feature as defined in subclause 5.3.4 are applicable as described in subclause 5.2.7. If no feature i indicated, the related property applies for all the features. |  |  |  |  |

R02 includes:

- 3GPP revised 29.122 which is available at http://ftp.3gpp.org//Specs/archive/29_series/29.122/29122-110.zip since Apr 27, 2018

  - Major change is that id and scsAsId  are removed

- Error condition in the procedure

- Procedure revised based on ARC-2017-0060R02 which is latest baseline.

  - For the clearly understanding, introduces resquest and response of API, e.g, request for DeviceTriggering, response to DeviceTriggering, notify on DeviceTriggeringDeliveryReportNotification because there is no DeviceTriggering Request, DeviceTriggering Response, DeviceTriggeringDeliveryReportNotification Request and DeviceTriggeringDeliveryReportNotification Response.

  - Step1 ~ step 3a: same as R01.

Step 3a shall not include deliveryResult attribute

  - Step3b is for just acknowledgement using DeviceTriggering message which shall be included deliveryResult attribute with exception. This is silimar to Device Trigger Confirm at Diameter protocol which is Tsp interface.

  - Step 4 is not revised.

  - Step5 is new step for response to step3.

Because step 3b is optional and IN CSE needs to know result of the step 3a. The deliveryResult attribute is different between step 3b and step 5.

The value for deliveryResult can be chosen among CONFIRMED, EXPIRED and UNCONFIRMED at step4. In step 5b, it is possible SUCCESS, UNKNOWN, FAILURE, EXPIRED. TERMINATE as defined at 3GPP TS 29.122

TS-0001 has clearly specified for the one of the values of trigerStatus is TRIGGER-DELIVERED at clause 9.6.49 for Resource Type triggerRequest. Without step 5, IN-CSE can not have a triggerStatus for TRIGGER-DELIVERED.

  - Step6 is slightly revised an optionality of attribute based on TS 29.122

  - Original Step 7 is removed. Because SCEP sends DeviceTriggeringDeliveryReportNotification to IN-CSE. In this case, IN-CSE does not need to respond due to same contents if IN-CSE response.

  - Step7 ~ step 8: conceptullly no change

- R03 includes:

- Off-line discussion results of essential parameters for implementation of oneM2M.

- R04:

- Corrects Figure at step 10.

- Describes considering that AE initiated triggering for:

  - Step 2

  - Step 3a, Step 3b

  - Step 5

  - Step 6

  - Step 7

  - Step 8 and 9

- triggerPortID replaces to Trigger-Recipient-ID for <triggerRequest> resource at step 3a

- General Exception at step 3b apply to step 5 and step 6.

### -----------------------Start of change 1-------------------------------------------

### 7.5.1	General Procedure for 3GPP Based Device Triggering

An IN-CSE may initiate a device trigger to an ASN/MN-CSE or ADN-AE hosted on a 3GPP UE to cause it to establish a connection to the IN-CSE, enrol to a MEF, register to the IN-CSE, update its PoA, or perform a CRUD operation on a specified resource.  The IN-CSE may initiate the device trigger itself (implicit) or it may be initiated by a request that the IN-CSE receives from an AE (explicit).

Whenever the IN-CSE sends a device trigger to an ASN/MN-CSE or ADN-AE hosted on a 3GPP UE, the device triggering procedure as described in 3GPP TS 29.122 [5] shall be used as the basis for the procedures defined below.

This procedure supports an ASN/MN-CSE or ADN-AE that is hosted on a 3GPP UE that is directly connected to a 3GPP access network.

Figure 7.5.1-1: General Procedure for Device Triggering

Pre-condition

The UE that hosts the ASN/MN-CSE or ADN-AE is available to receive the device trigger using one of the connectivity establishment methods described in clause 6.

Step 1 (Optional): Request targeted to ASN/MN-CSE or ADN-AE

An AE may initiate a device trigger to an ASN/MN-CSE or ADN-AE explicitly by creating or updating a <triggerRequest> resource as specified in clause 9.6.49 of TS-0001[1]. Alternatively, an AE may initiate a device trigger to an ASN/MN-CSE or ADN-AE implicitly by issuing a request to an IN-CSE that requires device triggering. For example, if an IN-CSE receives a request to perform a CRUD operation targeting an ASN/MN-CSE or ADN-AE hosted on a 3GPP UE that is not reachable by the IN-CSE, the IN-CSE may generate a trigger request.

Step-2: Determine if Device Triggering is required

The IN-CSE determines whether to send a device trigger to the targeted ASN/MN-CSE or ADN-AE.  Further details are provided in clause 8.3.3.2.1 of oneM2M TS-0001 [1].

If device trigger was initiated by an AE, initiating AE determines device trigger.

Step 3a: request for DeviceTriggering

The IN-CSE or AE sends the DeviceTriggering message that contains information as specified in 3GPP TS 29.122 [5]. Such information includes:

- URI shall be set to {apiRoot}/3gpp_device_triggering/v1/{scsAsId}/transactions.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- I

- triggerReferenceNumber shall be set to a value determined by the IN-CSE and shall be different from all other triggers issued by this IN-CSE during the lifetime of this request. This value shall also be stored in the triggerReference attribute of the <triggerRequest> resource to map this oneM2M trigger operation.

- tltrId is used to identify the trigger request.  It shall be assigned based on internal IN-CSE polices and shall be different from other tltrIds issued by this IN-CSE during the lifetime of this request.  Later, if the request needs to be recalled, this value will be used to reference the request that needs to be recalled.

- validityPeriod shall be set to either the triggerValidityTime attribute of the <triggerRequest> resource if the trigger request is initiated by an AE.

- triggerPayload shall be configured as described in clause 8.3.3.2.1 of oneM2M TS 0001 [1] and clause 9.2.1 of TS-0004 [4]. An empty payload indicates that the targeted ASN/MN-CSE or ADN-AE shall re-establish connectivity with the IN-CSE.

- externalId shall be set to the M2M-Ext-ID of the targeted UE hosting an ASN/MN-CSE or ADN-AE.

- applicationPortID shall be set to Trigger-Recipient-ID attribute of the <triggerRequest> resource, if specified.

- notificationDestination shall be configured with a URI that the SCEF can target Device Trigger notifications towards.  The value of this URI shall be based on internal IN-CSE policies or IN-CSE identifier.

- priority may be set to either PRIORITY or NO_PRIORITY per internal IN-CSE policies and/or agreements between the Service Provider and MNO or  the triggerPriority attribute of the <triggerRequest> resource.

- LIST OF PARAMETERS THAT THE SCEF GENERATES AND

General Exceptions:

- The SCEF is not reachable when IN-CSE tries to send DeviceTriggering message. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to ERROR_NSE_NOT_FOUND after a prior timeout period (IN-CSE local policy)

Step 3b (Optional): response to DeviceTriggering request

For the purpose of confirmation to process of Step 3a, the SCEF may send a DeviceTriggering response to the IN-CSE or AE that contains information with a response status code 201 Created to acknowledge the successful reception of the request as specified in 3GPP TS 29.122 [5].  Otherwise HTTP status code is error. The response includes the following information.

- In addition to parameters included in the request, the response also includes the following parameters:

- self parameter configured with a URI to the resource created by the SCEF for the Device Trigger request.  This URI will have a structure of {apiRoot}/3gpp_device_triggering/v1/{scsAsId}/transactions/{transactionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies and the {transactionId} segment is configured by the SCEF.

- deliveryResult configured with one of the following status for confirmation:

  - TRIGGERED: that request for device triggering request is accepted by the SCEF. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_TRIGGERED

  - REPLACED: that request for device triggering message is accepted by the SCEF for replace of device triggering. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_REPLACED.

  - : that rees devig message ismed to ase the IN-CSE shal triggerStat of the <trigrEXPIRED: that the validity period expired before a SCEF starts to deliver the device triggering message. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_EXPIRED.

  - UNCONFIRMED: that request for device triggering message is not confirmed to delivery. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_UNCONFIRMED.

General Exceptions:

- The SCEF sends response code in case of error. In this case the IN-CSE or AE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_FAILED.

  - Bad Request: If incorrect parameters were included in the request, the SCEF sends this response.

  - Unauthorized: Upon receipt of the request from the IN-CSE, SCEF will check whether the IN-CSE is authenticated including authorization for authentication credentials to access services. If the authentication fails, the SCEF sends this response.

  - Forbidden: Upon receipt of the request from the IN-CSE, after authentication, the SCEF will check whether the IN-CSE is authorized to invoke services. If the authorization fails, the SCEF sends this response.

  - Not Found: If the resource URI is incorrect, the SCEF sends this response.

  - Conflict: The operation is not allowed given the current status of the resource. This response is returned when the creation of a resource with an scsAsId and optional tltrId is requested and a resource with the same such identifiers already exists and has not been deleted; or the change cannot be made due to conflict with current state of the resource.

Step 4: Device Trigger Delivery procedure

The device trigger message shall be delivered to the UE hosting the ASN/MN-CSE or ADN-AE.

Step 5: response to DeviceTriggering

For the purpose of response to Step 3, the SCEF responds to the IN-CSE or AE using DeviceTriggering request that contains information as specified in 3GPP TS 29.122 [5].  In addition to parameters included in the request, the response also includes the following parameters:

- self parameter configured with a URI to the resource created by the SCEF for the Device Trigger request.  This URI will have a structure of {apiRoot}/3gpp_device_triggering/v1/{scsAsId}/transactions/{transactionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies and the {transactionId} segment is configured by the SCEF.

- deliveryResult configured with one of the following status for confirmation:

In case of skip processing Step 3b:

- deliveryResult is included in the HTTP response to indicate one of the following status for delivery:

  - TRIGGERED: that request for device triggering request is accepted by the SCEF. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_TRIGGERED

  - REPLACED: request for device triggering message is accepted by the SCEF for replace of device triggering. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_REPLACED.

  - EXPIRED: that the validity period expired to process the device triggering message. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_EXPIRED.

  - UNCONFIRMED: that request for device triggering message is not confirmed for delivery. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_UNCONFIRMED.

  - SUCCESS: that the device triggering delivery is successfully completed. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_DELIVERED.

  - UNKNOWN: that indicates any unspecified errors.

  - FAILURE: that this trigger encountered an error during delivery or processing and is deemed permanently undeliverable. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_FAILED

  - TERMINATE: indicates that the delivery of the device trigger request was terminated by the IN-CSE. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_TERMINATED.

In case of processing Step 3b:

- deliveryResult is included in the HTTP response to indicate one of the following status for delivery:

  - SUCCESS: that the device triggering delivery is successfully completed. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_DELIVERED.

  - UNKNOWN: that indicates any unspecified errors.

  - FAILURE: that this trigger encountered an error during delivery or processing and is deemed permanently undeliverable. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_FAILED

  - EXPIRED: that the validity period expired to process the device triggering request. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_EXPIRED.

  - TERMINATE: that the delivery of the device triggering request is terminated by the IN-CSE. In this case the IN-CSE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_TERMINATED.

General Exceptions:

The SCEF sends response code in case of error. In this case the IN-CSE or AE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_FAILED as described at step 3b.

Step 6: DeviceTriggeringDeliveryReportNotification report

The SCEF sends a DeviceTriggeringDeliveryReportNotification message to report to {notification_uri} at the IN-CSE or AE results of the trigger delivery outcome. This message is defined in 3GPP TS 29.122 [5] and shall include the following:

- transactionId is configured with the identifier of the device triggering transaction resource.

- transaction is configured with a link to the related device triggering transaction resource to which this notification is related

- result is configured with the delivery status of the device trigger. If the result is OK, the IN-CSE should update the triggerStatus with TRIGGER_SUCCESS. Otherwise, the triggerStatus should be set in accordance with diverse failures.

- tltrId is configured with the same value as in the request at step 3a.

General Exceptions:

The SCEF sends response code in case of error. In this case the IN-CSE or AE shall update the triggerStatus attribute of the <triggerRequest> to TRIGGER_FAILED as described at step 3b.

Step 7: Device TriggeringDeliveryReportNotification response

After receiving a Device TriggeringDeliveryReport Notification report, the IN-CSE or AE returns a HTTP response having a response code of 204 NO CONTENT and no payload.

Steps 8  (optional) and 9: IN-CSE Updates <triggerRequest> and Response to 1a or 1b

If device trigger was initiated by an AE via a <triggerRequest> resource, then AE shall update triggerStatus attribute of <triggerRequest> resource.

If device trigger was initiated by an in-CSE via a <triggerRequest> resource, then IN-CSE shall update triggerStatus attribute of <triggerRequest> resource.

Step 10: ASN/MN-CSE or ADN-AE performs trigger actions

If the trigger has no payload, the ASN/MN-CSE or ADN-AE shall re-establish connectivity with the IN-CSE.

Otherwise, based on the type of trigger request received, the ASN/MN-CSE or ADN-AE performs the corresponding trigger actions such as establish connectivity with the IN-CSE, enrol with the MEF, register to the IN-CSE, update its PoA, or execute a CRUD request on a specified resource.

Further details are described in clause 8.3.3.2.1 of oneM2M TS-0001 [1] and clause 9.2.1 of TS-0004 [4]

### -----------------------End of change 1---------------------------------------------

### CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?