| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #35 |
| Source:* | Dale Seed, Convida Wireless, Seed.Dale@ConvidaWireless.com |
| Date:* | 2018-05-14 |
|  | See Introduction |
| Reason for Change/s:* | Rel-2 |
| CR against: Release* | Active MNT maintenance Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) ARC-2018-0109-CR_Location_Policy STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: WI* | WI-0049 – Maintenance |
| CR against: TS/TR* | TS-0001 v2.19.0 |
| Clauses * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Type of change: * | Add further clarification |
| Impacted other TS/TR(s) | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

At TP34, PRO requested that ARC revisit a couple of attributes of the <locationPolicy> resource to add some more clarification since PRO was not sure what data types to use for these attributes.   The attributes in question were geographicalTargetArea  and aggregationForward.

This contribution proposes the following:

- Add clarification that geographicalTargetArea  is a polygon defined by a list of geographical coordinates.

- Remove the aggregationForward since the functionality of this attribute is not fully defined and it is not clear what the intended functionality or value of this attribute is.

---------------------------------------- Start of Change 1---------------------------------------------------

### 9.6.10	Resource Type locationPolicy

The <locationPolicy> resource represents the method for obtaining and managing geographical location information of an M2M Node.

The actual location or event result (in case Geo-fence-Based method) information shall be stored in a <contentInstance> resource which is a child resource of the <container> resource. The <container> resource includes the locationID attribute which holds the ID of this <locationPolicy> resource. A CSE can obtain location information based on the attributes defined under <locationPolicy> resource, and store the location information in the target <container> resource.

Based on the locationSource attribute, the method for obtaining location information of an M2M Node can be differentiated. The methods for obtaining location information shall be as follows:

- Network-based method: where the CSE on behalf of the AE obtains the target M2M Node's location information from an Underlying Network.

- Device-based method: where the ASN is equipped with any location capable modules or technologies (e.g. GPS) and is able to position itself.

- Sharing-based method: where the ADN has no GPS nor an Underlying Network connectivity. Its location information can be retrieved from either the associated ASN or a MN.

NOTE:	Geographical location information could include more than longitude and latitude.

Figure 9.6.10-0 shows the graphical information regarding the event types for the Geo-fence feature defined as the geofenceEventCriteria attribute. The time difference between t1 and t2 described in the figure below is defined by locationUpdatePeriod attribute.

Figure 9.6.10-0: The Event Types for the Geo-fence

Figure 9.6.10-1: Structure of <locationPolicy> resource

The <locationPolicy> resource shall contain the child resources specified in table 9.6.10-1.

Table 9.6.10-1: Child resources of <locationPolicy> resource

| Child Resources of <locationPolicy> | Child Resource Type | Multiplicity | Description | <locationPolicyAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | None |

The <locationPolicy> resource shall contain the attributes specified in table 9.6.10-2.

Table 9.6.10-2: Attributes of <locationPolicy> resource

| Attributes of <locationPolicy> | Multiplicity | RW/ RO/ WO | Description | <locationPolicyAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| locationSource | 1 | RW | Indicates the source of location information: - Network Based; - Device Based; - Sharing Based. | OA |
| locationInformationType | 1 | RW | - Indicate the types of location information: - Position fix (e.g. longitude and latitude); - Geo-fence event (e.g. entering and leaving). | OA |
| locationUpdatePeriod | 0..1(L) | RW | Indicates the period for updating location information. If the value is marked '0' or not defined, location information is updated only when a retrieval request to the <latest> child resource of the <container> indicated by locationContainerID is detected . If the attribute has more than one value and the hosting CSE of the resource is the target device, the value could be selected within the listed values depending on device's local context information (e.g. velocity, status of battery, range of the location etc.). Zero('0') shall not be stored with non-zero value(s). When the value is read, the first value in the list is the current active update period. | OA |
| locationTargetID | 0..1 | RW | The identifier to be used for retrieving the location information of a remote Node and this attribute is only used in the case that location information is provided by a location server. For example, when the remote Node is a 3GPP UE, locationTargetID could be M2M-Ext-ID or MSISDN. | OA |
| locationServer | 0..1 | RW | Indicates the identity of the location or Geo-fence server. This attribute is only used in that case location information is provided by a location server or Geo-fence server. | OA |
| locationContainerID | 1 | RO | ID of the <container> resource where the actual location information or event result of a M2M Node is stored. | OA |
| locationContainerName | 0..1 | WO | A name of the <container> resource where the actual location information of a M2M Node is stored. If it is not assigned, the Hosting CSE automatically assigns a name of the resource (see note). | OA |
| locationStatus | 1 | RO | Contains the information on the current status of the location request (e.g. location server fault). | OA |
| geographicalTargetArea | 0..1 | RW | Indicates area information where the Geo-fence feature is applied. The area is a polygon and represented as a list of geographical coordinates that define the perimeter corner points of the polygon. | OA |
| geofenceEventCriteria | 0..1 | RW | - Indicate the event type of Geo-fence feature: - Entering; - Leaving; - Inside; - Outside. | OA |
| NOTE: The created <container> resource related to this policy shall be stored only in the Hosting CSE. |  |  |  |  |

---------------------------------------- End of Change 1---------------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?