| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | ARC TP35 |
| Source:* | BeiXu，Huawei，Echo.xubei@huawei.com |
| Date:* | 2018-05-07 |
| Contact:* | BeiXu，Huawei，Echo.xubei@huawei.com |
| Reason for Change/s:* | Create the new content of clause 6. |
| CR against: Release* | <Release4> |
| CR against: WI* | Active < WI-0082 - 3GPP V2X Interworking > MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0055-V0_0_1 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

None.

### -----------------------Start of change 1-------------------------------------------

# 6	Reference Architecture

<Text> Describe oneM2M and the 3GPP V2X interworking architecture.

## 6.1 oneM2M interworking with 3GPP V2X architecture option1

Figure 6.1-1: oneM2M Interfaces to the 3GPP V2X Service option1

Figure 6.2-1 shows an architecture and functional mapping for the 3GPP Trust Domain which describes how oneM2M functional entities may access features and services that are exposed by 3GPP.  The oneM2M entities are shaded in grey.

The “V2X Applications” hosted on the UE may be deployed as follows:

- Application only: UE may be ADN as of oneM2M entity

- Application and CSE: UE may be ASN or MN as of oneM2M entity

- CSE only: UE may be MN as of oneM2M entity

- Neither application nor CSE: UE may host a NoDN.

The “V2X Control Function” may host on the IN-CSE as DMG CSF and can be located in the Service Layer. IN-CSE shall support to communicate with HSS via V4 reference point and communicate with other V2X Control Function in 3GPP Trust Domian via V6 reference point.

The “V2X Application server” can be splited to three logic functions:

- One function is  3GPP network status & QoS monitoring and report hosted in 3GPP Trust domain. At the same time, there is no standard interface between IN-CSE and 3GPP V2X AS.

- One function is Device Management, MBMS Group,  Location and 3GPP QoS hosted on the platform may be IN-CSE. 3GPP QoS management is new feature for oneM2M.

- The last function is application function hosted on an external network may be an IN-AE.

The “3GPP Trust Domain” in Figures5.1-1 and Figure 6.1-1 captures the functional entities that MUST be part of the 3GPP domain (the network). Although Figure 6.2-1 shows that the IN-CSE and IN-AE are outside of the 3GPP Domain, the IN-CSE may be part of the operator domain.

This figure also shows the oneM2M reference points Mcn and Mca. There is a reference point  for oneM2M which is the communication flow between oneM2M nodes in filed domain to support 3GPP PC5 mode. This reference point may be Mca between an ADN and a ASN/MN; or Mcc’’ between a ASN/MN and a ASN/MN; or Maa between an ADN and an ADN in Figure 6.1-2.

Figure 6.1-2: oneM2M reference point

## 6.2 oneM2M 3GPP V2X interworking architecture option2

Figure 6.2-2: oneM2M Interfaces to the 3GPP V2X Service option2

Figure 6.2-2 shows an architecture and functional mapping for the 3GPP Trust Domain which describes how oneM2M functional entities may access features and services that are exposed by 3GPP.  The oneM2M entities are shaded in grey. The differences from option1 in clause 6.1 are:

The “V2X Control Function” may hosted on the IN-CSE as DMG CSF and can be located in the Underlying Network using the Mcn reference point which is out of oneM2M scope. oneM2M shall support the communication between DMG CSFs via Mcc’ reference point according to 3GPP V6.

The “V2X Application server” can be splited to two logic functions:

- One function is about 3GPP network status & QoS monitoring and report hosted in 3GPP Trust domain. At the same time, there is no standard interface between IN-CSE and 3GPP V2X AS.

- The other function is  about Device Management, MBMS Group,  Location and 3GPP QoS hosted on the platform may be IN-CSE. 3GPP QoS management is new feature for oneM2M.

-----------------------End of change 1-------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?