| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#35 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com Jiaxin Yin, Huawei Technologies Co.,Ltd., yinjiaxin@huawei.com Jiadong Du, CAICT, dujiadong@caict.ac.cn |
| Date:* | 2018-05-11 |
| Reason for Change/s:* | The CR is to clarify the usage of transaction resource. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active WI-0050 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 |
| Clauses * | 9.6.48 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0001 v3.11.0 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

<Provide an introduction containing the problem(s) being solved, and a summary list of proposals.  Discuss any risk of breaking backwards compatibility with last published version of the impacted TS.>

### -----------------------Start of change 1-------------------------------------------

### 9.6.48	Resource Type transaction

The <transaction> resource is used to initiate and manage the atomic and consistent processing of a single oneM2M request primitive of a oneM2M transaction.

With the exception of the <request>, <delivery>, <transaction> and <transactionMgmt> resources, a <transaction> resource may be created as a child resource of any resource targeted by a oneM2M transaction.  A <transaction> create request may be originated by a CSE that hosts an associated <transactionMgmt> resource.  Alternatively, a <transaction> resource can be used independent of <transactionMgmt> resource for the case where an application wishes to create individual <transaction> resources itself.

In order to fulfil a transaction among multiple entities. Multiple <transaction> resources are created on their corresponding CSEs. Depending on the intention of the application that is coordinating the transaction, the <transaction> resources on different CSEs each represents the services need to be deployed by the entity. The requestPrimitive of different <transaction> resources may be different or the same to control the behaviour of devices which are part of the transaction. The time related attributes, transactionLockTime, transactionExecuteTime and transactionCommitTime may be different or the same as well to coordinate the behaviour of devices in a desired order or schedule.

The <transaction> resource supports the child resources specified in table 9.6.48-1.

Table 9.6.48-1: Child resources of <transaction> resource

| Child Resources of <transaction> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |

The <transaction> resource supports the attributes specified in table 9.6.48-2.

Table 9.6.48-2: Attributes of <transaction> resource

| Attributes of <transaction> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 |
| resourceID | 1 | RO | See clause 9.6.1.3 |
| resourceName | 1 | WO | See clause 9.6.1.3 |
| expirationTime | 1 | WO | See clause 9.6.1.3 The value of the expirationTime serves as the expiration time of the transaction. |
| parentID | 1 | RO | See clause 9.6.1.3 |
| creationTime | 1 | RO | See clause 9.6.1.3 |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 |
| accessControlPolicyIDs | 0..1 (L) | RO | See clause 9.6.1.3 |
| labels | 0..1 (L) | RO | See clause 9.6.1.3 |
| creator | 1 | RO | This attribute is configured with the CSE-ID of the <transactionMgmt> Hosting CSE or AE-ID of the AE that created this <transaction> resource. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 |
| transactionID | 1 | WO | This attribute is configured with the identifier of the transaction. This attribute is configured by the <transactionMgmt> Hosting CSE with the resource identifier of the <transactionMgmt> resource. If an AE creates the <transaction> resource, then this attribute is configured with an AE specified identifier. |
| transactionControl | 1 | RW | This attribute shall be used to configure or change the state of the transaction. The allowed values are: - LOCK - EXECUTE - COMMIT - ABORT Only the creator of this <transaction> resource is allowed to update this attribute. |
| transactionState | 1 | RO | This attribute contains the current state of the transaction. Only the Hosting CSE of this <transaction> resource shall be allowed to update this attribute. The allowed values are: - LOCKED - EXECUTED - COMMITTED - ERROR - ABORTED |
| transactionLockTime | 0..1 | WO | This attribute contains timing information that the <transaction> Hosting CSE itself shall use to schedule when it sets transactionControl to LOCK rather than relying on an UPDATE request to set the value of transactionControl to LOCK. |
| transactionExecuteTime | 0..1 | WO | This attribute contains timing information that the <transaction> Hosting CSE itself shall use to schedule when it sets transactionControl to EXECUTE rather than relying on an UPDATE request to set the value of transactionControl to EXECUTE. If the targeted resource is not locked at this scheduled time, the <transaction> Hosting CSE shall first lock the resource before executing the request primitive. |
| transactionCommitTime | 0..1 | WO | This attribute contains timing information that the <transaction> Hosting CSE itself shall use to schedule when it sets transactionControl to COMMIT rather than relying on an UPDATE request to set the value of transactionControl to COMMIT. If the targeted resource is not locked or the request primitive has not yet been executed at this scheduled time, the <transaction> Hosting CSE shall first lock the resource and execute the request primitive at this time before committal. |
| transactionLockType | 0..1 | WO | This attribute indicates the type of lock that is required on the targeted resource in order to perform the transaction. The following are the supported types of locks: - BLOCK_ALL - Block oneM2M request primitives not associated with this transaction from performing any CRUD operations on the targeted resource while it is locked for this transaction. This shall be the default value if this attribute is not configured. - ALLOW_RETRIEVES - Block oneM2M request primitives not associated with this transaction from performing any operation except RETRIEVE on the targeted resource while it is locked for this transaction |
| requestPrimitive | 1 | WO | This attribute contains the request primitive to be executed on the parent of this <transaction> resource. |
| responsePrimitive | 1 | RO | This attribute contains the oneM2M response primitive associated with this transaction. This attribute is updated by the <transaction> Hosting CSE after it executes the requestPrimitive on the parent resource. |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?