| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | PRO#35 |
| Source:* | Neeta Meshram(neeta@cdot.in), Suman Sheoran(ssheoran@cdot.in) Anupama Chopra(anupama@cdot.in), C-DOT |
| Date:* | 2018-04-14 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional) Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v2.19.0 |
| Clauses * | 10.2.13.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

According to TS-0004, clause 7.3.3.10 Announce the resource or attribute,

If CREATE Request that contains an announceTo attribute is received,

- Compose the CREATE Request primitive as follows:

- The link attribute is set to the URI of the original resource.

- If accessControlPolicyIDs attribute of the original resource is not present, accessControlPolicyIDs attribute is set to the same value with the parent resource or from the local policy of the original resource.

- Attributes marked with MA in TS-0001 [Error: Reference source not found]. Such attributes shall be included if present in the original resource and set to same value as in the original resource.

- Attributes marked with OA that are included in the announcedAttribute attribute. Such attributes shall be included if present in the original resource and set to same value as in the original resource.

- The resourceType attribute is set to the announced variant of the original resource (see the Error: Reference source not found)

- If announceTo attribute contains a list of CSE-ID(s) ,

- Check if the parent resource of the original resource has been announced to the announcement target CSE(s).

- If yes , announce the original resource by sending a CREATE Request to the CSE(s) specified in the announceTo and that targets the announced parent resource.

- If no, check if the CSE hosting the original resource has registered and created a <remoteCSE> resource to the announcement target CSE(s).

    - If yes , announce the original resource by sending a CREATE Request to the CSE(s) specified in the announceTo and that targets the <remoteCSE> of the CSE hosting the original resource.

    - If no, then check if the CSE hosting the original resource has been announced to the announcement target CSE(s) and created a <remoteCSEAnnc> resource as a child of the <CSEBase>.

      - If yes, announce the original resource by sending a CREATE Request to the CSE(s) specified in the announceTo and that targets the <remoteCSEAnnc> resource

      - If no, then CSE hosting the original resource shall perform the following steps.

        - Announce itself to the CSE(s) specified in the announceTo attribute such that its <remoteCSEAnnc> resource is present at the announcement target CSE(s).

        - Send a CREATE Request to the CSE(s) specified in the announceTo of the request and that target the <remoteCSEAnnc> resource.

- Otherwise, Send a CREATE Request to the CSE(s) represented by exact URI(s) in the announceTo of the request.

- Wait for the Response primitive

- Add the URI of successfully announced resource to the announceTo attribute of the resource

- Include updated announceTo attribute in the Content parameter in the Response to the received CREATE Request.

If UPDATE Request that adds the URI or CSE-ID into the announceTo attribute is received,

- Compose the CREATE Request primitive as follows:

- Link is set to the URI of the original resource.

- If accessControlPolicyIDs of the original resource is not present, accessControlPolicyIDs is set to the same value with the parent resource or from the local policy of the original resource.

- Attributes marked with MA and attributes marked with OA that are included in the announcedAttribute attribute. Such attributes shall be present in the original resource and set to same value as the original resource.

- If announceTo attribute contains a list of CSE-ID(s) ,

- Check if the parent resource of the original resource has been announced to the announcement target CSE(s).

- If yes , announce the original resource by sending a CREATE Request to the CSE(s) specified in the announceTo and that targets the announced parent resource.

- If no, check if the CSE hosting the original resource has registered and created a <remoteCSE> resource to the announcement target CSE(s).

    - If yes , announce the original resource by sending a CREATE Request to the CSE(s) specified in the announceTo and that targets the <remoteCSE> of the CSE hosting the original resource.

    - If no, then check if the CSE hosting the original resource has been announced to the announcement target CSE(s) and created a <remoteCSEAnnc> resource as a child of the <CSEBase>.

      - If yes, announce the original resource by sending a CREATE Request to the CSE(s) specified in the announceTo and that targets the <remoteCSEAnnc> resource

      - If no, then CSE hosting the original resource shall perform the following steps.

        - Announce itself to the CSE(s) specified in the announceTo attribute such that its <remoteCSEAnnc> resource is present at the announcement target CSE(s).

        - Send a CREATE Request to the CSE(s) specified in the announceTo of the request and that target the <remoteCSEAnnc> resource.

- Otherwise, Send a CREATE Request to the CSE(s) represented by exact URI(s) in the announceTo of the request.

- Wait for the Response primitive.

- Add the URI of successfully announced resource to the announceTo attribute of the resource

- Include updated announceTo attribute in the Content parameter in the Response to the received UPDATE Request.

If UPDATE Request that adds the attribute name into the announcedAttribute attribute is received,

- Compose the UPDATE Request primitive. The UPDATE Request shall provide the attribute name for the attribute to be announced, and the initial value for the attribute in the Content parameter. The initial value shall be the same with the value from the original resource. The attribute that will be announced shall be marked as OA.

- Send UPDATE Requests to all announced resources listed in the announceTo attribute.

- Wait for Response primitive.

Add the attribute name of the successfully announced attribute to the announcedAttribute attribute.

- Include updated announcedAttribute attribute in the Content parameter in the Response to the received UPDATE Request.

If an attribute(s) specified as MA (See TS-0001 [Error: Reference source not found]) or an attribute(s) included in the announcedAttribute attribute is updated:

- Compose an UPDATE Request primitive by including the updated attribute(s) with its associated updated value.

- Send the UPDATE Request to all CSE(s) represented by the URI(s) in the announceTo attribute of the original resource.

- Wait for Response primitive.

It says, If parent is not annouced, original Hosting CSE shall first check whether it is registered with Announcement Target CSE and If that is not the case then it shall go for checking whether it is announced on Announcement Target CSE.

TS-0001, clause 10.2.13.2	Procedure for AE and CSE to initiate Creation of an Announced Resource is not in line with TS-0004.

### -----------------------Start of change 1-------------------------------------------

#### 10.2.18.1		Procedure for AE and CSE to initiate Creation of an Announced Resource

This clause describes the procedure for an AE or a CSE to initiate the creation of an announced resource.

Figure 10.2.18.1-1 depicts how creation of and announced resource is initiated (clause 10.2.18.1) and the announced resource is created on an announcement target CSE (clause 10.2.18.4).

Figure 10.2.18.1-1: Announced resource CREATE procedures

The Originator of a Request for initiating resource announcement can be either an AE or a CSE. Two methods are supported for initiating the creation of an announced resource:

- CREATE: The Originator can initiate the creation of an announced resource during the creation of the original resource by providing announceTo attribute in the CREATE Request.

- UPDATE: The Originator can initiate the creation of an announced resource by using the UPDATE Request to update the announceTo attribute at the original resource.

Table 10.2.18.1-1: Initiate Resource Announcement: UPDATE or CREATE

| Initiate Resource Announcement: CREATE or UPDATE |  |
| --- | --- |
| Associated Reference Points | Mca and Mcc. |
| Information in Request message | All parameters defined in table 8.1.2-3 are applicable as indicated in that table. In addition, for the case of the CREATE procedure for a specific resource is described in clause 10.2. The Originator suggests the address(es) or the CSE-ID(s) to which the resource will be announced in the Content parameter. |
| Processing at the Originator before sending Request | Content: contains address where the resource needs to be announced (within announceTo attribute): - The Originator provides either the address(es) for the announced resource or the list of CSE-IDs of the remote CSEs where the original resource needs to be announced by including such information within the announceTo attribute of the UPDATE or CREATE Request. |
| Processing at the Receiver | Once the Originator has been successfully authorized, the Receiver (which shall be the original resource Hosting CSE) shall grant the Request after successful validation of the Request: - If the Request provides address(es) for the announced resource that are not already stored in the announceTo attribute or for newly created announceTo attribute, the Receiver shall announce the resource to the announcement target CSE. - If the Request provides a list of CSE-IDs of the remote CSEs that are not already stored in the announceTo attribute of for the newly created or updated announceTo attribute, the Receiver shall decide the location at the remote CSE(s) identified by CSE-ID(s) and announce the resource to the announcement target CSE. The original resource Hosting CSE shall first check if the parent resource of the original resource has a representation at the announcement target CSE. If that is the case, the announced resource shall be created as a child resource of that representation of the parent resource. If that is not the case, the original Hosting CSE shall next check if It is registered with Announcement target CSE and created <remoteCSE> resource. If that is the case, the announced resource shall be created as a child resource of the original Hosting CSE's <remoteCSE> resource. If that is also not the case, the original Hosting CSE shall next check if it has announced itself to the announcement target CSE. If that is the case, the announced resource shall be created as a child resource of the original Hosting CSE's <remoteCSEAnnc> resource. Otherwise, the original Hosting CSE shall first announce itself by creating a <remoteCSEAnnc> resource as a child resource of the <CSEBase> resource of the announcement target CSE. Next, the announced resource shall be created as a child resource of the original Hosting CSE's <remoteCSEAnnc> resource. |
| Information in Response message | On successful completion of resource announcement as in clause 10.2.18.4, the Receiver shall provide all parameters defined in table 8.1.3-1 that are applicable as indicated in that table in the Response message: - The Receiver shall provide the address(es) of the announced resource to the Originator by updating the content of the announceTo attribute in the original resource and by providing it in the UPDATE or CREATE Response message depending on the type of the Request. |
| Processing at Originator after receiving Response | According to clause 10.1.1.1 in case of CREATE Request. According to clause 10.1.3 in case of UPDATE Request. |
| Exceptions | All exceptions described in the basic procedures (clause 10.1.1) are applicable. |

-----------------------End of change 1-------------------------------------------

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?