| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #35 |
| Source:* | Catalina Mladin, Convida Wireless, Mladin.Catalina@ConvidaWireless.com |
| Date:* | 2018-05-14 |
|  | See Introduction |
| Reason for Change/s:* | Rel-3 |
| CR against: Release* | Active MNT maintenance Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: WI* | WI-0049 – Maintenance |
| CR against: TS/TR* | TS-0001 v3.11.0 |
| Clauses * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Type of change: * | Clean-up for Rel-3 Publication |
| Impacted other TS/TR(s) | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR addresses the clean-up of the editor’s notes in clause 10.2.7

add texts to explain<semanticFanOutPoint> resource is optional depending on the semanticSupportIndicator attribute of a <group> resource

R01 adds change 2 and includes agreements from ARC#35

### ------------------------ Start of Change 1---------------------------------------------------

- 10.2.7	Group management

- 10.2.7.1	Introduction

This clause describes different procedures for managing membership verification, creation, retrieval, update and deletion of the information associated with a <group> resource. Bulk management of all group member resources by invoking the corresponding operations upon the virtual resource <fanOutPoint> of a <group> resource are detailed.

This clause also describes the use of the virtual resource <semanticFanOutPoint> which is supported only if the group hosting CSE supports semantic discovery functionality.  This virtual resource, shall be the target of RETRIEVE requests only.

Figure 10.2.7.1-1 illustrates in a generic way how addressing a request to the  <fanOutPoint> virtual resource on the group Hosting CSE results in a fannig out of the request to the group member resources. The procedures in the figure apply to clauses 10.2.7.6 to 10.2.7.9.

Figure 10.2.7.1-1: Group content management procedures

If the group resource, whose fanOutPoint virtual sub-resource is addressed by the request, contains <group> resources as member resources, when fanning out the request, the Group Hosting CSE shall address the fanOutPoint virtual sub-resource of the member <group> resource in step 3 so that the member <group> resource Hosting CSE could further fan out the request to its members correspondingly.

- 10.2.7.2	Create <group>

This procedure shall be used for creating a <group> resource.

Table 10.2.7.2-1: <group> CREATE

| <group> CREATE |  |
| --- | --- |
| Associated Reference Point | Mcc, Mca and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <CSEBase>, <AE>, or <remoteCSE> where the <group> resource is intended to be Created Content: The representation of the <group> resource for which the attributes are described in clause 9.6.13 |
| Processing at Originator before sending Request | The Originator shall request to Create a <group> resource by using the CREATE operation. The request shall address <CSEBase>, <remoteCSE> or <AE> resource of a Hosting CSE. The Request shall also provide memberIDs and may provide expirationTime attributes. For members which are of type <group>, the originator shall suffix the ‘/fopt’ to that ‘memberID‘ during group creation if the originator wants to fan-out the group request to each member of that sub-<group>, else originator shall not suffix the ‘/fopt’ to that ‘memberID‘. The Originator may be an AE or a CSE |
| Processing at Receiver | For the CREATE procedure, the Receiver shall: Check if the Originator has CREATE permissions on the target resource Check the validity of the provided attributes Validate that there are no duplicate members present in the memberIDs attribute Validate that the resource type of every member on each member Hosting CSE conforms to the memberType attribute in the request, if the memberType attribute of the <group> resource is not 'mixed'. Set the memberTypeValidated attribute to TRUE upon successful validation. - Upon successful validation of the provided attributes, create a new group resource in the Hosting CSE. If the CSE supports semantic discovery functionality, the Hosting CSE shall also create and set the semanticSupportIndicator attribute to TRUE. If the registree Member Hosting CSEs and the Group Hosting CSE supports the same type of multicast communication, the Group Hosting CSE shall perform the procedures as specified in clause 10.2.7.13.1. Conditionally, in the case that the group resource contains temporarily. unreachable Hosting CSE of sub-group resources as member resource, set the memberTypeValidated attribute of the <group> resource to FALSE Respond to the Originator with the appropriate generic Response with the representation of the <group> resource if the memberTypeValidated attribute is FALSE, and the address of the created <group> resource if the CREATE was successful As soon as any Hosting CSE that hosts the unreachable resource becomes reachable, the memberType validation procedure shall be performed. If the memberType validation fails, the Hosting CSE shall deal with the <group> resource according to the policy defined by the consistencyStrategy attribute of the <group> resource provided in the request. or by default if the attribute is not provided |
| Information in Response message | The representation of the <group> resource if the memberTypeValidated attribute is FALSE |
| Processing at Originator after receiving Response | None |
| Exceptions | - No change from the basic procedure in clause 10.1.2 |

- 10.2.7.3	Retrieve <group>

This procedure shall be used for retrieving <group> resource.

Table 10.2.7.3-1: <group> RETRIEVE

| <group> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mcc, Mca and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <group> resource |
| Processing at Originator before sending Request | The Originator shall request to obtain <group> resource information by using the RETRIEVE operation. The request shall address the specific <group> resource of a Hosting CSE. The Originator may be an AE or a CSE |
| Processing at Receiver | - No change from the basic procedure in clause 10.1.3 |
| Information in Response message | - No change from the basic procedure in clause 10.1.3 |
| Processing at Originator after receiving Response | None |
| Exceptions | - No change from the basic procedure in clause 10.1.3 |

- 10.2.7.4	Update <group>

This procedure shall be used for updating an existing <group> resource.

Table 10.2.7.4-1: <group> UPDATE

| <group> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <group> resource |
| Processing at Originator before sending Request | The Originator shall request to update attributes of an existing <group> resource by using an UPDATE operation. The Request shall address the specific <group> resource of a CSE. If originator intends to update memberIDs attribute, for members which are of type <group>, originator shall suffix the ‘/fopt’ to that ‘memberID‘ during group update if the originator wants to fan-out the group request to each member of that sub-<group>, else originator shall not suffix the ‘/fopt’ to that ‘memberID‘. The Originator may be an AE or a CSE |
| Processing at Receiver | The UPDATE procedure shall be: Check if the Originator has UPDATE permissions on the <group> resource. Check the validity of provided attributes Validate that there are no duplicated members present in the memberIDs attribute Validate that the resource type of every member on each member Hosting CSE conforms to the memberType attribute in the request, if the memberType attribute of the <group> resource is not 'mixed'. Set the memberTypeValidated attribute to TRUE upon successful validation Upon successful validation of the provided attributes, update the <group> resource in the Hosting CSE Conditionally, if the memberIDs attribute changes and the group includes Multicast Group Information, Group Hosting CSE shall update or delete the Multicast Group Information according to the new group members and trigger the update or delete of <localMulticastGroup> on each member Hosting CSEs. The procedure is specified in the clause 10.2.7.15 or 10.2.7.16 correspondingly. Conditionally, in the case that the <group> resource contains temporarily unreachable Hosting CSE of sub-group resources as members resource set the memberTypeValidated attribute of the <group> resource to FALSE Respond to the Originator with the appropriate generic response with the representation of the <group> resource if the memberTypeValidated attribute is FALSE, and the address of the created <group> resource if the UPDATE is successful As soon as any Hosting CSE that hosts unreachable resource becomes reachable, the memberType validation procedure shall be performed. If the memberType validation fails, the Hosting CSE shall deal with the <group> resource according to the policy defined by the consistencyStrategy attribute of the <group> resource provided in the request, or by default if the attribute is not provided |
| Information in Response message | The representation of the <group> resource if the memberTypeValidated attribute is FALSE |
| Processing at Originator after receiving Response | None |
| Exceptions | - No change from the basic procedure in clause 10.1.4 |

- 10.2.7.5	Delete <group>

This procedure shall be used for deleting an existing <group> resource.

Table 10.2.7.5-1: <group> DELETE

| <group> DELETE |  |
| --- | --- |
| Associated Reference Point | Mcc, Mca and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <group> resource |
| Processing at Originator before sending Request | The Originator shall request to delete an existing <group> resource by using the DELETE operation. The request shall address the specific <group> resource of a Hosting CSE. The Originator may be an AE or a CSE This operation shall also delete the child virtual resources <fanOutPoint> and <semanticFanOutPoint> |
| Processing at Receiver | - Besides the basic procedure in clause 10.1.5, the receiver shall: Check if the group includes Multicast Group information, and perform the multicast message fan out procedure to delete the <localMulticastGroup> of member hosting CSEs as specified in the clause 10.2.7.16 |
| Information in Response message | - No change from the basic procedure in clause 10.1.5 |
| Processing at Originator after receiving Response | None |
| Exceptions | - No change from the basic procedure in clause 10.1.5 |

- 10.2.7.6	Create <fanOutPoint>

This procedure shall be used for creating the content of all members’ resources belonging to an existing <group> resource.

Table 10.2.7.6-1: <fanOutPoint> CREATE

| <fanOutPoint> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <fanOutPoint> virtual resource Content: The representation of the resource the Originator intends to create Group Request Identifier: The group request identifier Response Type: If the parameter is set to BlockingSynch, it indicates that the group hosting CSE shall return the aggregated response once. Otherwise if the parameter is set to nonBlockingRequestSynch, nonBlockingRequestAsynch or flexBlocking, it indicates that the Group Hosting CSE shall return the aggregated response in a batched mode Result Expiration Time: Indicates the maximum time limit in which the Group Hosting CSE has to respond the aggregated response. |
| Processing at Originator before sending Request | The Originator shall request to create the resource that have the same content in all member resources belonging to an existing <group> resource by using a CREATE operation. The Request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group Hosting CSE. The request may also address the address that results from appending a relative address to the <fanOutPoint> address in order to create the resources that have the same content under the corresponding child resources represented by the relative address with respect to all member resources. The Originator may be an AE or CSE. |
| Processing at Group Hosting CSE | For the CREATE procedure, the Group Hosting CSE shall: Check if the Originator has CREATE privilege in the <accessControlPolicy> resource referenced by the membersAccessControlPolicyIDs in the <group> resource. In the case members membersAccessControlPolicyIDs is not provided the access control policy defined for the <group> resource shall be used - Upon successful validation, obtain the IDs of all member resources from the attribute membersIDs of the addressed <group> resource - If the group includes Multicast Group Information, the group Hosting CSE shall perform the procedure as described in clause 10.2.7.13.2. If the request contains Group Request Target Members parameter, it shall check whether all members contained in this parameter are subset of memberIDs attribute of the addressed <group> resource. If true, the request shall be fanned out to the members contained in this parameter only. Generate fan out requests addressing the obtained address (appended with the relative address if any) to the member hosting CSEs as indicated in figure 10.2.7.1-1 which are not in the multicast group. The From parameter in the fanout request is set to ID of the Originator from the request from the original Originator. The Response Type parameter in the fanout request may be set by the group hosting CSE differently according to its local policy In the case that a member resource is a <group> resource and the request to be fanned out does not contain a group request identifier already, generate a unique group request identifier, include the group request identifier in all the requests to be fanned out and locally store the group request identifier If the group Hosting CSE determines that multiple member resources belong to one CSE according to the IDs of the member resources and no multicast group exists for these members, it may converge the requests accordingly before sending out. This may be accomplished by the group Hosting CSE creating a <group> resource on the members Hosting CSE to collect all the members on that members Hosting CSE - After receiving the responses from the members hosting CSEs, respond to the Originator with the aggregated results and the associated members list. Depending on the Response Type, the Group Hosting CSE shall: - blockingRequest: respond with the aggregated responses before the Result Expiration Time reaches and discard the member responses received after - nonBlockingRequestSynch: prepare the operationResult of the <request> resource and indicate that if all the member responses have been aggregated by setting the requestStatus of the <request> resource before the Result Expiration Time reaches. There may be multiple updates of the operationResult attribute. - nonBlockingRequestAsynch: notify with the aggregated response from all or part of the members before the Result Expiration Time reaches. There may be more than one notifications. - flexBlocking: continue aggregate the member response until the group hosting CSE determines to send the aggregated responses, if all member responses has been aggregated, respond the aggregated response as in the blockingRequest case. Otherwise, respond an acknowledgement together with the current aggregated member responses and the reference to the created <request> resource. Then continue aggregate and deliver the remaining member response to the Originator as defined in the nonBlockingRequestSynch or the nonBlockingRequestAsynch case. - After the Result Expiration Time, there shall not be any further updates to the aggregated responses. (See note) |
| Processing at Member Hosting CSE | For the CREATE procedure, the Member Hosting CSE shall: Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifiers stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the group request identifier until the expiration of the request expiration time or local policy Check if the original Originator has the CREATE permission on the addressed resource. Upon successful validation, perform the create procedures for the corresponding type of addressed resource as described in other sub-clauses of clause 10.2 Send the corresponding response to the Group Hosting CSE |
| Information in Response message | Converged responses from members hosting CSEs |
| Processing at Originator after receiving Response | None |
| Exceptions | Same request with identical group request identifier received Originator does not have the CREATE permission to access the <fanOutPoint> resource Members in Group Request Target Members request parameter are not subset of memberIDs attribute of the addressed <group> resource |
| NOTE: If Result Expiration Time is not provide in the original request from the Originator, the group hosting CSE may decide the timer based on its local policy. |  |

- 10.2.7.7	Retrieve <fanOutPoint>

This procedure shall be used for retrieving the content of all member resources belonging to an existing <group> resource.

Table 10.2.7.7-1: <fanOutPoint> RETRIEVE

| <fanOutPoint> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <fanOutPoint> virtual resource Content: The representation of the resource the Originator intends to retrieve Group Request Identifier: The group request identifier Response Type: If the parameter is set to BlockingSynch, it indicates that the group hosting CSE shall return the aggregated response once. Otherwise if the parameter is set to nonBlockingRequestSynch or nonBlockingRequestAsynch, it indicates that the Group Hosting CSE shall return the aggregated response in a batched mode. Result Expiration Time: Indicates the maximum time limit in which the Group Hosting CSE has to respond the aggregated response. |
| Processing at Originator before sending Request | The Originator shall request to obtain the resource or specific attributes of all member resources belonging to an existing <group> resource by using a RETRIEVE operation. The request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group Hosting CSE. The request may also address the address that results from appending a relative address to the <fanOutPoint> address in order to retrieve the corresponding attributes or child resources represented by the relative address with respect to all member resources. The Originator may be an AE or CSE |
| Processing at Group Hosting CSE | For the RETRIEVE procedure, the Group Hosting CSE shall: Check if the Originator has RETRIEVE permission in the <accessControlPolicy> resource referenced by the membersAccessControlPolicyIDs in the addressed <group> resource. In the case membersAccessControlPolicyIDs is not provided, the access control policy defined for the group resource shall be used Upon successful validation, obtain the IDs of all member resources from the membersIDs attribute of the addressed <group> resource If the group includes Multicast Group information, the group Hosting CSE shall perform the procedure as described in clause 10.2.7.13.2 If the request contains Group Request Target Members parameter, it shall check whether all members contained in this parameter are subset of memberIDs attribute of the addressed <group> resource. If true, the request shall be fanned out to the members contained in this parameter only. Generate fan out requests addressing the obtained address (appended with the relative address if any) to the members hosting CSEs as indicated in figure 10.2.7.1-1.The From parameter in the fanout request is set to ID of the Originator from the request from the original Originator. The Response Type parameter in the fanout request may be set by the group hosting CSE differently according to its local policy In the case that a member resource is a <group> resource, generate a unique group request identifier and the request to be fanned out does not contain a group request identifier already, include the group request identifier in all the requests to be fanned out and locally store the group request identifier If the group hosting CSE determines that multiple member resources belong to one CSE according to the IDs of the member resources and no multicast group exists for these members, it may converge the requests accordingly before sending out. This may be accomplished by the group Hosting CSE creating a <group> resource on the members Hosting CSE to collect all the members on that members Hosting CSE - After receiving the responses from the members hosting CSEs, respond to the Originator with the aggregated results and the associated members list. Depending on the Response Type, the Group Hosting CSE shall: - BlockingRequest: respond with the aggregated responses before the Result Expiration Time reaches and discard the member responses received after. - nonBlockingRequestSynch: prepare the operationResult of the <request> resource and indicate that if all the member responses have been aggregated by setting the requestStatus of the <request> resource before the Result Expiration Time reaches. There may be multiple updates of the operationResult attribute. - nonBlockingRequestAsynch: notify with the aggregated response from all or part of the members before the Result Expiration Time reaches. There may be more than one notifications. - flexBlocking: continue aggregate the member response until the group hosting CSE determines to send the aggregated responses. If all member responses has been aggregated, respond the aggregated response as in the blockingRequest case. Otherwise, respond an acknowledgement together with the current aggregated member responses and the reference to the created <request> resource. Then continue aggregate and deliver the remaining member response to the Originator as defined in the nonBlockingRequestSynch or the nonBlockingRequestAsynch case. - After the Result Expiration Time, there shall not be any further updates to the aggregated responses. (See note) |
| Processing at Member Hosting CSE | For the RETRIEVE procedure, the Member Hosting CSE shall: Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the request identifier until the expiration of the request expiration time or local policy Check if the original Originator has the RETRIEVE permission on the addressed resource. Upon successful validation, perform the retrieve procedures for the corresponding type of addressed resource as described in other sub-clauses of clause 10.2 Send the corresponding response to the group Hosting CSE |
| Information in Response message | Converged responses from members hosting CSEs |
| Processing at Originator after receiving Response | None |
| Exceptions | Same request with identical group request identifier received Originator does not have RETRIEVE permission to access the <fanOutPoint> resource Members in Group Request Target Members request parameter are not subset of memberIDs attribute of the addressed <group> resource |
| NOTE: If Result Expiration Time is not provide in the original request from the Originator, the group hosting CSE may decide the timer based on its local policy. |  |

- 10.2.7.8	Update <fanOutPoint>

This procedure shall be used for updating the content of all member resources belonging to an existing <group> resource.

Table 10.2.7.8-1: <fanOutPoint> UPDATE

| <fanOutPoint> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <group> resource Content: The representation of the resource the Originator intend to Update Group Request Identifier: The group request identifier Response Type: If the parameter is set to BlockingSynch, it indicates that the group hosting CSE shall return the aggregated response once. Otherwise if the parameter is set to nonBlockingRequestSynch or nonBlockingRequestAsynch, it indicates that the Group Hosting CSE shall return the aggregated response in a batched mode Result Expiration Time: Indicates the maximum time limit in which the Group Hosting CSE has to respond the aggregated response. |
| Processing at Originator before sending Request | The Originator shall request to update all member resources belonging to an existing <group> resource with the same data by using a UPDATE operation. The request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group Hosting CSE. The request may also address the address that results from appending a relative address to the <fanOutPoint> in order to update the corresponding child resources represented by the relative address with respect to all <members> resources. The Originator may be an AE or CSE |
| Processing at Group Hosting CSE | For the UPDATE procedure, the Group Hosting CSE shall: Check if the Originator has UPDATE permission in the <accessControlPolicy> resource referenced by the membersAccessControlPolicyIDs in the group resource. In the case members membersAccessControlPolicyIDs is not provided the access control policy defined for the group resource shall be used Upon successful validation, obtain the IDs of all member resources from the attribute membersIDs of the addressed <group> resource If the group includes Multicast Group information, the group Hosting CSE shall perform the procedure as described in clause 10.2.7.13.2 If the request contains Group Request Target Members parameter, it shall check whether all members contained in this parameter are subset of memberIDs attribute of the addressed <group> resource. If true, the request shall be fanned out to the members contained in this parameter only. Generate fan out requests addressing the obtained address (appended with the relative address if any) to the members hosting CSEs as indicated in figure10.2.7.1-1 which are not in the multicast group. The From parameter in the fanout request is set to ID of the Originator from the request from the original Originator. The Response Type parameter in the fanout request may be set by the group hosting CSE differently according to its local policy In the case that a member resource is a <group> resource and the request to be fanned out does not contain a group request identifier already, generate a unique group request identifier, include it in all the requests to be fanned out and locally store the group request identifier If the group Hosting CSE determines that multiple member resources belong to one CSE according to the IDs of the member resources and no multicast group exists for these members, it may converge the requests accordingly before sending out. This may be accomplished by the group Hosting CSE creating a <group> resource on the member Hosting CSE to collect all the members on that members Hosting CSE - After receiving the responses from the members hosting CSEs, respond to the Originator with the aggregated results and the associated members list. Depending on the Response Type, the Group Hosting CSE shall: - BlockingRequest: respond with the aggregated responses before the Result Expiration Time reaches and discard the member responses received after - nonBlockingRequestSynch: prepare the operationResult of the <request> resource and indicate that if all the member responses have been aggregated by setting the requestStatus of the <request> resource before the Result Expiration Time reaches. There may be multiple updates of the operationResult attribute. - nonBlockingRequestAsynch: notify with the aggregated response from all or part of the members before the Result Expiration Time reaches. There may be more than one notifications. - flexBlocking: continue aggregate the member response until the group hosting CSE determines to send the aggregated responses, if all member responses has been aggregated, respond the aggregated response as in the blockingRequest case. Otherwise, respond an acknowledgement together with the current aggregated member responses and the reference to the created <request> resource. Then continue aggregate and deliver the remaining member response to the Originator defined in the nonBlockingRequestSynch or the nonBlockingRequestAsynch case - After the Result Expiration Time, there shall not be any further updates to the aggregated responses. (See note) |
| Processing at Member Hosting CSE | For the UPDATE procedure, the Member Hosting CSE shall: Check if the request has a group request identifier. Check if the request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the request identifier until the expiration of the request expiration time or local policy Check if the original Originator has the UPDATE permission on the addressed resource. Upon successful validation, perform the update procedures for the corresponding type of addressed resource as described in other sub-clauses of clause 10.2 Send the corresponding response to the group Hosting CSE |
| Information in Response message | Converged responses from members hosting CSEs |
| Processing at Originator after receiving Response | None |
| Exceptions | Same request with identical group request identifier received Originator does not have the UPDATE permissions to access the <fanOutPoint> resource Members in Group Request Target Members request parameter are not subset of memberIDs attribute of the addressed <group> resource |
| NOTE: If Result Expiration Time is not provide in the original request from the Originator, the group hosting CSE may decide the timer based on its local policy. |  |

- 10.2.7.9	Delete <fanOutPoint>

This procedure shall be used for deleting the content of all member resources belonging to an existing <group> resource.

Table 10.2.7.9-1: <fanOutPoint> DELETE

| <fanOutPoint> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <fanOutPoint> virtual resource Content: The representation of the resource the Originator intends to delete Group Request Identifier: The group request identifier Response Type: If the parameter is set to BlockingSynch, it indicates that the group hosting CSE shall return the aggregated response once. Otherwise if the parameter is set to nonBlockingRequestSynch or nonBlockingRequestAsynch, it indicates that the Group Hosting CSE shall return the aggregated response in a batched mode Result Expiration Time: Indicates the maximum time limit in which the Group Hosting CSE has to respond the aggregated response. |
| Processing at Originator before sending Request | The Originator shall request to delete all member resources belonging to an existing <group> resource by using a DELETE operation. The request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group Hosting CSE. The request may also address the address that results from appending a relative address to the <fanOutPoint> in order to delete the corresponding child resources represented by the relative address with respect to all member resources. The Originator may be an AE or a CSE |
| Processing at Group Hosting CSE | For the DELETE procedure, the <group> Hosting CSE shall: Check if the Originator has DELETE permission in the <accessControlPolicy> resource referenced by the membersAccessControlPoliciIDs in the <group> resource. In the case membersAccessControlPolicyIDs is not provided the access control policy defined for the group resource shall be used Upon successful validation, obtain the IDs of all member resources from the attribute membersIDs of the addressed <group> resource If the group includes Multicast Group information, the group Hosting CSE shall perform the procedure as described in clause 10.2.7.13.2. Generate fan out requests addressing the obtained address (appended with the relative address if any) to the member hosting CSEs as indicated in figure 10.2.7.1-1 which are not in the multicast group. From parameter in the fanout request is set to ID of the Originator from the request from the original Originator. The Response Type parameter in the fanout request may be set by the group hosting CSE differently according to its local policy In the case that the member resources is a <group> resource and the request to be fanned out does not contain a group request identifier already, generate a unique group request identifier, include the group request identifier in all the requests to be fanned out and locally store the group request identifier If the <group> Hosting CSE determines that multiple member resources belong to one CSE according to the IDs of the member resources and no multicast group exists for these members, it may converge the requests accordingly before sending out. This may be accomplished by the group Hosting CSE creating a <group> resource on the member Hosting CSE to collect all the members on that member Hosting CSE - After receiving the responses from the members hosting CSEs, respond to the Originator with the aggregated results and the associated members list. Depending on the Response Type, the Group Hosting CSE shall: - BlockingRequest: respond with the aggregated responses before the Result Expiration Time reaches and discard the member responses received after - nonBlockingRequestSynch: prepare the operationResult of the <request> resource and indicate that if all the member responses have been aggregated by setting the requestStatus of the <request> resource before the Result Expiration Time reaches. There may be multiple updates of the operationResult attribute. - nonBlockingRequestAsynch: notify with the aggregated response from all or part of the members before the Result Expiration Time reaches. There may be more than one notifications. - flexBlocking: continue aggregate the member response until the group hosting CSE determines to send the aggregated responses, if all member responses has been aggregated, respond the aggregated response as in the blockingRequest case. Otherwise, respond an acknowledgement together with the current aggregated member responses and the reference to the created <request> resource. Then continue aggregate and deliver the remaining member response to the Originator as defined in the nonBlockingRequestSynch or the nonBlockingRequestAsynch case - After the Result Expiration Time, there shall not be any further updates to the aggregated responses. (See note) |
| Processing at Member Hosting CSE | For the DELETE procedure, the Members Hosting CSE shall: Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the group request identifier until the expiration of the request expiration time or local policy Check if the original Originator has the DELETE permission on the addressed resource. Upon successful validation, perform the delete procedures for the corresponding type of addressed resource as described in other sub-clauses of clause 10.2 Send the corresponding response to the Group Hosting CSE |
| Information in Response message | Converged responses from members hosting CSEs |
| Processing at Originator after receiving Response | None |
| Exceptions | Same request with identical group request identifier received Originator does not have the DELETE permissions to access the <fanOutPoint> resource |
| NOTE: If Result Expiration Time is not provide in the original request from the Originator, the group hosting CSE may decide the timer based on its local policy. |  |

- 10.2.7.10	Subscribe and Un-Subscribe <fanOutPoint> of a group

This procedure shall be used for receiving information about modifications of all member resources belonging to an existing <group> resource.

Table 10.2.7.10-1: <fanOutPoint> Subscribe/Un-subscribe

| <fanOutPoint> Subscribe/Un-subscribe |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or CSE that initiates the request To: The address of the <fanOutPoint> resource appended with the ID of the <subscription> resource to be created Group Request Identifier: The group request identifier |
| Processing at Originator before sending Request | The Originator shall request to create a subscription resource under all member resources belonging to an existing <group> resource by using a CREATE operation. The request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group Hosting CSE. The request may also address the address that results from appending a relative address to the <fanOutPoint> in order to create the corresponding subscription to the resource represented by the relative address with respect to all member resources. In both cases the targeted resource shall the parent of the newly created <subscription> resource(s). The request shall include notificationForwardingURI attribute if the Originator wants the group Hosting CSE to aggregate the notifications. The request shall include the required information and may include the optional information as described in subscription management clause 10.2.10. The Originator may be an AE or a CSE |
| Processing at Group Hosting CSE | The <group> Hosting CSE shall: Check if the Originator has CREATE privilege in the <accessControlPolicy> resource referenced by the membersAccessControlPolicyIDs in the group resource. In the case membersAccessControlPolicyIDs is not provided the access control policy defined for the group resource shall be used If the subscription resource in the request contains an notificationForwardingURI attribute, assign a URI to replace the notificationURI of the subscription resource which will be used to receive notifications from member hosting CSEs. The ID of the <group> resource shall be set to the groupID attribute of the <subscription> resource. The group Hosting CSE shall maintain the mapping of the generated notificationURI and the former notificationURI - Upon successful validation, obtain the IDs of all member resources from the attribute membersIDs of the addressed <group> resource - Generate fan out requests addressing the obtained address (appended with the relative address if any) to the member hosting CSEs as indicated in figure 10.2.7.1-1. From parameter in the request is set to ID of the Originator from the request from the original Originator If the group Hosting CSE determines that multiple members resources belong to one CSE according to the IDs of the member resources, it may converge the requests accordingly before sending out. This may be accomplished by the <group> Hosting CSE creating a <group> resource on the members Hosting CSE to collect all the members on that members Hosting CSE After receiving the responses from the member hosting CSEs, respond to the Originator with the aggregated results and the associated members list |
| Processing at Member Hosting CSE | For the subscribe/un-subscribe procedure, the Members Hosting CSE shall treat the request received from the group Hosting CSE as a normal SUBSCRIBE request on the addressed member resource as if it comes from the original Originator. Therefore the members Hosting CSE shall: - Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the group request identifier until the expiration of the request expiration time or local policy Check if the original Originator has the READ permission on the members resource Upon successful validation, perform the subscribe procedures for the corresponding type of member resource as described in clause 10.2.10 Send the corresponding response to the group Hosting CSE |
| Information in Response message | Converged responses from member hosting CSEs |
| Processing at Originator after receiving Response | None |
| Exceptions | Same request with identical request identifier received Originator does not have the access control privilege to access the <fanOutPoint> resource |

Un-subscribing to the members of a <group> resource uses the “Delete <fanOutPoint>” procedure defined in 10.2.7.9.

An typical example of how the subscription is established is as follows. The Originator is creating subscription resource on Member-1 resource, Member-2 resource and Member-3 resource. Member-2 resource and Member-3 resource are members of Group-2 resource. Member-1 resource and Group-2 resource are members of Group-1 resource. In this case, Group-2 resource is the sub-group of Group-1 resource.

Originator

Group-1

Member-1

Group-2

Member-2

Member-3

<subscription>

notificationURI = address-1

notificationForwardingURI = address-1

<subscription>

notificationURI = address-2

notificationForwardingURI = address-1

<subscription>

notificationURI = address-3

notificationForwardingURI = address-1

Figure 10.2.7.10-1: Example of subscription through group

Originator sends the <subscription> resource creation request to <fanOutPoint> of Group-1 resource. The Originator intends the Group-1 Hosting CSE to aggregate the notifications, thus, the Originator sets the notificationForwardingURI identical with notificationURI which is address-1 which is the address where the notification is supposed to be sent.

On receiving the request, the Group-1 Hosting CSE fans out the <subscription> creation request to address Member-1 resource and <fanOutPoint> resource of Group-2 resource. As notificationForwardingURI is set by the Originator, the Group-1 Hosting CSE allocates address-2 to receive aggregated notifications and put address-2 in the notificationURI of <subscription> resource to be fanned out.

On receiving the request, the Group-2 Hosting CSE fans out the <subscription> creation request to address Member-2 resource and Member-3 resource. As notificationForwardingURI is set, the Group-2 Hosting CSE allocates address-3 to receive aggregated notifications and put address-3 in the notificationURI of <subscription> resource to be fanned out. The mapping between address-2 and address-3 is maintained by the Group-2 Hosting CSE.

On receiving the request by any of the Member Hosting CSE, <subscription> resource is created.

- 10.2.7.11	Aggregate the Notifications by group

This procedure shall be used for the group Hosting CSE to aggregate the notifications from member hosting CSEs and forward the aggregated notification to the subscriber.

Table 10.2.7.11-1: Aggregation of Notifications by group

| Aggregate Notifications by group |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | The same as table 10.2.10.7-1 |
| Processing at Originator before sending Request (Member Hosting CSE) | Whenever the resource that is subscribed-to is modified in a way that matches the policies as is specified in clause 9.6.8, notification needs to be sent to the subscriber, the Members Hosting CSE shall: Notify the subscriber at the notificationURI and include the notificationForwardingURI in the notification, if it exists |
| Processing at Group Hosting CSE | For the notification procedure, the Group Hosting CSE shall: On receiving the notifications from the member hosting CSEs at the notificationURI generated by the group Hosting CSE during fanning out the <subscription> creation request, validate if the notification is sent from its member resource and contains a notificationForwardingURI attribute. - Upon successful validation, aggregate the notifications which have the same notificationForwardingURI for the duration specified in notifyAggregation or until the number of notifications specified in notifyAggregation are received, whichever occurs first. Send the aggregated notification to the subscriber according to the notificationForwardingURI in the notification. In the case the addressed group is the member of another group through which the subscription is created the notification shall be sent to the notificationURI assigned by the group hosting CSE which contains the addressed group as the sub-group according to the mapping of the notificationURI maintained by the addressed group. Wait for the response. After receiving the response, split the response and respond to the members hosting CSEs separately The group Hosting CSE may stop aggregating the notifications when the expirationTime of the corresponding subscription expires |
| Processing at Member Hosting CSE | The subscriber shall treat every notification extracted from the aggregated notification as a separate notification received from the subscribed resource and generate corresponding responses. The subscriber shall aggregate the responses to these notifications and send the aggregated response to the group Hosting CSE |
| Information in Response message | According to clause 10.1.6 |
| Processing at Originator after receiving Response | According to clause 10.1.6 |
| Exceptions | According to clause 10.1.6 |

The example of aggregating notification following example in clause 10.2.7.10 is as follows.

Originator

Group-1

Member-1

Group-2

Member-2

Member-3

Notification to address-1

Notification to address-2

Notification to address-3

Figure 10.2.7.11-1: Example of aggregating notifications

Member Hosting CSEs sends notifications to their corresponding Group Hosting CSEs. In this example, Member-2 Hosting CSE and Member-3 Hosting CSE send notifications to address-3 which is the address allocated by the Group-2 Hosting CSE. Member-1 Hosting CSE sends notifications to address-2 which is the address allocated by the Group-1 Hosting CSE.

On receiving notifications by the Group-2 Hosting CSE at address-3, Group-2 Hosting CSE aggregates the notification that has notificationForwardingURI as address-1, as Group-2 Hosting CSE maintains the mapping of address-3 and address-2, and sends the aggregated notification to address-2.

On receiving notifications by the Group-1 Hosting CSE at address-2, Group-1 Hosting CSE aggregates the notification that has notificationForwardingURI as address-1, as Group-1 Hosting CSE receives <subscription> resource with notificationURI address-1, Group-1 Hosting CSE send the aggregated notification to address-1.

- 10.2.7.12	Retrieve <semanticFanOutPoint>

This procedure shall be used for performing semantic discovery or semantic query procedure using the descriptor content of all member <semanticDescriptor> resources belonging to an existing <group> resource. The Hosting CSE support of  semantic discovery functionality via <semanticFanOutPoint> virtual resource is indicated by the semanticSupportIndicator attribute set to TRUE.

For detailed descriptions of the semantic discovery procedure see [14] Clause 6.2

- 10.2.7.13	Multicast Group Management Procedures

In case the multicasting is used to fan-out group operations to members of a <group> resource, the Group Hosting CSE shall maintain information specified in table 10.2.7.13-1 pertaining to the multicast group(s) that are applicable to a <group> resource, such as the mapping relationship between the member resources and the multicast address(es) and the target URI(s) in the fan out requests.

For each multicast group of a <group> resource, there may be multiple records of Multicast Group Information. The Group Hosting CSE should manage and index the multiple records by internal identifier for each Multicast Group Information. The definition of the internal identifier is out of scope of this specification.

Table 10.2.7.13-1: Multicast Group Information managed by Group Hosting CSE

| Information | Multiplicity | Description |
| --- | --- | --- |
| groupID | 1 | Indicates the <group> resource identifier which the Multicast Group Information belongs to. |
| multicastType | 1 | - Indicates the underlying networks multicast capability of the group members, the value is the same as the attribute multicastCapability of the <remoteCSE>. See clause 9.6.4. |
| externalGroupID | 0..1 | See Table 9.6.44-2. |
| multicastAddress | 1 | See Table 9.6.44-2. |
| multicastGroupFanoutTarget | 1 | See Table 9.6.44-2. |
| memberList | 1(L) | See Table 9.6.44-2. |
| groupServiceServerAddress | 0..1 | It shall be present only when the multicastType is 3GPP_MBMS_group, and it is the address of 3GPP group service server (e.g. SCEF); |
| TMGI | 0..1 | The Temporary Mobile Group Identity is allocated to identify the MBMS bearer service as specified in 3GPP TS 23.246[i.Error: Reference source not found]. It’s used to communicate with 3GPP networking with externalGroupID together. |
| TMGIExpiration | 0..1 | Indicates the duration of the TMGI expiration time as specified in 3GPP TS 23.468 [i.Error: Reference source not found]. |
| responseTimeWindow | 0..1 | Defines the upper bound on the amount of delay the Member Hosting CSE shall wait before sending a response message. The Member Hosting CSE shall wait a randomized time that is less than the value of this attribute. This randomized delay helps prevent network congestion caused by multiple Member Hosting CSEs responding at the same time as one another. |

- 10.2.7.13.1	Multicast Group Information and <localMulticastGroup> Creation Procedures

Figure 10.2.7.13.1-1: Multicast Group Information and <localMulticastGroup> creation procedures

Figure 10.2.7.13.1-1 illustrates how the Multicast Group Information and <localMulticastGroup> resource works on the Group Hosting CSE and the group members Hosting CSEs.

001: The Originator sends a group resource creation request to the Group Hosting CSE which includes member resource identifier list consisting of multiple resources hosted on Member Hosting CSEs.

For example, different member resources identifiers are:

- /CSExx1/aa

- /CSExx1/bb

- /CSExx2/cc

- /CSExx2/dd

002: The Group Hosting CSE shall create the group resource as requested.

003: The Group Hosting CSE shall return the group creation response to the Originator

004: The Group Hosting CSE shall check the multicastCapability attribute of each Member Hosting CSE’s <remoteCSE> resource to determine whether it could create a multicast group. If no member hosting CSEs support multicast capability or no more than one Member Hosting CSE supports the same multicast capability, then the Group Hosting CSE shall not create a multicast group..

If at least two Member Hosting CSEs support the same multicast capability, the Group Hosting CSE determines to create multicast group, and performs all the actions: assign the multicast type based on the multicast capability, and allocate multicast address and multicast address type to the member resources of the multicast group. For a guide to an allocation scheme of IPv4 and IPv6 multicast address spaces, reference the specificationdocuments such as RFC 3171 [i.Error: Reference source not found] and RFC 4291 [i.Error: Reference source not found].

- If the multicastType is 3GPP_MBMS_group, and the Member Hosting CSEs have the same externalGroupID, the Group Hosting CSE shall set the externalGroupID to the same value as the externalGroupID of the Member Hosting CSEs, set the responseTarget to the CSE-ID  of  the <CSEBase> resource of  the Group Hosting CSE, set the memberList to the members defined in the request, allocate a virtual fan out target for multicastGroupFanoutTarget according to the member resource identifiers, and establish the mapping relationship between the fanout target and the member resource identifiers.

- Note: Additional details for creating a 3GPP MBMS group are specified in clause 7.7.3.1 of TS-0026[15]

- If the multicastType is IP_multicast_group, the Group Hosting CSE shall set the memberList to the members defined in the request, allocate a virtual fan out target for multicastGroupFanoutTarget according to the member resource identifiers, establish the mapping relationship between the fanout target and the member resource identifiers and set the Group Hosting CSE resourceID as the responseTarget.

  - NOTE1: The current group based multicast can only be used when Member Hosting CSE and Group Hosting CSE have a registration relationship. For example, the Group Hosting IN-CSE can create a multicast group for Member Hosting MN/ASN-CSEs, and cannot create a multicast group for Member Hosting CSEs which are not registered to the IN-CSE..

  - NOTE2: The current group based multicast can only be applied between CSEs. Multicast to AEs is FFS

  - NOTE3:  The externalGroupID is pre-provisioned in the operator’s  network. The service provider and the operator need ensure that the externalGroupID assigned by the operator matches the externalGroupID attribute of each Member Hosting CSE’s <CSEBase> resource.

The Group Hosting CSE shall create Multicast Group Information locally and establish a mapping relationship between the multicast address and the fanout target according to the member resource identifiers on the Member Hosting CSE. The multicastGroupFanoutTarget should be set to / {groupHostingCSE-ID}/ {fanout-segment}. The multicastGroupFanoutTarget should be uniquely assigned by the Group Hosting CSE.

{fanout-segment} is a string assigned by the Group Hosting CSE.

In this example, the Multicast Group Information is illustrated in figure 10.2.7.13.1-2.

Figure 10.2.7.13.1-2: Multicast Group Information

005: The Group Hosting CSE shall send <localMulticastGroup> creation request which carries a mapping relationship between the multicastGroupFanoutTarget and the member resources to the Member Hosting CSEs to advertise them to join the multicast group corresponding to the multicast address in unicast mode.  If the Group Hosting CSE determines that multiple members resources belong to one Member Hosting CSE according to the IDs of the members resources, it may converge the requests accordingly before sending out. In this example, the Group Hosting CSE should send two group advertisement messages to the two Member Hosting CSEs respectively.

006: The Member Hosting CSEs receive the creation request and shall use a multicast management protocol such as MLD or IGMP to join the multicast group corresponding to the multicast address indicated in the multicast group advertisement.

The Member Hosting CSEs shall create the < localMulticastGroup > after successfully joining the multicast group and store  a mapping relationship between the multicastGroupFanoutTarget and the multicast address,  and a mapping relationship between the member list and the multicast address.

In this example, both the ASN-CSE1 and ASN-CSE2 create each <localMulticastGroup> resource to record to which multicast group its local member resources have joined.

Figure 10.2.7.13.1-3: <localMulticastGroup> resource in Member Hosting CSE

007: The members Hosting CSEs shall send the response to the Group Hosting CSE.

008: The Group Hosting CSE shall check the response messages from Member Hosting CSEs, if at least two members respond successfully, the Group Hosting CSE shall keep Multicast Group Information locally. Otherwise, the Group Hosting CSE shall delete the information.

NOTE: The Group Hosting CSE may create one or more sets of Multicast Group Information according to the member resources of the group resource created by the Originator.

- 10.2.7.13.2	Multicast Group member Fan out Procedures

Figure 10.2.7.13.2-1: Multicast Group Information fan out procedures

001: The Originator sends an access <fanOutPoint> request carrying the group resource identifier for accessing member resources to the Group Hosting CSE.

002: The Group Hosting CSE shall check whether there is Multicast Group Information, according to the group resource identifier. If the group has Multicast Group Information, check the multicast type:

Case A): The multicast type is 3GPP_MBMS_group.

- 003a The Group Hosting CSE shall get the externalGroupID based on the groupID in the Multicast Group Information, then send request message to the group service Server (eg. SCEF) to perform TMGI allocation and Group Message request of the 3GPP MBMS group message procedure, as specified in the oneM2M TS-0026[15].

Case B): The multicast type is IP_multicast_group.

- 003b: The Group Hosting CSE shall send the member resource access request to the Member Hosting CSEs in multicast mode according to the multicast address of the multicast group, which includes the multicastGroupFanoutTarget as the fan out target address corresponding to the member resource in the group resource. If Request Expiration Timestamp is not in the request from the Originator, the Group Hosting CSE shall set the Request Expiration Timestamp in the multicast request according to the local policy.

004: The Member Hosting CSE receives from the multicast address a member resource access request, which carries the fan out target matching the multicastGroupFanoutTarget of a <localMulticastGroup> resource that contains the same multicast address. It shall determine that the local member resource identifiers as the final targets of the request by the mapping relationship between the memberList and multicastGroupFanoutTarget in the <localMulticastGroup> resource, then replaces the multicastGroupFanoutTarget with the determined member resource identifiers and executes the operation indicated by the resource access request.

The Member Hosting CSEs shall not return any Acknowledgement when receiving the message from the multicast address.

005: The Member Hosting CSEs shall compose the response message for Group Hosting CSE about the access result: set To parameter value to the responseTarget of <localMulticastGroup> resource according to the Response Type in the request; and set From parameter value to CSE-ID of Member Hosting CSE.

006: The Member Hosting CSEs shall send the response message including the access results to the Group Hosting CSE.

007: The Group Hosting CSE shall determine a multicast response message according to the From and the Request Identifier in message, then aggregate the group member resource access results in the response messages from Member Hosting CSEs.

The Group Hosting CSE shall not return any response after parsing the Notification message content which is response message to the multicast request.

008: The Group Hosting CSE returns the aggregated group member resource access result to the Originator.

- 10.2.7.14	Create <localMulticastGroup>

This procedure shall be used for creating <localMulticastGroup> resource.

Table 10.2.7.14-1: <localMulticastGroup> CREATE

| <localMulticastGroup> CREATE |  |
| --- | --- |
| Associated Reference Point | Mcc and Mcc' |
| Information in Request message | From: Identifier of the group Hosting CSE that initiates the Request To: The address of the <localMulticastGroup> resource. Content: The representation of the <localMulticastGroup> resource for which the attributes are described in clause 9.6.44 |
| Processing at Originator before sending Request | Besides the basic procedures described in clause 10.1.1.2.The Multicast Group Information shall be created and maintained by the group Hosting CSE. The group Hosting CSE shall configure the group Hosting CSE as the only entity who has the CRUD privileges to the <localMulticastGroup> by configuring the corresponding <accessControlPolicy> resource. |
| Processing at Receiver | Besides the basic procedures described in clause 10.1.1.2, the receiver shall also comply with the multicast management protocol such as MLD or IGMP to join the multicast group and create the <localMulticastGroup> resource as specified in clause 10.2.7.13.1. |
| Information in Response message | According to clause 10.1.1.2. |
| Processing at Originator after receiving Response | None |
| Exceptions | If the Receiver responds with an error, the group Hosting CSE shall delete the receiver’s information from the Multicast Group Information maintained locally. |

- 10.2.7.15	Retrieve <localMulticastGroup>

This procedure shall be used for retrieving <localMulticastGroup> resource.

Table 10.2.7.15-1: <group> RETRIEVE

| <group> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mcc and Mcc' |
| Information in Request message | From: Identifier of the CSE that initiates the Request To: The address of the <localMulticastGroup> resource |
| Processing at Originator before sending Request | The Originator shall request to obtain <localMulticastGroup> resource information by using the RETRIEVE operation. The request shall address the specific <localMulticastGroup> resource of a Hosting CSE. |
| Processing at Receiver | - No change from the basic procedure in clause 10.1.3 |
| Information in Response message | - No change from the basic procedure in clause 10.1.3 |
| Processing at Originator after receiving Response | None |
| Exceptions | - No change from the basic procedure in clause 10.1.3 |

- 10.2.7.16	Update <localMulticastGroup>

This procedure shall be used for updating <localMulticastGroup> resource.

Table 10.2.7.16-1: <localMulticastGroup> UPDATE

| <localMulticastGroup> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mcc and Mcc' |
| Information in Request message | From: Identifier of the CSE that initiates the Request To: The address the <localMulticastGroup> resource. Content: The representation of the <localMulticastGroup> resource for which the attributes are described in clause 9.6.44 |
| Processing at Originator before sending Request | According to clause 10.1.4. |
| Processing at Receiver | According to clause 10.1.4. |
| Information in Response message | According to clause 10.1.4. |
| Processing at Originator after receiving Response | None |
| Exceptions | None |

- 10.2.7.17	Delete <localMulticastGroup>

This procedure shall be used for deleting an existing <localMulticastGroup> resource.

Table 10.2.7.17-1: <localMulticastGroup> DELETE

| <localMulticastGroup> DELETE |  |
| --- | --- |
| Associated Reference Point | Mcc, and Mcc' |
| Information in Request message | From: Identifier of the CSE that initiates the Request To: The address of the <localMulticastGroup> resource |
| Processing at Originator before sending Request | According to clause 10.1.5. |
| Processing at Receiver | Besides the basic procedures described in clause 10.1.5, the receiver shall also comply with the multicast management protocol such as MLD or IGMP to leave the multicast group. |
| Information in Response message | According to clause 10.1.5. |
| Processing at Originator after receiving Response | None |
| Exceptions | According to clause 10.1.5. |

### ---------------------------------------- End of Change 1---------------------------------------------------

### ------------------------------------ Start of Change 2---------------------------------------------------

- 9.6.13	Resource Type group

The <group> resource represents a group of resources of the same or mixed types. The <group> resource can be used to do bulk manipulations on the resources represented by the memberIDs attribute. The <group> resource contains an attribute that represents the members of the group and the <fanOutPoint> virtual resource that enables generic operations to be applied to all the resources represented by those members. By grouping <semanticDescriptor> resources across which a semantic description is distributed, another virtual resource (<semanticFanOutPoint>) enables semantic discovery procedures to be applied across the full logical tree in the description.

Members of a <group> resource may support unicast or multicast communication. In case multiple members of a <group> resource support multicast communications and they share the same multicast address, those members form a multicast group as a sub-set of the group. There may be multiple multicast groups corresponded to one group since the members of the group may use different multicast mechanisms (e.g. 3GPP MBMS vs. IP multicast) and different multicast addresses.

The <group> resource shall contain the child resources specified in table 9.6.13-1.

Table 9.6.13-1: Child resources of <group> resource

| Child Resources of <group> | Child Resource Type | Multiplicity | Description | <groupAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| fopt | <fanOutPoint> | 1 | See clause 9.6.14 | None |
| sfop | <semanticFanOutPoint> | 0..1 | See clause 9.6.14a | None |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |

The <group> resource shall contain the attributes specified in table 9.6.13-2.

Table 9.6.13-2: Attributes of <group> resource

| Attributes of <group> | Multiplicity | RW/ RO/ WO | Description | <groupAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3. | NA |
| memberType | 1 | WO | It is the resource type of the member resources of the group, if all member resources (including the member resources in any sub-groups) are of the same type. Otherwise, it is of type 'mixed'. | OA |
| currentNrOfMembers | 1 | RO | Current number of members in a group. It shall not be larger than maxNrOfMembers. | OA |
| maxNrOfMembers | 1 | RW | Maximum number of members in the <group>. | OA |
| memberIDs | 1 (L) | RW | List of member resource IDs referred to in the remaining of the present document as memberID. Each ID (memberID) should refer to a member resource or a (sub-) <group> resource of the <group> if memberID is suffixed with “/fopt”. A <group> resource with an empty member list is allowed. | OA |
| membersAccessControlPolicyIDs | 0..1 (L) | RW | List of IDs of the <accessControlPolicy> resources defining who is allowed to access the <fanOutPoint> and <semanticFanOutPoint> virtual resources. | OA |
| memberTypeValidated | 0..1 | RO | Denotes if the resource types of all members’ resources of the group have been validated by the Hosting CSE. In the case that the memberType attribute of the <group> resource is not 'mixed', then this attribute shall be set. | OA |
| consistencyStrategy | 1 | WO | This attribute determines how to deal with the <group> resource if the memberType validation fails. Its possible values are - ABANDON_MEMBER - ABANDON_GROUP - SET_MIXED Which means delete the inconsistent member if the attribute is ABANDON_MEMBER; delete the group if the attribute is ABANDON_GROUP; set the memberType to "mixed" if the attribute is SET_MIXED. If it is not given by the Originator at the creation procedure, default is " ABANDON_MEMBER " | OA |
| groupName | 0..1 | RW | Human readable name of the <group>. | OA |
| semanticSupportIndicator | 0..1 | RO | Indicator of support for sematic discovery functionality via <semanticFanOutPoint>. | OA |
| notifyAggregation | 0..1 | RW | This attribute specifies the number of messages and/or the duration that the group hosting CSE will aggregate notification messages when the subscriptions created specify aggregation of notifications i.e. specifying the notificationForwardingURI of the original <subscription> resource. | OA |

### ----------------------- end of Change 2---------------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?