| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #35 |
| Source:* | Dale Seed, Convida, Seed.Dale@convidawireless.com |
| Date:* | 2018-05-16 |
| Reason for Change/s:* | NIDD Configuration Delete Procedure |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS 0026 v 0_8_1 |
| Clauses * | Section 7.1.1.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

### TS0026 indicates that it is recommended to have the NIDD Configuration setup towards SCEF to be inititated before the UE hosting the ADN-AE or ASN/MN-CSE attaches to the network. This would mean that the entity may not be registered to the IN-CSE before an attach is initiated or when the NIDD Configuration is inititated by the IN-CSE. In this case, the design supports NIDD configuration for externalIDs that are not registered to the IN-CSE.

### However, TS0026 also states that when the <AE> or <remoteCSE> hosted on the UE is de-registered, NIDD Configuration delete shall be initiated. This contradicts with the statement that the NIDD Configuration setup can be initiated before the UE attaches.

This contribution proposes to initiate NIDD Configuration create when the UE gets enrolled using <serviceSubscribedNode> resource and initiate NIDD Configuration delete when the niddRequired is FALSE or the <serviceSubscribeNode> gets deleted.

### -----------------------Start of change 1-------------------------------------------

### 7.1.1.1 	SCEF Configuration for NIDD

The SCEF NIDD API supports an NIDD Configuration procedure that may be used by the IN-CSE to inform the SCEF that it expects Non-IP Data from a UE hosting an MN-CSE, ADN-AE, or ASN-CSE.  Figure 7.1.1.1-1 illustrates this procedure.  If the NIDD Configuration procedure is performed, the IN-CSE should perform the procedure before a UE attaches and attempts to establish a Non-IP PDN connection to the SCEF.

Figure 7.1.1.1-1: NIDD Configuration Request

Pre-conditions:

The IN-CSE is configured with the M2M-Ext-ID of a UE and an indication that the ASN/MN-CSE or ADN-AE hosted on this UE uses NIDD to exchange oneM2M primitives with the IN-CSE.  This information is configured in the deviceIdentifier and niddRequired attributes of the <serviceSubscribedNode> resource corresponding to the UE.

There is a relationship in place between the Service Provider and MNO allowing the IN-CSE to perform NIDD Configuration Requests to the underlying 3GPP network.  The method for establishing this relationship is outside the scope of the present document.

Step 1: IN-CSE determines to issue NIDD Configuration Request

If the niddRequired attribute of a <serviceSubscribedNode> resource associated with a UE hosting an ASN/MN-CSE or ADN-AE is set to TRUE, then the IN-CSE shall issue a NIDD Configuration Request to the proper SCEF.

Step 2 (Optional): DNS Query/Response

To determine which SCEF to contact, an IN-CSE may determine the IP address(es)/port(s) of the proper SCEF by performing a DNS query using the M2M-Ext-ID of the UE hosting the ASN/MN-CSE or ADN-AE.  This M2M-Ext-ID shall be configured in the deviceIdentifier attribute of the <serviceSubscribedNode> resource associated with the UE.  Alternatively, an IN-CSE may use a pre-configured SCEF identifier.  The method for pre-configuring a SCEF identifier into the IN-CSE is outside the scope of the present document.

Step 3: NIDD Configuration Request

The IN-CSE issues a NIDD Configuration Request for a particular ASN/MN-CSE or ADN-AE hosted on a UE. The fields of the API are populated as follows.

- externalId  shall be set to the M2M-Ext-ID of the UE hosting the targeted ASN/MN-CSE or ADN-AE.

- scsAsId shall be set to a value that is prearranged between the Service Provider and MNO.

- notificationDestination shall be set to a URI of the IN-CSE that the SCEF will deliver MO NIDD data to.

- tltrId is used to identify the NIDD Configuration Request.  It shall be assigned based on internal IN-CSE policies and shall be different from other tltrIds issued by this IN-CSE during the lifetime of this request.  The tltrId is also used in NIDD Configuration Update or Cancel requests.

- duration specifies the lifetime of the NIDD Configuration and shall be set per SLA between the Service Provider and MNO. The SCEF may change the NIDD Duration value.

- pdnEstablishmentOption may be used to indicate the IN-CSE’s default preference for how the SCEF should process a MT NIDD Submit Request from the IN-CSE if the UE has not yet established a Non-IP PDN connection to the SCEF.   This value shall be set based on SLA with the MNO.

- reliableDataService shall be set to TRUE or FALSE to indicate that Reliable Data Service is enabled or disabled based on IN-CSE preferences.

- rdsPorts shall be set to the source and destination ports used for MO and MT NIDD between the IN-CSE and the ASN/MN-CSE or ADN-AE hosted on the UE.  This field shall be set if reliableDataService is set to TRUE.

Step 4: Process NIDD Configuration Request

The SCEF processes the request.

Step 5: NIDD Configuration Response

The SCEF responds with a 201 CREATED indicating the request was accepted.  The message includes the following information.

- id is a SCEF-chosen identifier for this NIDD Configuration.

- tltrid is present if it was present in the NIDD Configuration request and is set to the same value.

- maximumPacketSize is set to the maximum supported NIDD packet size that can be transferred to the UE by the SCEF.  This value is configured by the SCEF per SLA with the MNO.

- reliableDataService indicates if the Reliable Data Service is enabled or not

- status is set to a value that indicates the NIDD configuration status (e.g. ACTIVE)

- self is configured with a link to the resource created by the SCEF for the request

If the response indicates that the request was accepted, the IN-CSE shall use the maximumPacketSize as a limit on the maximum size MT NIDD Submit Request it shall initiate towards the corresponding UE specified in the NIDD Configuration Request.

Step 6 (Optional): NIDD Configuration Delete Request

If the IN-CSE detects that <serviceSubscribedNode> is deleted or the <serviceSubscribedNode> niddRequired attribute is updated to FALSE, then the IN-CSE shall issue a NIDD Configuration Delete Request for the UE. The fields of the API are populated as follows.

- id shall be set to the id value assigned and returned to the IN-CSE in the NIDD Configuration response corresponding to the UE.

- externalId  shall be set to the M2M-Ext-ID of the UE hosting the targeted ASN/MN-CSE or ADN-AE.

- scsAsId shall be set to a value that is prearranged between the Service Provider and MNO.

- tltrId is configured with the same tltrId from the NIDD Configuration request.

Step 7 (Optional): Process NIDD Configuration Delete Request

The SCEF processes the request.

Step 8 (Optional): NIDD Configuration Delete Response

The SCEF responds with a 204 NO CONTENT that indicates the NIDD Configuration was cancelled.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?