| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#35 |
| Source:* | Catalina Mladin, Convida Wireless, Mladin.Catalina@convidawireless.com Dale Seed, Convida Wireless, Seed.Dale@convidawireless.com |
| Date:* | 2017-05-10 |
| Reason for Change/s:* | Provide solution for Key Issue in TR-0026 |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0046> MNT maintenance / < Work Item number(optional)> Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0026 V 4.0.0 |
| Clauses * | 10 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution provides two solutions for Key Issue 7 (maintaining time synchronization).

R01 addresses comments from ARC #35:

- Use “originating local time” instead of “Current Local Time”

- Address multiple hops via the editor’s note

- Indicate targets via pointOf Access , in case the teragets are not registrees.

### -----------------------Start of change 1-------------------------------------------

- 10.X Solution H : Time Synchronisation using beacons

- 10.X.1 Solution Description

This solution addresses Key Issue 7 which highlights that services requiring time-sensitive operations rely on the various Service Layer entities being time-synchronized.

CSEs can be configured by one or more registrees to send SL beacon messages to be used by themselves and /or other SL registrees. The beacons can be customized to be sent at specific frequencies (e.g. every 5 minutes) or in response to customized conditions or events (e.g. certain synchronization offset has been exceeded), and the settings can be specified by one or more registrees.

Time synchronization beacon messages contain information such as the local time of the originator of the beacon and other coordination scheduling information e.g. information used by receivers to program their sleep and low power schedules.

The solution relies upon the following elements:

- CSEs advertise their capability of providing time synchronization beacons, e.g. by supporting a dedicated <timeSynchBeacon> resource at the<CSEBase>.

- AEs and CSEs provide information during registration about the synchronization services requested, for example: beacon interval, target(s), beacon criteria, desired accuracy, registree’s time precision.

The CSE providing time synchronization services can provide accepted values (e.g. for beacon interval, targets, criteria and accuracy) with the response.

- The CSE provides a message with the time-synchronization information (beacon message) based on the configured parameters. The message includes: originator CSE-ID, target, CSE time, beacon event. The beacon can include a non-cacheable flag, indicating that the message should be discarded if a registree is not available, in order to avoid synchronizations with out of date beacons.  The beacon can also include a parameter indicating the number of hops the beacon is allowed to propagate in order to avoid flooding.

- The beacon receiver can extract a CSE Time component, compare it against the local time and an offset is computed.  The offset can then be used to synchronize the local time to the local time of the beacon originator.

Note: This solution allows for several options which are FFS:

- Allowing for cases where a hosting CSE provides synchronization services based on more than one <timeSynchBeacon> resource.

- Allowing for each <timeSynchBeacon> resource to be created and managed by the hosting CSE only. These operations can take into consideration information provided by other entities, as described in 2. Above.

- Allowing <timeSynchBeacon> resources to be created at a hosting CSE by entities requesting synchronization services. The hosting CSE uses local policies to accept these requests, then provides the requested service.

Editor’s note: Beacon delivery over multiple hops should be investigated further.

### 10.X.2 Solution Applicability

This solution applies to Key Issue 7

### 10.X.3 Solution Details

To implement this solution, the new <timeSynchBeacon> resource type needs to be introduced, as described in Table 10.x.3-1. Existing resource types <CSEBase> and <AE> are modified to include <timeSynchBeacon> as optional child resources.

Table 10.x.3-1 <timeSynchBeacon> Resource Attributes

| New Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| timeSynchBeaconInterval | 0..1 | RW | Frequency of the beacon, used for the periodic timeSynchBeaconCriteria |
| timeSynchBeaconTargetList | 0..1 (l) | RW | Targets for the beacons specified as a list of oneM2M pointOfAccess attributes |
| timeSynchBeaconCriteria | 0..1 | RW | Criteria for the beacon generation. For example: - Periodic - Loss of Synchronization – Beacon is sent as a result of a detection in a loss of synchronization exceeding a defined threshold and between the CSE issuing the beacon and one or more registrees. |

- 10.Y Solution I : Time Synchronization using request and response parameters

- 10.Y.1 Solution Description

This solution addresses the time synchronization Key Issue 7 by proposing additional optional message parameters to be added to the oneM2M primitives.

In this proposal new message parameters can be used to advertise as well as compare the local times of a synchronizing CSE and its registrees, compute offsets, and adjust local times of the CSE and/or its registrees such that they remain synchronized with one another.  These message parameters can be included in requests as well as in response messages, originated by the CSE or originated by the registrees.

When time synchronization services are enabled, the timing information might be included in all messages exchanged with a given registree, or only in select messages (e.g. one every n seconds or one in every n messages).  This allows providing adequate information for maintaining proper synchronization without introducing unnecessary overhead.

In this solution a Service Layer Ping message is also proposed, enabling the calculation of network latency for time compensation. In addition, registree entities can request the registrar CSE to perform the time compensation and correction on their behalf.

### 10.Y.2 Solution Applicability

This solution applies to Key Issue 7.

### 10.Y.3 Solution Details

To implement this solution, the <remoteCSE> and <AE> resource types are proposed to be modified to include a Boolean parameter (e.g. requestedTimeSynchServices attribute). When the attribute is set to TRUE, the registrar CSE is requested to provide time synchronization services to the registree. Note that the <CSEBase> resource type can also be modified to include a parameter (e.g. timeSynchServicesEnabled attribute) to indicate that it can provide these services.

To implement this solution, several new message parameters are also introduced, as described below:

•	Originating Local Time – Local time value captured and inserted into the message before it is sent by any originator.

•	SL Ping Request  - Flag indicating if the message is a Service Layer ping request

•	SL Ping Receipt Time- The local time at target when a SL ping request was received. This parameter is sent only in responses to a SL Ping Request, along with the Current Local Time at the receiver of the SL ping request.

•	Updated Time – A time value that is inserted into a message for the purposes of instructing the recipient of the message to change its local time to the value specified. This value should be specified only by a registrar CSE, if requestedTimeSynchServices is enabled.

Note 1:  The time compensation and correction capability of a CSE can take into account the amount of time it takes for a SL message exchanges between the two entities (i.e. the network latency).

Note 2:  The time compensation and correction capability of a CSE can be invoked based on different types of trigger conditions e.g. when the time offset between a registree’s and the CSE local times exceeds a threshold, when the  registree is hosted within a certain network domain or geographical region, etc.

Note 3:  This solution can be used in conjunction with solution H. The time compensation and correction capability of a CSE can be invoked based on trigger conditions specified by a registree in a <timeSynchBeacon> resource, with timeSynchBeaconCriteria = Loss of synchronization. When the loss of synchronization is detected (based on previous message exchanges, pings, etc.), the registrar CSE sends a message including the updated time parameter.

### -------------------- end of change 1 -----------------------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables. Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?