| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 32.2 |
| Source:* | Catalina Mladin, Convida, Mladin.Catalina@convidawireless.com; Yuan Tao, CNICG, taoyuan@cnicg.cn ; |
| Date:* | 2018-05-12 |
| Reason for Change/s:* | New potential solution for interworking with identification resolution systems using dedicated physical object resources |
| CR against: Release* | Release 3 |
| CR against: WI* | Active (WI-0069) MNT maintenance / < Work Item number(optional)> Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0044 Version 1.0.0 |
| Clauses * | 8.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution adds a new potential solution for enabling oneM2M platforms to provide support to identification resolution systems by using dedicated passive/tangible object resources with corresponding ID information.

The use case detailed in clause 7.2 indicates that “all information related to the physical entity and the M2M service platform should be compatible with the various identification systems.” For this purpose, having a dedicated resource to represent passive/tangible object will ensure that the necessary passive/tangible object information is available within the system, in addition to the mapping to other/external identification systems. This enables oneM2M platforms to provide, in addition to comprehensive services related to passive/tangible object, an interworking or mapping service with the external systems or between them.

## ----------------------- Start of change  1 -----------------------

## 8.1	Scenarios analysis based on oneM2M architecture

### 8.1.1	Creation of physical object representation and repository

A oneM2M AE captures physical/passive object using specialized resources (e.g. <physicalObjectResource> type) with information that describes it for use in within the system. These specialized resources may be stored at CSEs in a location associated with the AE or in repositories.

The creation procedure is described at a high level in Figure 8.1.1-1. Various attributes of the specialized resource may be populated at the time of creation or provided via updates. The details related to attribute creation is to be provided with the resource definitions in clause 8.2.

Figure 8.1.1-1 Creation procedures for the <poResource> resource

The oneM2M platform may interwork with external systems or databases based on different identification mechanisms and technologies such as ONS, Handle system, uCodeRP, etc. In order to enable interworking with these systems and access to the associated physical object information, the resource definitions in clause 8.2. provide links to these external resources

### 8.1.2	 Providing associated tangible identifiers and mapping information

A oneM2M AE captures the tangible identifier from tag attaching to a physical entity and also captures other related information (e.g. location, time). The procedures are shown in Figure 8.1.2-1 and assume that specific attributes of the resource representing the physical object (e.g. <physicalObjectResource>) are used to capture this information. The tangible identifiers may be provided via UPDATE operations to the associated physical object resource  and operation details are to be provided with the resource definitions in clause 8.2.

Figure 8.1.2-1 Creation procedures

- AEs (AE1, AE2) and potentially other information (e.g. location, time) of the physical entities.

- AEs optionally store the tangible identifiers, information (e.g. location, time) of the physical entities, identifier type, and the communication identifier of AE in the CSE.  There are five possible combinations: 1) tangible identifier, information of the physical entity; 2) tangible identifier, communication identifier; 3) tangible identifier, information of the physical entity, identifier type; 4) tangible identifier, communication identifier, identifier type; 5) identifier type, communication identifier.

### 8.1.3	Discovery of identifiers related information

The application wants to discover the tangible identifier related information from the CSE that provides the 	heterogeneous service. There are two cases depicted as follows:

Case 1: the CSE4 maintains the tangible identifier related mapping information, the discovery procedures are shown in 	Figure 8.1.3-1.

Figure 8.1.3-1 discovery procedures for case 1

- The discovery application AE3 initiates a request to discover device that interworks to a particular physical entity. The tangible identifier of the physical entity is included in the request.

- The CSE receives and processes the request. If the identifier type is not included in the request, the CSE can  recognize the type of tangible identifier. The CSE resolves the tangible identifier to a communication identifier of the device that interworks to the physical entity.

- The CSE forwards the discovery results to the AE.

Case 2: the mapping information is not recorded on the queried CSE. The CSE will interwork with external identification	resolution system(s) to assist with the identification resolution. The discovery procedure is shown in figure 8.1.2-2.

Figure 8.1.3-2 discovery procedures for case 2

- The discovery application AE initiates a request to discover the device that interworks to a particular physical entity.  The tangible identifier of the physical entity is included in the request.

- The CSE receives the request. If the identifier type is not included in the request, the CSE can recognize the type of tangible identifier.

- The CSE sends the tangible identifier to the corresponding external resolution system (e.g. Handle system),	and gets and resolution results from the resolution system. The results include the communication identifier of the device that interworks to the physical entity.

- The CSE responds with the discovery results to the AE.

## 8.2	Potential solutions

This section includes the changes needed in TS-0001 in existing resources, Figures and Tables to support the above scenarios. Impacted sections in TS-0001 will be shown using the applicable section number.

### 9.6.x	Resource Type <physicalObjectResource>

The <physicalObjectResource> resource represents information about a physical object data instances. It is used to share information with other entities and potentially to track the physical object.

Table 9.6.x-1: Child Resources of <physicalObjectResource> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <semanticDescriptor> | [variable] | 0..n | Clause 9.6.30 |
| <subscription> | [variable] | 0..n | Clause 9.6.8 |
| <flexContainer> | [variable] | 0..n | Clause 9.6.34 |

Table 9.6.x-2: Attributes of <physicalObjectResource> resource

| Attributes of <physicalObjectResource> | Multiplicity | RW/ RO/ WO | Description | <physicalObject ResourceAnnc> Attributes |
| --- | --- | --- | --- | --- |
| poID | 0..1 | RW | This attribute is a oneM2M identifier of the physical object | OA |
| tangibleID | 0..1(L) | RW | This parameter is a list of tangible identifiers associated with the specific physical entities. It is described as a list of tangible identifiers (e.g. 86.1000.11/70140703307739, 1.2.156.20000.2010319). It shall be present in case IDType parameter is null. | OA |
| IDType | 0..1(L) | RW | - It shall be present in the case tangibleID paremeter is null. If the tangibleID parameter is present, it represents the corresponding type of the tangible identifiers. If the tangibleID is not present, it represents the identifier type that is supported by the creation originator. The identifier type is expressed in string: - epc; - handle; - oid; - ucode; - mcode; - ecode. | OA |
| communicationID | 0..1 | RW | It represents the physical address (e.g. IP address, FQDN) used by the device that that interworks to the physical entity. | OA |
| associatedPoIDList | 0..1(L) | RW | A list of associated physical objects | OA |
| externalDatabaseList | 0..1(L) | RW | A list of associated external databases containing additional information | OA |
| poDefinition | 0..1 | WO | This contains an identifier reference (URI) to a schema definition which shall be used by the CSE to validate the syntax of the resource, if one or more [customAttributes] are used. If this attribute is not present, no [customAtrribute] specialized attributes are defined. |  |
| [customAttribute] | 0..n | RW | Specialization-specific attribute(s) used to create a specialization of the resource. Name and data type pairs are defined in the the schema provided in the poDefinition attribute. | OA |

- ----------------------- End of change  1 -----------------------