| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 32.2 |
| Source:* | Catalina Mladin, Convida, Mladin.Catalina@convidawireless.com; Yuan Tao, CNIC, taoyuan@cnicg.cn ; |
| Date:* | 2018-05-12 |
| Reason for Change/s:* | New potential solution for interworking with identification resolution systems using dedicated physical object resources |
| CR against: Release* | Release 3 |
| CR against: WI* | Active (WI-0069) MNT maintenance / < Work Item number(optional)> Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0044 Version 1.0.0 |
| Clauses * | 8 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution adds a new potential solution for enabling oneM2M platforms to provide support to identification resolution systems by using dedicated passive/tangible object resources with corresponding ID information.

The use case detailed in clause 7.2 indicates that “all information related to the physical entity and the M2M service platform should be compatible with the various identification systems.” For this purpose, having a dedicated resource to represent passive/tangible object will ensure that the necessary passive/tangible object information is available within the system, in addition to the mapping to other/external identification systems. This enables oneM2M platforms to provide, in addition to comprehensive services related to passive/tangible object, an interworking or mapping service with the external systems or between them.

Version R01 addresses comments received at ARC#35.

Version R02 introduces associatedPoIDAssocType.

## ----------------------- Start of change  1 -----------------------

## 8.1	Scenarios analysis based on oneM2M architecture

### 8.1.1	Creation of physical object representation and repository

A oneM2M AE captures physical/passive object using specialized resources (e.g. <physicalObjectResource> type) with information that describes it for use in within the system. These specialized resources may be stored at CSEs in a location associated with the AE or in repositories.

The creation procedure is described at a high level in Figure 8.1.1-1. Various attributes of the specialized resource may be populated at the time of creation or provided via updates. The details related to attribute creation are to be provided with the resource definitions in clause 8.2.

Figure 8.1.1-1 Creation procedures for the <poResource> resource

The oneM2M platform may interwork with external systems or databases based on different identification mechanisms and technologies such as ONS, Handle system, uCodeRP, etc. In order to enable interworking with these systems and access to the associated physical object information, the resource definitions in clause 8.2. provide links to these external resources.

### 8.1.2	Providing associated tangible identifiers and mapping information

A oneM2M AE captures the tangible identifiers (e.g. from tags attaching to a physical entity) and also captures other related information (e.g. location, time). The procedures are shown in Figure 8.1.2-1 and assume that specific attributes of the resource representing the physical object (e.g. <physicalObjectResource>) are used to capture this information.  Assuming the <physicalObjectResource> resources have already been created, the tangible identifiers may be provided via UPDATE operations to the associated physical object resource and operation details are to be provided with the resource definitions in clause 8.2.

Figure 8.1.2-1 Update procedures

- AEs (AE1, AE2) capture the tangible identifiers and potentially other information (e.g. location, time) associated with f the physical entities.

- If  <physicalObjectResource> resources have already been created (e.g. by an inventory application ) the AEs optionally update the resources with the tangible identifiers and identifier type. The AEs may also update other information (e.g. location, time) related to the physical entities.  The AEs may also provide their own communication identifier to be associated with the physical object at  the CSE. Alternatively, the AEs may start by first creating the necessary <physicalObjectResource>

### 8.1.3	Identifier-based discovery of Physical Object  information

When an application wants to discover Physical Object information based on the tangible identifier associated with a <physicalObjectresource> hosted by a CSE 	there are two cases  as follows:

Case 1: When the discovery request response requires only information maintained by the CSE,  as shown in 	Figure 8.1.3-1.

Figure 8.1.3-1 discovery procedures for case 1

- The discovery application AE initiates a request (e.g. to discover the communicationID of a device that interworks to the particular physical entity). The tangible identifier of the physical entity is included in the request.

- The CSE receives and processes the request. If the identifier type is not included in the request, the CSE may recognize the type of tangible identifier from the tangibleID. The CSE resolves the tangible identifier to a <physicalObjectResource>  and discovers the communicationID of the  device that interworks to the physical entity.

- The CSE forwards the discovery results to the  requesting AE.

Case 2:  When the discovery request response additional information which is not maintained  on the queried CSE. The CSE may interwork with external identification	resolution system(s) to assist with the identification resolution. The discovery procedure is shown in figure 8.1.2-2.

Figure 8.1.3-2 discovery procedures for case 2

- The discovery application AE initiates a request (e.g. to discover the the communicationID of a device that interworks to the particular physical entity).  The tangible identifier of the physical entity is included in the request.

- The CSE receives the request. If the identifier type is not included in the request, the CSE may recognize the type of tangible identifier from the tangibleID..

- The CSE sends the tangible identifier (tangibleID) to the corresponding external resolution system (e.g. Handle system),	and receives resolution results from the resolution system. The results may include the communication identifier of a device that interworks to the physical entity.

- The CSE may store the information received locally and responds with the discovery results to the requesting AE.

## 8.2	Potential solutions

This section includes the changes needed in TS-0001 in existing resources, Figures and Tables to support the above scenarios. Impacted sections in TS-0001 will be shown using the applicable section number.

### 9.6.x	Resource Type <physicalObjectResource>

The <physicalObjectResource> resource represents information about a physical object data instances. It is used to share information with other entities and potentially to track the physical object.

Table 9.6.x-1: Child Resources of <physicalObjectResource> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <semanticDescriptor> | [variable] | 0..n | Clause 9.6.30 |
| <subscription> | [variable] | 0..n | Clause 9.6.8 |
| <flexContainer> | [variable] | 0..n | Clause 9.6.34 |

Table 9.6.x-2: Attributes of <physicalObjectResource> resource

| Attributes of <physicalObjectResource> | Multiplicity | RW/ RO/ WO | Description | <physicalObject ResourceAnnc> Attributes |
| --- | --- | --- | --- | --- |
| poID | 0..1 | RW | This attribute is a oneM2M identifier of the physical object | OA |
| tangibleID | 0..1(L) | RW | This parameter is a list of tangible identifiers associated with the specific physical entities. It is described as a list of tangible identifiers (e.g. 86.1000.11/70140703307739, 1.2.156.20000.2010319). | OA |
| IDType | 0..1(L) | RW | - . If the tangibleID parameter is present, it represents the corresponding type of the tangible identifiers. If the tangibleID is not present, it represents the identifier type that is supported by the creation originator. The identifier type is expressed in string: - epc; - handle; - oid; - ucode; - mcode; - ecode. | OA |
| IDServiceCommAddrList | 0..1(L) | RW | It represents the physical address used by devices or applications that that interwork to the physical entity for the purpose of providing tangibleID information. For oneM2M devices, it represents the pointOfAccess attribute of AEs or CSEs. For non-oneM2M devices it represents physical address information (e.g. IP address, FQDN) for such devices, as provided by an external identification system. | OA |
| auxiliaryAEList | 0..1(L) | RW | List of AEs which provide services (e.g. object recognition camera) associated with this physical entity | OA |
| associatedPoIDList | 0..1(L) | RW | A list of associated physical objects | OA |
| associatedPoIDAssocType | 0..1(L) | RW | A list of qualifiers describing the type of association between this Physical Object and each of the Physical Objects in associatedPoIDList, e.g. component-of, sub-component, etc. | OA |
| externalDatabaseList | 0..1(L) | RW | A list of associated external databases containing additional information | OA |
| poDefinition | 0..1 | WO | This contains an identifier reference (URI) to a schema definition which shall be used by the CSE to validate the syntax of the resource, if one or more [customAttributes] are used. If this attribute is not present, no [customAtrribute] specialized attributes are defined. |  |
| [customAttribute] | 0..n | RW | Specialization-specific attribute(s) used to create a specialization of the resource. Name and data type pairs are defined in the the schema provided in the poDefinition attribute. | OA |

- ----------------------- End of change  1 -----------------------