| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | ARC TP35 |
| Source:* | BeiXu，Huawei，Echo.xubei@huawei.com |
| Date:* | 2018-05-07 |
| Contact:* | BeiXu，Huawei，Echo.xubei@huawei.com |
| Reason for Change/s:* | Create the new content of clause 6. |
| CR against: Release* | <Release4> |
| CR against: WI* | Active < WI-0082 - 3GPP V2X Interworking > MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0055-V0_0_1 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

R01: Update the  Figure 6.1-1 and Figure 6.2-1

R02: Change the should/shall normative verbs to can/could non- normative verbs.

### -----------------------Start of change 1-------------------------------------------

# 6	Reference Architecture

## 6.1 oneM2M interworking with 3GPP V2X architecture option1

Figure 6.1-1: oneM2M Interfaces to the 3GPP V2X Service option1

Figure 6.2-1 shows an architecture and functional mapping for the 3GPP Trust Domain which describes how oneM2M functional entities access features and services that are exposed by 3GPP.  The oneM2M entities are shaded in grey.

The “V2X Applications” hosted on the UE can be deployed as follows according to TR-0026[7]:

- Application only: UE could be ADN as of oneM2M entity

- Application and CSE: UE could be ASN or MN as of oneM2M entity

- CSE only: UE could be MN as of oneM2M entity

- Neither application nor CSE: UE could host a NoDN.

The “V2X Control Function” hosts on the IN-CSE as DMG CSF and is located in the Service Layer. IN-CSE can support to communicate with HSS via V4 reference point and communicate with other V2X Control Function in 3GPP Trust Domain via V6 reference point.

The “V2X Application server” is split into three logic functions:

- One function is 3GPP network status & QoS monitoring and report hosted in 3GPP Trust domain. It can communicate with IN-CSE via Vs reference point  according to 3GPP TR 23.795[3].

- One function is 3GPP V2X Device Management, 3GPP V2X MBMS Group, 3GPP V2X Location and 3GPP QoS hosted on the platform could be IN-CSE. 3GPP QoS management is new feature for oneM2M.

- The last function is application function hosted on an external network could be an IN-AE.

The “3GPP Trust Domain” in Figures5.1-1 and Figure 6.1-1 captures the functional entities that MUST be part of the 3GPP domain (the network). Although Figure 6.2-1 shows that the IN-CSE and IN-AE are outside of the 3GPP Domain, the IN-CSE would be part of the operator domain.

This figure also shows the oneM2M reference points Mcn and Mca. For 3GPP PC5 mode, there is PC5 reference point and V5 reference point between the UEs. PC5 reference point is used for user plane for communication for V2X Service of 3GPP which is out of oneM2M scope. V5 reference point is between the V2X service layers in the UE which can be supported in oneM2M scope. To mapping V5 reference point, there can be a new reference point for oneM2M which is the communication flow between oneM2M nodes in filed domain..

6.2 oneM2M 3GPP V2X interworking architecture option2

Figure 6.2-2: oneM2M Interfaces to the 3GPP V2X Service option2

Figure 6.2-2 shows an architecture and functional mapping for the 3GPP Trust Domain which describes how oneM2M functional entities access features and services that are exposed by 3GPP.  The oneM2M entities are shaded in grey. The differences from option1 in clause 6.1 are:

The “V2X Control Function” hosts on the IN-CSE as DMG CSF is located in the Underlying Network using the Mcn reference point which is out of oneM2M scope. oneM2M could support the communication between DMG CSFs via Mcc reference point according to 3GPP V6.

The “V2X Application server” is split into two logic functions:

- One function is 3GPP V2X AS which supports 3GPP network status & QoS monitoring and report hosted in 3GPP Trust domain. It can communicate with IN-CSE via Vs reference point according to 3GPP TR 23.795[3].

- The other function is about 3GPP V2X Device Management, 3GPP V2X MBMS Group, 3GPP V2X Location and 3GPP QoS hosted on the platform could be IN-CSE. 3GPP QoS management is new feature for oneM2M.

-----------------------End of change 1-------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?