| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 36 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2018-07-08 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <WI-0058> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026-V0.9.0 |
| Clauses * | 7.4.2 UE Availability after DDN Failure |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

3GPP TS 29.122, version 2.0.0 published at June 2018,  describes data model for monitoring event API at clause 5.3. Several APIs are defined for monitoring event.

- MonitoringEventSubscription

- MonitoringNotification  which is changed from MonitoringEventNotification at v2.0.0. It proposes to change step 8, step 9, step12 and step 13 at Figure.

- MonitoringEventReport

- idleStatusIndication

The following attributes has been removed at MonitoringEventSubscription message which uses in subscription request and subscription response i.e. IN-CSE sends and receives this message.

- tltrId

- scsAsId

It proposes to remove tltrId and scaAsId at step 2.

The following attributes has been removed at MonitoringNotification message which uses to report from SCEF to SCS, In-CSE.

- tltrId

- subscriptionId

It proposes to remove tltrId and subscriptionId at step 8 and step 12.

### -----------------------Start of change 1-------------------------------------------

### 7.4.2 	UE Availability after DDN Failure

The 3GPP SCEF functionality described in 3GPP TS 23.682 [2] supports APIs for monitoring of events such as UE Availability after a Downlink Data Notification (DDN) Failure. When communicating with UEs which sleep for a long time, if downlink packets are not delivered, the Underlying Network recognizes that the UE is not available by a lack of a response within a reasonable time.

Per this feature, the IN-CSE can subscribe and be notified every time the UE becomes reachable after the network fails to deliver a downlink packet. For example, when this option is set and the IN-CSE receives no response to downlink traffic towards a UE, the IN-CSE can assume that the network failed to deliver the packet because the UE was sleeping and not because of Network Issues. The network later sends a notification to the IN-CSE when the UE becomes reachable.

In the 3GPP Underlying Network, this feature involves an entry in the UE subscription, so it is an ongoing event that needs explicit deletion to cancel further reports and it is different than the UE Reachability Monitoring Request. The feature is particularly suitable when there is just one IN-CSE.

The IN-CSE may also request Idle Status Indication. If Idle Status Indication is supported by the Underlying Network, when the UE transitions into Idle mode, the report includes the time at which the UE transitioned into Idle mode, the active time and the periodic TAU/RAU time granted to the UE.

Figure 7.4.2-1: Procedures for AVAILABILITY_AFTER_DDN_FAILURE monitoring

Step 1: UE Attach and oneM2M Registration Procedures

The UE attaches to the 3GPP network and the ADN-AE(s) or ASN/MN-CSE hosted on the UE perform the oneM2M registration procedure, as detailed in clause 6.5. The IN-CSE hosts the corresponding <AE> or <remoteCSE> resources and an associated <node> resource for the registrees. During this procedure, a <schedule> resource  is created as a child of the <node> resource. The UE hosted ADN-AE(s) or ASN/MN may subscribe to the node <schedule> resource.

If an IN-AE is interested in the reachability status of the UE, it may subscribe to the <schedule> resource if the networkCoordinated attribute of <schedule> resource is set to TRUE (step 1b, optional).

Step 2: IN-CSE sends a MonitoringEventSubscription Request to SCEF to monitor the AVAILABILITY_AFTER_DDN_FAILURE

The MonitoringEventSubscription request from the IN-CSE to SCEF has specified in 3GPP TS 29.122 [5]Error: Reference source not found. The MonitoringEventSubscription request shaln includs:

- URI shall be set to {apiRoot}/3gpp_monitoring_event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- notificationDestination is set to a URI that the SCEF can target Availability after DDN Failure notifications towards.  The value of this URI shall be based on internal IN-CSE policies .

- monitoringType shall be set to AVAILABILITY_AFTER_DDN_FAILURE

- externalId shall be set to the M2M-Ext-ID of the UE

- idleStatusIndication shall be set to TRUE or FALSE. A setting of TRUE will result in the SCEF sending notifications to the IN-CSE when the UE transitions into PSM idle mode. These notifications will be in addition to any notifications sent to the IN-CSE when the UE transitions into connected mode or receives an eDRX paging occasion.  A setting of FALSE (default) will result in the SCEF not sending notifications when the UE transitions into PSM idle mode.  A setting of TRUE is only applicable for UEs supporting PSM.  How the IN-CSE determines whether a UE supports PSM, eDRX or both is out of scope for the current release of this specification and may be configured based on Service Provider and MNO policies.

- externalId shall be set to the M2M-Ext-ID of the UE

NOTE: 	It is recommended that Idle Status monitoring is enabled in conjunction with the Availability after DDN Failure monitoring. This enables the IN-CSE to update the <schedule> resource with updated timing information, once the UE transitions to idle again.

The following parameters are optional:

- maximumNumberOfReports may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.

- monitorExpireTime may be set to an absolute time at which the Availability after DDN Failure monitoring event request is considered to expire.  The IN-CSE shall configure this time based on Service Provider and MNO policies.

Step 3 and 4: MonitoringEventSubscription Request Handling in the Underlying Network

The SCEF handles the MonitoringEventSubscription Request together with the Mobile Core Network, as described in 3GPP TS23.682[2]. The SCEF sends a MonitoringEventSubscription Response message includes a self parameter configured with a URI to the resource created by the SCEF for the MonitoringEventSubscription request.to the IN-CSE with a response code of successful or unsuccessful.

General exceptions for response to unsuccessful are:

- The SCEF returns the following response codes in case of error.

  - Bad Request: If incorrect parameters are included in the request,.

  - Unauthorized: If the IN-CSE is not authorized as described in RFC 7235.

  - Forbidden: If the SCEF is able to understand the request but unable to fulfil the request.

  - Not Found: If the URI specified in the request is incorrect.

  - Conflict: The operation is not allowed given the current status of the resource. This response is returned when the creation of a resource with the same URI already exists; or the change cannot be made due to conflict with current state of the resource.

Step 5: DL Data is sent to the UE

Downlink data is sent to the UE which, in the Underlying Network, involves sending a Downlink Data Notification (DDN) message to the MME, and MME initiates paging of UE.

Step 6a: DDN Failure

The Underlying Network diagnoses a DDN Failure when, after DDN, no response to the UE paging is received and if the UE is in PSM mode (not every DDN failure triggers this event), the UE subscription is updated to reflect that a notification of availability should be sent after this DDN failure.

Step 6b: Request failure handling

The IN-CSE may also diagnose the failure when no response to the DL data has been received.  It is up to implementation if the IN-CSE changes the node <schedule> resource at this point to indicate that communications are not available (e.g. by using a keyword such as “NULL”), or just records it as a one-time response failure. Other alternatives for implementation are, for example, for the IN-CSE to change the node <schedule> resource to indicate that communications are not available only after a pre-provisioned number of requests fail. The IN-CSE may also buffer future requests or enlarge its buffer size.

Step 7: Available after DDN Failure

At a later time, the UE contacts the network, e.g., to perform a TAU, or as it executes a service request. The Underlying Network notes that the UE is available and that an availability notification is requested in the subscription. A Monitoring Indication that the UE is available is sent to the SCEF.

Step 8: SCEF sends a MonitoringNotification Report to IN-CSE for AVAILABILITY_AFTER_DDN_FAILURE

When the SCEF receives information of status change to available communicatetion in step 7, the SCEF  creates and sends a MonitoringNotification message for AVAILABILITY_AFTER_DDN_FAILURE to the IN-CSE as specified in 3GPP TS 29.122 [5].

Otherwise, the MonitoringNotification message is sent with appropriate information in accordance with the monitoringType.

The MonitoringNotification report for AVAILABILITY_AFTER_DDN_FAILURE shall include the following:

- subscription configured with a link to the subscription resource for which this notification corresponds to

- monitoringEventReport configured with one or more AVAILABILITY_AFTER_DDN_FAILURE monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [5]:

  - monitoringType configured with AVAILABILITY_AFTER_DDN_FAILURE

  - idleStatusInfo configured with information such as activeTime, edrxCycleLength, suggestedNumberOfDlPackets, idleStatusTimestamp, periodicAUTimer as defined in 3GPP TS 29.122 [5]

  - externalIDs configured with the same value of externalId as in the resuest at step 2.

Step 9: MonitoringNotification Response for AVAILABILITY_AFTER_DDN_FAILURE

- After receiving a MonitoringNotification for AVAILABILITY_AFTER_DDN_FAILURE, the IN-CSE returns a response having a response code based on the checked resuls.

Step 10a and 10b: Availability after DDN Failure Handling at the IN-CSE

The IN-CSE uses the information provided in the MonitoringNotification Report to update the node <schedule> to indicate that the UE is available.

A new scheduleElement shall be created that indicates that the node is available for communication starting immediately and will remain available for at least the duration specified by the Maximum Response Time parameter.

Later (step 10b, optional) IN-AE(s) or the IN-CSE may decide to resend data queued for the UE.

Step 11: UE transitions to Idle

UE transitions to idle mode. If Idle Status Indication was requested during MonitoringEventSubscription configuration, and the MCN supports Idle Status Indication, then the SCEF is provided with an indication which includes the time at which the UE transitioned into idle mode, the active time and the periodic TAU/RAU timer values.

Step 12: SCEF sends a MonitoringNotification Report to IN-CSE for AVAILABILITY_AFTER_DDN_FAILURE

When the SCEF receives information of status change to unavailable communicatetion in step 11, the SCEF  creates and sends a MonitoringNotification message for AVAILABILITY_AFTER_DDN_FAILURE to the IN-CSE as specified in 3GPP TS 29.122 [5].

Otherwise, the MonitoringNotification message is sent with appropriate information in accordance with the monitoringType.

The MonitoringNotification report for AVAILABILITY_AFTER_DDN_FAILURE includes the following:

- subscription configured with a link to the subscription resource for which this notification corresponds to

- monitoringEventReport configured with one or more AVAILABILITY_AFTER_DDN_FAILURE monitoring reports wherein each report includes the following fields:

  - monitoringType configured with AVAILABILITY_AFTER_DDN_FAILURE

  - idleStatusInfo configured with information such as activeTime, edrxCycleLength, suggestedNumberOfDlPackets, idleStatusTimestamp, periodicAUTimer as defined in 3GPP TS 29.122 [5]

  - externalID configured with the same value of externalId as in the resuest at step 2.

Step 13: MonitoringNotification Response

- After receiving a MonitoringNotification for AVAILABILITY_AFTER_DDN_FAILURE, the IN-CSE returns a response having a response code based on the checked results.

Step 14: UE Idle mode indication Handling at the IN-CSE

The IN-CSE uses the UE Idle status information to update the <schedule> child resource of the <node > resource.

The IN-CSE changes the <schedule> resource such that:

- the start of the scheduleElement is based on the Idle Timestamp, with a periodicity equal to the TAU/RAU Timer.

- the duration of the scheduleElement indicates the Active Time value.

When any traffic is received from the node or an Availability Notification is received for the node, any scheduleElement(s) that were created based on prior Idle Status Indications shall be deleted for the node. A new scheduleElement shall be created that indicates that the node is available for downlink communication starting immediately and will remain available for at least the duration specified by the Maximum Response Time parameter.

Step 15 (Optional): IN-CSE sends request to SCEF to delete MonitoringEventSubscription

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this MonitoringEventSubscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE or when the networkCoordinated attribute of the <schedule> resource affiliated with an ADN-AE or ASN/MN-CSE hosted on a UE is set to FALSE.

Step 16 and 17 (Optional): Response to delete MonitoringEventSubscription

The SCEF handles the request to delete MonitoringEventSubscription together with the 3GPP network entities. The SCEF responds to the IN-CSE with a response either a code of 200 OK or 204 NO CONTENT indicating that it has successfully deleted the MonitoringEventSubscription.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?