| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 36 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2018-07-08 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <WI-0058> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026-V0.9.0 |
| Clauses * | 7.4.3 UE Communication Failure |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

3GPP TS 29.122, version 2.0.0 published at June 2018,  describes data model for monitoring event API at clause 5.3. Several APIs are defined for monitoring event.

- MonitoringEventSubscription

- MonitoringNotification  which is changed from MonitoringEventNotification at v2.0.0. It proposes to change step 6 and step 7 at Figure.

- MonitoringEventReport

- FailureCause

The following attributes has been removed at MonitoringEventSubscription message which uses in subscription request and subscription response i.e. IN-CSE sends and receives this message.

- tltrId

- scsAsId

It proposes to remove tltrId and scaAsId at step 2.

The following attributes has been removed at MonitoringNotification message which uses to report from SCEF to SCS, In-CSE.

- tltrId

- subscriptionId

It proposes to remove tltrId and subscriptionId at step 6.

### -----------------------Start of change 1-------------------------------------------

### 7.4.3 	UE Communication Failure

In the context of 3GPP Interworking, the IN-CSE enables communications between IN-AEs and 3GPP UEs. Informing the IN-CSE that UEs have suffered communication failures in the Underlying Network helps optimize communications. For example, the IN-CSE may stop attempting to communicate with the UE if it is aware of repeated communication failures. The SCEF enables Communication Failure monitoring at the IN-CSE per 3GPP TS 23.682 [2].

The UE Communication Failure Monitoring flow is assumed to take place after the UE has attached to the 3GPP Network and registered with the IN-CSE.

Figure 7.4.3 -1: Procedure for COMMUNICATION_FAILURE monitoring

Step 1: UE Attach and oneM2M Registration Procedures

The UE attaches to the 3GPP network and the ADN-AE(s) or ASN/MN-CSE hosted on the UE perform the oneM2M registration procedure, as detailed in clause 6.5. The IN-CSE hosts the corresponding <AE> or <remoteCSE> resources and an associated <node> resource for the registree. During this procedure, a <schedule> resource is created as a child of the <node> resource. The UE hosted ADN-AE(s) or ASN/MN may subscribe to the node <schedule> resource.

If an IN-AE is interested in the communication failure status of the UE, it may subscribe to the node <schedule> resource (step 1b, optional).

Step 2: IN-CSE sends a MonitoringEventSubscription Request to SCEF to monitor the COMMUNICATION_FAILURE

This step may be triggered by an IN-CSE based on implementation options, for example when a type of communication requiring high reliability is scheduled to occur. The creation of one or more subscriptions to the <schedule> (step 1b, optional) may also be used to trigger the monitoring request if the networkCoordinated attribute of <schedule> resource is set to TRUE.

The MonitoringEventSubscription request from the IN-CSE to SCEF has specified in 3GPP TS 29.122 [5]. The MonitoringEventSubscription shall include:

- URI shall be set to {apiRoot}/3gpp_monitoring_event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- notificationDestination is set to a URI that the SCEF can target UE Communication Failure notifications towards.  The value of this URI shall be based on internal IN-CSE policies .

- monitoringType shall be set to COMMUNICATION_FAILURE

- externalId shall be set to M2M-Ext-ID of the UE

- The following parameters are optional:

- maximumNumberOfReports may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.

- monitorExpireTime may be set to an absolute time at which the Availability after DDN Failure monitoring event request is considered to expire.  The IN-CSE shall configure this time based on Service Provider and MNO policies.

- ipv4Addr and/or ipv6Addr may be set to identifiy for UE to monitor communicature failure at network level.

Step 3 and Step 4: MonitoringEventSubscription Request Handling in the Underlying Network

The SCEF handles the MonitoringEventSubscription Request together with the 3GPP network entities, as described in 3GPP TS 29.122[5]. The SCEF sends a MonitoringEventSubscription Response message to the IN-CSE with a response that contains information as specified in 3GPP TS 29.122 [5].

In addition to parameters included in the request, the response also includes a self parameter configured with a URI to the resource created by the SCEF for the MonitoringEventSubscription request.  This URI will have a structure of {apiRoot}/3gpp_monitoring_event/v1/{scsAsId}/subscriptions/{subscriptionId}.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies and the {subscriptionId} segment is configured by the SCEF.

General Exceptions:

- The SCEF returns the following response codes in case of error.

  - Bad Request: If incorrect parameters are included in the request,.

  - Unauthorized: If the IN-CSE is not authorized as described in RFC 7235.

  - Forbidden: If the SCEF is able to understand the request but unable to fulfil the request.

  - Not Found: If the URI specified in the request is incorrect.

  - Conflict: The operation is not allowed given the current status of the resource. This response is returned when the creation of a resource with the same URI already exists; or the change cannot be made due to conflict with current state of the resource.

Step 5: Detection of COMMUNICATION_FAILURE and reporting to SCEF

Later, when the UE communication failure occurs the condition is detected in the Underlying Network and the SCEF receives a Monitoring Event Report.

Step 6: SCEF sends MonitoringNotification Report to IN-CSE for COMMUNICATION_FAILURE

When the SCEF receives information of status change in step 5, the SCEF sends a MonitoringNotification Report to the IN-CSE for UE communication failure as specified in 3GPP TS 29.122 [5]Error: Reference source not found.

Otherwise, the MonitoringNotification message is sent with appropriate information in accordance with the monitoringType.

The MonitoringNotification report for COMMUNICATION_FAILURE shall include the following:

- subscription configured with a link to the subscription resource for which this notification corresponds to

- monitoringEventReport configured with one or more UE communication failure monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [5]:

  - monitoringType configured with COMMUNICATION_FAILURE

  - failureCause configured with a reason of communication failure such as bssgpCause, causeType, gmmCause, ranapCause, ranNasCause, s1ApCause and smCause.

  - externalIDs configured with the same value of externalId as in the rquest at step 2.

Step 7: MonitoringNotification Response for COMMUNICATION_FAILURE

After receiving a MonitoringNotification for COMMUNICATION_FAILURE, the IN-CSE returns a response having a response code based on the checked results.

Step 8: COMMUNICATION_FAILURE Handling at the IN-CSE

The IN-CSE uses the information provided in the Monitoring Notification Report to update the scheduleElement of the <schedule> resource to indicate that no communications are currently available (e.g. by using a keyword such as “NULL”).  Local IN-CSE policies may specify events/ thresholds further defining when the IN-CSE may provide the <schedule> resource update. For example, the update may be provided only after repeated communication failures are received within a timespan, or only if high reliability communications are expected.  It is recommended that UE Reachability monitoring is also enabled in conjunction with the Communication Failure monitoring. This enables the IN-CSE to provide updated timing information in the <schedule> resource, once the UE becomes reachable again.

Step 9: (Optional) Notifying subscribers

Optionally, notifications may send to the subscribers of the <schedule> resource.

Step 10 (Optional): IN-CSE sends request to SCEF to delete MonitoringEventSubscription

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this MonitoringEventSubscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE or when the networkCoordinated attribute of the <schedule> resource affiliated with an ADN-AE or ASN/MN-CSE hosted on a UE is set to FALSE.

Step 11 and 12 (Optional): Response to delete MonitoringEventSubscription

The SCEF handles the request to delete MonitoringEventSubscription together with the 3GPP network entities. The SCEF responds to the IN-CSE with a response either a code of 200 OK or 204 NO CONTENT indicating that it has successfully deleted the MonitoringEventSubscription.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?