| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 36 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2018-07-08 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <WI-0058> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026-V0.9.0 |
| Clauses * | 7.4.5 Detecting Change of IMSI-IMEI(SV) Association |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

3GPP TS 29.122, version 2.0.0 published at June 2018,  describes data model for monitoring event API at clause 5.3. Several APIs are defined for monitoring event.

- MonitoringEventSubscription

- MonitoringNotification  which is changed from MonitoringEventNotification at v2.0.0. It proposes to change step 6 and step 7 at Figure.

- MonitoringEventReport

- associationType

The following attributes has been removed at MonitoringEventSubscription message which uses in subscription request and subscription response i.e. IN-CSE sends and receives this message.

- tltrId

- scsAsId

It proposes to remove tltrId and scaAsId at step 2 and step 4.

The following attributes has been removed at MonitoringNotification message which uses to report from SCEF to SCS, In-CSE.

- tltrId

- subscriptionId

It proposes to remove tltrId and subscriptionId at step 6 and step 7.

### -----------------------Start of change 1-------------------------------------------

### 7.4.5	Detecting Change of IMSI-IMEI(SV) Association

The 3GPP SCEF Event Monitoring functionality described in 3GPP TS 29.122 [5] supports an API that allows the IN-CSE to be informed when the SIM card of one physical device is placed in another physical device.  This condition is detected by the underlying 3GPP network when the association between the International Mobile Subscriber Identity (IMSI) and International Mobile Equipment Identity (IMEI/IMEISV) changes.

An IN-CSE may request to receive notifications from an underlying 3GPP network when the association between the IMSI and IMEI(SV) changes for a given UE that hosts one or more ASN/MN-CSEs or ADN-AEs registered to the IN-CSE.  Based on this notification, the IN-CSE may then ignore incoming requests from these ASN/MN-CSEs or ADN-AEs.

Figure 7.4.5-1: Change of IMSI-IMEI(SV) Association Monitoring Request and Notification

Pre-conditions:

There is a relationship in place between the IN-CSE and MNO allowing the IN-CSE to request notifications for IMSI-IMEI(SV) association changes.

An ASN/MN-CSE or ADN-AE registers with the IN-CSE and configures the M2M-Ext-ID attribute of its <remoteCSE> or <AE> resource. The IN-CSE examines the M2M-Ext-ID and recognizes that it is associated with an MNO that it has a relationship with.  The relationship allows the IN-CSE to request notifications when the device’s IMSI-IMEI(SV) association changes.

Step 1: IN-CSE determines to monitor the CHANGE_OF_IMSI_IMEI_ASSOCIATION

The IN-CSE determines whether it wants a notification if the IMSI-IMEI(SV) association of a particular UE hosting one or more ASN/MN-CSEs or ADN-AEs changes.  This determination may be based on whether the IN-CSE has an established relationship with the underlying 3GPP network operator that supports this capability and provisioned policies.

Step 2: Request for MonitoringEventSubscription to monitor the CHANGE_OF_IMSI_IMEI_ASSOCIATION

The IN-CSE sends a request to monitor MonitoringEventSubscription which has specified in 3GPP TS 29.122 [5] when the device’s IMSI-IMEI(SV) association changes.  The MonitoringEventSubscription to monitor CHANGE_OF_IMSI_IMEI_ASSOCIATION shall include:

- URI shall be set to {apiRoot}/3gpp_monitoring_event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- notificationDestination is set to a URI that the SCEF can target Change of IMSI/IMEI notifications towards.  The value of this URI shall be based on internal IN-CSE policies .

- monitoringType shall be set to CHANGE_OF_IMSI_IMEI_ASSOCIATION.

- associationType shall be set to IMEI or IMEISV.  The type that is used is based on IN-CSE policies which may be based on the relationship between the Service Provider and MNO.

- externalId shall be set to the M2M-Ext-ID of the UE hosting an ADN-AE or ASN/MN-CSE, which may have been provided during ADN-AE or ASN/MN-CSE registration.

- The following parameters are optional:

- maximumNumberOfReports may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.

- monitorExpireTime may be set to an absolute time at which the CHANGE_OF_IMSI_IMEI_ASSOCIATION MonitoringEventSubscription request is considered to expire.  The IN-CSE shall configure this time based on Service Provider and MNO policies..

Step 3: Handling MonitoringEventSubscription

The SCEF processes the MonitoringEventSubscription Request together with the 3GPP network entities as described in 3GPP TS 29.122 [5].

Step 4: Response for MonitoringEventSubscription to monitor the CHANGE_OF_IMSI_IMEI_ASSOCIATION

The SCEF sends a MonitoringEventSubscription Response message to the IN-CSE with a response that contains information as specified in 3GPP TS 29.122 [5].

In addition to parameters included in the request, the response also includes a self parameter configured with a URI to the resource created by the SCEF for the MonitoringEventSubscription request.  This URI will have a structure of {apiRoot}/3gpp_monitoring_event/v1/{scsAsId}/subscriptions/{subscriptionId}.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies and the {subscriptionId} segment is configured by the SCEF.

General Exceptions:

- The SCEF returns the following response codes in case of error.

  - Bad Request: If incorrect parameters are included in the request,.

  - Unauthorized: If the IN-CSE is not authorized as described in RFC 7235.

  - Forbidden: If the SCEF is able to understand the request but unable to fulfil the request.

  - Not Found: If the URI specified in the request is incorrect.

  - Conflict: The operation is not allowed given the current status of the resource. This response is returned when the creation of a resource with the same URI already exists; or the change cannot be made due to conflict with current state of the resource.

Step 5: Detect change in IMSI-IMEI(SV) association

The underlying 3GPP network monitors and detects a IMSI-IMEI(SV) association change and reports Monitoring Event Report to SCEF.

Step 6: MonitoringNotification Reporrt to IN-CSE f CHANGE_OF_IMSI_IMEI_ASSOCIATION

When the SCEF receives information of status change in step 5, the SCEF sends a MonitoringNotification Report to the corresponding notificationDestination  of the IN-CSE that was configured in the MonitoringEventSubscription Request and that contains information as specified in 3GPP TS 29.122 [5]Error: Reference source not found.

Otherwise, the MonitoringNotification message is sent with appropriate information in accordance with the monitoringType.

The MontoringNotification report for CHANGE_OF_IMSI_IMEI_ASSOCIATION shall include:

- subscription configured with a link to the subscription resource for which this notification corresponds to

- monitoringEventReports configured with one or more change of IMSI-IMEI association monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [5]:

  - monitoringType configured with CHANGE_OF_IMSI_IMEI_ASSOCIATION

  - associationType configured with IMEI or IMEISV

  - externalIDs configured with the same value of externalID as in the request at step 2.

Step 7: MonitoringNotification Response for CHANGE_OF_IMSI_IMEI_ASSOCIATION

After receiving a Change of IMSI-IMEI Association MonitoringNotification, the IN-CSE returns a response having a response code based on the checked results.

Step 8: CHANGE_OF_IMSI_IMEI_ASSOCIATION handling at the IN-CSE

The IN-CSE shall stop servicing requests from any ASN/MN-CSEs or ADN-AEs having an M2M-Ext-ID matching the one indicated in the report.   To block these requests, the IN-CSE may tear down any active security associations (e.g. D/TLS sessions) between the IN-CSE and these ADN-AEs or ASN/MN-CSEs.  In addition, the IN-CSE may also deny new security association establishment requests from any ADN-AEs or ASN/MN-CSEs that have an M2M-Ext-ID matching the one indicated in the report until the restriction is removed via administrative means which are outside the scope of the present document.

Step 9 (Optional): Request to delete MonitoringEventSubscription  for CHANGE_OF_IMSI_IMEI_ASSOCIATION

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this MonitoringEventSubscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE or when the networkCoordinated attribute of the <schedule> resource affiliated with an ADN-AE or ASN/MN-CSE hosted on a UE is set to FALSE.

Step 10 and 11 (Optional): Response to delete MonitoringEventSubscription

The SCEF handles the request to delete MonitoringEventSubscription together with 3GPP network entities. The SCEF responds to the IN-CSE with a response either code of 200 OK or 204 NO CONTENT indicating that it has successfully deleted  the MonitoringEventSubscription.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?