| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 36 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2018-07-08 |
| Reason for Change/s:* | Create new content of Annex A |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <WI-0058> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026-V0.9.0 |
| Clauses * | Annex A Check for Monitoring Events |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The clause 7.4 describes monitoring events.

In MonitoringNotification aspects, the SCEF needs to check correct information of detected event from 3GPP network entities with montoringType which may be mixed at the SCEF. If the detected information is consistent with monitoringType, SCEF can proceed for further processing. Otherwise, the MonitoringEventNotification message is created with appropriate information in accordance with the monitoringType.

### -----------------------Start of change 1-------------------------------------------

Annex A (informative): Check for Monitoring Events

# A.1	General Concepts

A monitoring events provides functions that monitors LOSS_OF_CONNECTIVITY, UE_REACHABILITY, LOCATION_REPORTING, CHANGE_OF_IMSI_IMEI_ASSOCIATION, ROAMING_STATUS, COMMUNICATION_FAILURE, AVAILABILITY_AFTER_DDN_FAILURE and NUMBER_OF_UES_IN_AN_AREA

The procedures for each monitoring events are similar to use API which is MonitoringEventSubscription and MonitoringNotification. The basic procedures are:

- An IN-CSE, plays a SCS, requests MonitoringEventSubscription to a SCEF.

- A SCEF responds MonitoringEventSubscription to the IN-CSE.

- 3GPP network entities detect monitoring event and the SCEF receives detected information from the 3GPP network entities.

- The SCEF report MonitoringNotification to IN-CSE.

The information on MonitoringNotification API depends on monitoringType attribute in MonitoringEventSubscription API. Therefore, the information on MonitoringNotification should be consistent with monitoringType.

# A.2	Check for each Monitoring Events

This clause provides example of check for monitoring events which is consistent with monitoringType.

## A.2.1	Loss of connectivity

## A.2.2	UE Reachability

## A.2.3	Location Reporting

## A.2.4	Change of IMSI IMEI Association

## A.2.5	Roaming Status

## A.2.6	Communication Failure

## A.2.7	Availability after DDN Failure

# A.3	Check for Monitoring Events in holistic view

This clause provides example of check for monitoring events which is consistent with monitoringType holistic view..

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?