| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#36 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com |
| Date:* | 2018-06-25 |
| Reason for Change/s:* | The CR is to update node management from TS-0004 to TS-0026. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0058 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 |
| Clauses * | 7.9 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0026 v0.9.0 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Update the procedure according to the Annex B.2 Node Schedule Management of TS-0004.

### -----------------------Start of change 1-------------------------------------------

## 7.13 	Node Schedule Management

The <schedule> resource in oneM2M contains scheduling information. The usage of the <schedule> resource is different depending on the associated resource type, as follows:

- A child <schedule> resource of the <node> resource shall indicate the time periods when the node can communicate via the Underlying Network. If multiple Underlying Networks are supported, for each there can be a maximum of one <schedule> resource. One <schedule> resource may be used for multiple Underlying Networks.

The mgmtLink attribute of the <cmdhNwAccessRule> child resource of a <node> resource shall link to a <schedule> resource that is also a child of the same <node> resource.

In the context of 3GPP connectivity technologies, per 3GPP TS 23.682 [2Error: Reference source not found], the network reachability and UE reachability are both indications that the UE becomes reachable for receiving either an SMS or downlink data. The SCEF supports the capability to notify the IN-CSE of the network reachable status or the UE reachable status. The IN-CSE shall maintain a <schedule> resource of a UE and if the networkCoordinated attribute of the <schedule> is set to TRUE, then the IN-CSE shall coordinate the schedule based on the UE’s reachability. For example, the IN-CSE shall support synchronizing the start time of the scheduleElement attribute to be the same as the start time of the targeted UE idle status which the IN-CSE receives from the Underlying 3GPP Network.

Refer to the clause 9.6.9 Resource Type <schedule> of TS-0001[1]

Figure 7.13-1: Targeting UE based on the <schedule> of <node> resource

Pre-requisites:

- The <schedule> resource is a child resource of a <node> resource, and the networkCoordinated attribute of <schedule> is ‘True’

- If a request or response message that is targeting an entity or a resource in the 'to' parameter that is ASN/MN-CSE or ADN-AE hosted on a UE which has M2M-Ext-ID attribute of <remodeCSE> or <AE> resource, this processing shall be carried out

Step 1: The IN-AE sends a request or response message to the IN-CSE which includes the target resource identify of an ASN/MN-CSE or ADN-AE hosted on a UE.

Step 2: IN-CSE checks the local <schedule> of the targeted ASN/MN-CSE or ADN-AE node which indicates the pre-defined reachability schedule information of the targeted node  according to the target resource indentify . The IN-CSE also checks if the networkCoordinated attribute of the <schedule> resource is TRUE, to determine whether the schedule is coordinated with the UE’s underlying network schedule.  Based on this information, the IN-CSE determines whether the UE is available to receive a request or response message at the current time.

Case A: if the current time is within the period of <schedule>, then this indicates the target ASN/MN-CSE’s or ADN-AE’s current status is reachable and the following steps are applicable:

- Step 3a: IN-CSE sends the message to ASN/MN-CSE or ADN-AE directly after CMDH message processing is successful as specified in Annex H in TS-0004[4]; and then continues with the step 4).

- Step 4a: The ASN/MN-CSE or ADN-AE sends the response message to the IN-CSE if the message in Step 1 is a request;

- Step 5a: IN-CSE sends response message to the IN-AE if the message in Step 1 is a request;

Case B: if the current time is not in the period of <schedule>, then this indicates the target ASN/MN-CSE’s or ADN-AE’s current status is unreachable. The IN-CSE calculates the next reachable start time based on the <schedule>, and checks if the Operation Execution Time or the Request Expiration Timestamp in the IN-AE request message is earlier than the next reachable start time, or if the Operation Execution Time or Request Expiration Timestamp is not configured in the request.  If this is the case, then go to Step 3b,

- Step 3b: IN-CSE sends error response message to the IN-AE which indicates that the request cannot be delivered to the target ASN/MN-CSE or ADN-AE:

If the Operation Execution Time and Request Expiration Timestamp are not configured, the Response Status Code should be 6003 in the Table 6.6.3.7-1 of TS-0004[4].

If the Request Expiration Timestamp is configured, and Request Expiration Timestamp is earlier than the next reachable start time, the Response Status Code should be 6030 in the Table 6.6.3.7-1 of TS-0004[4].

If the Operation Execution Time are configured, the Operation Execution Time is earlier than the next reachable start time, the Response Status Code should be 6031 in the Table 6.6.3.7-1 of TS-0004[4].

Case C: if the current time is not in the period of <schedule>, then this indicates the target ASN/MN-CSE’s or ADN-AE’s current status is unreachable. The IN-CSE calculates the next reachable start time based on the <schedule>, and checks if and the Operation Execution Time and Request Expiration Timestamp in the IN-AE message are both in the period of <schedule>. For example, later than the next reachable start time and earlier than the next end time.  If this is the case, then go to Step 3c:

- Step 3c: IN-CSE buffers the message until the ASN/MN-CSE or ADN-AE is reachable again.

- Step 4c: IN-CSE forwards the message to the target ASN/MN-CSE or ADN-AE before the Operation Execution Time and Request Expiration Timestamp expire during the next reachable time after CMDH message processing is successful as specified in Annex H in TS-0004[4]; and then continues with the step 5c).

- Step 5c: the ASN/MN-CSE or ADN-AE sends the response message to the IN-CSE if the message in Step 1 is a request;

- Step 6c: IN-CSE sends the response message to the IN-AE if the message in Step 1 is a request.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?