| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | ARC#36 |
| Source:* | Convida Wireless |
| Date:* | 2017-07-09 |
| Contact:* | Catalina Mladin, Convida, Mladin.Catalina@convidawireles.com |
| Reason for Change/s:* | Introduces Key Issue for Simultaneous registration with multiple entities |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0046> MNT Maintenance / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0026 |
| Clauses/Sub Clauses* | Clause 9 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution provides further analysis of clause 6 and identifies a Key Issue for the Vehicular Domain related to registration to multiple entities simultaneously.

- -----------------------Start of change 1-------------------------------------------

## 9.Y Key Issue XYZ: Simultaneous registration with multiple entities

Clause 6.5 concludes with several requirements related to registration:

  - The M2M System shall provide the capability for an M2M device to maintain registration with multiple entities simultaneously.

  - The registration shall be able to include information that identifies the peer entity, and other information necessary for the establishment of the respective peer relationships (e.g. management privilege, subscription).

  - The M2M System shall be able to hold the complete set of information context about the peer entity for some registrations (i.e. "full registration").

  - The M2M System shall be to hold only a subset of information context about the peer entity for some registration (i.e. "lightweight registration").

  - The M2M System shall be able to perform "lightweight registration" at different entities pertaining to a common peer entity and to hold different sets of information about the common peer entity.

  - The M2M System shall be able to correlate the "full registration" and the "lightweight registration" pertaining to a common peer entity.

  - The M2M System shall be possible to distinguish the "full registrations" and the "lightweight registrations" pertaining to a common peer entity.

Further work in clauses 6.19-6.21 considers cases based on Edge/Fog deployments, and as such assume the possibility for nodes to receive services from peer nodes, in addition to hierarchies like the ones created by single registration relationships.

Registrations are performed in order to obtain services from a Service Layer. In a Release 3 implementation after registering, an AE or CSE receives all services it is authorized for from its registrar. Therefore, allowing multiple registrations requires that registration scope (i.e. the services to be provided by each registrar) to be defined. Otherwise multiple registrations of the same entity will result in resources from all registrars to be allocated/ planned, when in reality certain services might be provided only by a single registrar.  This results in the following issue:

- Support of multiple registrations for a single entity may introduce unnecessary resource allocations at the registrars, unless the registration procedure indicates which services are requested or granted, and the specific combinations of services to be provided by each registrar.

The use of multiple registrations of the same entity simultaneously (in Edge/Fog deployments ) has also been assumed in clause 6.19 in establishing requirements for reliability. Multiple registrations can ensure reliable services registration states by using registration states, e.g. active/inactive. This results in the following issue:

- The system needs to maintain registration status (e.g. active/inactive) along with context, in order to manage registrations in a multiple-registration system.

- e AE, in order e monitoring ger relevant.n time associated with ther regisratoion.hop around for another credi -----------------------End of change 1 ------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause?