| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #36 |
| Source:* | Dale Seed, Convida, Seed.Dale@convidawireless.com |
| Date:* | 2017-07-09 |
| Reason for Change/s:* | See Introduction |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS0001 v3_11_0 |
| Clauses * | Table 9.6.49-1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Currently in TS-0001, Table 9.6.49-1 mentions that the triggerValidityTime attribute of <triggerRequest> resource multiplicity as 0..1. However, Table 7.4.57.1-1 of  TS-0004 defines the triggerValidityTime attribute of <triggerRequest> resource as M(Mandatory) in Create operation.

The validityPeriod attribute of the DeviceTrigger API defined in 29.122 defines the cardinality as 1 for the Device Trigger procedure. The triggerValidityTime attribute maps to the validtyPeriod parameter over T8 interface.

This document proposes to change the multiplicity of the triggerValidityTime attribute set to be 1.

### -----------------------Start of change 1-------------------------------------------

### 9.6.49	Resource Type triggerRequest

The <triggerRequest> resource is used to initiate a device trigger request.  This resource type shall only be instantiated on an IN-CSE.

The successful creation of a <triggerRequesst> resource results in the IN-CSE initiating a trigger request to a targeted device (e.g. 3GPP UE).   A pending trigger request can be replaced with a new trigger request by updating the <triggerRequesst> resource.  A pending trigger request can be cancelled by deleting the <triggerRequesst> resource.

The <triggerRequest> resource shall contain the child resource specified in table 9.6.49-1.

Table 9.6.49-1: Child resources of <triggerReqeust> resource

| Child Resources of <trafficPattern> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 of TS-0001 |

The <triggerRequest> resource shall contain the attributes specified in table 9.6.49-2.

Table 9.6.49-1: Attributes of <triggerRequest> resource

| Attributes of <triggerRequest> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| Labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| M2M-Ext-ID | 1 | WO | M2M External Identifier of the device being triggered. See clause 7.1.8. This attribute shall be configured by the Originator when the resource is created. |
| Trigger-Recipient-ID | 1 | RW | Trigger-Recipient-ID of the ASN/MN-CSE or ADN-AE that is hosted on the device being triggered. See clause 7.1.10. This attribute shall be configured by the Originator when the resource is created and may also be updated when performing a trigger replace procedure. See clause 8.3.3.2.2. |
| triggerPurpose | 1 | RW | The purpose of the trigger. See clause 8.3.3.2.1. This attribute may be configured by the Originator when the resource is created and may also be updated when performing a trigger replace procedure. The allowed values are: - establishConnection - enrolmentRequest - registrationRequest - executeCRUD If not specified by the Originator, the default is "establishConnection". |
| triggerPayloadSerialization | 1 | RW | The type of serialization used to encode the trigger payload (i.e. XML, JSON or CBOR). This attribute may be configured by the Originator when the resource is created and may also be updated when performing a trigger replace procedure. See clause 8.3.3.2.2. If not specificied by the Originator, the default is "JSON". |
| triggerStatus | 1 | RO | The status of the trigger request. The Hosting CSE shall control the value of this attribute. The following values are valid values. - PROCESSING - ERROR-NSE-NOT-FOUND - TRIGGER-SUBMITTED - TRIGGER-DELIVERED - TRIGGER-FAILED |
| triggerValidityTime | 1 | RW | The time duration for which the trigger request is valid. After this time expires, the trigger shall be recalled (i.e. cancelled) by the Hosting CSE. This attribute may be configured by the Originator when the resource is created and may also be updated when performing a trigger replace procedure. See clause 8.3.3.2.2. |
| triggerInfoAE-ID | 0..1 | RW | When the triggerPurpose is “executeCRUD”, this attribute is mandatory otherwise it is not applicable. This attribute is configured with the AE-ID of the ASN/MN-AE that should perform the CRUD operation. When this attribute is configured, the trigger originator shall also configure the triggerInfoAddress, triggerInfoOperation and targetedResouceType attributes. |
| triggerInfoAddress | 0..1 | RW | When the triggerPurpose is “executeCRUD”, this attribute is mandatory otherwise it is not applicable. This attribute is configured with an unstructured CSE-Relative-Resource-ID of the resource that the ASN/MN-AE should perform the CRUD operation on. When this attribute is configured, the trigger originator shall also configure the triggerInfoAE-ID, triggerInfoOperation and targetedResouceType attributes. |
| triggerInfoOperation | 0..1 | RW | When the triggerPurpose is “executeCRUD”, this attribute is mandatory otherwise it is not applicable. This attribute is configured with the CRUD operation that the ASN/MN-AE should perform on the targeted resource specified by triggerInfoAddress. When this attribute is configured, the trigger originator shall also configure the triggerInfoAE-ID, triggerInfoAddress and targetedResouceType attributes. |
| targetedResourceType | 0..1 | RW | When the triggerPurpose is “executeCRUD”, this attribute is mandatory otherwise it is not applicable. This attribute is configured with the resource type of the targeted resource specified by triggerInfoAddress. When this attribute is configured, the trigger originator shall also configure the triggerInfoAE-ID, triggerInfoAddress and triggerInfoOperation attributes. |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?