| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | ARC 36 |
| Source:* | BeiXu, Huawei, Echo.xubei@huawei.com Can Zhao, Huawei, zhaocan20@huawei.com |
| Date:* | 2018-07-16 |
| Contact:* | BeiXu，Huawei，Echo.xubei@huawei.com |
| Reason for Change/s:* | Add a new KI and a related solution for TR-0026 |
| CR against: Release* | <Release> Only ONE Release shall be indicated |
| CR against: WI* | Active < WI-0046: Vehicular Domain Enablement > MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0026-Vehicular_Domain_Enablement-V4_0_0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Add a new solution for TR-0026.

### -----------------------Start of change 1-------------------------------------------

## 10.x Solution H: V2V network interface switching management

### 10.x.1 Solution Description

In order to address the Key Issue 8, we can rely on the IN-CSE to plan communication mode for V2V communication in platoon according to platoon information, network status information and map information. The basic procedure is described as below:

Step1: an ADN-AE, or a MN/ASN-CSE that is hosted on a vehicle registers to the IN-CSE, and creates its supportCommunicationMode  and routeAreaInformation for communication mode planning. In platooning use case, the routeAreaInformation can be a route path or a geographical area where the vehicle is estimated to pass.

Step2a and 2b: an ADN-AE, or a MN/ASN-CSE, or an IN-AE creates <group> resource to IN-CSE, including memberID and memberRoles (to indicate the leader vehicle of the platoon).

Step3: IN-CSE plans the group communication modes according to the one or more segments of the routeAreaInformation.

Step4: IN-CSE create <areaCommunicationModeList> resource to an ADN-AE, or a MN/ASN-CSE that is hosted on the leader vehicle of the platoon. The <areaCommunicationModeList> resource indicates the relation between the planed communication mode and the route segment.

Step5: an ADN-AE, or a MN/ASN-CSE, that is hosted on the leader vehicle of the platoon informs member vehicles of the platoon to switch its current using communication mode of V2V communication according to the  IN-CSE planed communication mode in a specified location through V2V communication.

Step6: member vehicle updates its current using communication mode and using this mode for V2V communication which is out of the oneM2M scope.

Figure 10.x.1-1. Solution Procedures for V2V communication mode management

The proposed solution procedures in Error: Reference source not found also can be used for non-group situation. The IN-CSE can plan a vehicle’s communication mode when the vehicle registers with the supportCommunicationModes and routeAreaInformation.

Note: How to link the V2V communication mode to CMDH policy is FFS.

### 10.x.2 Solution Applicability

This solution applies to Key Issue X.

### 10.x.3 New Resources and Procedures

#### 10.x.3.1 Introduction

To implement this solution, a new resource <areaCommunicationModelist> is proposed, and <group> resource and <AE> resource need to be modified. The <areaCommunicationModelist> maintains a list of communication modes that can be used in a specified area. These are further detailed in the following clauses.

#### 10.x.3.2 New <areaCommunicationModelist> resource type

An <areaCommunicationModelist> resource shall only be created in the IN-CSE.

Table 10.x.3.2-1: Resource specific attributes of <areaCommunicationModelist> resource

| Attributes of < areaCommunicationModelist > | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| expirationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| accessControlPolicyIDs | 0..1 (L) | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| Labels | 0..1 (L) | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| planedCommunicationModelist | 0..1 | RW | List of arrays which consist of two elements: communication mode and area information. Communication mode indicates the communication mode that oneM2M node should be used, pre-defined values are: PC5, unicast over LTE-Uu and MBMS. Area information indicates the geographic area where the communication mode is used. |

#### 10.x.3.3 Modified <group> resource

New attribute as shown in Error: Reference source not found.  The memberRoles attribute is used to indicate the leader of a group, and the leader should be one of members of the group.

Table 10.x.3.3-1: New attributes of <group> resource

| Attributes of <group> | Multiplicity | RW/ RO/ WO | Description | <groupAnnc> Attributes |
| --- | --- | --- | --- | --- |
| memberRoles | 0..1 | RW | List of arrays which consist of two elements: role and memberID. The role is the role of an oneM2M node in a group. The memberID should be one of the memberIDs. This attribute is used when the members’ roles of a group is need to specified, e.g. the leader vehicle of a platoon. | OA |

#### 10.x.3.4 Modified <AE> and <remoteCSE> resource

New attribute as shown in Error: Reference source not found.  The supportCommunicationModes attribute indicates the oneM2M node supported communication modes which can be one or more of PC5, unicast over LTE-Uu and MBSM. The routeAreaInformation attribute indicates the area information which is requested by a terminal for communication mode planning.

Table 10.x.3.4-2: New attributes of <AE> and <remoteCSE> resource

| Attributes of <AE> and <remoteCSE> | Multiplicity | RW/ RO/ WO | Description | <AEAnnc> and <remoteCSEAnnc> Attributes |
| --- | --- | --- | --- | --- |
| supportCommunicationModes | 0..n | RW | Indicates the oneM2M node supported communication mode, e.g. supported V2V communication mode, pre-defined values are: - PC5 - Unicast over LTE-Uu - MBMS | OA |
| routeAreaInformation | 0..1 | RW | Indicates the route information that the oneM2M node may pass through, e.g. the route of a platoon. When routeAreaInformation is exit, the IN-CSE should plan communication mode according to this attribute for the oneM2M node. | OA |

-----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?