input.doc

| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC36 |
| Source:* | Catalina Mladin, Convida, mladin.catalina@convidawireless.com Quang Ly, Convida, Ly.Quang@convidawireless.com Dale Seed, Convida, Seed.Dale@convidawireless.com Bei(Echo) Xu, Huawei, echo.xubei@huawei.com Bob Flynn, Convida, Flynn.Bob@convidawireless.com |
| Date:* | 2018-07-10 |
| Reason for Change/s:* | Request to move action triggering contents of TR-0021 into TS-0001 |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0035 Action Triggering MNT maintenance / < Work Item number(optional)> Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001, v3.11.0 |
| Clauses * | 9.6.1, 9.6.X, 10.2.Z |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution contains new resource types for supporting action triggering as agreed in TR-0021 and proposes moving them to TS-0001.

R01 includes:

- Corected release,

- Added cosigners and their proposed by the changes

- Additional changes meant to clarify attributes and procedures.

R02 corrected an upload issue

R03 addresses comments from ARC#36:

- Added a summary of the resources and their reole, i.e. object, subject, etc at the beginning os 10.2.Z

- Corrected resource references to resource IDs

- Described how the CRUD primitive is constructed

- Clarified that the operation will come from the <action> creator.

Not yet done: more clarifications about object resources on other CSEs – unclear what to be done there

R04 introduces:

- editorials at the beginning of 9.6.x section

- replacement of the operation attribute with actionPrimitive attribute

- clarifications for actionResult in the table, changed both actionResult and outputResourceID to RW, objectResoureID to optional

- changes in 10.2.Z to reflect the introduction of actionPrimitive attribute

R05 accepts previous markups is changes 2 and 3 (new text) and includes some clarifications

### -----------------------Start of change 1-------------------------------------------

## 9.6	Resource Types

### 9.6.1      Overview

- 9.6.1.1	Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary – i.e. not announced – resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in Table 9.6.26.1-1 "Announced Resource Types".

Among the resource types listed in Table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

- container;

- contentInstance;

- flexContainer;

- timeSeries;

- timeSeriesInstance.

Table 9.6.1.1-1: Resource Types

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| accessControlPolicy | Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.2 |
| AE | Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE | subscription, container, flexContainer, group, accessControlPolicy, pollingChannel, semanticDescriptor, timeSeries, transaction, transactionMgmt, triggerRequest, crossResourceSubscription, backgroundDataTransfer, semanticMashupInstance | CSEBase | 9.6.5 |
| container | Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule | container, flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor, timeSeries, transaction | AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, CSEBase, flexContainer, flexContainerAnnc | 9.6.6 |
| contentInstance | Represents a data instance in the <container> resource | semanticDescriptor, transaction | Container, containerAnnc | 9.6.7 |
| flexContainer | A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure | container, flexContainer, subscription, semanticDescriptor, timeSeries, transaction | AE, AEAnnc, container, containerAnnc, flexContainer, flexContainerAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.35 |
| CSEBase | The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself | remoteCSE, remoteCSEAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery, schedule, notificationTargetPolicy, flexContainer, timeSeries, AEContactList, transaction, transactionMgmt, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupInstance | None specified | 9.6.3 |
| delivery | Forwards requests from CSE to CSE | subscription, transaction | CSEBase | 9.6.11 |
| eventConfig | Defines events that trigger statistics collection | subscription, transaction | statsConfig | 9.6.24 |
| execInstance | Contains all execution instances of the same Management Command | subscription, transaction | mgmtCmd | 9.6.17 |
| fanOutPoint (V) | Virtual resource containing target for group request It is used for addressing bulk operations to all the resources that belong to a group | None specified | group | 9.6.14 |
| group | Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group | fanOutPoint, subscription, semanticFanOutPoint, semanticDescriptor, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.13 |
| latest (V) | Virtual resource that points to most recently created <contentInstance> and <timeSeriesInstance> child resource within a <container> and a <timeSeries> resource | None specified | container, timeSeries | 9.6.27 |
| locationPolicy | Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information | subscription, transaction | CSEBase | 9.6.10 |
| mgmtCmd | Management Command resource represents a method to execute management procedures required by existing management protocols | execInstance, subscription, transaction | CSEBase | 9.6.16 |
| mgmtObj | Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note) | subscription, mgmtObj, transaction | node, mgmtObj, mgmtObjAnnc | 9.6.15 Annex D |
| m2mServiceSubscriptionProfile | Data pertaining to the M2M Service Subscription | serviceSubscribedNode, subscription, transaction | CSEBase | 9.6.19 |
| node | Represents specific Node information | mgmtObj, subscription, semanticDescriptor, schedule, transaction | CSEBase | 9.6.18 |
| notificationTargetMgmtPolicyRef | Represents a list of notification targets and the deletion policy | subscription, transaction | subscription | 9.6.31 |
| notificationTargetPolicy | Represents a notification target deletion policy with pre-defined action and deletion rules | subscription, policyDeletionRules, transaction | CSEBase | 9.6.32 |
| notificationTargetSelfReference (V) | Virtual resource used to remove the Notification Target | None specified | subscription | 9.6.34 |
| oldest (V) | Virtual resource that points to first created <contentInstance> and <timeSeriesInstance> child resource within a <container> and a <timeSeries> resource | None specified | container, timeSeries | 9.6.28 |
| pollingChannel | Represent a channel that can be used for a request-unreachable entity | pollingChannelURI | remoteCSE, AE | 9.6.21 |
| pollingChannelURI (V) | Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity | None specified | pollingChannel | 9.6.22 |
| policyDeletionRules | Represents a set of rules which is associated with notification target removal policy | subscription, transaction | notificationTargetPolicy | 9.6.33 |
| remoteCSE | Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource | container, containerAnnc, flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, pollingChannel, timeSeries, timeSeriesAnnc, remoteCSEAnnc, nodeAnnc, AEAnnc, locationPolicyAnnc, transaction, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupJobProfileAnnc, semanticMashupInstance, semanticMashupInstanceAnnc | CSEBase | 9.6.4 |
| request | Expresses/access context of an issued Request | subscription, transaction | CSEBase | 9.6.12 |
| schedule | Contains scheduling information for delivery of messages | subscription, transaction | subscription, CSEBase, - node | 9.6.9 |
| serviceSubscribedNode | Node information | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.20 |
| statsCollect | Defines triggers for the IN-CSE to collect statistics for applications | subscription, transaction | CSEBase (in IN-CSE) | 9.6.25 |
| statsConfig | Stores configuration of statistics for applications | eventConfig, subscription, transaction | CSEBase (in IN-CSE) | 9.6.23 |
| subscription | Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribe-to resource | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | accessControlPolicy, accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, request, schedule, scheduleAnnc, semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig, flexContainer, flexContainerAnnc, timeSeries, timeSeriesAnnc | 9.6.8 |
| serviceSubscribedAppRule | Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE | subscription, transaction | CSEBase | 9.6.29 |
| semanticDescriptor | Stores semantic description pertaining to a resource and potentially sub-resources. | subscription, transaction | AE, container, contentInstance, group, node, flexContainer, timeSeries | 9.6.30 |
| semanticFanOutPoint | Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource | transaction | group | 9.6.14a |
| dynamicAuthorizationConsultation | Represents consultation information used by a CSE when performing consultation-based dynamic authorization | transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.40 |
| timeSeries | Stores and Shares Time Series Data instances among entities. | timeSeriesInstance, subscription, semanticDescriptor, latest, oldest, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, container, containerAnnc, flexContainer, flexContainerAnnc | 9.6.36 |
| timeSeriesInstance | Represents a Time Series Data instance in the <timeSeries> resource | transaction | timeSeries, timeSeriesAnnc | 9.6.37 |
| authorizationDecision | Represents an access control decision point | subscription, transaction | CSEBase | 9.6.41 |
| authorizationPolicy | Represents an access control policy retrieval point | subscription, transaction | CSEBase | 9.6.42 |
| authorizationInformation | Represents an access control information point | role token subscription, transaction | CSEBase | 9.6.43 |
| localMulticastGroup | Stores local multicast group information of member hosting CSE. | transaction | CSEBase | 9.6.44 |
| AEContactList | Contains information about a CSE that has resources that referencing an AE-ID | AEContactListPerCSE, subscription | CSEBase | 9.6.45 |
| AEContactListPerCSE | Contains information about a CSE that has resources that referencing an AE resource identifier for tracking purposes | None specified | AEContactList | 9.6.46 |
| transactionMgmt |  | subscription | CSEBase, AE, remoteCSE | 9.6.47 |
| transaction |  | None specified | All non-virtual resource types with the exception of the following: request, delivery, pollingChannel, transactionMgmt, transaction | 9.6.48 |
| triggerRequest | Used by an AE to initiate, replace or recall a device trigger request | subscription | AE | 9.6.49 |
| ontologyRepository | Represents the collection of the managed ontologies and the semantic validation service | ontology, semanticValidation, subscription | CSEBase | 9.6.50 |
| ontology | Store the representation of an ontology | subscription | ontologyRepository | 9.6.51 |
| semanticValidation | A virtual resource as the interface to perform semantic validation on the received <semanticDescriptor> resource against the referenced ontology. | None specified | ontologyRepository | 9.6.52 |
| semanticMashupJobProfile | Represents the profile and description of a semantic mashup service | semanticMashupInstance, semanticDescriptor, subscription | CSEBase, remoteCSE | 9.6.53 |
| semanitcMashupInstance | Represents a semantic mashup instance | semanticMashupResult, semanticDescriptor, mashup, subscription | semanticMashupJobProfile, AE, remoteCSE, CSEBase | 9.6.54 |
| mashup | A virtual resource use to trigger the calculation and generation of new mashup result | Not specified | semanticMashupInstance | 9.6.55 |
| semanticMashupResult | Represent semantic mashup results | semanticDescriptor, subscription | semanticMashupInstance | 9.6.56 |
| multimediaSession | Stores a representation of a multimedia session information requested by a registering AE | subscription, accessControlPolicy | AE | 9.6.57 |
| crossResourceSubscription | represents the cross-resource subscription information related to multiple subscribed-to resources. Such a resource shall include a list of subscribed-to resources as its attribute, or shall be created as a child resource of a <group> resource where member resources shall be the subscribed-to resources. | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, subscriptionLinkDeletion | CSEBase, remoteCSE, AE | 9.6.58 |
| subscriptionLinkDeletion | Virtual resource used to remove a <subscription> resource from the subscriptionResourcesAsTarget list of a <crossResourceSubscription> resource. | None specified | crossResourceSubscription | 9.6.59 |
| backgroundDataTransfer | Stores information for a background data transfer request | None specified | AE, remoteCSE, CSEBase | 9.6.60 |
| action | Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource | dependency | CSEBase, remoteCSE, node, accessControlPolicy, AE, container, flexContainer, group, timeSeries, transaction | 9.6.X |
| dependency | Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource | None specified | action | 9.6.Y |
| NOTE: See clause 9.6.12 for a summary of specializations of <mgmtObj>. |  |  |  |  |

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 9.6.X	Resource Type action

The <action> resource enables the system to specify which actions or operations to be performed on system resources (e.g. devices or services) based on monitored events. The events may include state changes of subject resources, requests from applications, etc. Once the event occurs, and if specified <dependency> conditions are met, the Hosting CSE sends a primitive defined by <action> resource attributes.

These child resources and attributes provide information about:

- The subject resource, which is the resource monitored to determine if a primary event occurs. Action is to be triggered if the primary event is the change of the state of the subject resource .

- The object resource, which is the resource which is the target of the triggered action

- The input resource, which is a resource which may provide input parameters for the action. Alternatively, an input value may be provided.

- Conditions to be monitored in order to determine if the primary event occurs

- Dependencies to be evaluated in order to determine if the action is to be conditionally performed.

- Parameters providing priorities of the action and priorities of the dependencies.

The <action> resource shall contain the child resources specified in table 9.6.X-1.

Table 9.6.X-1: Child resources of <action> resource

| Child Resources of <action> | Child Resource Type | Multiplicity | Description | <actionAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <dependency> | 0..n | See clause 9.6.Y | <dependencyAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |

The <action> resource shall contain the attributes specified in table 9.6.X-2.

Table 9.6.X-2: Attributes of <action> resource

| Attributes of <action> | Multiplicity | RW/ RO/ WO | Description | <actionAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| Labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| stateTag | 1 | RO | See clause 9.6.1.3. | OA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| Creator | 1 | RO | See clause 9.6.1.3. | NA |
| actionSubjectResource | 0..1 | RW | The resource that is the subject to be monitored for triggering the evaluation of the eventCriteriaevalCriteria attribute. The action indicated by the operation attribute Action is to be triggered based on the change of the state of the resource indicated the actionSubject Resource attribute.The subject can be any resource in the system. If the attribute is missing the subject is the parent resource. | OA |
| actionPriority | 0..1 | RW | Attribute indicating the priority of the action when compared with the other actions with the same event expression (subjectResourceID and evalCriteria), with highest priority indicated by the lowest value. Local policies are applied for equal or unspecified priorities. | OA |
| subjectResourceID | 0..1 | RW | The resourceID of the resource that is the subject of monitoring for evaluation of the evalCriteria attribute. The subject can be any resource in the system. If the attribute is missing the subject is the parent resource of this <action> resource. | OA |
| evalCriteria | 0..1 | RW | This attribute provides the conditions determining if the action is to be conditionally triggered to the object resource. The evalCriteria attribute, in conjunction with the subjectResourceID, forms the event expression that is being monitored. See further description below and in table 9.6.X-3 | OA |
| evalMode | 1 | RW | Attribute provides the macro control mode of the evaluation of the <action> resource. Some values are: 0 = off 1 = once 2 = periodic 3 = continuous | OA |
| evalControlParam | 0..1 | RW | When evalMode= periodic, evalControlParam represents the periodicity. When evalMode= continuous the evaluation is restarted as soon as an action has been triggered. In this case, if evalControlParam is specified, it determines the number of times the Hosting CSE shall trigger the event , otherwise it is repeated indefinitely. When evalMode is off or once this attribute can be ignored. | OA |
| evalPriority | 0..1 (L) | RW | List of priorities to be applied for the evaluation of the dependencies . This is an ordered list of the resourceIDs of the child <dependency> resources. | OA |
| objectResourceID | 0..1 | RW | The resourceID of the target of the primitive to be sent when the event specified by evalCriteria occurs at the subject resource. The target may be specified also as the URI of a resource attribute. | MA |
| actionPrimitive | 1 | RW | This attribute stores the entire content of the primitive associated with this action with its parameters (e.g. the op parameter indicating the operation to be performed on the resource identified by objectResourceID), with some parameters being overwritten by the Hosting CSE when the action is triggered as follows: - the to and resource type parameters will be derived from objectResourceID attribute. - the content parameter is derived from the input attribute. When objectResourceID or content the input attributes are empty, the respective parameters are maintained as indicated by actionPrimitive. | MA |
| input | 0..1 (L) | RW | Input parameters for the operation, used by the Hosting CSE to overwrite the content parameter of the actionPrimitive before sending the primitive, at the time that the action is triggered. The attribute could be one of the following: - a string - a resourceID - the URI of a resource attribute. The Hosting CSE overwrites the content parameter of actionPrimitive with the following (respectively): - the given string - the resource representation of the resource with the given resourceID. - the value of the given resource attribute | OA |
| outputResourceID | 0..1 | RW | The resourceID of a resource where the results of the action are stored. | OA |
| actionResult | 1 | RW | The result of the action. If outputResourceID is set, the Hosting CSE performs a RETRIEVE based on it when the response primitive of the action primitive is received and stores its content. Otherwise, the response primitive of the action primitive is stored in this attribute. | OA |

The conditions represented in the evalCriteria attribute determine if the action is to be conditionally triggered.

The evalCriteria attribute is comprised of triples (subject, operator, threshold) with the parameters shown and described in table 9.6.X-3.

Table 9.6.4-3: Parameters in evalCriteria triple

| Name | Description |
| --- | --- |
| subject | This parameter shall indicate an attribute of the subjectResourceID resource e.g. “tempContainer.content” |
| operator | This parameter is a keyword used to construct the evaluation logic, e.g. ‘equals’, ‘not equals’, ‘greater than’, ‘less than’, ‘greater or equal’, ‘less or equal’, ‘string match’, etc. |
| threshold | This parameter provides a value used to evaluate the criteria. The value type shall match the type of the attribute used as subject. |

### 9.6.Y	Resource Type dependency

The <dependency> resource represents  conditions that determines if the action or operation specified in the parent <action> resource is triggered. One or more <dependency> resources may exist to provide complex events for  triggering. The <dependency> resources provide conditions that always need to be checked before the action or operation is triggered. The difference between the evalCriteria  specified by an <action> resource and the one specified by a  <dependency> resource is that the former applies to the subject resource and triggers an evaluation, while  the latter applies to other resources which provide conditions used in evaluation.

The <dependency> resource shall contain the attributes specified in table 9.6.Y-2.

Table 9.6.Y-2: Attributes of <dependency> resource

| Attributes of <action> | Multiplicity | RW/ RO/ WO | Description | <actionAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. If no accessControlPolicyIDs value is configured, the accessControlPolicyIDs of the parent resource shall be applied for privilege checking. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| stateTag | 1 | RO | See clause 9.6.1.3. | OA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3. | NA |
| logic | 0..1 | RW | Indicates if the dependency is SUFFICIENT or NECESSARY. The operand of the related parent <action> resource needs to match: At least one of the SUFFICIENT dependencies OR All the NECESSARY dependencies | OA |
| evalCriteria | 0..1 | RW | Attribute providing conditions determining if the dependency matches the criteria. The evalCriteria attribute in conjunction with the dependentResource attribute forms the dependency to be evaluated. See further description in table 9.6.X-3. | OA |
| dependentResourceID | 1 | RW | The resourceID of the resource for which the evalCriteria is evaluated for the dependency. | MA |

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

### 10.2.Z	Action Triggering

#### 10.2.Z.1	Introduction

This clause describes the procedure for managing oneM2M action triggering via the <action> and <dependency> resources.

A oneM2M <action> resource is created whenever an Originator desires a primitive to be sent in response to the detection of a oneM2M event. An event has a subject resource associated with it – a change on this subject resource causes the Hosting CSE to conditionally trigger the procedure as outlined in 10.2.Z.2. This is referred to as “action triggering”. The subject resource is identified by the subjectResourceID attribute of the <action> resource and the evalCriteria attribute of the <action> resource determines if an action is conditionally triggered. Collectively, this combination is known as the event expression and the event is the primary event when it is triggered.

When an <action> resource is created, the Hosting CSE checks the evalMode attribute to see if an event is enabled for monitoring. The evalMode attribute has one of four valid values: off, once, periodic, or continuous. If evalMode is off, then no event is being monitored by the Hosting CSE. For all other values, the Hosting CSE will begin monitor events .

When evalMode is once, the Hosting CSE will immediately enable monitoring the event and will only allow the primary event to trigger once. After the event triggers, the Hosting CSE will disable further monitoring.

When evalMode is either periodic or continuous, the Hosting CSE will also evaluate the evalControlParam attribute to determine how to monitor the event. For periodic evalMode, the evalControlParam attribute determines the periodicity of monitoring the event. The Hosting CSE will start monitoring the event at the indicated periodicity and only allow the event to trigger once within the indicated period. For continuous evalMode, the evalControlParam attribute determines the number of times the Hosting CSE should trigger an event. When in this mode, the Hosting CSE resumes monitoring for the occurrence of the next event upon the triggering of a prior event; hence, there is no lapse in monitoring.

To handle more complex events, one or more  <dependency> resources  may be created as a child resources of the <action> resource. Within the <dependency> resource, the dependentResourceID attribute identifies the resource that the event dependents on. The evalCriteria attribute of the <dependency> resource determines whether the dependency condition is met. The logic attribute determines whether the dependency condition is SUFFICIENT or NECCESARY to the action triggering. Finally, the evalPriority attribute of the <action> resource determines the priorities to the dependency conditions to be applied when evaluating if the action or operation should be triggered based on the primary event. The dependency conditions will be evaluated by the Hosting CSE in their evalPriority priority order.

Once a Hosting CSE has determined that an event has triggered, it then uses the following attributes to create the  associated action : actionPrimitive, objectResourceID, and input. The actionPrimitive attribute provides the main content for the primitive that the Hosting CSE sends when the event is triggered. If present, the objectResourceID attribute is used to derive to and resourceType parameters which are overwritten in the primitive to be sent. If present, the input attribute is used to derive content parameter which is overwritten in the primitive to be sent. If these attributes are not present, the primitive sent maintains the parameters provided in the actionPrimitive.

Both the outputResourceID and actionResult attributes of the <action> resource provide the results of the action that is performed in response to the triggering of an event. The outputResourceID attribute identifies the resource reference in which the result of the action is stored. If the outputResourceID attribute is specified, the Hosting CSE retrieves  it and stores it in the actionResult attribute, providing the actual result of the action. If the outputResource attribute is not specified  the Hosting CSE stores the response to the operation in the actionResult attribute.

#### 10.2.Z.2	Action Triggering Procedure

This clause describes the procedure a Hosting CSE performs to manage the monitoring of events and, when they occur, the conditional triggering of actions and operations specified, as detailed by the following the steps:

Step 1:	Create <action> resource either as the child resource of a subject resource or elsewhere in the resource tree. The subject resource can be any of the resources defined in oneM2M and is indicated by the subjectResourceID attribute. One or more <dependency> resources may be created as child resources of the <action> resource.

Changes to the subject resource trigger the evaluation of the evalCriteria.

Step 2:	The content of subject resource is updated

Step 3:	The Hosting CSE gets the updated result and evaluates if the updated subject resource, and if it matches the criteria defined in the evalCriteria attribute of the <action> resource the primary event is considered triggered. If the subject resource does not match the criteria defined in the evalCriteria attribute of the <action> resource, the Hosting CSE returns to step 2, waiting for further subject resource updates.

Step 4:	After a primary event is triggered in step 3, if there is at least one <dependency> child resource of the <action> resource, the Hosting CSE performs the following step for each <dependency> child resource(s) in the order provided in the evalPriority attribute, or in an order based on local policy if the attribute is missing.

For each of the <dependency> child resources, the Hosting CSE uses the evalCriteria attribute and the logic attribute values provided in the current <dependency> resource. The Hosting CSE retrieves the dependent resource specified by the dependentResourceID attribute, evaluates it against the criteria defined in the evalCriteria attribute to check if the dependency is met.

If the dependency is met (i.e. the resource identified by dependentResourceID meets the criteria specified by evalCriteria):

- if the logic attribute is of value SUFFICIENT, the Hosting CSE proceeds to step 5.

- if the logic attribute is of value NECESSARY, the hosting CSE repeats step 4 for the next <dependency> child resource.

If the dependency is not met (i.e. the resource identified by dependentResourceID does not meet the criteria specified by evalCriteria), the hosting CSE repeats step 4 for the next <dependency> child resource.

Step 5:	The Hosting CSE retrieves the input resource if the input attribute of the <action> resource is a resourceID.

NOTE: The procedure outlined in step 4 means that step 5 is reached when, after the primary event is triggered, either:

- the SUFFICIENT dependency with the highest evalPriority priority is met; OR

- all the NECCESARY dependencies are met.

Step 6:	The Hosting CSE composes the request primitive for the object resource for triggering the action by populating it with the content of the actionPrimitive attribute.

If the objectResource attribute is present, the Hosting CSE determines its type (e.g. by performing a retrieve operation). The Hosting CSE overwrittes the to and resourceType parameters of the primitive to be sent with the values determined from objectResourceID.

If the input attribute is present and is a string, the Hosting CSE overwrites the content parameter of the primitive to be sent with the string. If the input attribute is present and is a resourceID, the Hosting CSE retrieves the resource and overwrites the content parameter of the primitive to be sent with the retrieved resource representation. If the input attribute is present and is a URI of a resource attribute , the Hosting CSE retrieves the resource and overwrites the content parameter of the primitive to be sent with the value of the given resource attribute .If the input attribute is not present, the content parameter of the primitive to be sent remain populated as indicated by the actionPrimitive attribute.

Step 7:	The Hosting CSE sends the composed primitive request to the object resource.

Step 8:	After receiving the response of the request from step 7, if the outputResourceID attribute of the <action> resource is set, the Hosting CSE retrieves the resource representation of the resource identified by outputResourceID and stores the representation as the actionResult attribute. If the outputResourceID attribute is not set the Hosting CSE stores the response primitive received in the actionResult attribute.

### -----------------------End of change 3-------------------------------------------