| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | ARC 37 |
| Source:* | BeiXu, Huawei, Echo.xubei@huawei.com Can Zhao, Huawei, zhaocan20@huawei.com |
| Date:* | 2018-07-16 |
| Contact:* | BeiXu，Huawei，Echo.xubei@huawei.com |
| Reason for Change/s:* | Add a solution for the KI V2V network interface switching management for TR-0026 |
| CR against: Release* | <Release> Release 4 |
| CR against: WI* | Active < WI-0046: Vehicular Domain Enablement > MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0026-Vehicular_Domain_Enablement-V4_0_0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Add a new solution for TR-0026.

R02：

Add the background about the solution

Add new resource <routeInfo>,update the step 1 to create the new resource insteand of registration.

Update the step 3 and 5,and delete the step 6.

Key Issue 8: V2V network interface switching management.

This results in the following issues:

- The IN-CSE does not have the supported communication mode of each vehicle and routing path of the platoon to plan the communication mode of the routing path.

- The IN-CSE lacks resources to store the relationship between planned communication mode and route segments.

- The IN-CSE cannot distinguish the leader vehicle role in a platooning group.

### -----------------------Start of change 1-------------------------------------------

## 10.x Solution H: V2V network interface switching management

### 10.x.1 Solution Description

In order to address the Key Issue 8, we can rely on the IN-CSE to plan communication mode for V2V communication in a platoon group, according to group information (e.g. Member ID, lead vehicle ID, supported communication mode for each member and route information of the platoon), network status information (e.g. network coverage, congestion status) and map information (e.g. road topology, building location). The basic procedure is described as below:

Step1: an ADN-AE, or a MN/ASN-CSE creates the <node> to the IN-CSE with supportedCommunicationModes and creates the <routeInfo> resource with routeInformation for platooning.

Step2a and 2b: an ADN-AE, or a MN/ASN-CSE, or an IN-AE sends a <group> resource CREATE request to IN-CSE, including memberID (the resourceID of <node>), memberRoles (to indicate the lead vehicle of the platoon) and routeInformation for platooning. The routeInformation is the filter condition of the group. IN-CSE keeps the members which have the same routeInformation with the routeInformation in the request, and creates the <group> resource.

Step3: IN-CSE plans the group communication modes since there is routeInformation in the request of step 2, according to the information for platooning in step 1 and 2, and supplementary information, such as network information, map information and so on. IN-CSE sets the planedCommunicationModelist as the planned group communication modes and creates <areaCommunicationModeList> child resource of <node> when the lead is an ADN-AE. IN-CSE prepares the <areaCommunicationModeList> resource of <node> CREATE request when the lead is an MN/ASN-CSE.

Editor’s note: The procedure should be FFS that the IN-CSE determines the route segments according to the network information such as MBMS deployment of network which is pre-configuration.

Step4a: If the ADN-AE subscribes the <node> resource, the IN-CSE sends a notification to the AE that is hosted on the lead vehicle of the platoon about the <areaCommunicationModeList> child resource creation of <node> resource.

Step4b: The IN-CSE sends <areaCommunicationModeList> resource CREATE to the MN/ASN-CSE that is hosted on the lead vehicle of the platoon.

Step5: The ADN-AE or MN/ASN-CSE, as the lead vehicle, informs other members to switch the communication mode according to the <areaCommunicationModeList> resource.

Note: The procedure of communication between filed nodes is FFS.

Figure 10.x.1-1. Solution Procedures for V2V communication mode management

.

Note: How to link the V2V communication mode to CMDH policy is FFS.

### 10.x.2 Solution Applicability

This solution applies to Key Issue 8.

### 10.x.3 New Resources and Procedures

#### 10.x.3.1 Introduction

To implement this solution, new resources <routeInfo> and < communicationModeCapability> are proposed, and <group>, <node> resource need to be modified. The <areaCommunicationModelist> maintains a list of communication modes that can be used in a specified area. These are further detailed in the following clauses.

#### 10.x.3.2 New <areaCommunicationModelist> resource type

An <areaCommunicationModelist> resource shall only be created by the IN-CSE. When the parent resourceID is one member of a group, the <areaCommunicationModeList> resource is used to indicate the communication mode for route of the group members.

Table 10.x.3.2-1: Resource specific attributes of <areaCommunicationModelist> resource

| Attributes of < areaCommunicationModelist > | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| expirationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| accessControlPolicyIDs | 0..1 (L) | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| Labels | 0..1 (L) | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| planedCommunicationModelist | 0..1 | RW | List of arrays which consist of two elements: communication mode and area information. Communication mode indicates the communication mode that oneM2M node should be used, pre-defined values are: PC5, unicast over LTE-Uu and MBMS. Area information indicates the geographic area where the communication mode is used. |

#### 10.x.3.3 Modified <group> resource

New attribute as shown in Error: Reference source not found.  The memberRoles attribute is used to indicate the leader of a group, and the leader should be one of members of the group.

Table 10.x.3.3-1: New attributes of <group> resource

| Attributes of <group> | Multiplicity | RW/ RO/ WO | Description | <groupAnnc> Attributes |
| --- | --- | --- | --- | --- |
| memberRoles | 0..1 | RW | List of arrays which consist of two elements: role and memberID. The role is the role of an oneM2M node in a group: - 0：Lead The memberID should be one of the memberIDs. This attribute is used when the members’ roles of a group is need to specified, e.g. the lead vehicle of a platoon. | OA |

#### 10.x.3.4 Modified <node> resource

New child resource and attribute as shown in Error: Reference source not found.

Table 10.x.3.4-2: New attributes of <node> resource

| Attributes of <node> | Multiplicity | RW/ RO/ WO | Description | <nodeAnnc> Attributes |
| --- | --- | --- | --- | --- |
| supportedCommunicationModes | 1..n | RW | Indicates the oneM2M node supported communication mode capability, e.g. supported V2V communication mode, pre-defined values are: - PC5 - Unicast over LTE-Uu - MBMS | OA |

Table 10.x.3.4-2: Child resources of <node> resource

| Child Resources of <node> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <routeInfo> | 0..1 | See table 10.x.3.5-1 |
| [variable] | <areaCommunicationModelist> | 0..1 | See table 10.x.3.2-1 |

#### 10.x.3.5 New <routeInfo> resource type

The attribute as shown in Error: Reference source not found.  The routeInformation attribute indicates the area information which is requested by a terminal for communication mode planning

Table 10.x.3.5-3: Resource specific attributes of <routeInfo> resource

| Attributes of <routeInfo> | Multiplicity | RW/ RO/ WO | Description | <routeInfoAnnc> Attributes |
| --- | --- | --- | --- | --- |
| routeInformation | 1 | RW | Indicates the route information that the oneM2M node may pass through as specified in the ETSI ITS TS 102 894-2 Annex A A-113, e.g. the route of a platoon or a geographical area where the platoon is estimated to pass. When routeInformation is exit, the IN-CSE should plan communication mode according to this attribute for the oneM2M node. | OA |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

## Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	oneM2M TS-0002: "Requirements".

[i.3]	oneM2M TS-0001: "Functional Architecture"

[i.4]	oneM2M TS-0011: "Common Terminology".

[i.5]	Recommendation ITU-T X.1362: "Simple encryption procedure for Internet of Things (IoT) environments".

[i.6]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[i.7]	ISO/IEC 29192:2012 "Lightweight cryptography".

[i.8]	PRESERVE: "V2X Security Architecture v2".

[i.9]	3GPP TR 33.885: "Study on security aspects for LTE support of Vehicle-to-Everything (V2X) services".

[i.10]	3GPP TS 33.185: "Security aspect for LTE support of Vehicle-to-Everything (V2X) services".

[i.11]	EVITA: "Secure on-board architecture specification".

[i.12]	EVITA: "Secure on-board protocols specification".

[i.13]	Trusted Computing Group: "TCG TPM 2.0 Automotive Thin Profile".

[i.14]	oneM2M TS-0003: "Security Solutions".

[i.15]	GSMA: "SGP.01 - Embedded SIM Remote Provisioning Architecture".

[i.16]	oneM2M TS-0004: "Service Layer Core Protocol Specification".

[i.17]	W. Chen, Telcordia Technologies: “Vehicular Communications and Networks: Architectures, Protocols, Operation and Deployment”

[i.18]	AUTOPILOT: http://autopilot-project.eu/

[i.19]	SAREF: The Smart Appliances REFerence (SAREF) ontology.

NOTE:	Available at http://ontology.tno.nl/saref/.

[i.20]	HERE: Vehicle Sensor Data Cloud Ingestion Interface Specification(v2.0.2)

NOTE:	Available at https://lts.cms.here.com/static-cloud-content/Company_Site/2015_06/Vehicle_Sensor_Data_Cloud_Ingestion_Interface_Specification.pdf

[i.21]	ETSI TR 102 863 : “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Local Dynamic Map (LDM); Rationale for and guidance on standardization”

[i.22]	5GAA White paper - Toward fully connected vehicles: Edge computing for advanced automotive communications, Dec. 2017

[i.xx]        ETSI TS 102 894-2 : “Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionary”

NOTE:	Available at http://5gaa.org/news/toward-fully-connected-vehicles-edge-computing-for-advanced-automotive-communications/

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?