| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #37 |
| Source:* | Bareau Cyrille, Orange, cyrille.bareau@orange.com Bonnardel Gregory, Orange, gbonnardel.ext@orange.com Gharout Said, Orange, said.gharout@orange.com Goluch Maciej, Orange, maciej.goluch@orange.com Kraft Andreas, DT, A.Kraft@telekom.de Strzemecki Paweł, Orange, pawel.strzemecki2@orange.com |
| Date:* | 2018-09-15 |
| Reason for Change/s:* | See the introduction below |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0079 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v. 3.12.0 |
| Clauses * | D |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR brings new Management Object [storage] which shall be used to manage device’s storage

### -----------------------Start of change 1-------------------------------------------

# D.X	Resource storage

The [storage] resource is used to share information regarding the storage on the device. The resource for [storage] Management Object  is to be recreated after each storage media change.

The [storage] resource is a specialization of the <mgmtObj>resource.

Figure D.X-1: Structure of [storage] resource

The [storage] resource shall contain the child resources specified in table D.X-1.

Table D.X-1: Child resources of [storage] resource

| Child Resources of [storage] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 |

The [storage] resource shall contain the attributes specified in table D.X-2.

Table D.X-2: Attributes of [storage] resource

| Attributes of [storage] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1(L) | RW | See clause 9.6.1.3. |
| mgmtDefinition | 1 | WO | See clause 9.6.15. Has fixed value "storage" to indicate the resource is for memory management. |
| objectIDs | 0..1 (L) | WO | See clause 9.6.15. |
| objectPaths | 0..1 (L) | WO | See clause 9.6.15. |
| description | 0..1 | RW | See clause 9.6.15. |
| UUID | 0..1 | WO | The uuid of the storage device. This attribute is a specialization of [objectAttribute] attribute. |
| storageType | 0..1 | RO | Indicates the type of storage. 0 indicates internal and 1 indiciates external |
| writeSpeed | 0..1 | RO | Indicates the write speed of storage device. This attribute is a specialization of [objectAttribute] attribute. |
| readSpeed | 0..1 | RO | Indicates the read speed of storage device. This attribute is a specialization of [objectAttribute] attribute. |
| availStorage | 1 | RO | Indicates the current available amount of memory. 1MB = 106 bytes. This attribute is a specialization of [objectAttribute] attribute. |
| totalStorage | 1 | RO | Indicates the total amount of memory available for the. 1MB = 106 bytes. This attribute is a specialization of [objectAttribute] attribute. |
| storagePresence | 0..1 | RW | Indicates current presence status of memory card. 0 indicates card is ejected, 1 indicates card is inserted. This attribute is a specialization of [objectAttribute] attribute. |
| status | 0..1 | RW | Indicates current operation status of storage. 1 – indicates storage is ready, 0 indicates storage is busy. This attribute is a specialization of [objectAttribute] attribute. |
| numberOfMounts | 0..1 | RO | Indicates number of successful mounts of the storage. This attribute is a specialization of [objectAttribute] attribute. |
| numberOfForcedUnmounts | 0..1 | RO | Indicates number of forced unmounts of the storage. This attribute is a specialization of [objectAttribute] attribute. |
| fileSystem | 0..1 | RW | Indicates the filesystem type used on the mounted storage. This attribute is a specialization of [objectAttribute] attribute. |
| storageName | 0..1 | RW | Indicates name of the mounted storage. This attribute is a specialization of [objectAttribute] attribute. |
| mountingPoint | 0..1 | RW | Indicates mounting point of the mounted storage. This attribute is a specialization of [objectAttribute] attribute. |
| format | 0..1 | RW | The action that allows to format the mounted storage. The action is triggered by assigning value "TRUE" to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| unmount | 0..1 | RW | The action that allows to safety eject storage device. The action is triggered by assigning value "TRUE" to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| mountOptions | 0..1 | RO | Indicates additional file system specific and file system independent mount options that indicate specific behaviours of the mount point as well as the capabilities of the underlying file system. |
| writable | 1 | RO | Indicates whether the storage volume is mounted as read/write (“TRUE”) or read-only (“FALSE”). This attribute is a specialization of [objectAttribute] attribute. |

### -----------------------Start of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

#### 9.6.1.2	Resource Type Specializations

##### 9.6.1.2.1	Specializations of <mgmtObj>

Table 9.6.1.2-1 lists specializations of the <mgmtObj> resource type in which the mgmtDefinition attribute contains an enumerated value that provides further definition of the resource.

Table 9.6.1.2.1-1: <mgmtObj> Specializations

| Resource specialization | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| activeCmdhPolicy | Provides a link to the currently active set of CMDH policies | None specified | node | D.12.1 |
| areaNwkDeviceInfo | Provides information about the Node in the M2M Area Network | subscription | node | D.6 |
| areaNwkInfo | Describes the list of Nodes attached behind the MN node and its physical or underlying relation among the nodes in the M2M Area Network | subscription | node | D.5 |
| battery | Provides the power information of the node (e.g. remaining battery charge) | subscription | node | D.7 |
| cmdhBuffer | Defines CMDH buffer usage limits | subscription | cmdhPolicy | D.12.8 |
| cmdhDefaults | Defines CMDH default values | cmdhDefEcValue, cmdhEcDefParamValues subscription | cmdhPolicy | D.12.2 |
| cmdhEcDefParamValues | Represent a specific set of default values for the CMDH related parameters | subscription | cmdhDefaults | D.12.4 |
| cmdhDefEcValue | Defines a value for the Event Category parameter of an incoming request when it is not defined | subscription | cmdhDefaults | D.12.3 |
| cmdhLimits | Defines limits for CMDH related parameter values | subscription | cmdhPolicy | D.12.5 |
| cmdhNetworkAccessRules | Defines rules for the usage of underlying networks | cmdhNwAccessRule, subscription | cmdhPolicy | D.12.6 |
| cmdhNwAccessRule | Defines a rule for the usage of underlying networks | subscription | cmdhNetworkAccessRules | D.12.7 |
| cmdhPolicy | A set of rules defining which CMDH parameters will be used by default | cmdhDefaults, cmdhLimits, cmdhNetworkAccessRules, cmdhBuffer, subscription | node | D.12 |
| deviceCapability | Contains information about the capability supported by the Node | subscription | node | D.9 |
| deviceInfo | Contains information about the identity, manufacturer and model number of the device | subscription | node | D.8 |
| eventLog | Contains information about the log of events of the Node | subscription | node | D.11 |
| firmware | Provides information about the firmware of the Node (e.g. name, version) | subscription | node | D.2 |
| memory | Provides the memory (typically RAM) information of the node (e.g. the amount of total volatile memory) | subscription | node | D.4 |
| reboot | Used to reboot or reset the Node | subscription | node | D.10 |
| software | Provides information about the software of the Node | subscription | node | D.3 |
| registration | To convey the service layer configuration information | subscription | node | Clause 7.1 in [10] |
| dataCollection | To convey the application configuration information | subscription | node | Clause 7.2 in [10] |
| authenticationProfile | To convey the configuration information regarding establishing mutually-authenticated secure communications | subscription | node | Clause 7.1 in [10] |
| myCertFileCred | To configure a certificate or certificate chain | subscription | authenticationProfile | Clause 7.1 in [10] |
| trustAnchorCred | To identify a trust anchor certificate for validation of certificates | subscription | authenticationProfile | Clause 7.1 in [10] |
| MAFClientRegCfg | To convey instructions regarding the MAF Client Registration procedure | subscription | authenticationProfile | Clause 7.1 in [10] |
| storage | To manage available storage memory on the device | subscription | node | D.X |

### -----------------------End of change 2--------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?