| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#37 |
| Source:* | Dale Seed, Convida, seed.dale@convidawireless.com |
| Date:* | 2018-09-09 |
| Reason for Change/s:* | See introduction |
| CR against: Release* |  |
| CR against: WI* | Active (WI-0083 - oneM2M Service Subscribers and Users) MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0054 v0.2.0 |
| Clauses * | 7.2.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | n/a |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes to add a <serviceSubscribedUserProfile> resource type.  The motivation for defining this new resource type is to support the capability for a M2M Service User profile in the oneM2M architecture.

This profile can contain information for a given M2M Service User such as its M2M-User-ID.  Additional information is envisioned to be defined in future contributions applicable to this work item which is the justification for defining a new resource type rather than an attribute.

A oneM2M <m2mServiceSubscriptionProfile> resource can have a <serviceSubscribedUserProfile> child resource for each authorized M2M Service User associated with the M2M Service Subscription.  A <serviceSubscribedAppRule> resource can have an allowedUsers attribute that can be configured with a list of <serviceSubscribedUserProfile> resource identifiers, wherein the allowedUsers represent the users that are allowed to originate requests via the registered AEs specified by the <serviceSubscribedAppRule>.

### -----------------------Start of change 1-------------------------------------------

## M2M Service User

### Solution Applicability

The definition of the M2M Service User Identifier is applicable to the current limitation that oneM2M lacks the capability to identify a M2M Service User (i.e. the entity authorized by a M2M Service Subscriber to use M2M Services offered by the M2M Service Provider which the M2M Service Subscriber has established a M2M Service Subscription with).

### Solution Description

### M2M Service User Identifier (M2M-User-ID)

A M2M Service User is a stakeholder that is authorized by a M2M Service Subscriber to use M2M Services offered by the M2M Service Provider which the M2M Service Subscriber has established a M2M Service Subscription with.  A M2M-User-ID uniquely identifies a M2M Service User and is assigned by a M2M Service Provider.  A M2M-User-ID is globally unique.  When used internally within the M2M Service Provider domain that assigned it, a M2M-User-ID is sufficient to be unique within that M2M Service Provider domain.  When used externally outside the M2M Service Provider domain that assigned it, a M2M-User-ID is extended to make it globally unique.  The IN-CSE can perform this task by adding or removing segments (i.e. the M2M Service Provider Identifier) of the M2M-User-ID.

Table 7.2.2-1: M2M-User-ID Identifier Format

| Identifier Name | Format | Format | Rule of use |
| --- | --- | --- | --- |
| M2M-User-ID | SP-Relative | The SP-Relative-M2M-User-ID begins with a slash character '/' and is followed by a sequence of characters that includes any of the unreserved characters defined in the clause 2.3 of the IETF RFC 3986 [i.3]. The SP-Relative-M2M-User-ID is unique within the context of the M2M-SP Domain to which the M2M Service User has a service subscription with. The M2M-SP assigns the SP-Relative-M2M-User-ID and is responsible for guaranteeing that the SP-Relative-M2M-User-ID is unique in the context of the M2M-SP Domain. Examples: - /bobjones - /bob123 | On the Mca and Mcc reference points: refers to M2M Service Users from the internal M2M Service Provider Domain |
|  | Absolute | Concatenation according to the format {M2M-SP-ID}{SP-Relative-M2M-User-ID} where {M2M-SP-ID} and {SP-relative- M2M-User-ID} are placeholders for the M2M-SP-ID and the SP-relative-M2M-User-ID format of the M2M-User-ID, respectively. | On the Mca and Mcc reference points: refers to M2M Service Users from external M2M Service Provider Domains |

### Resource Type serviceSubscribedUserProfile

The <serviceSubscribedUserProfile> resource contains user profile information for a given M2M Service User such as its M2M-User-ID.  A oneM2M <m2mServiceSubscriptionProfile> resource can have a <serviceSubscribedUserProfile> child resource for each authorized M2M Service User associated with the M2M Service Subscription.  A <serviceSubscribedAppRule> resource can have an allowedUsers attribute that can be configured with a list of <serviceSubscribedUserProfile> resource identifiers, wherein the allowedUsers represents the users that are allowed to originate requests via the registered AEs specified by the <serviceSubscribedAppRule>.

Figure 7.2.2-1: < serviceSubscribedUserProfile> Resource

Table 7.2.2-2: Attributes of <serviceSubscribedUserProfile> resource

| Attributes of <serviceSubscribedUserProfile> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See TS-0001 [i.2] clause 9.6.1.3. |
| resourceID | 1 | RO | See TS-0001 [i.2] clause 9.6.1.3. |
| resourceName | 1 | WO | See TS-0001 [i.2] clause 9.6.1.3. |
| parentID | 1 | RO | See TS-0001 [i.2] clause 9.6.1.3. |
| expirationTime | 1 | RW | See TS-0001 [i.2] clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See TS-0001 [i.2] clause 9.6.1.3. |
| creationTime | 1 | RO | See TS-0001 [i.2] clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See TS-0001 [i.2] clause 9.6.1.3 |
| lastModifiedTime | 1 | RO | See TS-0001 [i.2] clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See TS-0001 [i.2] clause 9.6.1.3. |
| M2M-User-ID | 1 | RW | The M2M-User-ID that is assigned to this M2M Service User by the M2M Service Provder. |

Table 7.2.2-3: New <serviceSubscribedAppRule> resource attributes

| Attributes of <serviceSubscribedAppRule> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| allowedUsers | 0..1 (L) | RW | A list of <serviceSubscribedUserProfile> resource identifiers. Each user that corresponds to a referenced <serviceSubscribedUserProfile> resource represents a user that is allowed to register an AE specified by this <serviceSubscribedAppRule> resource and issue requests via the AE. |

### Including M2M-User-ID in From parameter

Requests from an Originator to a Receiver that are sent over the Mca and Mcc reference points contain a From parameter that is configured with the AE-ID of an AE or a CSE-ID of a CSE that originates the request.  When the From parameter is configured with an AE-ID, a M2M-Subscriber-ID or M2M-User-ID can also be appended to indicate a corresponding M2M Service Subscriber or M2M Service User currently using the AE that is originating the request.  For example, /SAE099:/bob123 is a From parameter configured with an AE-ID (i.e. /SAE099) and a M2M-User-ID (i.e. /bob123).

NOTE 1: In the above example, a colon character is used as a delimiter.  It is left to Stage 3 to define the format of a From parameter that includes an AE-ID and M2M-Subscriber-ID or M2M-User-ID.

NOTE 2: The data type currently used by Stage 3 for the From parameter is m2m:ID which equates to xs:anyURI.  Hence including a M2M-Subscriber-ID or M2M-User-ID should be feasible without having to change the data type of the From parameter.

### M2M-Subscriber-ID and M2M-User-ID Security Analysis

NOTE: Analysis of the impacts of M2M-Subscriber-ID and M2M-User-ID on the oneM2M security framework (e.g. credentials, Remote Security Provisioning Frameworks, Security Association Establishment, authorization, etc.) is FFS.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?