| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#37 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com |
| Date:* | 2018-08-25 |
| Reason for Change/s:* | The CR is to fix the issues of UE reachability. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active WI-0058 MNT maintenance / WI-0058 Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 |
| Clauses * | 7.9 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0026 v0.9.0 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

In 3GPP TS 29.122V15, maxUEAvailabilityTime this parameter may be included to identify the timestamp until which a UE using a power saving mechanism is expected to be reachable for SM delivery.

When "monitoring-Type" is "UE_REACHABILITY" and the “ReachabilityType” is “SMS” or “SMS_AND_DATA”, the maxUEAvailabilityTime will be include in the notification from 3GPP to SCS.

But in the Step3 clause 7.4.1 of oneM2M TS-0026, oneM2M sets ReachabilityType as DATA, so maxUEAvailabilityTime shall not be included in the response in Step7.

Table 5.3.2.3.2-1: Definition of type MonitoringEventReport

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- |
| imeiChange | AssociationType | 0..1 | If "monitoring-Type" is "CHANGE_OF_IMSI_IMEI_ASSOCIATION", this parameter shall be included to identify the event of change of IMSI-IMEI or IMSI-IMEISV association is detected. Refer to 3GPP TS 29.336 [11] Subclause 8.4.22. | Change_of_IMSI_IMEI_association_notification |
| externalIds | array(ExternalId) | 0..N | External identifier (NOTE 2) |  |
| idleStatusInfo | IdleStatusInfo | 0..1 | If "idleStatusIndication" in the "MonitoringEventSubscription"sets to "true", this parameter shall be included to indicate the information when the UE transitions into idle mode. | Ue-reachability_notification, Availability_after_DDN_failure_notification |
| locationInfo | LocationInfo | 0..1 | If "monitoringType" is "LOCATION_REPORTING", this parameter shall be included to indicate the user location related information. | Location_notification |
| lossOfConnectReason | Integer | 0..1 | If "monitoring-Type" is "LOSS_OF_CONNECTIVITY", this parameter shall be included if available to identify the reason why loss of connectivity is reported. Refer to 3GPP TS 29.336 [11] Subclause 8.4.58. | Loss_of_connectivity_notification |
| maxUEAvailabilityTime | DateTime | 0..1 | If "monitoring-Type" is "UE_REACHABILITY", this parameter may be included to identify the timestamp until which a UE using a power saving mechanism is expected to be reachable for SM delivery. Refer to Subclause 5.3.3.22 of 3GPP TS 29.338 [34]. | Ue-reachability_notification |
| msisdns | array(Msisdn) | 0..N | Identifies the MS internal PSTN/ISDN number (NOTE 2) |  |
| monitoringType | MonitoringType | 1 | Identifies the type of monitoring type as defined in clause 5.3.2.4.3. |  |
| uePerLocationReport | UePerLocationReport | 0..1 | If "monitoringType" is "NUMBER_OF_UES_IN_AN_AREA", this parameter shall be included to indicate the number of UEs found at the location. | Number_of_UEs_in_an_area_notification |
| plmnId | PlmnId | 0..1 | If "monitoringType" is "ROAMING_STATUS" and "plmnIIndication" in the "MonitoringEventSubscription" sets to "true", this parameter shall be included to indicate the UE's serving PLMN. | Roaming_status_notification |
| reachabilityType | ReachabilityType | 0..1 | If "monitoring-Type" is "UE_REACHABILITY", this parameter shall be included to identify the reachability of the UE. Refer to 3GPP TS 29.336 [11] Subclause 8.4.20. | Ue-reachability_notification |
| roamingStatus | Boolean | 0..1 | If "monitoringType" is "ROAMING_STATUS", this parameter shall be set to "true" if the UE is on roaming status. Set to false or omitted otherwise. | Roaming_status_notification |
| failureCause | FailureCause | 0..1 | If "monitoringType" is "COMMUNICATION_FAILURE", this parameter shall be included to indicate the reason of communication failure. | Communication_failure_notification |
| NOTE 1: Properties marked with a feature as defined in subclause 5.3.4 are applicable as described in subclause 5.2.7. If no features are indicated, the related property applies for all the features. NOTE 2: Identifies the user(s) for which the event occurred. Can be omitted if the event occurred for all users identified by the surrounding structure. |  |  |  |  |

##### 5.3.2.4.4	Enumeration: ReachabilityType

The enumeration ReachabilityType represents a reachability type. It shall comply with the provisions defined in table 5.3.2.4.4-1. If reachabilityType set to "SMS", the monitoring event request from SCS/AS shall be only for one-time monitoring request.

Table 5.3.2.4.4-1: Enumeration ReachabilityType

| Enumeration value | Description | Applicability (NOTE) |
| --- | --- | --- |
| SMS | The SCS/AS requests to be notified when the UE becomes reachable for sending SMS to the UE | Ue-reachability_notification |
| DATA | The SCS/AS requests to be notified when the UE becomes reachable for sending downlink data to the UE | Ue-reachability_notification |
| SMS_AND_DATA | The SCS/AS requests to be notified when the UE becomes reachable for sending SMS and downlink data to the UE | Ue-reachability_notification |
| NOTE: Properties marked with a feature as defined in subclause 5.3.4 are applicable as described in subclause 5.2.7. If no feature are indicated, the related property applies for all the features. |  |  |

In oneM2M TS-0026

Step 3: request for Monitoring Event Subscription to monitor the UE reachability

The IN-CSE sends a MonitoringEventSubscription request to the SCEF to monitor the reachability of a UE. The creation of one or more subscriptions to the node <schedule> in step 2 may be used to trigger the monitoring request if the networkCoordinated attribute of the <schedule> resource is set to TRUE.

The MonitoringEventSubscription request from the IN-CSE to the SCEF shall comply with Error: Reference source not found3GPP TS 29.122 [5]. The MonitoringEventSubscription request shall include:

- URI shall be set to {apiRoot}/3gpp_monitoring_event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- notificationDestination shall be set to a URI that the SCEF can target UE Reachability notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

- tltrId is used to identify the monitoring request.  It shall be assigned based on internal IN-CSE policies and shall be different from other tltrIds issued by this IN-CSE during the lifetime of this request.  Later, when reports are received, the value shall be used to associate them to the original request

- monitoringType shall be set to UE_REACHABILITY

- reachabilityType shall be set to DATA

- idleStatusIndication shall be set to TRUE or FALSE. A setting of TRUE will result in the SCEF sending notifications to the IN-CSE when the UE transitions into PSM idle mode. These notifications will be in addition to any notifications sent to the IN-CSE when the UE transitions into connected mode or receives an eDRX paging occasion.  A setting of FALSE (default) will result in the SCEF not sending notifications when the UE transitions into PSM idle mode.  A setting of TRUE is only applicable for UEs supporting PSM.  How the IN-CSE determines whether a UE supports PSM, eDRX or both is out of scope for the current release of this specification and may be configured based on Service Provider and MNO policies.

externalId shall be set to the M2M-Ext-ID of the UE.

### -----------------------Start of change 1-------------------------------------------

### 7.4.1 UE Reachability monitoring

The 3GPP SCEF functionality described in 3GPP TS 29.122 [5] supports APIs for monitoring specific events such as UE Reachability status. This allows an SCS to request to receive reports when a device becomes reachable for receiving either SMS or downlink data.  In the 3GPP interworking architecture of oneM2M, the UE hosts an ADN with one or more AEs or an ASN/MN-CSE. The UE Monitoring flow in Figure 7.4.1-1 takes place after the UE has attached to the 3GPP Network and the ADN-AE(s) or ASN/MN-CSE register with the IN-CSE.

Figure 7.4.1 -1: UE Reachability monitoring

Step 1: UE attaches to 3GPP network and registers to IN-CSE

The UE attaches to the 3GPP network and the ADN-AE(s) or ASN/MN-CSE hosted on the UE perform the oneM2M registration procedure, as detailed in clause 6.5. The IN-CSE hosts the corresponding <AE> or <remoteCSE> resources for the registree and an associated <node> resource.

Step 2 (Optional): Subscribe to <schedule>

As described in clause 6.3, if at the time of ADN-AE(s) or ASN/MN-CSE registration to the IN-CSE a child <schedule> resource of the <node> resource representing the UE does not exist, the IN-CSE shall create it.  If an IN-AE, ADN-AE(s) or ASN/MN-CSE is interested in receiving notifications when this <schedule> resource is updated by the IN-CSE, based on UE reachability notifications from the underlying 3GPP network, it may subscribe to this <schedule> resource.

Step 3: request for Monitoring Event Subscription to monitor the UE reachability

The IN-CSE sends a MonitoringEventSubscription request to the SCEF to monitor the reachability of a UE. The creation of one or more subscriptions to the node <schedule> in step 2 may be used to trigger the monitoring request if the networkCoordinated attribute of the <schedule> resource is set to TRUE.

The MonitoringEventSubscription request from the IN-CSE to the SCEF shall comply with Error: Reference source not found3GPP TS 29.122 [5]. The MonitoringEventSubscription request shall include:

- URI shall be set to {apiRoot}/3gpp_monitoring_event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- notificationDestination shall be set to a URI that the SCEF can target UE Reachability notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

- tltrId is used to identify the monitoring request.  It shall be assigned based on internal IN-CSE policies and shall be different from other tltrIds issued by this IN-CSE during the lifetime of this request.  Later, when reports are received, the value shall be used to associate them to the original request

- monitoringType shall be set to UE_REACHABILITY

- reachabilityType shall be set to DATA

- idleStatusIndication shall be set to TRUE or FALSE. A setting of TRUE will result in the SCEF sending notifications to the IN-CSE when the UE transitions into PSM idle mode. These notifications will be in addition to any notifications sent to the IN-CSE when the UE transitions into connected mode or receives an eDRX paging occasion.  A setting of FALSE (default) will result in the SCEF not sending notifications when the UE transitions into PSM idle mode.  A setting of TRUE is only applicable for UEs supporting PSM.  How the IN-CSE determines whether a UE supports PSM, eDRX or both is out of scope for the current release of this specification and may be configured based on Service Provider and MNO policies.

- externalId shall be set to the M2M-Ext-ID of the UE.

The following parameters are optional:

- maximumNumberOfReports may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.

- monitorExpireTime may be set to an absolute time at which the UE reachability monitoring event request is considered to expire.  The IN-CSE shall configure this time based on Service Provider and MNO policies.

Step 4: Handling in the 3GPP Network Entities

The SCEF processes the MonitoringEventSubscription request together with the 3GPP network entities, as described in 3GPP TS 29.122 [5].

Step 5: response to Monitoring Event Subscription

The SCEF sends a MonitoringEventSubscription response that contains information as specified in 3GPP TS 29.122 [5] to the IN-CSE.

- In addition to parameters included in the request, the response also includes a self parameter configured with a URI to the resource created by the SCEF for the MonitoringEventSubscription request.  This URI will have a structure of {apiRoot}/3gpp_monitoring_event/v1/{scsAsId}/subscriptions/{subscriptionId}.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies and the {subscriptionId} segment is configured by the SCEF.

General Exceptions:

- The SCEF returns the following response codes in case of error.

  - Bad Request: If incorrect parameters are included in the request,.

  - Unauthorized: If the IN-CSE is not authorized as described in RFC 7235.

  - Forbidden: If the SCEF is able to understand the request but unable to fulfil the request.

  - Not Found: If the URI specified in the request is incorrect.

  - Conflict: The operation is not allowed given the current status of the resource. This response is returned when the creation of a resource with the same URI already exists; or the change cannot be made due to conflict with current state of the resource.

Step 6: Detection of UE changing reachability mode

Later, when the UE transitions to connected mode (for a UE using Power Saving Mode or extended idle mode DRX) or the UE becomes reachable for paging (for a UE using extended idle mode DRX) or the UE transitions to idle mode (for a UE using Power Saving Mode), the 3GPP network entities (e.g. HSS) detect the condition and send a Monitoring Event Report for UE reachability with Idle Status Information including Idle Status Timestamp, Periodic RAU/TAU timer, Active Time, eDRX Cycle Length) to the SCEF.

Step 7: SCEF sends Monitoring Event Notification to IN-CSE

When the SCEF receives information of status change in step 6, the SCEF  creates and sends a MonitoringEventNotification message for UE reachability to the IN-CSE as specified in 3GPP TS 29.122 [5].

Otherwise, the MonitoringEventNotification message is sent with appropriate information in accordance with the monitoringType.

The MonitoringEventNotification request for UE_REACHABILITY includes the following:

- subscriptionId configured with the identifier of the subscription for which this notification corresponds to

- subscription configured with a URI to the subscription resource for which this notification corresponds to

- tltrId configured with the same value of tltrId as in the request at step 3.

- monitoringEventReports configured with one or more UE Reachability monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [5]:

  - monitoringType configured with UE_REACHABILITY

  - idleStatusInfo configured with information such as activeTime, edrxCycleLength, suggestedNumberOfDlPackets, idleStatusTimestamp, periodicAUTimer as defined in 3GPP TS 29.122 [5]

  - reachabilityType configured with DATA  as defined in 3GPP TS 29.122 [5]

- externalID configured with the same value of externalId as in the request at step 3

Step 8: UE Reachability Monitoring Event Notification Response

After receiving a UE Reachability Monitoring Event Notification, the IN-CSE checks Monitoring Event Notification message and returns a response having a response code based on the checked results.

Step 9: UE Reachability Monitoring Event Notification Handling at the IN-CSE

The IN-CSE uses the information provided in the UE Reachability Monitoring Event Report as follows:

- If idleStatusInfo information is provided in the report, the IN-CSE shall change the scheduleElement of the UE’s node <schedule> resource such that the duration of the scheduleElement is set to the value of the activeTime parameter configured in the idleStatusInfo.

- If maxUEAvailabilityTime is provided in the report, the IN-CSE shall change the scheduleElement of the UE’s node <schedule> resource such that the duration of the scheduleElement is set to the value of maxUEAvailabilityTime.

Step 10a and 10b: (Optional) Notifications are sent to the entities which subscribed to changes in the <schedule> resources. This may include notifications sent to the ADN-AE(s) or ASN/MN-CSE hosted on the UE, if they have subscriptions to their respective <schedule> resources.

Step 11: Notification of schedule update

Upon receiving notification of <schedule> resource updates, the ADN-AE(s) or ASN/MN-CSE hosted on the UE process the notification and should update their local <schedule> resources (if applicable) with the same values.

Step 12 (Optional): request to delete Monitoring Event Subscription

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this Monitoring Event Subscription.  This step may be triggered by the IN-CSE when the <node> or <schedule> resources associated with a UE are deleted or when the networkCoordinated attribute of the <schedule> resource affiliated with a UE is set to FALSE.

Step 13 and 14 (Optional): Handling in 3GPP Network Entities and response to delete Monitoring Event Subscription

The SCEF processes the request to delete the Monitoring Event Subscription together with the 3GPP Network Entities.

The SCEF responds to the IN-CSE with a response either a code of 200 OK or 204 NO CONTENT indicating that it has successfully deleted the Monitoring Event Subscription.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?