| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#37 |
| Source:* | NTT |
| Date:* | 2018-09-17 |
| Contact:* | Kei Harada(harada.kei@lab.ntt.co.jp), NTT, Hiroyuki Maeomichi(maeomichi.hiroyuki@lab.ntt.co.jp), NTT, |
| Reason for Change/s:* | To add the introducion for each clause of TR-0056 |
| CR against: Release* | R4 |
| CR against: WI* | Active < WI-0087> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No STE Small Technical Enhancements / < WI-0087> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0056 |
| Clauses * | 5.1, 6.1, 7.1, 8.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | none |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR expresses introduction for each clause to make it clear what should be written in each clause.

## -----------------------Start of change 1-------------------------------------------

# 5	Features of Each Releases

## 5.1	Introduction

This clause summarizes features of both Rel-2A and Rel-3 based on documents published by Technical Plenary, reports for each F2F meeting written by WG chairs and so on.

## 5.2	Rel-2A

Rel-2A is a point release version of Rel-2 for ITU-T recommendation so the official release date does not exist but the Control Document was approved on June 2018.

There are official documents which introduce Rel-2 but not Rel-2A, but there is no big difference between Rel-2 and Rel-2A so this clause is based on official documents about Rel-2 (TP-2017-0126_THE ROLE OF oneM2M in UNLOCKING the IOT potential).

5.2.1	Home domain enablement

TS-0023 defines the Home Appliance Information Model (HAIM) for the home domain devices. For the reasons of interworking with external technologies and efficiency, the principle of the HAIM is designed based on HGI SDT 3.0. There are 13 concrete models for home appliances such as air conditioner, oven, refrigerator and so on.

TS-0023 also defines how to map each HAIM classe to oneM2M resources with specialization of <flexContainer>. Moreover, there is a mapping table between HAIM and oneM2M Base Ontology.

5.2.2	Industrial domain enablement

Considering requirements from industrial domain, the number of oneM2M resource types increases in Rel-2A. Increasing resource types means increasing oneM2M function so oneM2M is able to cover Real-time data collection, keeping redundancy and fault tolerance.

5.2.3	Semantic Interoperability

The first Semantic function in oneM2M is <semanticDescriptor> in Rel-2. This resource enables semantic description / discovery.Base Ontology is newly introduced  in TS-0012.

5.2.4	Dynamic authorizations and end to end security

The Dynamic Authorization reference model and processes including Dynamic Authorization System (DAS) are written in TS-0003, and authorization parameters and their transport are written in TS-0001 and TS-0004.

5.2.5	oneM2M as generic interworking framework

Some TSs express how to interwork between oneM2M and other specifications like Alljoyn / AllSeen (TS-0021) , OIC (TS-0024) and LightWeight M2M (LWM2M) (TS-0014).

These TSs explain mapping of data, architecture, process, security function and so on. About message protocol layer, TS-0008, 0009, 0010 amd 0020 explain about Protocol Binding.

5.2.6	Application developer APIs and guidelines

Eight Developer Guides got started to be written. These guides are Technical Reports and will be released independent from the timing of oneM2M Release-N.

## 5.3	Rel-3

5.3.1	Smart City and Automotive Enablement

5.3.2	Industrial Domain Enablement

5.3.3	Management

5.3.4	Semantics

5.3.5	Security

5.3.6	oneM2M Interworking

5.3.7	Market Adoption

## -----------------------End of change 1-------------------------------------------

## -----------------------Start of change 2-------------------------------------------

# 6	Updated and NewTSs

## 6.1	Introduction

This clause shows TSs which are included in both Rel-2A and Rel-3 and TSs which are included in only Rel-3. The former is called Updated TSs and the latter is called New TSs here.

Following sections summrizes short descriptions about contents or updated parts of each TSs.

## 6.2	Updated TSs

### 6.2.1	TS-0001

- 22 resource types like <transaction> and <semanticMashupInstance> are newly added in Rel-3.

- Rel-3 has less gaps with TS-0004 because of the gap analysis activity.

- There is no resource tree pictures at Rel-3 because of optimisation of space.

- Concept of transaction is introduced and many resource types become to be able to have <transaction> resource as a child in Rel-3.

- About <node> resource, hostedAELinks attribute and hostedAServiceLinks attributes are added in Rel-3.

Relating this, figures expressing relationship between IN/MN (Fig.9.6.18-1in Rel-3) and ADN and between IPE  (Fig.9.6.18-2 in Rel-3) are modified.

- <trafficPattern> resource type is deleted in Rel-3, so the general procedure for configuration of traffic patterns (clause 8.3.5.3) is changed to the procedure without using <trafficPattern>.

- 6.2.2	TS-0003

### 6.2.3	TS-0004

6.2.4		TS-0005

6.2.5		TS-0006

6.2.6		TS-0008

6.2.7		TS-0009

6.2.8		TS-0011

6.2.9		TS-0012

### 6.2.10	TS-0014

6.2.11		TS-0016

### 6.2.12	TS-0022

### 6.2.13	TS-0023

### 6.2.14	TS-0024

6.2.15		TS-0025

### 6.2.16	TS-0031

6.2.17		TS-0032

## 6.3	New TSs

### 6.3.1	TS-0017

### 6.3.2	TS-0018

### 6.3.3	TS-0019

### 6.3.4	TS-0026

### 6.3.5	TS-0030

### 6.3.6	TS-0033

### 6.3.7	TS-0034

### 6.3.8	TS-0035

### -----------------------End of change 2-------------------------------------------

## -----------------------Start of change 3-------------------------------------------

# 7	Changes about oneM2M Resources

## 7.1	Introduction

This clause expresses  relatively detaied topics, changes about oneM2M resources themselves.

Short introductions, attributes and child resources of each oneM2M resources are written in the table including updated points.

Moreover, differences about CRUD processes between Rel-2A and Rel-3 are also expressed in this clause.

## 7.2	Resource Types

| Resource Type | Short Introduction | Reference (minor) | Attribute (Rel-2A) | Multiplicity (Rel-2A) | Child Resource (Rel-2A) | Changes on Attributes (+: added , -: deleted) | Changes on Child Resource (+: added , -: deleted) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <accessControlPolicy> | The Access Control Policies (ACPs) shall be used by the CSE to control access to the resources. | TS-0003 | resourceType | 1 | <subscription> | + authorizationDecisionResourceIDs + authorizationPolicyResourceIDs + authorizationInformationResourceIDs | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | labels | 0..1(L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | announcedTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | priviledges | 1 |  |  |  |
|  |  |  | selfPrivileges | 1 |  |  |  |
| <CSEBase> | A <CSEBase> resource shall represent a CSE. |  | resourceType | 1 | <remoteCSE> <remoteCSEAnnc> <node> <AE> <container> <flexContainer> <group> <accessControlPolicy> <subscription> <mgmtCmd> <locationPolicy> <statsConfig> <statsCollect> <request> <delivery> <schedule> <role> <token> <m2mServiceSubscriptionProfile> <serviceSubscribedAppRule> <notificationTargetPolicy> <dynamicAuthorizationConsultation> <timeSeries> | + contentSerialization | + <authorizationDecision> + <authorizationPolicy> + <authorizationInformation> + <localMulticastGroup> + <transactionMgmt> + <transaction> + <ontologyRepository> + <semanticMashupJobProfile> + <semanticMashupInstance> + <AEContactList> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | cseType | 0..1 |  |  |  |
|  |  |  | CSE-ID | 1 |  |  |  |
|  |  |  | supportedResourceType | 1 (L) |  |  |  |
|  |  |  | pointOfAccess | 1 (L) |  |  |  |
|  |  |  | nodeLink | 0..1 |  |  |  |
|  |  |  | notificationCongestionPolicy | 0..1 |  |  |  |
|  |  |  | e2eSecInfo | 0..1 |  |  |  |
|  |  |  | supportedReleaseVersions | 0..1 (L) |  |  |  |
| <remoteCSE> | A <remoteCSE> resource shall represent both a Registree CSE and a Registrar CSE. For example, when CSE1 (Registree CSE) registers with CSE2 (Registrar CSE), there will be two <remoteCSE> resources created: one in CSE1: <CSEBase1>/<remoteCSE2> and one in CSE2: <CSEBase2>/<remoteCSE1>. |  | resourceType | 1 | <container> <containerAnnc> <flexContainer> <flexContaineAnnc> <group> <groupAnnc> <accessControlPolicy> <accessControlPolicyAnnc> <subscription> <pollingChannel> <scheduleAnnc> <nodeAnnc> <dynamicAuthorizationConsultation> <timeSeries> <timeSeriesAnnc> <remoteCSEAnnc> <AEAnnc> <locationPolicyAnnc> | + contentSerialization + descendantCSEs + multicastCapability + externalGroupID + triggerEnable + activityPatternElements | + <transactionMgmt> + <transaction> + <ontologyRepositoryAnnc> + <semanticMashupJobProfile> + <semanticMashupJobProfileAnnc> + <semanticMashupInstance> + <semanticMashupInstanceAnnc> - <scheduleAnnc> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | cseType | 0..1 |  |  |  |
|  |  |  | pointOfAccess | 0..1 (L) |  |  |  |
|  |  |  | CSEBase | 1 |  |  |  |
|  |  |  | CSE-ID | 1 |  |  |  |
|  |  |  | M2M-Ext-ID | 0..1 |  |  |  |
|  |  |  | Trigger-Recipient-ID | 0..1 |  |  |  |
|  |  |  | requestReachability | 1 |  |  |  |
|  |  |  | nodeLink | 0..1 |  |  |  |
|  |  |  | e2eSecInfo | 0..1 |  |  |  |
|  |  |  | triggerReferenceNumber | 0..1 |  |  |  |
|  |  |  | supportedReleaseVersions | 0..1 (L) |  |  |  |
| <AE> | An <AE> resource shall represent information about an Application Entity. |  | resourceType | 1 | <semanticDescriptor> <subscription> <container> <flexContainer> <group> <accessControlPolicy> <schedule> <pollingChannel> <trafficPattern> <dynamicAuthorizationConsultation> <timeSeries> | + M2M-Ext-ID + trigger-Recipient-ID + triggerReferenceNumber + registrationStatus + trackRegistrationPoints + activityPatternElements + triggerEnable + sessionCapabilities | + <transactionMgmt> + <transaction> + <triggerRequest> + <multimediaSession> + <semanticMashupInstance> - <schedule> - <trafficPattern> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | appName | 0..1 |  |  |  |
|  |  |  | App-ID | 1 |  |  |  |
|  |  |  | AE-ID | 1 |  |  |  |
|  |  |  | pointOfAccess | 0..1 (L) |  |  |  |
|  |  |  | ontologyRef | 0..1 |  |  |  |
|  |  |  | requestReachability | 1 |  |  |  |
|  |  |  | nodeLink | 0..1 |  |  |  |
|  |  |  | contentSerialization | 0..1 (L) |  |  |  |
|  |  |  | e2eSecInfo | 0..1 |  |  |  |
|  |  |  | supportedReleaseVersions | 0..1(L) |  |  |  |
| <container> |  |  | resourceType | 1 | <semanticDescriptor> <contentInstance> <subscription> <container> <flexContainer> <latest> <oldest> |  | + <timeSeries> + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | stateTag | 1 |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | maxNrOfInstances | 0..1 |  |  |  |
|  |  |  | maxByteSize | 0..1 |  |  |  |
|  |  |  | maxInstanceAge | 0..1 |  |  |  |
|  |  |  | currentNrOfInstances | 1 |  |  |  |
|  |  |  | currentByteSize | 1 |  |  |  |
|  |  |  | locationID | 0..1 |  |  |  |
|  |  |  | ontologyRef | 0..1 |  |  |  |
|  |  |  | disableRetrieval | 0..1 |  |  |  |
| <contentInstance> |  |  | resourceType | 1 | <semanticDescriptor> |  | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | stateTag | 1 |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | contentInfo | 0..1 |  |  |  |
|  |  |  | contentSize | 1 |  |  |  |
|  |  |  | contentRef | 0..1 |  |  |  |
|  |  |  | ontologyRef | 0..1 |  |  |  |
|  |  |  | content | 1 |  |  |  |
| <subscription> |  |  | resourceType | 1 | <schedule> <notificationTargetMgmtPolicyRef> <notificationTargetSelfReference> | + associatedCrossResourceSub The multiplicity of nortificationForwardingURI changed into 0..1 (L) in Rel-3. | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | eventNotificationCriteria | 0..1 |  |  |  |
|  |  |  | expirationCounter | 0..1 |  |  |  |
|  |  |  | notificationURI | 1 (L) |  |  |  |
|  |  |  | groupID | 0..1 |  |  |  |
|  |  |  | notificationForwardingURI | 0..1 |  |  |  |
|  |  |  | batchNotify | 0..1 |  |  |  |
|  |  |  | rateLimit | 0..1 |  |  |  |
|  |  |  | preSubscriptionNotify | 0..1 |  |  |  |
|  |  |  | pendingNotification | 0..1 |  |  |  |
|  |  |  | notificationStoragePriority | 0..1 |  |  |  |
|  |  |  | latestNotify | 0..1 |  |  |  |
|  |  |  | notificationContentType | 1 |  |  |  |
|  |  |  | notificationEventCat | 0..1 |  |  |  |
|  |  |  | subscriberURI | 0..1 |  |  |  |
| <schedule> |  |  | resourceType | 1 | <subscription> | + networkCoodinated | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | scheduleElement | 1 (L) |  |  |  |
| <locationPolicy> |  |  | resourceType | 1 | <subscription> | + authID + retrieveLastKnownLocation + locationUpdateEventCriteria | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | locationSource | 1 |  |  |  |
|  |  |  | locationInformationType | 1 |  |  |  |
|  |  |  | locationUpdatePeriod | 0..1(L) |  |  |  |
|  |  |  | locationTargetID | 0..1 |  |  |  |
|  |  |  | locationServer | 0..1 |  |  |  |
|  |  |  | locationContainerID | 1 |  |  |  |
|  |  |  | locationContainerName | 0..1 |  |  |  |
|  |  |  | locationStatus | 1 |  |  |  |
|  |  |  | geographicalTargetArea | 0..1 |  |  |  |
|  |  |  | geofenceEventCriteria | 0..1 |  |  |  |
|  |  |  | aggregationForward | 0..1 |  |  |  |
| <delivery> |  |  | resourceType | 1 | <subscription> | Nothing | Nothing |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | stateTag | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | source | 1 |  |  |  |
|  |  |  | target | 1 |  |  |  |
|  |  |  | lifespan | 1 |  |  |  |
|  |  |  | eventCat | 1 |  |  |  |
|  |  |  | deliveryMetaData | 1 |  |  |  |
|  |  |  | aggregatedRequest | 1 |  |  |  |
| <request> |  |  | resourceType | 1 | <subscription> | Nothing | Nothing |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | stateTag | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | operation | 1 |  |  |  |
|  |  |  | target | 1 |  |  |  |
|  |  |  | originator | 1 |  |  |  |
|  |  |  | requestID | 1 |  |  |  |
|  |  |  | metaInformation | 1 |  |  |  |
|  |  |  | primitiveContent | 0..1 |  |  |  |
|  |  |  | requestStatus | 1 |  |  |  |
|  |  |  | operationResult | 1 |  |  |  |
| <group> |  |  | resourceType | 1 | <semanticDescriptor> <subscription> <fanOutPoint> <semanticFanOutPoint> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | memberType | 1 |  |  |  |
|  |  |  | currentNrOfMembers | 1 |  |  |  |
|  |  |  | maxNrOfMembers | 1 |  |  |  |
|  |  |  | memberIDs | 1 (L) |  |  |  |
|  |  |  | membersAccessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | memberTypeValidated | 0..1 |  |  |  |
|  |  |  | consistencyStrategy | 1 |  |  |  |
|  |  |  | groupName | 0..1 |  |  |  |
|  |  |  | semanticSupportIndicator | 0..1 |  |  |  |
|  |  |  | notifyAggregation | 0..1 |  |  |  |
| <fanOutpoint> |  |  | (Virtual Resoure) |  |  |  |  |
| <SemanticFanOutpoint> |  |  | (Virtual Resoure) |  |  |  |  |
| <mgmtObj> |  |  | resourceType | 1 | <subscription> <semanticDescriptor> | + mgmtSchema | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | mgmtDefinition | 1 |  |  |  |
|  |  |  | objectIDs | 0..1 (L) |  |  |  |
|  |  |  | objectPaths | 0..1 (L) |  |  |  |
|  |  |  | mgmtLink | 0..1 (L) |  |  |  |
|  |  |  | [objectAttribute] | 0..n |  |  |  |
|  |  |  | description | 0..1 |  |  |  |
| <mgmtCmd> |  |  | resourceType | 1 | <subscription> <execInstance> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | description | 0..1 |  |  |  |
|  |  |  | cmdType | 1 |  |  |  |
|  |  |  | execReqArgs | 0..1 |  |  |  |
|  |  |  | execEnable | 1 |  |  |  |
|  |  |  | execTarget | 1 |  |  |  |
|  |  |  | execMode | 0..1 |  |  |  |
|  |  |  | execFrequency | 0..1 |  |  |  |
|  |  |  | execDelay | 0..1 |  |  |  |
|  |  |  | execNumber | 0..1 |  |  |  |
| <execInstance> |  |  | resourceType | 1 | <subscription> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | execStatus | 1 |  |  |  |
|  |  |  | execResult | 1 |  |  |  |
|  |  |  | execDisable | 0..1 |  |  |  |
|  |  |  | execTarget | 1 |  |  |  |
|  |  |  | execMode | 0..1 |  |  |  |
|  |  |  | execFrequency | 0..1 |  |  |  |
|  |  |  | execDelay | 0..1 |  |  |  |
|  |  |  | execNumber | 0..1 |  |  |  |
|  |  |  | execReqArgs | 0..1 (L) |  |  |  |
| <node> |  |  | resourceType | 1 | <semanticDescriptor> <mgmtObj> as defined in the specialization [memory] <mgmtObj> as defined in the specialization [battery] <mgmtObj> as defined in the specialization [areaNwkInfo] <mgmtObj> as defined in the specialization [areaNwkDeviceInfo] <mgmtObj> as defined in the specialization [firmware] <mgmtObj> as defined in the specialization [software] <mgmtObj> as defined in the specialization [deviceInfo] <mgmtObj> as defined in the specialization [deviceCapability] <mgmtObj> as defined in the specialization [reboot] <mgmtObj> as defined in the specialization [eventLog] <mgmtObj> as defined in the specialization [cmdhPolicy] <mgmtObj> as defined in the specialization [activeCmdhPolicy] <subscription> <trafficPattern> | + hostedAELinks + hostedServiceLinks + roamingStatus + networkID | + <transaction> + <schedule> - <trafficPattern> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | nodeID | 1 |  |  |  |
|  |  |  | hostedCSELink | 0..1 |  |  |  |
|  |  |  | mgmtClientAddress | 0..1 |  |  |  |
| <m2mServiceSubscriptionProfile> |  |  | resourceType | 1 | <subscription> <serviceSubscribedNode> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
| <serviceSubscribedNode> |  |  | resourceType | 1 | <subscription> | + niddRequired | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | nodeID | 1 |  |  |  |
|  |  |  | CSE-ID | 0..1 |  |  |  |
|  |  |  | deviceIdentifier | 0..1 (L) |  |  |  |
|  |  |  | ruleLinks | 0..1 ((L) |  |  |  |
| <pollingChannel> |  |  | resourceType | 1 | <pollingChannelURI> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
| <pollingChannelURI> |  |  | (Virtual Resoure) |  |  |  |  |
| <statsConfig> |  |  | resourceType | 1 | <eventConfig> <subscription> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
| <eventConfig> |  |  | resourceType | 1 | <subscription> | + eventResourceTypes + eventResourceIDs | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | eventID | 1 |  |  |  |
|  |  |  | eventType | 1 |  |  |  |
|  |  |  | eventStart | 0..1 |  |  |  |
|  |  |  | eventEnd | 0..1 |  |  |  |
|  |  |  | operationType | 0..1 (L) |  |  |  |
|  |  |  | dataSize | 0..1 |  |  |  |
| <statsCollect> |  |  | resourceType | 1 | <subscription> | The multiplicity of collectedEntityID changed into 0..1 (L) in Rel-3. | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | statsCollectID | 1 |  |  |  |
|  |  |  | collectingEntityID | 1 |  |  |  |
|  |  |  | collectedEntityID | 0..1 |  |  |  |
|  |  |  | statsRuleStatus | 1 |  |  |  |
|  |  |  | statModel | 1 |  |  |  |
|  |  |  | collectPeriod | 0..1 |  |  |  |
|  |  |  | eventID | 0..1 |  |  |  |
| <xxxAnnc> (Announced Resources) |  |  | resourceType resourceID resourceName parentID creationTime lastModifiedTime expirationTime link | Universal Attributes (Mandatory) |  | Nothing | Nothing |
|  |  |  | accessControlPolicyIDs stateTag labels | Common Attributes (Conditionally Mandatory) |  | + registrationStatus | Nothing |
| accessControlPolicyAnnc |  |  |  |  | subscription | Nothing | Nothing |
| AEAnnc |  |  |  |  | subscription, container, containerAnnc, flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc semanticDescriptor, semanticDescriptorAnnc, scheduleAnnc, trafficPatternAnnc, timeSeries, timeSeriesAnnc | Nothing | - <scheduleAnnc> - <trafficPatternAnnc> |
| containerAnnc |  |  |  |  | container, containerAnnc, flexContainer, flexContainerAnnc, contentInstance, contentInstanceAnnc, subscription, semanticDescriptor, semanticDescriptorAnnc | Nothing | + <timeSeries> + <timeSeriesAnnc> |
| contentInstanceAnnc |  |  |  |  | semanticDescriptor, semanticDescriptorAnnc | Nothing | Nothing |
| flexContainerAnnc |  |  |  |  | container, containerAnnc, flexContainer, flexContainerAnnc, subscription, semanticDescriptor, semanticDescriptorAnnc | Nothing | + <timeSeries> + <timeSeriesAnnc> |
| groupAnnc |  |  |  |  | subscription, semanticDescriptor, semanticDescriptorAnnc | Nothing | Nothing |
| locationPolicyAnnc |  |  |  |  | None specified | Nothing | Nothing |
| mgmtObjAnnc |  |  |  |  | subscription | Nothing | Nothing |
| nodeAnnc |  |  |  |  | mgmtObjAnnc, subscription, semanticDescriptor, semanticDescriptorAnnc, trafficPatternAnnc | Nothing | + <schduleAnnc> - <trafficPatternAnnc> |
| remoteCSEAnnc |  |  |  |  | container, containerAnnc, flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, scheduleAnnc, timeSeries, timeSeriesAnnc, remoteCSEAnnc, nodeAnnc, AEAnnc, locationPolicyAnnc | Nothing | - <contentInstanceAnnc> - <timeSeriesInstanceAnnc> - <mgmrObjAnnc> |
| scheduleAnnc |  |  |  |  | None specified | None specified | None specified |
| trafficPatternAnnc |  |  |  |  | scheduleAnnc subscription | <trafficPatternAnnc> is deleted in Rel-3. |  |
| semanticDescriptorAnnc |  |  |  |  | subscription | Nothing | Nothing |
| timeSeriesAnnc |  |  |  |  |  |  |  |
| timeSeriesInstanceAnnc |  |  |  |  |  |  |  |
| <latest> |  |  | (Virtual Resoure) |  |  |  |  |
| <oldest> |  |  | (Virtual Resoure) |  |  |  |  |
| <serviceSubscribedAppRule> |  |  | resourceType | 1 | <subscription> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | applicableCredIDs | 1 (L) |  |  |  |
|  |  |  | allowedApp-IDs | 0..1 (L) |  |  |  |
|  |  |  | allowedAEs | 0..1 (L) |  |  |  |
|  |  |  | allowedRole-IDs | 0..1 (L) |  |  |  |
| <semanticDescriptor> |  |  | resourceType | 1 | <subscription> | + semanticValidated + validationEnable | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | descriptorRepresentation | 1 |  |  |  |
|  |  |  | semanticOpExec | 0..1 |  |  |  |
|  |  |  | descriptor | 1 |  |  |  |
|  |  |  | ontologyRef | 0..1 |  |  |  |
|  |  |  | relatedSemantics | 0..1(L) |  |  |  |
| <notificationTargetMgmtPolicyRef> |  |  | resourceType | 1 | <subscription> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | notificationTargetURI | 1 (L) |  |  |  |
|  |  |  | notificationlPolicyID | 0..1 |  |  |  |
| <nortificationTargetPolicy> |  |  | resourceType | 1 | <policyDeletionRules> <subscription> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | action | 1 |  |  |  |
|  |  |  | policyLabel | 1 |  |  |  |
|  |  |  | rulesRelationship | 0..1 |  |  |  |
| <policyDeletionRules> |  |  | resourceType | 1 | <subscription> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | deletionRules | 0..1(L) |  |  |  |
|  |  |  | deletionRulesRelation | 0..1 |  |  |  |
| <notificationTargetSelfReference> |  |  | (Virtual Resoure) |  |  |  |  |
| <flexContaner> |  |  | resourceType | 1 | <semanticDescriptor> <subscription> <container> <flexContainer> | + nodeLink | + <transaction> + <timeSeries> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 0..1 (note) |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 0..1 (note) |  |  |  |
|  |  |  | lastModifiedTime | 0..1 (note) |  |  |  |
|  |  |  | stateTag | 1 |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 0..1 |  |  |  |
|  |  |  | containerDefinition | 1 |  |  |  |
|  |  |  | ontologyRef | 0..1 |  |  |  |
|  |  |  | contentSize | 1 |  |  |  |
|  |  |  | [customAttribute] | 0..n |  |  |  |
| <timeSeries> |  |  | resourceType | 1 | <semanticDescriptor> <timeSeriesInstance> <subscription> | + contentInfo | + <transaction> + <latest> + <oldest> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | stateTag | 1 |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | creator | 1 |  |  |  |
|  |  |  | maxNrOfInstances | 0..1 |  |  |  |
|  |  |  | maxByteSize | 0..1 |  |  |  |
|  |  |  | maxInstanceAge | 0..1 |  |  |  |
|  |  |  | currentNrOfnstances | 1 |  |  |  |
|  |  |  | currentByteSize | 1 |  |  |  |
|  |  |  | periodicInterval | 0..1 |  |  |  |
|  |  |  | missingDataDetect | 0..1 |  |  |  |
|  |  |  | ontologyRef | 0..1 |  |  |  |
|  |  |  | missingDataMaxNr | 0..1 |  |  |  |
|  |  |  | missingDataList | 0..1(L) |  |  |  |
|  |  |  | missingDataCurrentNr | 0..1 |  |  |  |
|  |  |  | missingDataDetectTimer | 0..1 |  |  |  |
| <timeSeriesInstance> |  |  | resourceType | 1 | none | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dataGenerationTime | 1 |  |  |  |
|  |  |  | content | 1 |  |  |  |
|  |  |  | contentSize | 1 |  |  |  |
|  |  |  | sequenceNr | 0..1 |  |  |  |
| <role> |  |  | resourceType | 1 | <subscription> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | roleID | 1 |  |  |  |
|  |  |  | issuer | 1 |  |  |  |
|  |  |  | holder | 1 |  |  |  |
|  |  |  | notBefore | 1 |  |  |  |
|  |  |  | notAfter | 1 |  |  |  |
|  |  |  | roleName | 0..1 |  |  |  |
|  |  |  | tokenLink | 0..1 |  |  |  |
| <token> |  |  | resourceType | 1 | <subscription> | Nothing | + <transaction> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | tokenID | 1 |  |  |  |
|  |  |  | tokenObject | 1 |  |  |  |
|  |  |  | version | 0..1 |  |  |  |
|  |  |  | issuer | 0..1 |  |  |  |
|  |  |  | audience | 0..1 (L) |  |  |  |
|  |  |  | holder | 0..1 |  |  |  |
|  |  |  | notBefore | 0..1 |  |  |  |
|  |  |  | notAfter | 0..1 |  |  |  |
|  |  |  | tokenName | 0..1 |  |  |  |
|  |  |  | permissions | 0..1 (L) |  |  |  |
|  |  |  | extension | 0..1 |  |  |  |
| <dynamicAuthorizationConsultation> |  |  | resourceType | 1 | none | Nothing | + <transaction> + <subscription> |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationEnabled | 1 |  |  |  |
|  |  |  | dynamicAuthorizationPoA | 1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationLifetime | 0..1 |  |  |  |
| <trafficPattern> |  |  | resourceType | 1 | <subscription> <schedule> | <trafficPattern> is deleted in Rel-3. |  |
|  |  |  | resourceID | 1 |  |  |  |
|  |  |  | resourceName | 1 |  |  |  |
|  |  |  | parentID | 1 |  |  |  |
|  |  |  | creationTime | 1 |  |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |  |
|  |  |  | expirationTime | 1 |  |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |  |
|  |  |  | labels | 0..1 (L) |  |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |  |
|  |  |  | announceTo | 0..1(L) |  |  |  |
|  |  |  | announcedAttribute | 0..1(L) |  |  |  |
|  |  |  | providedToNSE | 0..1 |  |  |  |
|  |  |  | periodicIndicator | 0..1 |  |  |  |
|  |  |  | periodicDurationTime | 0..1 |  |  |  |
|  |  |  | periodicIntervalTime | 0..1 |  |  |  |
|  |  |  | stationaryIndication | 0..1 |  |  |  |
|  |  |  | dataSizeIndicator | 0..1 |  |  |  |
|  |  |  | validityTime | 0..1 |  |  |  |
|  |  |  | targetNetwork | 1 |  |  |  |

| Resource Type | Short Introduction | Reference (minor) | Attribute (Rel-3) | Multiplicity (Rel-3) | Child Resource (Rel-3) | memo |
| --- | --- | --- | --- | --- | --- | --- |
| <xxxAnnc> (Announced Resources) |  |  | resourceType resourceID resourceName parentID creationTime lastModifiedTime expirationTime link | Universal Attributes (Mandatory) |  | Nothing |
|  |  |  | accessControlPolicyIDs stateTag labels | Common Attributes (Conditionally Mandatory) |  | Following attribute is added in Rel-3 as a common Attribute. ・ registrationStatus |
| timeSeriesAnnc |  |  |  |  |  | <timeSeriesAnnc> is newly added in Rel-3. This announced resource can have following Child Resources. ・ <timeSeriesInstance> ・ <timeSeriesInstanceAnnc> ・ <subscription> ・ <semanticDescriptor> ・ <semanticDescriptorAnnc> |
| timeSeriesInstanceAnnc |  |  |  |  |  | <timeSeriesInstanceAnnc> is newly added in Rel-3. Child Resources of this announced resource are not specified. |
| <authorizationDecision> |  |  | resourceType | 1 | <subscription> <transaction> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | decision | 0..1 |  |  |
|  |  |  | status | 0..1 |  |  |
|  |  |  | to | 0..1 |  |  |
|  |  |  | from | 0..1 |  |  |
|  |  |  | operation | 0..1 |  |  |
|  |  |  | requestedResourceType | 0..1 |  |  |
|  |  |  | filterUsage | 0..1 |  |  |
|  |  |  | roleIDs | 0..1 (L) |  |  |
|  |  |  | tokenIDs | 0..1 (L) |  |  |
|  |  |  | tokens | 0..1 (L) |  |  |
|  |  |  | requestTime | 0..1 |  |  |
|  |  |  | originatorLocation | 0..1 |  |  |
|  |  |  | originatorIP | 0..1 |  |  |
| <authorizationPolicy> |  |  | resourceType | 1 | <subscription> <transaction> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | policies | 0..1 (L) |  |  |
|  |  |  | combiningAlgorithm | 0..1 |  |  |
|  |  |  | status | 0..1 |  |  |
|  |  |  | to | 0..1 |  |  |
| <authorizationInformation> |  |  | resourceType | 1 | <role> <token> <subscription> <transaction> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | status | 0..1 |  |  |
|  |  |  | from | 0..1 |  |  |
|  |  |  | roleIDs | 0..1 (L) |  |  |
|  |  |  | tokenIDs | 0..1 (L) |  |  |
| <localMulticastGroup> |  |  | resourceType | 1 | <transaction> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | announceTo | 0..1(L) |  |  |
|  |  |  | announcedAttribute | 0..1(L) |  |  |
|  |  |  | externalGroupID | 0..1 |  |  |
|  |  |  | multicastAddress | 1 |  |  |
|  |  |  | multicastGroupFanoutTarget | 1 |  |  |
|  |  |  | memberList | 1(L) |  |  |
|  |  |  | responseTarget | 1 |  |  |
|  |  |  | responseTimeWindow | 0..1 |  |  |
|  |  |  | TMGI | 0..1 |  |  |
| <AEContactList> |  |  | resourceType | 1 | <subscription> <AEContactListPerCSE> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | numberImpactedCSEs | 1 |  |  |
| <AEContactListPerCSE> |  |  | resourceType | 1 | Nothing |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | CSE-ID | 1 |  |  |
|  |  |  | AE-IDList | 0..1(L) |  |  |
| <transactionMgmt> |  |  | resourceType | 1 | <subscription> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | creator | 1 |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | transactionLockTime | 0..1 |  |  |
|  |  |  | transactionExecuteTime | 0..1 |  |  |
|  |  |  | transactionCommitTime | 0..1 |  |  |
|  |  |  | transactionExpirationTime | 0..1 |  |  |
|  |  |  | transactionMode | 0..1 |  |  |
|  |  |  | transactionLockType | 0..1 |  |  |
|  |  |  | transactionControl | 0..1 |  |  |
|  |  |  | transactionState | 1 |  |  |
|  |  |  | transactionMaxRetries | 0..1 |  |  |
|  |  |  | transactionMgmtHandling | 0..1 |  |  |
|  |  |  | requestPrimitives | 1(L) |  |  |
|  |  |  | responsePrimitives | 1(L) |  |  |
| <transaction> |  |  | resourceType | 1 | <subscription> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | creator | 1 |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | transactionID | 1 |  |  |
|  |  |  | transactionControl | 1 |  |  |
|  |  |  | transactionState | 1 |  |  |
|  |  |  | transactionLockTime | 0..1 |  |  |
|  |  |  | transactionExecuteTime | 0..1 |  |  |
|  |  |  | transactionCommitTime | 0..1 |  |  |
|  |  |  | transactionLockType | 0..1 |  |  |
|  |  |  | requestPrimitive | 1 |  |  |
|  |  |  | responsePrimitive | 1 |  |  |
| <triggerRequest> |  |  | resourceType | 1 | <subscription> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | M2M-Ext-ID | 1 |  |  |
|  |  |  | Trigger-Recipient-ID | 1 |  |  |
|  |  |  | triggerPurpose | 1 |  |  |
|  |  |  | triggerPayloadSerialization | 1 |  |  |
|  |  |  | triggerStatus | 1 |  |  |
|  |  |  | triggerValidityTime | 0..1 |  |  |
|  |  |  | triggerInfoAE-ID | 0..1 |  |  |
|  |  |  | triggerInfoAddress | 0..1 |  |  |
|  |  |  | triggerInfoOperation | 0..1 |  |  |
|  |  |  | targetedResourceType | 0..1 |  |  |
| <ontologyRepository> |  |  | resourceName | 1 | <ontology> <semanticValidation> <subscription> |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creator | 0..1 |  |  |
| <ontology> |  |  | resourceName | 1 | <subscription> |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creator | 0..1 |  |  |
|  |  |  | description | 0..1 |  |  |
|  |  |  | ontologyFormat | 1 |  |  |
|  |  |  | ontologyContent | 1 |  |  |
|  |  |  | semanticOpExec | 0..1 |  |  |
| <semanticValidation> |  |  | (Virtual Resoure) |  |  |  |
| <semanticMashupJobProfile> |  |  | resourceType | 1 | <semanticMashupInstance> <semanticDescriptor> <subscription> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | stateTag | 1 |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creator | 0..1 |  |  |
|  |  |  | memberFilter | 1 |  |  |
|  |  |  | smiID | 0..1(L) |  |  |
|  |  |  | inputDescriptor | 0..1 |  |  |
|  |  |  | outputDescriptor | 1 |  |  |
|  |  |  | functionDescriptor | 1 |  |  |
| <semanticMashupInstance> |  |  | resourceType | 1 | <semanticMashupResult> <semanticDescriptor> <subscription> <mashup>(mandatory) |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | stateTag | 1 |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creator | 0..1 |  |  |
|  |  |  | smjpID | 1 |  |  |
|  |  |  | smjpInputParameter | 1 |  |  |
|  |  |  | memberStoreType | 1 |  |  |
|  |  |  | mashupMember | 0..1(L) |  |  |
|  |  |  | resultGenType | 1(L) |  |  |
|  |  |  | periodForResultGen | 0..1 |  |  |
| <mashup> |  |  | (Virtual Resoure) |  |  |  |
| <semanticMashupResult> |  |  | resourceType | 1 | <semanticDescriptor> <subscription> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | stateTag | 1 |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creator | 0..1 |  |  |
|  |  |  | smjpInputParameter | 0..1 |  |  |
|  |  |  | mashupResultFormat | 1 |  |  |
|  |  |  | mashupResult | 1 |  |  |
| <multimediaSession> |  |  | resourceType | 1 | <subscription> <accessControlPolicy> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |
|  |  |  | sessionOriginatorID | 1 |  |  |
|  |  |  | acceptedSessionDescriptions | 1(L) |  |  |
|  |  |  | offeredSessionDescriptions | 1(L) |  |  |
|  |  |  | sessionState | 1 |  |  |
| <crossResourceSubscription> |  |  | resourceType | 1 | <schedule> <notificationTargetMgmtPolicyRef> <notificationTargetSelfReference>(mandatory) <subscriptionLinkDeletion>(mandatory) |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | stateTag | 1 |  |  |
|  |  |  | announceTo | 0..1 (L) |  |  |
|  |  |  | announcedAttribute | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | creator | 1 |  |  |
|  |  |  | expirationCounter | 0..1 |  |  |
|  |  |  | notificationURI | 1 (L) |  |  |
|  |  |  | notificationContentType | 1 |  |  |
|  |  |  | notificationEventCat | 0..1 |  |  |
|  |  |  | subscriberURI | 0..1 |  |  |
|  |  |  | regularResourcesAsTarget | 0..1(L) |  |  |
|  |  |  | subscriptionResourcesAsTarget | 0..1(L) |  |  |
|  |  |  | timeWindowType | 1 |  |  |
|  |  |  | timeWindowSize | 1 |  |  |
|  |  |  | eventNotificationCriteriaSet | 0..1(L) |  |  |
| <subscriptionLinkDeletion> |  |  | (Virtual Resoure) |  |  |  |
| <backgroundDataTransfer> |  |  | resourceType | 1 | <subscription> |  |
|  |  |  | resourceID | 1 |  |  |
|  |  |  | resourceName | 1 |  |  |
|  |  |  | parentID | 1 |  |  |
|  |  |  | creationTime | 1 |  |  |
|  |  |  | lastModifiedTime | 1 |  |  |
|  |  |  | expirationTime | 1 |  |  |
|  |  |  | accessControlPolicyIDs | 0..1 (L) |  |  |
|  |  |  | dynamicAuthorizationConsultationIDs | 0..1 (L) |  |  |
|  |  |  | labels | 0..1 (L) |  |  |
|  |  |  | announceTo | 0..1(L) |  |  |
|  |  |  | volumePerNode | 1 |  |  |
|  |  |  | numberOfNodes | 1 |  |  |
|  |  |  | desiredTimeWindow | 0..1 |  |  |
|  |  |  | transferSelectionGuidance | 0..1(L) |  |  |
|  |  |  | geographicInformation | 0..1 |  |  |
|  |  |  | groupLink | 0..1 |  |  |
|  |  |  | memberIDs | 0..1 (L) |  |  |

## 7.3	CRUD processes

## -----------------------End of change 3-------------------------------------------

## -----------------------Start of change 4-------------------------------------------

# 8	Short Guide for Developers

## 8.1	Introduction

This clause gathers the pitfalls for users, service providers or developers about each release.

## -----------------------End of change 4-------------------------------------------