| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC 37 |
| Source:* | Youngjin Na, Hyundai Motor, yjra@hyundai.com Minbyeong Lee, Hyundai Motor, minbyeong.lee@hyundai.com Joonyoung Kim, Hyundai Motor, jkim@hyundai.com |
| Date:* | 2018-09-10 |
| Reason for Change/s:* | Action triggering improvement |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0035 Action triggering> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| _CR against: TS/TR* | TR-0021 v0.5.0 |
| Clauses * | 7.2.1, 7.2.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0001 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution contains several proposals for action triggering in TR-0021.

1. Action triggering based on multiple selective primary events

To handle multiple events, the  <dependency> resource is defined. According to the current mechanism, action triggering based on multiple events should be evaluated based on both evalCriteria for subject resource and depedent resource. However there is a case where an action triggering is invoked when at least one of evalCriteria among multiple events is satisfied. For example, ecall for vehicles can be triggered when airbag deployment signal is occured or an ecall button is pressed. Or remote air conditioning in the vehicle can be triggered when the temperature is higher than predefined point (for cooling) or the humidity is bigger than predefined point (for removing humidity in the grass). In order to consider these cases, for action triggering of multiple selective events subjectResourceID and evalCriteria should have 0..1(L) type multiplicity and evalCriteriaLogic should be defined such as OR logic.

2. Action triggering for multiple, ordered, and time-based operations

In addition, there exist a case where multiple operations should be performed based on specific order or specific time. For example, when vehicle is near to the home (satisfying evalCriteria), multiple operations can be happened such as 1) turning on air conditioning, and 2) turinging on the light. In this case, the operation 1) can be done befor operation 2) because operation 1) needs some operation time. And operation 2) can be done with detailed operation time. For this, operationResourceID and operation should have 0..1(L) type multiplicity and  objectResouceIDs should have ordered list and triggering time information.

3. Introducing time window mechanism into <dependency> resource

One or more <dependency> may exist to provide complex events for triggering. Multiple events may occur at difference time intervals. Thus time widnow mechanism can be used in determining whethere to send action triggering or not within a given timeframe. The resource host issues action triggering when evalCriteria is met within the time window. The timeWindowSize can be set to zero. To implement such a time window with zero value, when one of the designated event conditions is triggered, the resource host has to check if other event criteria are also met. The action triggering is sent if the conditions are substantially simultaneously satisfied.

### -----------------------Start of change 1-------------------------------------------

### 7.2.1	Action resource type

An <action> resource represents information about an action that is to be operated on a certain resource depending on certain conditions.

Figure 7.2.1-1: Structure of <action> resource

The <action> resource contains the child resources specified in table 7.2.1-1.

Table 7.2.1-1: Child resources of <action> resource

| Child Resources of <action> | Child Resource Type | Multiplicity | Description | <actionAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <dependency> | 0..n | See clause 7.2.2 | <dependencyAnnc> |

The <action> resource contains the attributes specified in table 7.2.1-2.

Table 7.2.1-2: Attribute of <action> resource

| Attributes of <action> | Multiplicity | RW/ RO/ WO | Description | <actionAnnc> Attributes |
| --- | --- | --- | --- | --- |
| subjectResourceID | 0..1 (L) | RW | The list of the resourceID of the resource that is the subject of monitoring for evaluation of the evalCriteria attribute. The action indicated by the operation attribute is to be triggered based on the change of the state of the resource indicated by the subjectResourceID attribute.The subject can be any resource in the system. If the attribute is missing the subject is the parent resource of this <action> resource. | OA |
| priority | 0..1 | RW | The attribute indicates the priority of the action when compared with the other actions with the same actionSubjectResource, with highest priority indicated by the lowest value. | OA |
| evalCriteriaLogic | 0..1(L) | RW | Indicates the logical operation (AND/OR) to be used for different evalCriteria of subjectResourceID. | OA |
| evalCriteria | 0..1 (L) | RW | This attribute provides the conditions determining if the action is to be conditionally triggered to the object resource. The evalCriteria attribute, in conjunction with the subjectResourceID, forms the event expression that is being monitored. | OA |
| evalMode | 1 | RW | Attribute provides the macro control mode of the evaluation of the <action> resource. Some values are: 0 = off 1 = once 2 = periodic 3 = continuous | OA |
| evalControlParam | 0..1 | RW | When evalMode= periodic, evalControlParam represents the periodicity. When evalMode= continuous the evaluation is restarted as soon as an action has been triggered. In this case, if evalControlParam is specified, it indicates the number of times to repeat an evaluation in continuous mode, otherwise it is repeated indefinitely. When evalMode is off or once this attribute can be ignored. | OA |
| evalPriority | 0..1 | RW | Prioritized list of the dependencies for evaluation. This is an ordered list of The resourceIDs of the child <dependency> resources. | OA |
| objectResourceID | 0..1 (L) | RW | List of object resources that the operation is going to operate on. The resource could be any of the resource in the system. This is an ordered list of objectResouceIDs. | MA |
| TriggeringTimeStamp | 0..1 (L) | RW | Attribute provides the time when the the operation for object resources is triggered. | OA |
| operation | 0..1 (L) | RW | The operation that is going to be performed. The operation could be Create, Update, Retrieve, Delete or Notify. | MA |
| input | 0..1 | RW | Input parameter for the operation. The attribute could be the value of the input parameter or could be the reference to the resource which is the input. | OA |
| outputResource | 0..1 | RO | The reference to the resource where the result of the action can be retrieved. | OA |
| actionResult | 1 | RO | The result of the action. If outputResource is set, the actionResult is the retrieved resource representation of the outputResource. Otherwise, it’s the result of the request. | OA |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 7.2.2	Dependency resource type

An <dependency> resource represents the condition that determines if the related parent <action> resource can be operated.

Figure 7.2.2-1: Structure of <dependency> resource

The <dependency> resource contains the attributes specified in table 7.2.2-1.

Table 7.2.2-1: Attribute of <dependency> resource

| Attributes of <dependency> | Multiplicity | RW/ RO/ WO | Description | <dependencyAnnc> Attributes |
| --- | --- | --- | --- | --- |
| logic | 0..1 | RW | Indicates if the dependency is SUFFICIENT or NECESSARY. The operand of the related parent <action> resource needs to match: Either of the sufficient dependency OR All of the necessary dependency | OA |
| filterCriteria | 0,,1 | RW | The filterCriteria to decide if the dependency matches the criteria. | OA |
| dependentResource | 1 | RW | The resourceID of the resource that the filterCriteria applies on. | MA |
| timeWindowSize | 0..1 (L) | RW | Indicates the time duration (e.g. in seconds) defining the time window size of corresponding action triggering shall be generated. | OA |

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?