| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #37 |
| Source:* | Dale Seed, Convida Wireless, Seed.Dale@ConvidaWireless.com Catalina Mladin, Convida Wireless, Mladin.Catalina@ConvidaWireless.com |
| Date:* | 2018-09-09 |
| Reason for Change/s:* | See Introduction |
| CR against: Release* | Rel-3 |
| CR against: WI* | Active (3GPP Interworking) MNT maintenance Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 v0.10.0 |
| Clauses * | Section 7.12 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This Contribution is proposed to use activityPatternElements attribute of <AE> or <remoteCSE> resource as an input to derive network parameters(maximumResponseTime and maximumLatency) to be configured over T8 interface in the Network Parameter Configuration procedure.

ARC-2018-0257-3GPP_Interworking_and_schedule.PPT details the need/motivation for this change.

R01 – Added text to address the case where the IN-CSE aggregates multiple activityPatternElements associated with the ADN-AE(s) and ASN/MN-CSE hosted on a UE.

### -----------------------Start of change 1-------------------------------------------

## 7.12 	Network Parameter Configuration

The 3GPP SCEF functionality described in 3GPP TS 23.682 [2] supports an API for Network Parameter Configuration which may be used by the IN-CSE to suggest to the 3GPP Mobile Network specific configuration parameters. The procedure may be used by the IN-CSE to influence certain aspects of UE/network behaviour such as the UE's PSM and extended idle mode DRX. For this purpose, parameter values may be suggested for Maximum Latency and Maximum Response Time for a UE. The Mobile Core Network may choose to accept, reject or modify (via the SCEF) the suggested configuration parameter value.

NOTE: Once the network provides the SCEF with the configured values, the MME could later change the values.  For example, the UE can be roaming and the visited network might not allow certain values.  In this scenario, the SCEF can request a reachability notification with the Idle Status Indication set to indicate that it wants to be notified of the UE’s active timer and periodic TAU/RAU timer when the UE enters the idle state.

In the 3GPP interworking architecture of oneM2M, the UE can host an ADN-AE or an ASN/MN-CSE. The UE Monitoring flow in Figure 7.12-1 takes place after the UE has attached to the 3GPP Network and the ADN-AE or ASN/MN-CSE is registered with the IN-CSE.

Figure 7.12 -1: Network Parameter Configuration flow

Step 0: UE Attach and oneM2M Registration Procedures

The UE attaches to the 3GPP network and the ADN-AE(s) or ASN/MN-CSE hosted on the UE perform the oneM2M registration procedure, as detailed in clause 6.3. The IN-CSE hosts the corresponding <AE> or <remoteCSE> resources and an associated <node> resource for the registree. During this procedure, a <schedule> resource is created as child of the <node> resource. The UE hosted ADN-AE(s) or ASN/MN may subscribe to the node <schedule> resource. If an IN-AE is interested in the reachability status of the UE, it may also subscribe to the <schedule> resources

Step 1: IN-CSE sends to the SCEF a Network Parameter Configuration request

This step is triggered by the creation of one or more <AE> or <remoteCSE> resources with the activityPatternElements attribute set (step 0a) or the modification of one or more  of the activityPatternElements attributes during an update of the <AE> or <remoteCSE> resources (step 0b).

The IN-CSE determines the SCEF based on the M2M-Ext-ID’s of the registree ASN/MN-CSE or ADN-AEs (e.g. either a DNS lookup on the M2M-Ext-ID or the based on the domain portion of the M2M-Ext-ID’s.).  The IN-CSE provides the network pattern to the SCEF, the fields of the API are populated as follows.

  - An HTTP POST method shall be used

  - URI shall be set to {apiRoot}/3gpp-network-parameter-configuration/v1/{scsAsId}/configurations/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

  - The request payload shall include a NpConfiguration data structure as specified in 3GPP TS 29.122 [5] with the following attributes:

    - externalId shall be set to M2M-Ext-ID

    - supportedFeatures – shall be set to a string value of “0” indicating no support for notifications over Websockets or notification test events.

    - maximumLatency – This value tells the network how long the UE is allowed to sleep.  Setting it to 0 will disable PSM, extended idle mode DRX, and S-GW buffering.  The IN-CSE shall extract the the active periodicity defined in the scheduleElement(s) of all the applicable activityPatternElements attributes of the <AE> resource(s) of the ADN-AE(s) and <remoteCSE> resource of the ASN/MN-CSE hosted by the UE, if configured. The IN-CSE shall set Maximum Latency to be approximately the periodicity of the active periods defined in the scheduleElement(s) of all the applicable activityPatternElements attributes of the <AE> resource(s) of the ADN-AE(s) and <remoteCSE> resource of the ASN/MN-CSE hosted by the UE, if configured. If there is no periodicity it is recommended not to utilize this parameter.

    - maximumResponseTime – When the UE uses PSM, Maximum Response Time tells the network how long the UE should stay reachable after a TAU.  When the UE uses eDRX, Maximum Response Time is used by the network to determine when to send a reachability notification before a UE’s paging occasion.  The IN-CSE extracts a duration of activity from the scheduleElement of all the applicable activityPatternElements attributes of the <AE> resource(s) of the ADN-AE(s) and <remoteCSE> resource of the ASN/MN-CSE hosted by the UE, if configured. The IN-CSE shall set Maximum Response Time to reflect this duration of activity, indicating how long the UE should stay reachable for downlink communications.

    - notificationDestination shall be set to a URI that the SCEF can target Network Parameter notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

    - msisdn, externalGroupId, requestTestNotification, groupReportingGuardTime, websockNotifConfig and suggestedNumberOfDlPackets are not supported by the present document and shall not be included.

General exceptions and error handling:

- If the SCEF is not reachable when the IN-CSE tries to send Network Parameter Configuration request then the IN-CSE may attempt to retry Network Parameter Configuration requests some time in the future in case the the SCEF does become reachable.  The conditions for when the IN-CSE should retry a Network Parameter Configuration request are not defined in the present document.

Step 2 and 3: Network Parameter Configuration Handling in the Underlying Network

The SCEF processes the request and sends a response to the IN-CSE to acknowledge the request has been accepted.  This message is defined in 3GPP TS 29.122 [5] and includes the following information.

- A response code of 201 CREATED

- The URI of the Network Parameter Configuration resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-network-parameter-configuration/v1/{scsAsId}/configurations/{configurationId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {configurationId} segment is configured by the SCEF.

- The response payload will include a NpConfiguration data structure as specified in 3GPP TS 29.122 [5] that includes the attributes present in the request along with the following additional attributes.  The response may include updated values of maximumLatency and maximumResponseTime if the network chose to use values that were different than the values that were suggested by the IN-CSE:

  - self is configured with a URI to the Network Parameter Configuration resource created by the SCEF for the request

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 4: Network Parameter Configuration Handling at the IN-CSE

The IN-CSE processes information received in the response which may include updated values of maximumLatency and  maximumResponseTime if the network chose to use values that were different than the values that were suggested by the IN-CSE.

Note that IN-CSE shall not update the activityPatternElements attribute based on the response received from SCEF. Any further updates to the UE reachability shall be reflected in the <schedule> resource.

Step 5: (Optional) If IN-AEs have subscribed to changes in the <schedule> resources, a notification will be sent to the subscribers when UE changes it reachability status such as transitioning to connected mode or idle mode. Notifications will also be sent to the UE hosted ADN-AE or ASN-CSE/MN-CSE, if subscriptions to their respective <schedule> resources have been created.

### -----------------------End of change 1-------------------------------------------