| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #37 |
| Source:* | Dale Seed, Convida Wireless, Seed.Dale@ConvidaWireless.com |
| Date:* | 2018-09-09 |
| Reason for Change/s:* | See Introduction |
| CR against: Release* | Rel-3 |
| CR against: WI* | Active (3GPP Interworking) MNT maintenance Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 v0.10.0 |
| Clauses * | Section 7.4.5 and 7.4.6 and 7.4.7.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes a correction for sending of a DELETE request of the Monitoring Event Subscription for IMSI, Roaming and Location Monitoring procedures.

The current procedure states that the DELETE request is sent when a <schedule> resource networkCoordinated attribute is updated to FALSE or when the ADN-AE(s) or ASN/MN-CSE de-register.

This contribution proposes to remove the <schedule> case for the DELETE since the <schedule> resource is not related to these procedures.

---------------------------------------- Start of Change 1--------------------------------------------------

### 7.4.5	Detecting Change of IMSI-IMEI(SV) Association

The 3GPP SCEF Event Monitoring functionality described in 3GPP TS 23.682 [2] supports an API that allows the IN-CSE to be informed when the SIM card of one physical device is placed in another physical device.  This condition is detected by the underlying 3GPP network when the association between the International Mobile Subscriber Identity (IMSI) and International Mobile Equipment Identity (IMEI/IMEISV) changes.

An IN-CSE may request to receive notifications from an underlying 3GPP network when the association between the IMSI and IMEI(SV) changes for a given UE that hosts one or more ASN/MN-CSEs or ADN-AEs registered to the IN-CSE.  Based on this notification, the IN-CSE may then ignore incoming requests from these ASN/MN-CSEs or ADN-AEs.

Figure 7.4.5-1: Change of IMSI-IMEI(SV) Association Monitoring Request and Notification

Pre-conditions:

There is a relationship in place between the IN-CSE and MNO allowing the IN-CSE to request notifications for IMSI-IMEI(SV) association changes.

An ASN/MN-CSE or ADN-AE registers with the IN-CSE and configures the M2M-Ext-ID attribute of its <remoteCSE> or <AE> resource. The IN-CSE examines the M2M-Ext-ID and recognizes that it is associated with an MNO that it has a relationship with.  The relationship allows the IN-CSE to request notifications when the device’s IMSI-IMEI(SV) association changes.

Step 1: IN-CSE requests notification for changes in IMSI-IMEI(SV) association

The IN-CSE determines whether it wants a notification if the IMSI-IMEI(SV) association of a particular UE hosting one or more ASN/MN-CSEs or ADN-AEs changes.  This determination may be based on whether the IN-CSE has an established relationship with the underlying 3GPP network operator that supports this capability and provisioned policies.

Step 2: Monitoring Request

The IN-CSE sends a request to be notified when the device’s IMSI-IMEI(SV) association changes.  The Monitoring Request contains information as specified in 3GPP TS 29.122 [5]Error: Reference source not found which includes the following:

- externalId shall be set to the M2M-Ext-ID of the UE hosting an ADN-AE or ASN/MN-CSE, which may have been provided during ADN-AE or ASN/MN-CSE registration.

- scsAsId shall be set to a value that is prearranged between the Service Provider and MNO.

- notificationDestination is set to a URI that the SCEF can target Change of IMSI/IMEI notifications towards.  The value of this URI shall be based on internal IN-CSE policies .

- tltrId is used to identify the Change of IMSI-IMEI(SV) Association Request.  It shall be assigned based on internal IN-CSE policies and shall be different from other tltrIds issued by this IN-CSE during the lifetime of this request.  Later, when a report is received, the value will be used to reference the original request so that the IN-CSE knows what M2M-Ext-ID the report pertains to.

- monitoringType shall be set to CHANGE_OF_IMSI_IMEI_ASSOCIATION.

- associationType shall be set to IMEI or IMEISV.  The type that is used is based on IN-CSE policies which may be based on the relationship between the Service Provider and MNO.

- maximumNumberOfReports and monitorExpireTime shall not be configured; thus indicating that a monitoring request may be cancelled after receiving one report.

Step 3: Process Monitoring Request

The SCEF processes the request

Step 4: Monitoring Response

The SCEF sends a Monitoring Event Subscription Response message to the IN-CSE with a response code of 201 CREATED to acknowledge acceptance of the Monitoring Event Subscription Request.

The message includes the following information.

- tltrId (same as in the request)

- id is a SCEF-chosen identifier for this Monitoring Event Subscription.

- self is configured with a link to the resource created by the SCEF for the request

Step 5: Detect change in IMSI-IMEI(SV) association

The underlying 3GPP network monitors and detects a IMSI-IMEI(SV) association change.

Step 6: Monitoring Indication

The SCEF sends a Monitoring Indication to the corresponding notificationDestination  of the IN-CSE that was configured in the Monitoring Request and that contains information as specified in 3GPP TS 29.122 [5]Error: Reference source not found. Such information includes:

- subscriptionId configured with the identifier of the subscription for which this notification corresponds to

- subscription configured with a link to the subscription resource for which this notification corresponds to

- tltrId (same as in the request)

- monitoringEventReports configured with one or more change of IMSI-IMEI association monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [5]:

  - externalID configured with the M2M-Ext-ID of the UE

  - monitoringType configured with CHANGE_OF_IMSI_IMEI_ASSOCIATION

  - associationType configured with IMSI-IMEI or IMSI-IMEISV

Step 7: Monitoring Indication Response

After receiving a Change of IMSI-IMEI Association Monitoring Event Notification, the IN-CSE returns a response having a response code of 204 NO CONTENT.

Step 8: Process Notification

The IN-CSE shall stop servicing requests from any ASN/MN-CSEs or ADN-AEs having an M2M-Ext-ID matching the one indicated in the report.   To block these requests, the IN-CSE may tear down any active security associations (e.g. D/TLS sessions) between the IN-CSE and these ADN-AEs or ASN/MN-CSEs.  In addition, the IN-CSE may also deny new security association establishment requests from any ADN-AEs or ASN/MN-CSEs that have an M2M-Ext-ID matching the one indicated in the report until the restriction is removed via administrative means which are outside the scope of the present document.

Step 9 (Optional): IN-CSE sends request to SCEF to delete Monitoring Event Subscription

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this Monitoring Event Subscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE.

Step 10 and 11 (Optional): Monitoring Event Subscription Delete Request Handling in the Underlying Network.

The SCEF handles the Monitoring Event Subscription Delete Request together with the Mobile Core Network. The SCEF sends a Monitoring Event Subscription Delete Response message to the IN-CSE with a response code of 204 NO CONTENT to acknowledge the Monitoring Event Subscription has been deleted.

---------------------------------------- End of Change 1---------------------------------------------------

---------------------------------------- Start of Change 2--------------------------------------------------

### 7.4.6	Scheduling communication based on Roaming Indications

The 3GPP SCEF Monitoring functionality described in 3GPP TS 23.682 [2] supports an API to allow an IN-CSE to be informed when the roaming status of a UE in the underlying 3GPP network changes.

An IN-CSE may request to receive indications from an underlying 3GPP network when the roaming status of a 3GPP UE hosting an ASN/MN-CSE or ADN-AE changes.  Based on these indications, the IN-CSE may buffer requests for ASN/MN-CSEs or ADN-AEs that are hosted on roaming UEs and send these requests when they are no longer roaming.  The determination of whether to buffer requests may be based on system policies that are outside the scope of the current document.  An IN-CSE may also make the roaming status of ASN/MN-CSEs or ADN-AEs available to IN-AEs such that they can use this status to determine whether to avoid and or delay communications until they are no longer roaming.

Figure 7.4.6-1: Request for Roaming Status Indications

Pre-conditions:

There is a relationship in place between the IN-CSE and MNO allowing the IN-CSE to request Roaming Status Reports from the underlying 3GPP network.  The method for establishing this relationship is outside the scope of the present document.

An ASN/MN-CSE or ADN-AE registers with the IN-CSE and configures the M2M-Ext-ID attribute of its <remoteCSE> or <AE> resource. The IN-CSE examines the M2M-Ext-ID and recognizes that it is associated with an MNO that it has a relationship with.

The ASN/MN-CSE or ADN-AE or IN-CSE creates a <node> resource. This <node> resource has roamingStatus and networkID attributes.

An IN-AE may subscribe to the IN-CSE to receive notifications when the roaming status of a ASN/MN-CSE or ADN-AE changes.  This subscription is made to the roamingStatus and/or networkID attributes of the <node> resource.

The IN-CSE may be configured with policies to control whether to buffer requests targeting ASN/MN-CSEs or ADN-AEs hosted on 3GPP UEs that are roaming.  The method for configuring these policies is outside the scope of the present document.

Step 1: IN-CSE determines to issue Network Status Request

If the IN-CSE can resolve the M2M-Ext-ID to a SCEF and IN-CSE policies allow for it, the IN-CSE issues a Roaming Status Request.

NOTE:	These IN-CSE policies are outside the scope of the present document.

Step 2: Roaming Status Request

The IN-CSE requests roaming status reports for a particular ASN/MN-CSE or ADN-AE hosted on a UE. The fields of the API are populated as follows.

- externalId shall be set to the M2M-Ext-ID of the UE hosting the targeted ASN/MN-CSE or ADN-AE.

- scsAsId shall be set to a value that is prearranged between the Service Provider and MNO.

- notificationDestination shall be set to a URI that the SCEF can target Roaming Status notifications towards.  The value of this URI shall be based on internal IN-CSE policies .

- tltrId is used to identify the Roaming Status Report request.  It shall be assigned based on internal IN-CSE policies and shall be different from other tltrIds issued by this IN-CSE during the lifetime of this request.  Later, when reports are received, the value shall be used to reference the original request so that the IN-CSE knows what request the report pertains to.

- maximumNumberOfReports shall be configured with the maximum number of roaming event reports that the SCEF shall generate before stopping.

- monitorExpireTime is used to indicate for how long reports will be sent.  For example, if duration is set to “1 day”, reports will be sent every time the roaming status changes within a 1 day period.  After the duration expires, the SCEF will stop sending reports.

- NOTE: 	If no monitorExpireTime is provided, then only one Roaming Status Report will be provided to the IN-CSE

- monitoringType shall be set to ROAMING_STATUS

- pLMNIndication shall be set to TRUE or FALSE to indicate whether the IN-CSE wants to know the identity of the PLMN that the UE is attached to.

Step 3: Process Roaming Status Request

The SCEF processes the request

Step 4: Roaming Status Response

The SCEF sends a Monitoring Event Subscription Response message to the IN-CSE with a response code of 201 CREATED to acknowledge acceptance of the Monitoring Event Subscription Request.

The message includes the following information.

- tltrId (same as in the request)

- id is a SCEF-chosen identifier for this Monitoring Event Subscription.

- self is configured with a link to the resource created by the SCEF for the request

Step 5: Detect Roaming Status Change

When the UE roams, a report will be sent to the SCEF.

Step 6: Roaming Status Report

The SCEF sends a Monitoring Indication to the corresponding notificationDestination of the IN-CSE that was configured in the Monitoring Request and that contains information as specified in 3GPP TS 29.122 [5]Error: Reference source not found. Such information includes:

- subscriptionId configured with the identifier of the subscription for which this notification corresponds to

- subscription configured with a link to the subscription resource for which this notification corresponds to

- tltrId (same as in the request)

- monitoringEventReports configured with one or more change of Roaming Status monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [5]:

  - externalID configured with the M2M-Ext-ID of the UE

  - monitoringType configured with ROAMING_STATUS

  - plmnId if pLMNIIndication in the Monitoring Event Subscription was set to TRUE, this parameter is included and indicate the UE's serving PLMN

  - roamingStatus is configured with ROAMING or NOT_ROAMING

Step 7: Roaming Status Acknowledgement

After receiving a Change of Roaming Status Monitoring Event Notification, the IN-CSE returns a response having a response code of 204 NO CONTENT.

Step 8: Process Network Status Report

The IN-CSE shall update the roamingStatus and networkID attributes of the <node> resource that is linked to <AE> or <remoteCSE> resources having a M2M-Ext-ID attribute that matches the M2M-Ext-ID configured in the Roaming Status Request.

In addition, the IN-CSE may determine to delay or reject the processing of requests targeting ASN/MN-CSEs or ADN-AEs hosted on roaming UEs via one or more of the following approaches.  How the IN-CSE makes this determination is outside the scope of the current document and may be based on policies and agreements with the mobile network operator.

- An IN-CSE may reject requests that target ASN/MN-CSEs or ADN-AEs hosted on roaming UEs.  If an IN-CSE rejects a request due to a roaming UE, it shall return a corresponding response code informing the cause of rejection is due to lack of accessibility to a roaming network node.

- An IN-CSE may delay the processing of requests (i.e. buffer) that target ASN/MN-CSEs or ADN-AEs hosted on roaming UEs.

  - If the request is a blocking request, the IN-CSE should not delay the processing of the request and should instead reject the request with a corresponding response code informing the cause of rejection is due to the destination node is roaming.

  - If the request includes an Event Category that is set to immediate the IN-CSE should not delay the processing of the request and should instead reject the request with a corresponding response code informing the cause of rejection is due to the destination node is roaming.  In this case, the IN-AE may decide to resubmit the request with the Event Category set to “bestEffort” or “latest” to indicate the IN-CSE may buffer the request.

Step 9 (Optional): Roaming Status Request Cancellation

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this Monitoring Event Subscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE.

Steps 10 and 11 (Optional): Process Roaming Status Cancellation Request

The SCEF handles the Monitoring Event Subscription Delete Request together with the Mobile Core Network. The SCEF sends a Monitoring Event Subscription Delete Response message to the IN-CSE with a response code of 204 NO CONTENT to acknowledge the Monitoring Event Subscription has been deleted

---------------------------------------- End of Change 2---------------------------------------------------

---------------------------------------- Start of Change 3--------------------------------------------------

#### 7.4.7.3 Location updating triggered by location change

Figure 7.4.7.3-1: Location updating triggered by location change

Step 1: Request for creation of <locationPolicy>

The IN-AE sends <locationPolicy> CREATE request which shall be included  following parameters as described in 3GPP TS29.122 [5]:

  - locationSource shall be set to Network Based.

  - locationUpdatePeriod shall not be present or set to 0.

  - locationTargetID shall be set to the M2M-Ext-ID of the targeted UE hosting an ASN/MN-CSE or ADN-AE.

  - locationServer shall not be present.

  - locationContainerID shall not be present.

  - locationInformationType shall be set to position fix.

  - retrieveLastKnownLocation shall not be configured.

  - locationUpdateEventCriteria shall be set to LocationChange

Step 2: Responds to creation <locationPolicy>

IN-CSE shall create the <locationPolicy> resource and corresponding <container> resource for storing location information and return a response based on the procedure specified in clause 10.2.9 of oneM2M TS-0001[1].

Step 3: Request Monitoring Event Subscription to get Current Location

The IN-CSE sends Monitoring Event Subscription to SCEF. The request contains information as specified in 3GPP TS 29.122 [5]. Such information includes :

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload shall include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [5] with the following attributes:

  - externalId shall be set to the locationTargetID.

  - notificationDestination shall be set to a URI that the SCEF can target Location Reporting notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - monitoringType shall be set to LOCATION_REPORTING

  - supportedFeatures shall be set to a string value of “5” indicating support for Location Reporting notifications.

  - maximumNumberOfReports is optional and may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - monitorExpireTime is optional and may be set to an absolute time at which the monitoring event request is considered to expire.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - locationType shall be set to CURRENT_LOCATION

  - accuracy shall be set set by the IN-CSE (e.g. CGI_ECGI, ENODEB, TA_RA, PLMN or TWAN_ID)

  - maximumDetectionTime shall be set per IN-CSE pre-provisioning. This value should be set to a value that is longer than the length of time of inactive communications as configured in the <schedule> resource

  - msisdn, ipv4Addr, ipv6Addr, externalGroupId, requestTestNotification, websockNotifConfig, groupReportGuardTime, maximumLatency, maximumResponseTime are not supported by the present document and shall not be included.

Step 4: Process Monitoring Event Subscription to get Current Location

The SCEF handles request for Monitoring Event Subscription together with 3GPP network entities based on the procedure defined in 3GPP TS 29.122 [5].

Step 5: Response to Monitoring Event Subscription for Current Location

The SCEF sends a Monitoring Event Subscription Response message to the IN-CSE to acknowledge acceptance of the Monitoring Event Subscription Request.

The message includes the following information.

- A response code of 201 CREATED

- The URI of the Monitoring Event Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {subscriptionId} segment is configured by the SCEF.

- The response payload will include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [5] that includes the attributes present in the request along with the following additional attributes:

  - self is configured with a URI to the resource created by the SCEF for the request

  - monitoringEventReport may be included if a monitoring event report is available.

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 6: Request for Creation of <subscription>

The IN-AE sends the <subscription> CREATE request of <container> which stores location information to the IN-CSE.

Step 7: Response for Creation of <Subscription>

The IN-CSE sends the response to the IN-AE.

Step 8: Detect Current Location change of UE

The 3GPP network entities send a Monitoring Event Report for the current Indication to SCEF when detecting a change in location of the UE hosting an ASN/MN-CSE or ADN-AE.

Step 9: Report Monitoring Notification for Current Location from SCEF  to IN-CSE

The SCEF sends a Monitoring Notification Report to the IN-CSE that contains information as specified in 3GPP TS 29.122 [5]Error: Reference source not found.

Otherwise, the Monitoring Notification message is sent with appropriate information in accordance with the monitoringType.

The Monitoring Notification report for LOCATION_REPORTING for current location shall include:

- An HTTP POST method is used

- URI is set to {notification_uri}.  The {notification_uri} is configured by the IN-CSE in the Monitoring Event Subscription Request.

- The request payload will include a MonitoringNotification data structure as specified in 3GPP TS 29.122 [5] with the following attributes:

  - subscription configured with a URI to the subscription resource for which this notification corresponds to

  - configResults is used for group-based monitoring and shall be ignored by the IN-CSE if present since group-based monitoring is not supported by the present document.

  - cancelind shall be ignored by the IN-CSE if present since it is not supported by the present document

  - monitoringEventReports configured Location Reporting monitoring report(s) wherein a report includes the following fields as defined in 3GPP TS 29.122 [5]:

    - externalID configured with the same value of M2M-Ext-ID of the UE as step 3.

    - monitoringType configured with LOCATION_REPORTING

    - locationInfo configured with the UE’s current location information (e.g. ageOfLocationInfo, cellId, enodeBId, routingAreaId, trackingAreaId, plmmId and twanId)

Step 10: Response to Monitring Notification for Current Location

After receiving a Location Reporting Monitoring Notification, the IN-CSE returns a response having a response code of 204 NO CONTENT.

Step 11: Handling Monitoring Notification for Current Location at the IN-CSE

The IN-CSE shall create a new <contentInstance> child resource of the <container> resource created in Step 2.  The IN-CSE shall store the UE’s current location in this <contentInstance> resource.

Step 12: Notify <contentInstance>

IN-CSE sends a NOTIFY request to the IN-AE.  The NOTIFY includes the <contentInstance> with the UE’s current location.

Step 13 (Optional): The IN-AE sends a request to update the locationUpdatePeriod  to 0 or <locationPolicy> is deleted.

Step 14 (Optional): The IN-CSE sends a response to the IN-AE.

Step 15 (Optional): IN-CSE requests to SCEF to delete Monitoring Event Subscription

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this Monitoring Event Subscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE.

The request is configured as follows.

- An HTTP DELETE method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId} which is identical to step 3.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies. The {subscriptionId} corresponds to the one configured by the SCEF and returned to the IN-CSE when the MonitoringEventSubscription was created.

- The request shall not contain a payload

Step 16 and 17 (Optional): Monitoring Event Subscription Delete Request Handling in the Underlying Network.

The SCEF handles request to delete the Monitoring Event Subscription together with the 3GPP network entities. The SCEF responds to the IN-CSE with a response code of 204 NO CONTENT to acknowledge the Monitoring Event Subscription has been deleted.

---------------------------------------- End of Change 3---------------------------------------------------