| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC37 |
| Source:* | Bei(Echo) Xu, Huawei, echo.xubei@huawei.com Catalina Mladin, Convida, mladin.catalina@convidawireless.com |
| Date:* | 2018-09-11 |
| Reason for Change/s:* | Add operation for new added <action> and <dependency> resource |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0035 Action Triggering MNT maintenance / < Work Item number(optional)> Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001, v3.11.0 |
| Clauses * | 10.2.Z |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

Add C/R/U/D operation for new added resources <action> and <dependency> based on the CR ARC-2018-0206.

### -----------------------Start of change 1-------------------------------------------

## 9.6	Resource Types

### 9.6.1      Overview

- 9.6.1.1	Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary – i.e. not announced – resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in Table 9.6.26.1-1 "Announced Resource Types".

Among the resource types listed in Table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

- container;

- contentInstance;

- flexContainer;

- timeSeries;

- timeSeriesInstance.

Table 9.6.1.1-1: Resource Types

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| accessControlPolicy | Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.2 |
| AE | Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE | subscription, container, flexContainer, group, accessControlPolicy, pollingChannel, semanticDescriptor, timeSeries, transaction, transactionMgmt, triggerRequest, crossResourceSubscription, backgroundDataTransfer, semanticMashupInstance, action | CSEBase | 9.6.5 |
| container | Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule | container, flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, CSEBase, flexContainer, flexContainerAnnc | 9.6.6 |
| contentInstance | Represents a data instance in the <container> resource | semanticDescriptor, transaction | Container, containerAnnc | 9.6.7 |
| flexContainer | A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure | container, flexContainer, subscription, semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, flexContainer, flexContainerAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.35 |
| CSEBase | The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself | remoteCSE, remoteCSEAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery, schedule, notificationTargetPolicy, flexContainer, timeSeries, AEContactList, transaction, transactionMgmt, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupInstance, action | None specified | 9.6.3 |
| delivery | Forwards requests from CSE to CSE | subscription, transaction | CSEBase | 9.6.11 |
| eventConfig | Defines events that trigger statistics collection | subscription, transaction | statsConfig | 9.6.24 |
| execInstance | Contains all execution instances of the same Management Command | subscription, transaction | mgmtCmd | 9.6.17 |
| fanOutPoint (V) | Virtual resource containing target for group request It is used for addressing bulk operations to all the resources that belong to a group | None specified | group | 9.6.14 |
| group | Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group | fanOutPoint, subscription, semanticFanOutPoint, semanticDescriptor, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.13 |
| latest (V) | Virtual resource that points to most recently created <contentInstance> and <timeSeriesInstance> child resource within a <container> and a <timeSeries> resource | None specified | container, timeSeries | 9.6.27 |
| locationPolicy | Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information | subscription, transaction | CSEBase | 9.6.10 |
| mgmtCmd | Management Command resource represents a method to execute management procedures required by existing management protocols | execInstance, subscription, transaction | CSEBase | 9.6.16 |
| mgmtObj | Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note) | subscription, mgmtObj, transaction | node, mgmtObj, mgmtObjAnnc | 9.6.15 Annex D |
| m2mServiceSubscriptionProfile | Data pertaining to the M2M Service Subscription | serviceSubscribedNode, subscription, transaction | CSEBase | 9.6.19 |
| node | Represents specific Node information | mgmtObj, subscription, semanticDescriptor, schedule, transaction, action | CSEBase | 9.6.18 |
| notificationTargetMgmtPolicyRef | Represents a list of notification targets and the deletion policy | subscription, transaction | subscription | 9.6.31 |
| notificationTargetPolicy | Represents a notification target deletion policy with pre-defined action and deletion rules | subscription, policyDeletionRules, transaction | CSEBase | 9.6.32 |
| notificationTargetSelfReference (V) | Virtual resource used to remove the Notification Target | None specified | subscription | 9.6.34 |
| oldest (V) | Virtual resource that points to first created <contentInstance> and <timeSeriesInstance> child resource within a <container> and a <timeSeries> resource | None specified | container, timeSeries | 9.6.28 |
| pollingChannel | Represent a channel that can be used for a request-unreachable entity | pollingChannelURI | remoteCSE, AE | 9.6.21 |
| pollingChannelURI (V) | Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity | None specified | pollingChannel | 9.6.22 |
| policyDeletionRules | Represents a set of rules which is associated with notification target removal policy | subscription, transaction | notificationTargetPolicy | 9.6.33 |
| remoteCSE | Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource | container, containerAnnc, flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, pollingChannel, timeSeries, timeSeriesAnnc, remoteCSEAnnc, nodeAnnc, AEAnnc, locationPolicyAnnc, transaction, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupJobProfileAnnc, semanticMashupInstance, semanticMashupInstanceAnnc, action | CSEBase | 9.6.4 |
| request | Expresses/access context of an issued Request | subscription, transaction | CSEBase | 9.6.12 |
| schedule | Contains scheduling information for delivery of messages | subscription, transaction | subscription, CSEBase, - node | 9.6.9 |
| serviceSubscribedNode | Node information | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.20 |
| statsCollect | Defines triggers for the IN-CSE to collect statistics for applications | subscription, transaction | CSEBase (in IN-CSE) | 9.6.25 |
| statsConfig | Stores configuration of statistics for applications | eventConfig, subscription, transaction | CSEBase (in IN-CSE) | 9.6.23 |
| subscription | Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribe-to resource | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | accessControlPolicy, accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, request, schedule, scheduleAnnc, semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig, flexContainer, flexContainerAnnc, timeSeries, timeSeriesAnnc | 9.6.8 |
| serviceSubscribedAppRule | Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE | subscription, transaction | CSEBase | 9.6.29 |
| semanticDescriptor | Stores semantic description pertaining to a resource and potentially sub-resources. | subscription, transaction | AE, container, contentInstance, group, node, flexContainer, timeSeries | 9.6.30 |
| semanticFanOutPoint | Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource | transaction | group | 9.6.14a |
| dynamicAuthorizationConsultation | Represents consultation information used by a CSE when performing consultation-based dynamic authorization | transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.40 |
| timeSeries | Stores and Shares Time Series Data instances among entities. | timeSeriesInstance, subscription, semanticDescriptor, latest, oldest, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, container, containerAnnc, flexContainer, flexContainerAnnc | 9.6.36 |
| timeSeriesInstance | Represents a Time Series Data instance in the <timeSeries> resource | transaction | timeSeries, timeSeriesAnnc | 9.6.37 |
| authorizationDecision | Represents an access control decision point | subscription, transaction | CSEBase | 9.6.41 |
| authorizationPolicy | Represents an access control policy retrieval point | subscription, transaction | CSEBase | 9.6.42 |
| authorizationInformation | Represents an access control information point | role token subscription, transaction | CSEBase | 9.6.43 |
| localMulticastGroup | Stores local multicast group information of member hosting CSE. | transaction | CSEBase | 9.6.44 |
| AEContactList | Contains information about a CSE that has resources that referencing an AE-ID | AEContactListPerCSE, subscription | CSEBase | 9.6.45 |
| AEContactListPerCSE | Contains information about a CSE that has resources that referencing an AE resource identifier for tracking purposes | None specified | AEContactList | 9.6.46 |
| transactionMgmt |  | subscription | CSEBase, AE, remoteCSE | 9.6.47 |
| transaction |  | actions | All non-virtual resource types with the exception of the following: request, delivery, pollingChannel, transactionMgmt, transaction | 9.6.48 |
| triggerRequest | Used by an AE to initiate, replace or recall a device trigger request | subscription | AE | 9.6.49 |
| ontologyRepository | Represents the collection of the managed ontologies and the semantic validation service | ontology, semanticValidation, subscription | CSEBase | 9.6.50 |
| ontology | Store the representation of an ontology | subscription | ontologyRepository | 9.6.51 |
| semanticValidation | A virtual resource as the interface to perform semantic validation on the received <semanticDescriptor> resource against the referenced ontology. | None specified | ontologyRepository | 9.6.52 |
| semanticMashupJobProfile | Represents the profile and description of a semantic mashup service | semanticMashupInstance, semanticDescriptor, subscription | CSEBase, remoteCSE | 9.6.53 |
| semanitcMashupInstance | Represents a semantic mashup instance | semanticMashupResult, semanticDescriptor, mashup, subscription | semanticMashupJobProfile, AE, remoteCSE, CSEBase | 9.6.54 |
| mashup | A virtual resource use to trigger the calculation and generation of new mashup result | Not specified | semanticMashupInstance | 9.6.55 |
| semanticMashupResult | Represent semantic mashup results | semanticDescriptor, subscription | semanticMashupInstance | 9.6.56 |
| multimediaSession | Stores a representation of a multimedia session information requested by a registering AE | subscription, accessControlPolicy | AE | 9.6.57 |
| crossResourceSubscription | represents the cross-resource subscription information related to multiple subscribed-to resources. Such a resource shall include a list of subscribed-to resources as its attribute, or shall be created as a child resource of a <group> resource where member resources shall be the subscribed-to resources. | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, subscriptionLinkDeletion | CSEBase, remoteCSE, AE | 9.6.58 |
| subscriptionLinkDeletion | Virtual resource used to remove a <subscription> resource from the subscriptionResourcesAsTarget list of a <crossResourceSubscription> resource. | None specified | crossResourceSubscription | 9.6.59 |
| backgroundDataTransfer | Stores information for a background data transfer request | None specified | AE, remoteCSE, CSEBase | 9.6.60 |
| action | Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource | dependency | CSEBase, remoteCSE, node, AE, container, flexContainer, group, timeSeries, transaction | 9.6.X |
| dependency | Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource | None specified | action | 9.6.Y |
| NOTE: See clause 9.6.12 for a summary of specializations of <mgmtObj>. |  |  |  |  |

### resource

### resource

9.6.4	Resource Type remoteCSE

A <remoteCSE> resource shall represent a Registree CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase> resource of Registrar CSE.

Similarly <remoteCSE> resource shall also represent a Registrar CSE. <remoteCSE> resource shall be located directly under the <CSEBase> resource of Registree CSE.

For example, when CSE1 (Registree CSE) registers with CSE2 (Registrar CSE), there will be two <remoteCSE> resources created: one in CSE1: <CSEBase1>/<remoteCSE2> and one in CSE2: <CSEBase2>/<remoteCSE1>.

Note that the creation of the two resources does not imply mutual registration. The <CSEBase1>/<remoteCSE2> does not mean CSE2 registered with CSE1 in the example above.

The <remoteCSE> resource shall contain the child resources specified in table 9.6.4-1. The <remoteCSE> resource may contain <remoteCSEAnnc> child resources.

Table 9.6.4-1: Child resources of <remoteCSE> resource

| Child Resources of <remoteCSE> | Child Resource Type | Multiplicity | Description | <remoteCSEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> |
| [variable] | <containerAnnc> | 0..n | Announced variant of <container>. See clause 9.6.6 | <containerAnnc> |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 | <flexContainer> |
| [variable] | <flexContaineAnnc> | 0..n | Announced variant of <flexContainer>. See clause 9.6.35 | <flexContainerAnnc> |
| [variable] | <group> | 0..n | See clause 9.6.13 | <group> |
| [variable] | <groupAnnc> | 0..n | Announced variant of <group>. See clause 9.6.13 | <groupAnnc> |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 | <accessControlPolicy> |
| [variable] | <accessControlPolicyAnnc> | 0..n | Announced variant of <accessControlPolicy>. See clause 9.6.2 | <accessControlPolicyAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <pollingChannel> | 0..1 | See clause 9.6.21. If requestReachability is FALSE, the CSE that created this <remoteCSE> resource should create a <pollingChannel> resource and perform long polling. The <pollingChannel> shall be utilized by the parent resource. | None |
| [variable] | <nodeAnnc> | 0..n | Announced variant of <node>. This announced resource is associated with a <node> resource that is hosted on a CSE which is represented by the parent <remoteCSE> or <remoteCSEAnnc> resource. See clause 9.6.18 for <node>. | <nodeAnnc> |
| [variable] | <dynamicAuthorizationConsultation> | 0..n | See clause 9.6.40 |  |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries> |
| [variable] | <timeSeriesAnnc> | 0..n | Announced variant of <timeSeries>. See clause 9.6.36 | <timeSeriesAnnc> |
| [variable] | <remoteCSEAnnc> | 0..n | Announced variant of <remoteCSE> defined in the present clause 9.6.4. | <remoteCSEAnnc> |
| [variable] | <AEAnnc> | 0..n | Announced variant of <AE>. See clause 9.6.5 | <AEAnnc> |
| [variable] | <locationPolicyAnnc> | 0..n | Announced variant of <locationPolicy>. See clause 9.6.10 | <locationPolicyAnnc> |
| [variable] | <transactionMgmt> | 0..n | See clause 9.6.47 | <transactionMgmt> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <ontologyRepositoryAnnc> | 0..1 | Announced variant of <ontologyRepository>. See clause 9.6.50 | <ontologyRepositoryAnnc> |
| [variable] | <semanticMashupJobProfile> | 0..n | See clause 9.6.53 | <semanticMashupJobProfile> |
| [variable] | <semanticMashupJobProfileAnnc> | 0..n | Announced variant of <semanticMashupJobProfile>. See clause 9.6.53 | <semanticMashupJobProfileAnnc> |
| [variable] | <semanticMashupInstance> | 0..n | See clause 9.6.54 | <semanticMashupInstance> |
| [variable] | <semanticMashupInstanceAnnc> | 0..n | Announced variant of <semanticMashupInstance >. See clause 9.6.54. | <semanticMashupInstanceAnnc> |
| [variable] | <action> | 0..n | See clause 9.6.x | <actionAnnc> |

### 9.6.5	Resource Type AE

An <AE> resource shall represent information about an Application Entity registered to a CSE.

The <AE> resource shall contain the child resources specified in table 9.6.5-1.

Table 9.6.5-1: Child resources of <AE> resource

| Child Resources of <AE> | Child Resource Type | Multiplicity | Description | <AEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 | <flexContainer> <flexContainerAnnc> |
| [variable] | <group> | 0..n | See clause 9.6.13 | <group> <groupAnnc> |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 | <accessControlPolicy> <accessControlPolicyAnnc> |
| [variable] | <pollingChannel> | 0..1 | See clause 9.6.21 When the AE is request-unreachable, the AE should create this <pollingChannel> resource and perform long polling. The <pollingChannel> shall be utilized by the parent resource | None |
| [variable] | <dynamicAuthorizationConsultation> | 0..n | See clause 9.6.40 | None |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries> <timeSeriesAnnc> |
| [variable] | <transactionMgmt> | 0..n | See clause 9.6.47 | <transactionMgmt> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <triggerRequest> | 0..n | See clause 9.6.49 | None |
| [variable] | <multimediaSession> | 0..n | See Clause 9.6.57. This resources holds information describing the established multimedia session | None |
| [variable] | <semanticMashupInstance> | 0..n | See clause 9.6.54 | <semanticMashupInstance> <semanticMashupInstanceAnnc> |
| [variable] | <action> | 0..n | See clause 9.6.x | None |

### 9.6.6	Resource Type container

The <container> resource represents a container for data instances. It is used to share information with other entities and potentially to track the data. A <container> resource has no associated content. It has only attributes and child resources.

The <container> resource shall contain the child resources specified in table 9.6.6-1.

Table 9.6.6-1: Child resources of <container> resource

| Child Resources of <container> | Child Resource Type | Multiplicity | Description | <containerAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <contentInstance> | 0..n | See clause 9.6.7 | <contentInstance>, <contentInstanceAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 | <flexContainer> <flexContainerAnnc> |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries>, <timeSeriesAnnc> |
| la | <latest> | 1 | See clause 9.6.27 | None |
| ol | <oldest> | 1 | See clause 9.6.28 | None |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <action> | 0..n | See clause 9.6.x | None |

### 9.6.13	Resource Type group

The <group> resource represents a group of resources of the same or mixed types. The <group> resource can be used to do bulk manipulations on the resources represented by the memberIDs attribute. The <group> resource contains an attribute that represents the members of the group and the <fanOutPoint> virtual resource that enables generic operations to be applied to all the resources represented by those members. By grouping <semanticDescriptor> resources across which a semantic description is distributed, another virtual resource (<semanticFanOutPoint>) enables semantic discovery procedures to be applied across the full logical tree in the description.

Members of a <group> resource may support unicast or multicast communication. In case multiple members of a <group> resource support multicast communications and they share the same multicast address, those members form a multicast group as a sub-set of the group. There may be multiple multicast groups corresponded to one group since the members of the group may use different multicast mechanisms (e.g. 3GPP MBMS vs. IP multicast) and different multicast addresses.

The <group> resource shall contain the child resources specified in table 9.6.13-1.

Table 9.6.13-1: Child resources of <group> resource

| Child Resources of <group> | Child Resource Type | Multiplicity | Description | <groupAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| fopt | <fanOutPoint> | 1 | See clause 9.6.14 | None |
| sfop | <semanticFanOutPoint> | 0..1 | See clause 9.6.14a | None |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <action> | 0..n | See clause 9.6.x | None |

### 9.6.18	Resource Type node

The <node> resource represents specific information that provides properties of an M2M Node that can be utilized by other oneM2M operations. The <node> resource has specialization of the <mgmtObj> as its child resources. These resources represent the Node's context information (e.g. memory and battery), network topology, device information, device capability etc. The specialized <mgmtObj> resources are used to perform management of the Node.

This node specific information stored in these resources such as [memory] and [battery] can be obtained either by the existing device management technologies (OMA DM [Error: Reference source not found], BBF TR-069 [Error: Reference source not found]) or any other way (e.g. JNI [Error: Reference source not found]).

For the case when the <node> resource belongs to an ADN, please see figure 9.6.18-1 in conjunction with the description of nodeLink attribute in the <AE> resource (clause 9.6.5).

Figure 9.6.18-1: Relationship between IN/MN and ADN

The <node> resource shall contain the child resources specified in table 9.6.18-1.

Table 9.6.18-1: Child resources of <node> resource

| Child Resources of <node> | Child Resource Type | Multiplicity | Description | <nodeAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [memory] | 0..1 | This resource provides the memory (typically RAM) information of the node. (E.g. the amount of total volatile memory), See clause D.4. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [battery] | 0..n | The resource provides the power information of the node. (E.g. remaining battery charge). See clause D.7. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [areaNwkInfo] | 0..n | This resource describes the list of Nodes attached behind the MN node and its physical or underlying relation among the nodes in the M2M Area Network. This attribute is defined in case the Node is MN. See clause D.5. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [areaNwkDeviceInfo] | 0..n | This resource describes the information about the Node in the M2M Area Network. See clause D.6. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [firmware] | 0..n | This resource describes the information about the firmware of the Node include name, version etc. See clause D.2. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [software] | 0..n | This resource describes the information about the software of the Node. See clause D.3. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [deviceInfo] | 0..n | The resource contains information about the identity, manufacturer and model number of the device. See clause D.8. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [deviceCapability] | 0..n | The resource contains information about the capability supported by the Node. See clause D.9. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [reboot] | 0..1 | The resource is the place to reboot or reset the Node. See clause D.10. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [eventLog] | 0..1 | The resource contains the information about the log of events of the Node. See clause D.11. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [cmdhPolicy] | 0..n | The resource(s) contain(s) information about CMDH policies that are applicable to the CMDH processing on the CSE hosted on the node represented by this <node> resource and identified by the hostedCSELink attribute of this <node> resource. See clause D.12. | NA |
| [variable] | <mgmtObj> as defined in the specialization [activeCmdhPolicy] | 0..1 | This resource defines which of the present [cmdhPolicy] resource(s) shall be active for the CMDH processing on the CSE hosted on the node represented by this <node> resource and identified by the hostedCSELink attribute of this <node> resource. See clause D.12. | NA |
| [variable] | <subscription> | 0..n | See clause 9.6.8. | <subscription> |
| [variable] | <schedule> | 0..n | See clause 9.6.9. | <scheduleAnnc> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <action> | 0..n | See clause 9.6.x | None |

### 9.6.35	Resource Type flexContainer

The <flexContainer> resource type is a customizable container for data instances. It is a template for the definition of flexible specializations of data containers. Like a <container> resource, specializations of this <flexContainer> resource type are used to share information with other entities and potentially to track the data. While the <container> resources includes data to be made accessible to oneM2M entities inside <contentInstance> children, a specialization of the <flexContainer> resource includes associated content directly inside the <flexContainer> by means of one or more [customAttribute] attribute(s). The attribute name and attribute data type of [customAttribute] attributes are defined explicitly for each specialization of <flexContainer>, i.e. the specific set of attribute name and type are defined in a corresponding XSD-file.

Example usage of <flexContainer>: As a specialization of <flexContainer> that includes two [customAttribute] attributes, named "temperature"(xs:float type) and "humidity"(xs:positiveInteger type) can be specified in some TS. The actual data types of [customAttribute] will be described both in the specification document or XSD file which are referred by the value of containerDefinition attribute.

The <flexContainer> resource shall contain the child resource specified in table 9.6.35-1.

Table 9.6.35-1: Child resources of <flexContainer> resource

| Child Resources of <flexContainer> | Child Resource Type | Multiplicity | Description | <flexContainerAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |
| [variable] | <flexContainer> | 0..n | <flexContainer> resource can include any of its specializations as child resource | <flexContainer> <flexContainerAnnc> |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries>, <timeSeriesAnnc> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <action> | 0..n | See clause 9.6.x | None |

### 9.6.36	Resource Type timeSeries

The <timeSeries> resource represents a container for Time Series Data instances. It is used to share information with other entities and potentially to track, detect and report the missing data in Time Series. A <timeSeries> resource has no associated content. It has only attributes and child resources.

Table 9.6.36-1: Child resources of <timeSeries> resource

| Child Resources of <timeSeries> | Child Resource Type | Multiplicity | Description | <timeSeriesAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <timeSeriesInstance> | 0..n | See clause 9.6.37 | <timeSeriesInstance>, <timeSeriesInstanceAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| la | <latest> | 1 | See clause 9.6.27 | None |
| ol | <oldest> | 1 | See clause 9.6.28 | None |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <action> | 0..n | See clause 9.6.x | None |

### 9.6.48	Resource Type transaction

The <transaction> resource is used to initiate and manage the atomic and consistent processing of a single oneM2M request primitive of a oneM2M transaction.

With the exception of the <request>, <delivery>, <transaction> and <transactionMgmt> resources, a <transaction> resource may be created as a child resource of any resource targeted by a oneM2M transaction.  A <transaction> create request may be originated by a CSE that hosts an associated <transactionMgmt> resource.  Alternatively, a <transaction> resource can be used independent of <transactionMgmt> resource for the case where an application wishes to create individual <transaction> resources itself.

The <transaction> resource supports the child resources specified in table 9.6.48-1.

Table 9.6.48-1: Child resources of <transaction> resource

| Child Resources of <transaction> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <action> | 0..n | See clause 9.6.x |

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 10.2.Z	Action Triggering

#### 10.2.z.3 <action> Resource Procedures

##### 10.2.z.3.1 Create <action>

The hosting CSE shall use this procedure to create an <action> resource according to the resource identifier of the parent resource.

Table 10.2.z.3.1-1: <action> CREATE

| <action> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: Content: The resource content shall provide the information as defined in clause 9.6.x |
| Processing at Originator before sending Request | According to clause 10.1.2 |
| Processing at Receiver | According to clause 10.1.2 After successfully creating the <action> resource, the Receiver shall perform the procedure specified in 10.2.Z.2. |
| Information in Response message | According to clause 10.1.2 |
| Processing at Originator after receiving Response | According to clause 10.1.2 |
| Exceptions | According to clause 10.1.2 |

##### 10.2.z.3.2	Retrieve <action>

This procedure shall be used for retrieving the attributes of an <action> resource.

Table 10.2.z.3.2-1: <action> RETRIEVE

| <action> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, and Mcc |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.3 |
| Processing at Receiver | According to clause 10.1.3 |
| Information in Response message | All parameters defined in table 8.1.3-1 apply. |
| Processing at Originator after receiving Response | According to clause 10.1.3 |
| Exceptions | According to clause 10.1.3 |

##### 10.2.z.3.3	Update <action>

This procedure shall be used for updating attributes of an <action> resource.

Table 10.2.z.3.3-1: <action> UPDATE

| <action> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.4 |
| Processing at Receiver | According to clause 10.1.4 After successfully updating the <action> resource, the Receiver shall perform the procedure specified in 10.2.Z.2. |
| Information in Response message | According to clause 10.1.4 |
| Processing at Originator after receiving Response | According to clause 10.1.4 |
| Exceptions | According to clause 10.1.4 |

##### 10.2.z.3.4	Delete <action>

This procedure shall be used for deleting an <action> resource.

Table 10.2.z.3.4-1: <action> DELETE

| <action> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, and Mcc |
| Information in Request message | All parameters defined in table 8.1.2-2 apply |
| Processing at Originator before sending Request | According to clause 10.1.5 |
| Processing at Receiver | According to clause 10.1.5 After successfully deleting the <action> resource, the Receiver shall stop the event monitoring procedure specified in 10.2.Z.2. |
| Information in Response message | According to clause 10.1.5 |
| Processing at Originator after receiving Response | According to clause 10.1.5 |
| Exceptions | According to clause 10.1.5 |

#### 10.2.z.4 <dependency> Resource Procedures

##### 10.2.z.4.1 Create <dependency>

The hosting CSE shall use this procedure to create an <dependency> resource according to the resource identifier of the parent resource.

Table 10.2.z.4.1-1: <dependency> CREATE

| <dependency> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: Content: The resource content shall provide the information as defined in clause 9.6.x |
| Processing at Originator before sending Request | According to clause 10.1.2 |
| Processing at Receiver | According to clause 10.1.2 After successfully creating the <dependency> resource, the Receiver shall perform the procedure specified in 10.2.Z.2. |
| Information in Response message | According to clause 10.1.2 |
| Processing at Originator after receiving Response | According to clause 10.1.2 |
| Exceptions | According to clause 10.1.2 |

##### 10.2.z.4.2	Retrieve <dependency>

This procedure shall be used for retrieving the attributes of an <dependency> resource.

Table 10.2.z.4.2-1: <dependency> RETRIEVE

| <dependency> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, and Mcc |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.3 |
| Processing at Receiver | According to clause 10.1.3 |
| Information in Response message | All parameters defined in table 8.1.3-1 apply. |
| Processing at Originator after receiving Response | According to clause 10.1.3 |
| Exceptions | According to clause 10.1.3 |

##### 10.2.z.4.3	Update <dependency>

This procedure shall be used for updating attributes of an <dependency> resource.

Table 10.2.z.4.3-1: <dependency> UPDATE

| <dependency> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.4 |
| Processing at Receiver | According to clause 10.1.4 After successfully updating the <dependency> resource, the Receiver shall perform the procedure specified in 10.2.Z.2. |
| Information in Response message | According to clause 10.1.4 |
| Processing at Originator after receiving Response | According to clause 10.1.4 |
| Exceptions | According to clause 10.1.4 |

##### 10.2.z.4.4	Delete <dependency>

This procedure shall be used for deleting an <dependency> resource.

Table 10.2.z.4.4-1: <dependency> DELETE

| <dependency> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, and Mcc |
| Information in Request message | All parameters defined in table 8.1.2-2 apply |
| Processing at Originator before sending Request | According to clause 10.1.5 |
| Processing at Receiver | According to clause 10.1.5 After successfully deleting the <dependency> resource, the Receiver shall perform the procedure specified in 10.2.Z.2. |
| Information in Response message | According to clause 10.1.5 |
| Processing at Originator after receiving Response | According to clause 10.1.5 |
| Exceptions | According to clause 10.1.5 |

### -----------------------End of change 2-------------------------------------------