| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#37 |
| Source:* | Bingxuan Wang, CMCC, wangbingxuan@chinamobile.com |
| Date:* | 2018-09-11 |
| Reason for Change/s:* | Update the Summary of levels of driving automation |
| CR against: Release* | Rel.3 |
| CR against: WI* | Active <WI-0046> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0026 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

<Provide an introduction containing the problem(s) being solved, and a summary list of proposals.  Discuss any risk of breaking backwards compatibility with last published version of the impacted TS.>

### -----------------------Start of change 1-------------------------------------------

## 5.4    Levels of Driving Automation

As mentioned in previous clauses, vehicles can exchange data on its own state, as well as receive data from surroundings (infrastructure - traffic lights, roads, other participants - vehicles, pedestrians, cyclists, etc.) and its state.

For vehicles exchanging data with its surroundings, we can distinguish number of levels of connectivity.

One is connected car - meaning car is connected to the Internet, typically for use of mobile network services. Examples of this are use of mobile network to Wi-Fi AP gateway, enabling passengers within vehicle to access internet via Wi-Fi. Other use is for collecting data from GPS unit within car to send information on current vehicle's location and speed. Data collected in this way is typically used for road management and routing advice (detecting congestion in roads).

Next level of connectivity is cooperative car - where car is communicating with other vehicles, either directly or indirectly (via RSU, or some other infrastructure element), with the goal of creating LDM (Local Dynamic Map) which contains all vehicles in vicinity and their current state (speed, acceleration/deceleration, changing lanes, …). Further, each vehicle is announcing (via local broadcast, within range of used communication technology) its own state. Based on received information from other vehicles, and own sensors, vehicle can make decision on actions it needs to take. Typically, cooperative driving is providing driver with additional information on state of traffic in immediate vicinity, and vehicle can also take some of actions (keeping distance, braking) itself.

Next stage is automated driving, which refers to the capability of the vehicle to drive from one location to another, without intervention from humans, and in a safe way, without incurring damage to surroundings (pedestrians, buildings, other vehicles) and to its (vehicle's) passengers. Here vehicle has all information like in cooperative driving (state of other vehicles in vicinity), but gets information from other participants in traffic as well - infrastructure (roads, cameras,  traffic lights, etc.) but also pedestrians, cyclists, etc.

Different levels of automation in driving are shown in Error: Reference source not found. From top to bottom we can see increasing levels of automation, with ' Full Driving Automationn' being autonomous driving.

Table 5.4-1 S    ummary of levels of driving automation

| SAE Level | Name | Definition | DDT |  | DDT fallback | ODD |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sustained lateral and longitudinal vehicle motion control | OEDR(Object and Event Detection and Response) |  |  |
| Driver performs part or all of the DDT |  |  |  |  |  |  |
| 0 | No Driving Automation | The performance by the driver of the entire DDT, even when enhaced by active safety systems. | Driver | Driver | Driver | N/A |
| 1 | Driver Assistance | The sustained and ODD-specific execution by a driving automation system of either the lateral or the longitudinal vehicle motion control subtask of the DDT(but not both simultaneously) with the expectation that the driver performs the remainder of the DDT. | Driver and system | Driver | Driver | Limited |
| 2 | Partial Driving Automation | The sustained and ODD-specific execution by a driving automation system of both the lateral and longitudinal vehicle motion control subtasks of the DDT with the expectation that the driver completes the OEDR subtask and supervises the driving automation system. | System | Driver | Driver | Limited |
| ADS (“System”) performs the entire DDT (while engaged) |  |  |  |  |  |  |
| 3 | Conditional Driving Automation | The sustained and ODD-specific performance by an ADS of the entire DDT with the expectation that the DDT fallback-ready user is receptive to ADS-issued requests to intervene, as well as to DDT performance-relevant system failures in other vehicle systems, and will respond appropriately. | System | System | Fallback-ready user ( becomes the driver during fallback ) | Limited |
| 4 | High Driving Automation | The sustained and ODD-specific performance by an ADS of the entire DDT and DDT fallback without any expectation that a user will respond to a request to intervene. | System | System | System | Limited |
| 5 | Full Driving Automation | The sustained and unconditional (i.e., not ODD-specific) performance by an ADS of the entire DDT and DDT fallback without any expectation that a user will respond to a request to intervene. | System | System | System | Unlimited |

DDT(Dynamic Driving Task): all of the real-time operational and tactical functions required to operate a vehicle in on-road traffic, excluding the strategic functions such as trip scheduling and selection of destinations and waypoints,  including without limitation: Motion control via steering, acceleration and deceleration; Monitoring the driving environment via object and event detection, recognition, classification and response execution,etc.

DDT Fallback: given the circumstances that the occurrence of a DDT performance-relevant system failure or upon operational design domain(ODD) exit, the response by the user to perform the DDT or the response to achieve a minimal risk condition by the user or an ADS.

Using IoT for vehicular domain, vehicles will be able to go beyond cooperative driving, and move into automated driving, levels 3-5 in Table 5.4-1.

- Rest of the present document will refer to this figure, when applicable.

### -----------------------End of change 1---------------------------------------------

# 3	Definitions, symbols, abbreviations  and acronyms

## 3.1	Definitions

<defined term>: <definition>

<defined term>[N]: <definition>

## 3.2	Symbols

<symbol>	<Explanation>

## 3.3	Abbreviations and Acronyms

ADS	Automated Driving System

DDT                    Dynamic Driving Task

ODD                    Operational Design Domain

OEDR                  Object and Event Detection and Response

### ---End of changes to Definitions, Symbols, Abbreviations, Acronyms ---

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?