| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC37 |
| Source:* | Bob Flynn Convida, Flynn.Bob@convidawirelesss.com Dale Seed, Convida, Seed.Dale@convidawireless.com Zhuo Chen, Convida, Chen.Zhuo@convidawireless.com |
| Date:* | 2018-09-20 |
| Reason for Change/s:* | Rework announcement of resources to remote CSEs that are 1 or more hops away |
| CR against: Release* | Release 2 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision) Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) ARC-2018-0291 STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001, v2.20.0 |
| Clauses * | 9.6.1.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introdution

### -----------------------Start of change 1-------------------------------------------

#### 9.6.1.1	Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary - i.e. not announced - resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in table 9.6.26.1-1 "Announced Resource Types".

Among the resource types listed in table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

- container;

- contentInstance;

- flexContainer;

- timeSeries;

- timeSeriesInstance.

Table 9.6.1.1-1: Resource Types

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| accessControlPolicy | Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources | subscription | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.2 |
| AE | Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE | subscription, container, flexContainer, group, accessControlPolicy, schedule, pollingChannelsemanticDescriptor, timeSeries | CSEBase | 9.6.5 |
| container | Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule | container, flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor | AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, CSEBase, flexContainer, flexContainerAnnc | 9.6.6 |
| contentInstance | Represents a data instance in the <container> resource | semanticDescriptor | Container, containerAnnc | 9.6.7 |
| flexContainer | A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure | container, flexContainer, subscription, semanticDescriptor | AE, AEAnnc, container, containerAnnc, flexContainer, flexContainerAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.35 |
| CSEBase | The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself | remoteCSE, remoteCSEAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery, schedule, notificationTargetPolicy, flexContainer, timeSeries | None specified | 9.6.3 |
| delivery | Forwards requests from CSE to CSE | subscription | CSEBase | 9.6.11 |
| eventConfig | Defines events that trigger statistics collection | subscription | statsConfig | 9.6.24 |
| execInstance | Contains all execution instances of the same Management Command | subscription | mgmtCmd | 9.6.17 |
| fanOutPoint (V) | Virtual resource containing target for group request It is used for addressing bulk operations to all the resources that belong to a group | None specified | group | 9.6.14 |
| group | Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group | fanOutPoint, subscription, semanticFanOutPoint, semanticDescriptor | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.13 |
| latest (V) | Virtual resource that points to most recently created <contentInstance> child resource within a <container> resource | None specified | container | 9.6.27 |
| locationPolicy | Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information | subscription | CSEBase | 9.6.10 |
| mgmtCmd | Management Command resource represents a method to execute management procedures required by existing management protocols | execInstance, subscription | CSEBase | 9.6.16 |
| mgmtObj | Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note) | subscription, schedule, semanticDescriptor | node, mgmtObjAnnc | 9.6.15 Annex D |
| m2mServiceSubscriptionProfile | Data pertaining to the M2M Service Subscription | serviceSubscribedNode, subscription | CSEBase | 9.6.19 |
| node | Represents specific Node information | mgmtObj, subscription, semanticDescriptor | CSEBase | 9.6.18 |
| notificationTargetMgmtPolicyRef | Represents a list of notification targets and the deletion policy | subscription | subscription | 9.6.31 |
| notificationTargetPolicy | Represents a notification target deletion policy with pre-defined action and deletion rules | subscription, policyDeletionRules | CSEBase | 9.6.32 |
| notificationTargetSelfReference (V) | Virtual resource used to remove the Notification Target | None specified | subscription | 9.6.34 |
| oldest (V) | Virtual resource that points to first created <contentInstance> child resource within a <container> resource | None specified | container | 9.6.28 |
| pollingChannel | Represent a channel that can be used for a request-unreachable entity | pollingChannelURI | remoteCSE, AE | 9.6.21 |
| pollingChannelURI (V) | Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity | None specified | pollingChannel | 9.6.22 |
| policyDeletionRules | Represents a set of rules which is associated with notification target removal policy | subscription | notificationTargetPolicy | 9.6.33 |
| remoteCSE | Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource | container, containerAnnc, contentInstanceAnnc, flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, pollingChannel, schedule, timeSeries, timeSeriesAnnc, timeSeriesInstanceAnnc, mgmtObjAnnc, nodeAnnc, AEAnnc, locationPolicyAnnc | CSEBase | 9.6.4 |
| request | Expresses/access context of an issued Request | subscription | CSEBase | 9.6.12 |
| schedule | Contains scheduling information for delivery of messages | subscription | subscription, remoteCSE, AE, trafficPattern | 9.6.9 |
| serviceSubscribedNode | Node information | subscription | m2mServiceSubscriptionProfile | 9.6.20 |
| statsCollect | Defines triggers for the IN-CSE to collect statistics for applications | subscription | CSEBase (in IN-CSE) | 9.6.25 |
| statsConfig | Stores configuration of statistics for applications | eventConfig, subscription | CSEBase (in IN-CSE) | 9.6.23 |
| subscription | Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribe-to resource | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef | accessControlPolicy,accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, request, schedule, scheduleAnnc, semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig, flexContainer, flexContainerAnnc, timeSeries, timeSeriesAnnc | 9.6.8 |
| serviceSubscribedAppRule | Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE | subscription | CSEBase | 9.6.29 |
| semanticDescriptor | Stores semantic description pertaining to a resource and potentially sub-resources. | subscription | AE, container, contentInstance, group, node, flexContainer, timeSeries, mgmtObj | 9.6.30 |
| semanticFanOutPoint | Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource | None specified | group | 9.6.14a |
| trafficPattern | Represents the communication pattern and the mobility pattern of a Field Domain Node. | schedule, subscription | Node, AE | 9.6.41 |
| dynamicAuthorizationConsultation | Represents consultation information used by a CSE when performing consultation-based dynamic authorization | None Specified | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.40 |
| timeSeries | Stores and Shares Time Series Data instances among entities. | timeSeriesInstance, subscription, semanticDescriptor | AE, AEAnnc, remoteCSE, remoteCESAnnc, CSEBase | 9.6.36 |
| timeSeriesInstance | Represents a Time Series Data instance in the <timeSeries> resource | None specified | timeSeries, timeSeriesAnnc | 9.6.37 |
| NOTE: See clause 9.6.12 for a summary of specializations of <mgmtObj>. |  |  |  |  |

### -----------------------End of change 1-------------------------------------------