| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#37 |
| Source:* | Dale Seed, Convida, seed.dale@convidawireless.com |
| Date:* | 2018-09-09 |
| Reason for Change/s:* | See introduction |
| CR against: Release* |  |
| CR against: WI* | Active (WI-0083 - oneM2M Service Subscribers and Users) MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0054 v0.2.0 |
| Clauses * | 7.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | n/a |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes to refine the definition of the existing M2M Service Subscription Identifier which is currently not fully nor well defined in the current oneM2M specifications.

### -----------------------Start of change 1-------------------------------------------

## M2M Service Subscription Identifier

### Solution Applicability

oneM2M currently defines the M2M Service Subscription Identifier, however this identifier is not currently used by any oneM2M defined resources or procedures.  The format of this identifier is also not explicitly defined.  This identifier is well suited to be associated with a <m2mServiceSubscriptionProfile> resource since this would enable the correlation of a M2M Service Subscription Identifier with a M2M Service Subscriber and its associated M2M Service Users, M2M Nodes, AEs and CSEs.  This could in turn enable additional value-add capabilities.  For example, an IN-CSE could generate charging records that include a M2M Service Subscription Identifier which could then be used by external billing and charging systems to more easily reconcile charging records for a given M2M Service Subscription.

### Solution Description

The M2M Service Subscription Identifier (M2M-Subscription-ID) enables the M2M Service Provider to bind a M2M Service Subscriber, M2M Service Users, AEs, M2M Nodes and CSEs, as well as administrative information (e.g.  billing address, etc.), to a particular M2M Service Subscription between an M2M Service Subscriber and the M2M Service Provider. The M2M-Subscription-ID is unique for every M2M Service Subscriber and enables the M2M Service Provider to identify the service subscription.  The M2M-Subscription-ID is assigned by the M2M Service Provider and is globally unique.  When used internally within the M2M Service Provider domain that assigned it, a M2M-Subscription-ID is sufficient to be unique within that M2M Service Provider domain.  When used externally outside the M2M Service Provider domain that assigned it, a M2M-Subscription-ID is extended to make it globally unique.  The IN-CSE can perform this task by adding or removing segments (i.e. the M2M Service Provider Identifier) of the M2M-Subscription-ID.  The M2M-Subscription-ID can differ from a M2M Underlying Network Subscription Identifier.  There can be multiple M2M Service Subscription Identifiers per M2M Underlying Network subscription.

Care should be taken (e.g. proper configuration of ACPs) to not expose the M2M-Subscription-ID to untrusted entities.

Table 7.3.2-1: M2M-Subscription-ID Identifier Format

| Identifier Name | Format | Format | Rule of use |
| --- | --- | --- | --- |
| M2M-Subscription-ID | SP-Relative | The SP-Relative-M2M-Subscription-ID begins with a slash character '/' and is followed by a sequence of characters that includes any of the unreserved characters defined in clause 2.3 of the IETF RFC 3986 [i.3]. The SP-Relative-M2M-Subscription-ID is unique within the context of the M2M-SP Domain to which the M2M Service Subscriber has a service subscription with. The M2M-SP assigns the SP-Relative-M2M-Subscription-ID and is responsible for guaranteeing that the SP-Relative-M2M-Subscription-ID is unique in the context of the M2M-SP Domain. Example: - /subscription783567 | Uniquely identifies a M2M Service Subscription |
|  | Absolute | Concatenation according to the format {M2M-SP-ID}{SP-Relative-M2M-Subscription-ID} where {M2M-SP-ID} and {SP-relative- M2M-Subscription-ID} are placeholders for the M2M-SP-ID and the SP-relative-M2M-Subscription-ID format of the M2M-Subscription-ID respectively. | Uniquely identifies a M2M Service Subscription |

The <m2mServiceSubscriptionProfile> resource represents an M2M Service Subscription. The M2M-Subscription-ID can be added as an attribute to the <m2mServiceSubscriptionProfile> resource.  This provides the capability to uniquely identify a M2M Service Subscription.

Table 7.3.2-1: New <m2mServiceSubscriptionProfile> resource attributes

| Attributes of <m2mServiceSubscriptionProfile> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| M2M-Subscription-ID | 1 | WO | The identifier that is assigned to this M2M Service Subscription by the M2M Service Provder. |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?