| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | ARC#38 |
| Source:* | Convida Wireless, KDDI |
| Date:* | 2018-11-23 |
| Contact:* | Catalina Mladin, Convida, Mladin.Catalina@convidawireles.com Kenichi Yamamoto, KDDI, kc-yamamoto@kddi.com-std.jp |
| Reason for Change/s:* | Introduces a new section for capturing the Usecases in scope |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0080> MNT Maintenance / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0052 V0.4.0 |
| Clauses/Sub Clauses* | New Clause 7.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution introduces a Key Issue based cases used collected in relevant use cases and requirements.

- -----------------------Start of change 1-------------------------------------------

  - Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	NIST Special Publication 800-145, The NIST Definition of Cloud Computing, September 2011 (https://csrc.nist.gov/publications/detail/sp/800-145/final)

[i.3]	NIST Special Publication 800-191 (Draft), The NIST Definition of Fog Computing, August 2017 (https://csrc.nist.gov/publications/detail/sp/800-191/draft)

[i.4]	OpenFog Reference Architecture for Fog Computing, February 2017 (OPFRA001.020817)

[i.5]	ETSI GS MEC 002 (V1.1.1): “Mobile Edge Computing (MEC); Technical Requirements”.

[i.6]	ETSI GS MEC 003 (V1.1.1): “Mobile Edge Computing (MEC); Framework and Reference Architecture”.

[i.7]	ETSI GS MEC 009 (V1.1.1): “Mobile Edge Computing (MEC); General principles for Mobile Edge Service APIs”.

[i.8]	ETSI GS MEC 010-1 (V1.1.1): “Mobile Edge Computing (MEC); Mobile Edge Management; Part 1: System, host and platform management”.

[i.9]	ETSI GS MEC 010-2 (V1.1.1): “Mobile Edge Computing (MEC); Mobile Edge Management; Part 2: Application lifecycle, rules and requirements management”.

[i.10]	ETSI GS MEC 011 (V1.1.1): “Mobile Edge Computing (MEC); Mobile Edge Platform Application Enablement”.

[i.11]	ETSI GS MEC 012 (V1.1.1): “Mobile Edge Computing (MEC); Radio Network Information API”.

[i.12]	ETSI GS MEC 013 (V1.1.1): “Mobile Edge Computing (MEC); Location API”.

[i.13]	ETSI GS MEC 014 (V1.1.1): “Mobile Edge Computing (MEC); UE Identity API”.

[i.14]	ETSI GS MEC 015 (V1.1.1): “Mobile Edge Computing (MEC); Bandwidth Management API”.

[i.15]	ETSI GS MEC 016 (V1.1.1): “Mobile Edge Computing (MEC); UE application interface”.

[i.16]	NFV World Congress 2017 - Enabling the 5G Cloud.

(http://www.etsi.org/multi-access-edge-computing/mec-presentations-at-industry-events)

[i.17]	3GPP TS 36.413: “LTE; Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)”.

[i.18]	3GPP TS 36.214: "LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements”.

[i.19]	3GPP TS 36.331:"LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[i.20]	3GPP TS 29.171: "LTE; Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface”.

[i.21]	3GPP TS 23.401: “LTE; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[i.22]	3GPP TS 32.602: "Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Telecommunication management; Configuration Management(CM); Basic CM Integration Reference Point (IRP); Information Service (IS)".

[i.23]	3GPP TS 32.662: "Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Telecommunication management; Configuration Management (CM); Kernel CM Information Service (IS)".

[i.24]	3GPP TS 32.332: "Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Telecommunication management; Notification Log (NL) Integration Reference Point (IRP); Information Service (IS)".

[i.25]	3GPP TS 32.111-2: "Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[i.26]	SCF 084.07.01: "Small cell zone services - RESTful bindings".

[i.27]	SCF 152.07.01: "Small cell services API".

[i.28]	OMA-TS-REST-NetAPI-ZonalPresence-V1-0-20160308-C: "RESTful Network API for Zonal Presence".

[i.29]	OMA-TS-REST-NetAPI-ACR-V1-0-20151201-C: "RESTful Network API for Anonymous Customer Reference Management"

[i.30]	MEC Deployments in 4G and Evolution Towards 5G.
(http://www.etsi.org/images/files/ETSIWhitePapers/etsi_wp24_MEC_deployment_in_4G_5G_FINAL.pdf)

[i.31]				3GPP TS23.203: Policy and charging control architecture

[i.32]	3GPP TS 29.122: T8 reference point for Northbound APIs (Release 15)

[i.33]	oneM2M TS-0001 Functional Architecture

[i.34]	oneM2M TS-0026 3GPP Interworking

[i.35]				3GPP TS 29.214: Policy and Charging Control over Rx reference point (Release 15)

[i.36]				oneM2M TR-0001 Use Cases Collection

[i.37]				oneM2M TR-0018 Industrial Domain Enablement

[i.38] 				oneM2M TR-0026 Vehicular Domain Enablement

[i.39]				oneM2M TS-0002 Requirements.

[i.40]				oneM2M TS-0025 Product Service Profiles.

[i.41]				oneM2M TS-0011 Common Terminology

- -----------------------End of change 1-------------------------------------------

- -----------------------Start of change 2-------------------------------------------

- Analysis

Editor’s Note: This section details Key Issues for employing Edge and Fog technologies for oneM2M deployments.

  - Key Issue X: Common Service awareness

Editor’s Note: Each Key Issue description references either optimization scenarios (section 7.2), use-cases from one of the relevant TRs (e.g. TR-0001, TR-0018, TR-0026, etc.) or requirements (TS-0002) and concludes with a succinct statement defining the issue.

- oneM2M Specifications (i.e. TS-0011 [i.41]) provide the following service-related definitions:

M2M Application Service: realized through the service logic of an M2M Application and is operated by the User or an M2M Application Service Provider

M2M Common Services: set of oneM2M specified functionalities that are widely applicable to different application domains made available through the set of oneM2M specified interfaces

M2M Service: consists of one or more M2M Application Services and one or more M2M Common Services

Although oneM2M specifications support identifiers for M2M Application Services, there are no such identifiers for M2M Common Services or M2M Services. There are also no resource or attribute-level mapings for these concepts.

- According to TS-0001 [i.33] “Registration is the process of delivering AE or CSE information to another CSE in order to use M2M Services”.

Moreover, for Vehicular Enablement the following issue has been identified in TR-0026 [i.38]:

- “Support of multiple registrations for a single entity may introduce unnecessary resource allocations at the registrars, unless the registration procedure indicates which services are requested or granted, and the specific combinations of services to be provided by each registrar.”

- Usecases collected within the scope of Edge/Fog technology support introduce a number of service-related requirements. For example, the Smart Transportation with Edge/Fog computing use case (clause 6.21 TR-0026 [i.38]) introduces requirements also captured in TS-0002 [i.39]:

OSR-0162: The oneM2M System shall enable the sharing and discovery of service capability information across fog/edge networks.

OSR-0163: The oneM2M System shall enable to request services provided by fog/edge nodes.

OSR-0164: The oneM2M System shall enable service migration among fog/edge nodes.

OSR-0165: The oneM2M System shall enable the orchestration of services provided by Fog/Edge nodes in a dynamic fashion to satisfy operational requirements for availability, scalability, interoperability, etc.

Noting that these requirements refer to “services” rather than “M2M Application Services”, do not provide the capability to selectively provide or manage  M2M Common Services or combinations of M2M Common Services and M2M Application Services , namely M2M Services.

- Finally, it is to be noted that other procedures and terminology may be linked (semantically or logically) to the term “service” and therefore provide additional context. For example: M2M service subscription; established advanced functionality (e.g. Security) which may be construed as being composed of several services (e.g. Authorization, Identity management, Access control, etc.); Product Service Profiles (TS-0025 [i.40]) which are further defined by “fundamental feature set” and “extendable feature set”.

The following Key Issues are drawn by combining several of the points A-D identified above:

- The oneM2M specifications need to provide clear relationships between the concept of “M2M Service” and oneM2M resources, procedures or architectural entities.  Specifically, it is recommended:

  - Providing identifiers for M2M Common Services and/or M2M Services, along resource or attribute-level mapings for these concepts.

  - Identifying “M2M Service” relationships (if any) with concepts/terms including, but not limited to:

    - Registration procedures for CSEs and AEs

    - M2M service subscription and the m2mServiceSubscriptionProfile resource.

    - Product Service Profiles

  - Key Issue N

- -----------------------End of change 2-------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause?