| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC #38 |
| Source:* | Catalina Mladin, Convida Wireless, Mladin.Catalina@convidawireless.com Saïd Gharout, Convida Orange, said.gharout@orange.com |
| Date:* | 2018-11-12 |
| Reason for Change/s:* | See the introduction below |
| CR against: Release* | Release 3/4 |
| CR against: WI* | Active WI-0079 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v. 3.12.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution seeks to clarify how the operations which use filter criteria work when the criteria is used to determine a group of resources to determine the targets of the operations. The contribution is introduced in conjunction with PRO-2018-0244 and it clarifies details missed before for U and D opeartions, and adds the create operations

R01 adds cosigners, R02 addresses offline comments

### -----------------------Start of change 1-------------------------------------------

### 8.1.2	Request

Requests over the Mca and Mcc reference points, from an Originator to a Receiver, shall contain mandatory and may contain optional parameters. Certain parameters may be mandatory or optional depending upon the Requested operation. In this clause, the mandatory parameters are detailed first, followed by those that are operation dependent, and then by those that are optional:

- To: Address of the target resource or target attribute for the operation. The To parameter shall conform to clause 9.3.1.

NOTE 1:	To parameter can be known either by pre-provisioning (clause 11.2) or by discovery (clause 10.2.6 for discovery). Discovery of <CSEBase> resource is not supported in this release of the document. It is assumed knowledge of <CSEBase> resource is by pre-provisioning only.

NOTE 2:	The term target resource refers to the resource which is addressed for the specific operation. For example, the To parameter of a Create operation for a resource <example> would be "/m2m.provider.com/exampleBase". The To parameter for the Retrieve operation of the same resource <example> is "/m2m.provider.com/exampleBase/example".

NOTE 3:	For Retrieve operation (clause 10.1.3), the To parameter can be the URI of an attribute to be retrieved.

- From: Identifier representing the Originator.

The From parameter is used by the Receiver to check the Originator identity for access privilege verification.

- Operation: operation to be executed: Create (C), Retrieve (R), Update (U), Delete (D), Notify (N).

The Operation parameter shall indicate the operation to be executed at the Receiver:

- Create (C): To is the address of the target resource where the new resource (parent resource).

- Retrieve (R): an existing To addressable resource is read and provided back to the Originator.

- Update (U): the content of an existing To addressable resource is replaced with the new content as in Content parameter. If some attributes in the Content parameter do not exist at the target resource, such attributes are created with the assigned values. If some attributes in the Content parameter are set to NULL, such attributes are deleted from the addressed resource.

- Delete (D): an existing To addressable resource and all its sub-resources are deleted from the Resource storage.

- Notify (N): information to be sent to the Receiver, processing on the Receiver is not indicated by the Originator.

- Request Identifier: request Identifier (see clause 7.1.7).

Example usage of request identifier includes enabling the correlation between a Request and one of the many received Responses.

Operation dependent Parameters:

- Content: resource content to be transferred.

The Content parameter shall be present in Request for the following operations:

- Create (C): Content is the content of the new resource with the resource type ResourceType.

- Update (U): Content is the content to be replaced in an existing resource. For attributes to be updated at the resource, Content includes the names of such attributes with their new values. For attributes to be created at the resource, Content includes names of such attributes with their associated values. For attributes to be deleted at the resource, Content includes the names of such attributes with their value set to NULL.

- Notify (N): Content is the notification information.

The Content parameter may be present in Request for the following operations:

- Retrieve (R): Content is the list of attribute names from the resource that needs to be retrieved. The values associated with the attribute names shall be returned.

- Resource Type: type of resource.

The ResourceType parameter shall be present in Request for the following operations:

- Create (C): Resource Type is the type of the resource to be created.

Optional Parameters:

- Role IDs: optional, required when role based access control is applied. A list of Role-IDs that are allowed by the service subscription shall be provided otherwise the request is considered not valid.

The Role IDs parameter shall be used by the Receiver to check the Access Control privileges of the Originator.

- Originating Timestamp: optional originating timestamp of when the message was built.

Example usage of the originating timestamp includes: to measure and enable operation (e.g. message logging, correlation, message prioritization/scheduling, accept performance requests, charging, etc.) and to measure performance (distribution and processing latency, closed loop latency, SLAs, analytics, etc.)

- Request Expiration Timestamp: optional request message expiration timestamp. The Receiver CSE should handle the request before the time expires. If a Receiver CSE receives a request with Request Expiration Timestamp with the value indicating a time in the past, then the request shall be rejected.

Example usage of the request expiration timestamp is to indicate when request messages (including delay-tolerant) should expire and to inform message scheduling/prioritization. When a request with set expiration timestamp demands an operation on a Hosting CSE different than the current Receiver CSE, then the current CSE shall keep trying to deliver the Request to the Hosting CSE until the request expiration timestamp time, in line with provisioned policies.

- Result Expiration Timestamp: optional result message expiration timestamp. The Receiver CSE should return the result of the request before the time expires.

Example usage of the result expiration timestamp: An Originator indicates when result messages (including delay-tolerant) should expire and informs message scheduling/prioritization. It can be used to set the maximum allowed total request/result message sequence round trip deadline.

- Response Type: optional response message type: Indicates what type of response shall be sent to the issued request and when the response shall be sent to the Originator:

- nonBlockingRequestSynch: In case the request is accepted by the Receiver CSE, the Receiver CSE responds, after acceptance, with an Acknowledgement confirming that the Receiver CSE will further process the request. The Receiver CSE includes in the response to an accepted request a reference that can be used to access the status of the request and the result of the requested operation at a later time. Processing of Non-Blocking Requests is defined in clause 8.2.2 and in particular for the synchronous case in clause 8.2.2.2.

- nonBlockingRequestAsynch {optional list of notification targets}: In case the request is accepted by the Receiver CSE, the Receiver CSE shall respond, after acceptance, with an Acknowledgement confirming that the Receiver CSE will further process the request. The result of the requested operation needs to be sent as notification(s) to the notification target(s) provided optionally within this parameter as a list of entities or to the Originator when no notification target list is provided. When an empty notification target list is provided by the Originator, no notification with the result of the requested operation shall be sent at all. Processing of Non-Blocking Requests is defined in clause 8.2.2 and in particular for the asynchronous case in clause 8.2.2.3.

- blockingRequest: In case the request is accepted by the Receiver CSE, the Receiver CSE responds with the result of the requested operation after completion of the requested operation. Processing of Blocking Requests is defined in clause 8.2.1. This is the default behaviour when the Response Type parameter is not given the request.

- flexBlocking {optional list of notification targets}: When Response Type in the request received by the Receiver CSE is set to flexBlocking, it means that the Originator of the request has the capability to accept the following types of responses: nonBlockingRequestSynch, nonBlockingRequestAsynch and blockingRequest.

The Receiver CSE shall make the decision to respond using blocking or non-blocking based on its own local context (memory, processing capability, etc.) if not defined in the resource handling procedure.

If the Receiver CSE choose to respond using non-blocking mode or blocking mode, based on the presence of notification targets in the request:

- If the notification targets are provided in the request and the Receiver CSE is responding, the Receiver CSE shall choose and respond with nonBlockingRequestAsynch, nonBlockingRequestSynch or blockingRequest mode.

- If notification targets are not provided, the Receiver CSE shall choose and respond with nonBlockingRequestSynch or blockingRequest mode.

  - No Response: In case the request is accepted by the Receiver CSE or AE, the Receiver CSE or AE does not respond with the result of the requested operation after completion of the requested operation.  Note, in this case the Result Content parameter should not be included in the request.

Example usage of the response type set to nonBlockingRequestSynch: An Originator that is optimized to minimize communication time and energy consumption wants to express a Request to the receiver CSE and get an acknowledgement on whether the Request got accepted. After that the Originator may switch into a less power consuming mode and retrieve a Result of the requested Operation at a later time.

Further example usage of response type set to nonBlockingRequestSynch: When the result content is extremely large, or when the result consists of multiple content parts from a target group which are to be aggregated asynchronously over time.

- Result Content: optional result content: Indicates what are the expected components of the result of the requested operation. This shall be indicated in the Content parameter of the response message. Settings of Result Content depend on the requested operation specified in Operation. This parameter is not applicable when Response Type has a value of No Response. Possible values of Result Content are:

- attributes: A representation of the targeted resource including all its attributes shall be returned as content, without the address(es) of the child resource(s) or their descendants. For example, if the request is to retrieve a <container> resource, the address(es) of the <contentInstance> child-resource(s) is not provided. This setting shall be only valid for Create, Retrieve, Update, or Delete operation. If the Originator does not set Result Content parameter in a Create, Retrieve or Update request message, this setting shall be the default value when the Receiver processes the request message

- modified-attributes: This setting shall be only valid for a Create or Update operation. A representation of the targeted resource including only the assigned or modified attributes relative to what was provided by the Originator of the request shall be returned as content, without the address(es) of the child resource(s) or their descendants.

- hierarchical-address: Representation of the address of the created resource. This setting shall only be valid for a Create operation. The address shall be in hierarchical address scheme.

- hierarchical-address+attributes: Representation of the address in hierarchical address scheme and the attributes of the created resource. This setting shall only be valid for a Create operation.

- -	attributes+child-resources: Representation of the requested resource, along with a nested representation of all of its child resource(s), and their descendants, in line with any provided filter criteria as given in the Filter Criteria parameter shall be returned as content. If there is no filter criteria parameter in the request message, then all children/descendants are returned along with their attributes. For example, if the request is to retrieve a <container> resource that only has <contentInstance> children, the attributes of that <container> resource and a representation of all of its <contentInstance> child-resource(s), including their attributes, are provided.

The originator may request to limit the maximum number of allowed nesting levels. The originator may also include an offset that indicates the starting point of the direct child resource. The offset shall start at 1. The hosting CSE shall return all direct child resources and their descendants, or up to the maximum nesting level specified in a request subject to maximum size limit that may be imposed by the hosting CSE. The offset, maximum number/size and maximum level shall be specified in Filter Criteria as offset, limit, and level condition, respectively, by the Originator.

The hosting CSE shall list parent resources before their children. This means that the originator of the request will not receive a discovered resource without having received its parents. The hosting CSE shall also ensure that proper nesting representation of all the children is incorporated in its listing for parents and children.

Nested processing is applicable at every level in the resource tree. If a direct child resource and all its descendants cannot be included in the returned content due to size limitations imposed by the hosting CSE then the direct child resource shall not be included in the response.

An indication shall be included in the response signalling if the returned content is partial. If the indication is for partial content, the response shall include an offset for the direct child resource where processing can restart for the remaining direct child resources

This shall be only valid for a Retrieve/Delete operation.

- child-resources: A nested representation of the resource's child resource(s) their descendants and their attributes shall be returned as content. The resources that are returned are subject to any filter criteria that are given in the Filter Criteria parameter (if there are no filter criteria then all children and their descendants are returned). The attributes of the parent resource are not returned, but all the attributes of the children are returned. For example, if the request is to retrieve a <container> resource that only has <contentInstance> children, only a representation of all of its <contentInstance> child-resource(s) is provided.

The offset, maximum number/size and maximum level shall be specified in Filter Criteria as offset, limit, and level condition, respectively, by the Originator. Processing of direct child resources, size limitations, maximum nesting level, and offset for the starting of direct child resource processing of the attributes+child-resources option shall apply to this option as well.

This shall be only valid for a Retrieve/Delete operation.

- attributes+child-resource-references: Representation of the requested resource, along with the address(es) of the child resource(s), and their descendants shall be returned as content. For example, if the request is to retrieve a <container> resource, the <container> resource and the address(es) of the <contentInstance> child-resource(s) are provided.

The offset, maximum number/size and maximum level shall be specified in Filter Criteria as offset, limit, and level condition, respectively, by the Originator. Processing of child resources, size limitations, maximum nesting level, and offset for the starting of child resource processing of the attributes+child-resources option shall apply to this option as well.

This shall be only valid for a Retrieve/Delete operation.

- child-resource-references: Address(es) of the child resources and their descendants, without any representation of the actual requested resource shall be returned as content. For example, if the request is to retrieve a <container> resource, only the address(es) of the <contentInstance> child-resource(s) is provided.

The offset, maximum number/size and maximum level shall be specified in Filter Criteria as offset, limit, and level condition, respectively, by the Originator. Processing of child resources, size limitations, maximum nesting level, and offset for the starting of child resource processing of the attributes+child-resources option shall apply to this option as well.

This shall be only valid for a Retrieve/Delete operation.

This option can be used within the context of resource discovery mechanisms (see clause 10.2.6).

- nothing: Nothing shall be returned as operational result content. If the Originator does not set the Result Content parameter in a Delete request message, this setting shall be the default value when the Receiver processes the request message. This setting shall be valid for a Create, Update, Delete, or Notify operation.

EXAMPLE:	If the request is to delete a resource, this setting indicates that the response shall not include any content.

- original-resource: Representation of the original resource pointed by the link attribute in the announced resource shall be returned as content, without the address(es) of the child resource(s). This shall be only valid for a Retrieve operation where the To parameter targets the announced resource.

- semantic-content: Representation of semantic information that is the result of a semantic query as indicated by the setting of the Semantic Query Indicator parameter.

Note that for any of the above options, Discovery access control is applied against discovery related procedures, while Retrieve access control procedures is applied against non-discovery related Retrieve operations.

Note that the fitter criteria usage governs the purpose of a Retrieve operation.

Table 8.1.2-1: Summary of Result Content Values

| Value | Create | Retrieve | Update | Delete | Notify |
| --- | --- | --- | --- | --- | --- |
| Attributes | default | default | default | valid | n/a |
| modified-attributes | valid | n/a | valid | n/a | n/a |
| hierarchical-address | valid | n/a | n/a | n/a | n/a |
| hierarchical-address+attributes | valid | n/a | n/a | n/a | n/a |
| attributes+child-resources | n/a | valid | n/a | valid | n/a |
| child-resources | n/a | valid | n/a | valid | n/a |
| attributes+child-resource-references | n/a | valid | n/a | valid | n/a |
| child-resource-references | n/a | valid | n/a | valid | n/a |
| Nothing | valid | n/a | valid | default | valid |
| original-resource | n/a | valid | n/a | n/a | n/a |
| semantic-content | n/a | valid | n/a | n/a | n/a |

- Result Persistence: optional result persistence: indicates the time for which the response may persist to. The parameter is used in case of non-blocking request where the result attribute of the <request> resource should be kept at the CSE, for example, with the purpose of sharing, tracking and analytics.

In the case the response of a request is required to be kept in the CSE, for example the procedures of <request> resource, <delivery> resource and <group> resource, the Result Persistence indicates the time duration for which the CSE keeps the response available after receiving it.

Example usage of result persistence includes requesting sufficient persistence for analytics to process the response content aggregated asynchronously over time. If a result expiration time is specified, then the result persistence lasts beyond the result expiration time.

- Operation Execution Time: optional operation execution time: indicates the time when the specified operation Operation is to be executed by the target CSE. A target CSE shall execute the specified operation of a Request having its operational execution time indicator set, starting at the operational execution time. If the execution time has already passed or if the indicator is not set, then the specified operation shall be immediately executed, unless the request expiration time, if set, has been reached.

Example usage of operational execution time includes asynchronous distribution of flows, which are to be executed synchronously at the operational execution time.

NOTE 6:	Time-based flows could not be supported depending upon time services available at CSEs.

- Event Category: optional event category: Indicates the event category that should be used to handle this request. Event categories are impacting how Requests to access remotely hosted resources are processed in the CMDH CSF. Selection and scheduling of connections via CMDH are driven by policies that can differentiate event categories.

Example usage of "event category" set to specific value X: When the request is demanding an operation to be executed on a Hosting CSE that is different from the current Receiver CSE, the request may be stored in the current Receiver CSE that is currently processing the request on the way to the Hosting CSE until it is allowed by provisioned policies for that event category X to use a communication link to reach the next CSE on a path to the Hosting CSE or until the request expiration timestamp is expired.

The following values for Event Category shall have a specified pre-defined meaning:

- Event Category = immediate: Requests of this category shall be sent as soon as possible and shall not be subject to any further CMDH processing, i.e. the request will not be subject to storing in CMDH buffers when communication over an underlying network is possible. In particular, CMDH processing will respect values for Request Expiration Timestamp, Result Expiration Timestamp given in the original request and not fill in any default values if they are missing.

- Event Category = bestEffort: Requests of this category can be stored in CMDH buffers at the discretion of the CSE that is processing the request for an arbitrary time and shall be forwarded via Mcc on a best effort basis. The CSE does not assume any responsibility to meet any time limits for delivering the information to the next CSE. Also the maximum amount of buffered requests for this category is at the discretion of the processing CSE.

- Event Category = latest:

- If this category is used in a request asking for a CRUD operation on a resource, the following shall apply:
CRUD requests using this category shall undergo normal CMDH processing as outlined further below in the present document and in oneM2M TS-0004 [Error: Reference source not found] with a maximum buffer size of one pending request for a specific pair of From and To parameters that appear in the request. If a new request message is received by the CSE with a pair of parameters From and To that has already been buffered for a pending request, the newer request will replace the buffered older request.

- If this category is used in a notification request triggered by a subscription, the following shall apply:
Notification requests triggered by a subscription using this category shall undergo normal CMDH processing as outlined further below in the present document and in oneM2M TS-0004 [Error: Reference source not found] with a maximum buffer size of one pending notification request per subscription reference that appears in a notification request. If a new notification request is received by the CSE with a subscription reference that has already been buffered for a pending notification request, the newer request will replace the buffered older request.

- If no further CMDH policies are provisioned for this event category, the forwarding process shall follow the 'bestEffort' rules defined above.

The M2M Service Provider shall be able to provision CMDH policies describing details for the usage of the specific Underlying Network(s) and the applicable rules as defined in the [cmdhPolicy] resource type for other Event Category values not listed above.

- Delivery Aggregation: optional delivery aggregation on/off: Use CRUD operations of <delivery> resources to express forwarding of one or more original requests to the same target CSE(s). When this parameter is not given in the request, the default behaviour is determined per the provisioned CMDH policy if available. If there is no such CMDH policy, then the default value is "aggregation off".

NOTE 7:	Since Delivery Aggregation is optional, there could be a default value to be used when not present in the Request. This parameter could not be exposed to AEs via Mca.

Example usage of delivery aggregation set on: The CSE processing a request shall use aggregation of requests to the same target CSE by requesting CREATE of a <delivery> resource on the next CSE on the path to the target CSE.

- Group Request Identifier: optional group request identifier: Identifier optionally added to the group request that is to be fanned out to each member of the group in order to detect loops and avoid duplicated handling of operation in case of loops of group and common members between groups that have parent-child relationship.

- Group Request Target Members: optional group request target members: Indicates subset of members of a group for which fanout is to be executed. Example usage of Group Request Target Members: if fanout operation failed for some of the members then the Originator may use this parameter to execute fanout for failed members of a previous fanout operation.

- Filter Criteria: optional filter criteria: conditions for filtered operations which are described in table 8.1.2-2. This is used for resource discovery (clause 10.2.6) and general Create, Retrieve, Update, Delete requests (clauses 10.1.3, 10.1.4 and 10.1.5).

The Filter Criteria set includes matching conditions and filter handling conditions. Matching conditions are evaluated against resources and, when true, determine the matched resources which compose the matching result. The filter handling conditions provide additional input used to determine the filtering result (e.g. maximum number of resources to be included in the filtering result). The filtering result may be composed of one or more resources and determines the target resources for the operation.

Example usage of retrieve requests with filter criteria using modifiedSince condition tag: if a target resource is modified since 12:00 then the Hosting CSE will identify it as a matched resource.

- Discovery Result Type: Optional Discovery result format: indicates the preference of the Originator for the format of returned information in the result of discovery-based operations. This parameter applies to requests where the filterUsage parameter is set (see filterUsage in table 8.1.2-2 and clause 10.2.6)This parameter shall take on one of the following values reflecting the options in clause 9.3.1:

- Hierarchical addressing method

- Non-hierarchical addressing method.

- Note that for discovery-based operations: For Retrieve request containing the Result Content parameter set to “attributes+child-resource-references” or “child-resource-references”, the addressing method applies to the child resource references which are child resource identifiers. For Create, Update and Delete requests and for  Retrieve requests without the Result Content parameter, the addressing method applies to the target resource identifiers.

- For example, if Discovery Result Type is set to Non-hierarchical addressing method, then the request Originator indicates that the result should be in the form of one or more Non-hierarchical addresses.

The absence of the parameter implies that the result shall be in the form of a Hierarchical address.

- Token Request Indicator: Optional parameter used to indicate that the Originator supports the Token Request procedure, and the Originator may attempt the Token Request procedure if the Receiver provides a Token Request Information parameter in the response.

- Tokens: Optional parameter used to transport ESData-protected Tokens applicable to the request for use in Indirect Dynamic Authorization.

- Token IDs: Optional parameter used to transport Token-IDs applicable to the request for use in Indirect Dynamic Authorization.

- Local Token IDs: Optional parameter used to transport Local-Token-IDs applicable to the request for use in Indirect Dynamic Authorization.

- Authorization Signature Indicator: Optional parameter used to indicate the capability for creating AuthorRelMapRecord when Originator is an AE. If the Hosting CSE does not support this parameter, the Hosting CSE should ignore it. The details of the AuthorRelMapRecord are described in clause 7.3.2.2 of oneM2M TS-0003 [Error: Reference source not found].

- Authorization Signature: Optional parameter used to transport the signatures for Token(s) or TokenID(s) generated using the certificate of the AE or a MIC generated using a symmetric key shared between the AE and DAS server.

- Authorization Relationship Indicator: Optional parameter used to indicate that the relationship between the AE and the Token(s) are maintained in the DAS server.

- Semantic Query Indicator: Optional parameter used to indicate whether a RETRIEVE request is a semantic query or a semantic resource discovery. If the request contains this parameter with the value set to “TRUE”, the request shall be processed as a semantic query based on the SPARQL query statement included in the “semanticsFilter” condition tag; other Filter Criteria and the following parameters shall be ignored: Discovery Result Type, Delivery Aggregation. The parameter Result Content shall be set to semantic-content to indicate that the response message contains the result of a semantic query request.  If it is not set or set to “FALSE” the request shall be processed as a semantic resource discovery.

- Release Version Indicator: This parameter is used to indicate the oneM2M release version that this request message conforms to.  Starting with Release 2 this parameter is mandatory. The release version indicated shall apply to all oneM2M defined request parameters and certain types of content carried in the Content request parameter.   Within the Content request parameter, the release version indicated shall apply to all oneM2M defined elements (e.g. notifications) and resource types with the exception of <flexContainer> and <mgmtObj> specializations which have their own version implicitly defined by their respective containerDefinition and mgmtSchema attributes.  In addition, the release version indicated does not apply to resource types or specializations defined external to oneM2M.

- Vendor Information: This optional parameter is available to convey vendor specific information. The use of this parameter is not defined by oneM2M specifications.

Table 8.1.2-2: Filter Criteria conditions

| Condition tag | Multiplicity | Description |
| --- | --- | --- |
| Matching Conditions |  |  |
| createdBefore | 0..1 | The creationTime attribute of the matched resource is chronologically before the specified value. |
| createdAfter | 0..1 | The creationTime attribute of the matched resource is chronologically after the specified value. |
| modifiedSince | 0..1 | The lastModifiedTime attribute of the matched resource is chronologically after the specified value. |
| unmodifiedSince | 0..1 | The lastModifiedTime attribute of the matched resource is chronologically before the specified value. |
| stateTagSmaller | 0..1 | The stateTag attribute of the matched resource is smaller than the specified value. |
| stateTagBigger | 0..1 | The stateTag attribute of the matched resource is bigger than the specified value. |
| expireBefore | 0..1 | The expirationTime attribute of the matched resource is chronologically before the specified value. |
| expireAfter | 0..1 | The expirationTime attribute of the matched resource is chronologically after the specified value. |
| labels | 0..1 | The labels attribute of the matched resource matches the specified value. |
| labelsQuery | 0..1 | The value is an expression for the filtering of labels attribute of resource when it is of key-value pair format. The expression is about the relationship between label-key and label-value which may include equal to or not equal to, within or not within a specified set etc. For example, label-key equals to label value, or label-key within {label-value1, label-value2}. Details are defined in [3] |
| childLabels | 0..1 | A child of the matched resource has labels attributes matching the specified value. The evaluation is the same as for the labels attribute above. Details are defined in [3]. |
| parentLabels | 0..1 | The parent of the matched resource has labels attributes matching the specified value. The evaluation is the same as for the labels attribute above. Details are defined in [3]. |
| resourceType | 0..n | The resourceType attribute of the matched resource is the same as the specified value. It also allows differentiating between normal and announced resources. |
| childResourceType | 0..n | A child of the matched resource has the resourceType attribute the same as the specified value. |
| parentResourceType | 0..1 | The parent of the matched resource has the resourceType attribute the same as the specified value. |
| sizeAbove | 0..1 | The contentSize attribute of the <contentInstance> matched resource is equal to or greater than the specified value. |
| sizeBelow | 0..1 | The contentSize attribute of the <contentInstance> matched resource is smaller than the specified value. |
| contentType | 0..n | The contentInfo attribute of the <contentInstance> matched resource matches the specified value. |
| attribute | 0..n | This is an attribute of resource types (clause 9.6). Therefore, a real tag name is variable and depends on its usage and the value of the attribute can have wild card *. E.g. creator of container resource type can be used as a filter criteria tag as "creator=Sam", "creator=Sam*", "creator=*Sam". |
| childAttribute | 0..n | A child of the matched resource meets the condition provided. The evaluation of this condition is similar to the attribute matching condition above. |
| parentAttribute | 0..n | The parent of the matched resource meets the condition provided. The evaluation of this condition is similar to the attribute matching condition above. |
| semanticsFilter | 0..n | Both semantic resource discovery and semantic query use semanticsFilter to specify a query statement that shall be specified in the SPARQL query language [Error: Reference source not found]. When a CSE receives a RETRIEVE request including a semanticsFilter, and the Semantic Query Indicator parameter is also present in the request, the request shall be processed as a semantic query; otherwise, the request shall be processed as a semantic resource discovery. In the case of semantic resource discovery targeting a specific resource, if the semantic description contained in the <semanticDescriptor> of a child resource matches the semanticFilter, the URI of this child resource will be included in the semantic resource discovery result. In the case of semantic query, given a received semantic query request and its query scope, the SPARQL query statement shall be executed over aggregated semantic information collected from the semantic resource(s) in the query scope and the produced output will be the result of this semantic query. Examples for matching semantic filters in SPARQL to semantic descriptions can be found in [Error: Reference source not found]. |
| filterOperation | 0..1 | Indicates the logical operation (AND/OR) to be used for different condition tags. The default value is logical AND. |
| contentFilterSyntax | 0..1 | Indicates an identifier for the syntax usedin contentFilterQuery for content-based discovery. |
| contentFilterQuery | 0..1 | The query string shall be specified when contentFilterSyntax parameter is present. |
| Filter Handling Conditions |  |  |
| filterUsage | 0..1 | Indicates how the filter criteria is used. If provided, possible values are 'discoveryBasedOperation' and 'IPEOnDemandDiscovery'. If this parameter is not provided, the Retrieve operation is a generic retrieve operation and the content of the child resources fitting the filter criteria is returned. If this parameter is provided, the matching conditions in this table are evaluated against resources and, when true, determine the matching result. Other filter handling conditions provide additional input used to determine the filtering result, which may be composed of zeroor more resources. If filterUsage is 'discoveryBasedOperation' and the operation is a RETRIEVE, the request is for resource discovery (clause 10.2.6), i.e. only the addresses of the resources in the filtering result are returned. If filterUsage is ‘discoveryBasedOperation’ and the operation is CREATE, UPDATE or DELETE, the operation is applied to all the resources in the filtering result. If filterUsage is 'IPEOnDemandDiscovery', the other filter conditions are sent to the IPE as well as the discovery Originator ID. When the IPE successfully generates new resources matching with the conditions, then the resource address(es) shall be returned. This value shall only be valid for the Retrieve request targeting an <AE> resource that represents the IPE. |
| limit | 0..1 | The maximum number of resources to be included in the filtering result. This may be modified by the Hosting CSE. When it is modified, then the new value shall be smaller than the suggested value by the Originator. |
| level | 0..1 | The maximum level of resource tree that the Hosting CSE shall perform the operation starting from the target resource (i.e. To parameter). This shall only be applied for Retrieve operation. The level of the target resource itself is zero and the level of the direct children of the target is one. |
| offset | 0..1 | The number of direct child and descendant resources that a Hosting CSE shall skip over and not include within a Retrieve response when processing a Retrieve request to a targeted resource. |
| applyRelativePath | 0..1 | This attribute contains a resource tree relative path (e.g. ../tempContainer/LATEST). This condition applies after all the matching conditions have been used (i.e. a matching result has been obtained). The attribute determines the set of resource(s) in the final filtering result. The filtering result is computed by appending the relative path to the path(s) in the matching result. All resources whose Resource-IDs match that combined path(s) shall be returned in the filtering result. If the relative path does not represent a valid resource, the outcome is the same as if no match was found, i.e. there is no corresponding entry in the filtering result. |

The rules when multiple matching conditions are used together shall be as follows:

- Different condition tags shall use the "AND/OR" logical operation based on the filterOperation specified;

e.g. createdBefore = "time1" AND unmodifiedSince = "time2" if filterOperation = "AND" or "NULL", or createdBefore = "time1" OR unmodifiedSince = "time2" if filterOperation = "OR".

- Same condition tags shall use the "OR" logical operation, i.e. filterOperation doesn't apply to same conditions.

- No mixed AND/OR filter operation will be supported.

Once the Request is delivered, the Receiver shall analyze the Request to determine the target resource.

If the target resource is addressing another M2M Node, the Receiver shall route the request appropriately.

If the target resource is addressing the Receiver, it shall:

- Check the existence of To addressed resource.

- Identify the resource type by Resource Type.

- Check the privileges for From Originator to perform the requested operation.

- Perform the requested operation (using Content content when provided) according to the provided request parameters as described above.

- Depending on the request result content, respond to the Originator with indication of successful or unsuccessful operation results. In some specific cases (e.g. limitation in the binding protocol or based on application indications), the Response could be avoided.

Table 8.1.2-3 summarizes the parameters specified in this clause for the Request message, showing any differences as applied to C, R, U, D or N operations. "M" indicates mandatory, "O" indicates optional, "N/A" indicates "not applicable".

Table 8.1.2-3: Summary of Request Message Parameters

| Request message parameter |  | Operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Create | Retrieve | Update | Delete | Notify |
| Mandatory | Operation - operation to be executed | M | M | M | M | M |
|  | To - the address of the target resource on the target CSE | M | M | M | M | M |
|  | From - the identifier of the message Originator | O See note 1 | M | M | M | M |
|  | Request Identifier - uniquely identifies a Request message | M | M | M | M | M |
| Operation dependent | Content - to be transferred | M | O | M | N/A | M |
|  | Resource Type - of resource to be created | M | N/A | N/A | N/A | N/A |
| Optional | Originating Timestamp - when the message was built | O | O | O | O | O |
|  | Request Expiration Timestamp - when the request message expires | O | O | O | O | O |
|  | Result Expiration Timestamp - when the result message expires | O | O | O | O | O |
|  | Operational Execution Time - the time when the specified operation is to be executed by the target CSE | O | O | O | O | O |
|  | Response Type - type of response that shall be sent to the Originator | O | O | O | O | O |
|  | Result Persistence - the duration for which the reference containing the responses is to persist | O | O | O | O | N/A |
|  | Result Content - the expected components of the result | O | O | O | O | N/A |
|  | Event Category - indicates how and when the system should deliver the message | O | O | O | O | O |
|  | Delivery Aggregation - aggregation of requests to the same target CSE is to be used | O | O | O | O | O |
|  | Group Request Identifier - Identifier added to the group request that is to be fanned out to each member of the group | O | O | O | O | O |
|  | Group Request Target Members-indicates subset of members of a group | O | O | O | O | N/A |
|  | Filter Criteria - conditions for filtered retrieve operation | O | O | O | O | O |
|  | Discovery Result Type - format of information returned for Discovery operation | N/A | O | N/A | N/A | N/A |
|  | Token Request Indicator - indicating that the Originator may attempt Token Request procedure (for Dynamic Authorization) if initiated by the Receiver | O | O | O | O | O |
|  | Tokens - for use in dynamic authorization | O | O | O | O | O |
|  | Token IDs - for use in dynamic authorization | O | O | O | O | O |
|  | Role IDs - for use in role based access control | O | O | O | O | O |
|  | Local Token IDs - for use in dynamic authorization | O | O | O | O | O |
|  | Authorization Signature Indicator – for use in Authorization Relationship Mapping | O | O | O | O | N/A |
|  | Authorization Signature – for use in Authorization Relationship Mapping | O | O | O | O | N/A |
|  | Authorization Relationship Indicator - for use in Authorization Relationship Mapping | O | O | O | O | N/A |
|  | Semantic Query Indicator – for use in semantic queries | N/A | O | N/A | N/A | N/A |
|  | Release Version Indicator – the oneM2M release version that this request message conforms to. | M See note 2 | M See note 2 | M See note 2 | M See note 2 | M See note 2 |
|  | Vendor Information | O | O | O | O | O |
| NOTE: 1.From parameter is optional in case of an AE CREATE request and mandatory for all other requests. 2. Release Version Indicator parameter is not present for the case when a request is targeting a Rel-1 entity and mandatory for all other cases. |  |  |  |  |  |  |

### 8.1.3	Response

The Response received by the Originator of a Request accessing resources over the Mca and Mcc reference points shall contain mandatory and may contain optional parameters. Certain parameters may be mandatory or optional depending upon the Requested operation (CRUDN) or the mandatory response code. In this clause, the mandatory parameters are detailed first, followed by those that are conditional, and then by those that are optional:

Mandatory Parameters:

- Response Status Code: response status code: This parameter indicates that a result of the requested operation is successful, unsuccessful, acknowledgement or status of processing such as authorization timeout, etc.:

  - A successful code indicates to the Originator that the Requested operation has been executed successfully by the Hosting CSE.

  - An unsuccessful code indicates to the Originator that the Requested operation has not been executed successfully by the Hosting CSE.

  - An acknowledgement indicates to the Originator that the Request has been received and accepted by the attached CSE, i.e. by the CSE that received the Request from the issuing Originator directly, but the Request operation has not been executed yet. The success or failure of the execution of the Requested operation is to be conveyed later.

Details of successful, unsuccessful and acknowledge codes are provided in clause 6.6 of oneM2M TS-0004 [Error: Reference source not found].

- Request Identifier: Request Identifier. The Request Identifier in the Response shall match the Request Identifier in the corresponding Request.

Conditional Parameters:

- Content: resource content:

  - If Response Status Code is successful then:

    - The Content response parameter may be present in a Create/Update/Delete Response and the information in this Content response parameter depends on the value of the Result Content and/or Discovery Result Type request parameters of the corresponding Request. If the value of the Result Content request parameter is “nothing” or if the Result Content request parameter is not present in a Delete Request, the Content response parameter shall not be present. Otherwise, the Content response parameter shall be present in the following cases:

    - When the FilterUsage parameter is not present, the Content parameter shall be present in a Retrieve Response and the information in this Content parameter depends on the Result Content value of the corresponding Retreive Request. If Response Status Code is unsuccessful then the Content parameter may be present in a Response to provide more error information.

    - When the FilterUsage parameter is set, the Content parameter shall be present in Create, Retreieve, Update and Delete operations and the information in this Content parameter depends on the Discovery Result Type value of the corresponding request. If Response Status Code is unsuccessful then the Content parameter may be present in a Response to provide more error information.

    - For filtered Retrieve requests or discovery-based operations which result in no target resources being identified, the Content indicates an empty result.

  - If Response Status Code is acknowledgment, then the Content parameter:

    - Shall contain the address of a <request> resource if the response was an acknowledgement of a non-blocking request and the <request> resource type is supported by the Receiver CSE.

    - Is not present otherwise.

- Content Status: This parameter shall be present in the response to a Retrieve operation when the returned content is partial. More specifically, this parameter takes the value of partial depending on the Content parameter.

  - If Response Code is successful then and the Content parameter is present due to the following case:

    - Retrieve (R): Content is the retrieved resource content or aggregated contents of discovered resources and the retrieved content is partial

Then Content Status parameter shall be present in the response for a Retrieve (R) operation

- Content Offset: This parameter includes the point where a Hosting CSE left off with processing a Retrieve operation that resulted in a response with partial content (i.e. due to reaching the limit on the number of resources allowed in a response). This parameter shall be expressed as a number which can be used in a subsequent Retrieve request. The parameter shall be used by the Hosting CSE to skip over the specified number of direct child and descendant resources of a targeted resource and retrieve the remaining direct child and descendant resources. Its value depends on the information included in the Content Status parameter

  - If Content Status parameter is complete, then this parameter shall not be included

  - If Content Status parameter is partial, then this shall include the offset where processing can restart for the remaining descendant resources in the resource tree.

  - Then Content Offset parameter shall be present in the response for a Retrieve (R) operation.

Optional parameters:

- To: ID of the Originator or the Transit CSE.

- From: ID of the Receiver.

The To and From parameters can be used in the response for specific protocol bindings (e.g. MQTT):

- Originating Timestamp: originating timestamp of when the message was built.

- Result Expiration Timestamp: result expiration timestamp. The Receiver shall echo the result expiration timestamp if set in the Request message, or may set the result expiration timestamp itself.

Example usage of the Receiver setting the result expiration timestamp is when the value of the delivery time is dependent upon some changing Receiver context e.g. Result message deadline for aircraft position based upon velocity.

- Event Category: event category: Indicates the event category that should be used to handle this response. The definition of event category is the same as in the case of requests in clause 8.1.2.

Example usage of "event category" set to specific value X: When the response is targeted to an entity that is different from the Transit CSE currently processing the response message and is not an AE registered with the Transit CSE that is currently processing the response message, the response may be stored in the Transit CSE that is currently processing the response on the way to the destination of the response message until it is allowed by provisioned policies for that event category X to use a communication link to reach the next CSE on a path to the destination of the response message or until the result expiration timestamp is expired.

- Token Request Information: Optional parameter which may be used for requesting Tokens from Dynamic Authorization Systems.

- Assigned Token Identifiers: Optional parameter containing the mapping from assigned Local-Token-IDs to corresponding Token-IDs.

- Authorization Signature Request Information: Optional parameter used to request the Authorization Signature(s) of the Token(s) from an AE which is the holder of the Token(s).

- Release Version Indicator: This parameter is used to indicate the oneM2M release version that this response message is compliant with.  Starting with Release 2 this parameter is mandatory. The release version indicated shall apply to all oneM2M defined response parameters and certain types of content carried in the Content response parameter.   Within the Content response parameter, the release version indicated shall apply to all oneM2M defined elements (e.g. notifications) and resource types with the exception of <flexContainer> and <mgmtObj> specializations which have their own version implicitly defined by their respective containerDefinition and mgmtSchema attributes.   The release version indicated does not apply to resource types or specializations defined external to oneM2M.

- Vendor Information: This optional parameter is available to convey vendor specific information. The use of this parameter is not defined by oneM2M specifications.

- Table 8.1.3-1 summarizes the parameters specified in this clause for the Response messages, showing any differences as applied to successful C, R, U, D or N operations, and unsuccessful operations. "M" indicates mandatory, "O" indicates optional, "N/A" indicates "not applicable".

Table 8.1.3-1: Summary of Response Message Parameters

| Response message parameter/success or not | Ack |  |  | successful Operation |  |  |  |  |  |  |  |  |  | unsuccessful Operation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Create |  | Retrieve |  | Update | Delete |  |  | Notify |  | Create / Retrieve / Update / Delete |  | Notify |
| Response Status Code - successful, unsuccessful, ack |  | M | M |  | M |  | M |  |  | M | M |  | M |  | M |  |
| Request Identifier - uniquely identifies a Request message |  | M | M |  | M |  | M |  |  | M | M |  | M |  | M |  |
| Content - to be transferred |  | O (address of <request> resource if response is ACK of a non-blocking request) | O (The address and/or the content of the created resource. More than one entry may be provided for discovery-based operations) |  | M (the retrieved resource content or aggregated contents or an address list) |  | O (The content replaced in an existing resource. The content of the new attributes created. The name of the attributes deleted. More than one entry may be provided for discovery-based operations) |  |  | O (The content actually deleted. More than one entry may be provided for discovery-based operations) | O (see note 1, end-to-end security protocol message) |  | O (Additional error info) |  | O (see note, additional error info secured using ESPrim) |  |
| To - the identifier of the Originator or the Transit CSE that sent the corresponding non-blocking request |  | O | O |  | O |  | O |  |  | O | O |  | O |  | O |  |
| From - the identifier of the Receiver |  | O | O |  | O |  | O |  |  | O | O |  | O |  | O |  |
| Originating Timestamp - when the message was built |  | O | O |  | O |  | O |  |  | O | O |  | O |  | O |  |
| Result Expiration Timestamp - when the message expires |  | O | O |  | O |  | O |  |  | O | O (see note) |  | O |  | O (see note) |  |
| Event Category - what event category shall be used for the response message |  | O | O |  | O |  | O |  |  | O | O |  | O |  | O |  |
| Content Status |  | N/A | N/A |  | O |  | N/A |  |  | N/A | N/A |  | N/A |  | N/A |  |
| Content Offset |  | N/A | N/A |  | O |  | N/A |  |  | N/A | N/A |  | N/A |  | N/A |  |
| Token Request Information |  | N/A | N/A |  | N/A |  | N/A |  |  | N/A | N/A |  | O |  | O |  |
| Assigned Token Identifiers |  | N/A | O |  | O |  | O |  |  | O | O |  | O |  | O |  |
| Authorization Signature Request Information |  | N/A | N/A |  | N/A |  | N/A |  |  | N/A | N/A |  | O |  | N/A |  |
| Release Version Indicator - the oneM2M release version that this response message conforms to |  | M (See note 2) | M (See note 2) |  | M (See note 2) |  | M (See note 2) |  |  | M (See note 2) | M (See note 2) |  | M (See note 2) |  | M (See note 2) |  |
| Vendor Information |  | O | O |  | O |  | O |  |  | O | O |  | O |  | O |  |
| NOTE: 1.This parameter is present if the response contains an end-to-end security protocol message. Otherwise this parameter is not applicable. 2. Release Version Indicator parameter is not present for the case when a response is targeting a Rel-1 entity and mandatory for all other cases. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### ********************** End of Change 1******************************************

### ********************** Start of Change 2******************************************

- 10.2.6	Discovery and Discovery-based Operations

- 10.2.6.1	Discovery without Result Content parameter

This is the resource discovery procedure which returns matching resource identifiers. Note that the returned information is the difference compared to the other discovery mechanism in the present document which involves the Result Content parameter (clause 10.2.6.2).

The resource discovery procedures allow discovering of resources residing on a CSE. The filterUsage parameter shall be set in this case.and the use of the Filter Criteria parameter allows limiting the scope of the results.

Resource discovery shall be accomplished using the RETRIEVE method by an Originator which shall also include the root of where the discovery begins: e.g. <CSEBase>. The unfiltered result of the resource discovery procedure includes all the child resources under the root of where the discovery begins, which the Originator has a Discover access right on. The unfiltered results do not include any resources whose status is marked as “INACTIVE”, as well as any child resources of these “INACTIVE” resources.

Filter criteria conditions may be provided as parameters to the RETRIEVE method. The filter criteria conditions describe the rules for resource discovery, e.g. resource types, creation time and matching string. The filter criteria can also contain the parameters for specifying the maximum number of discovered resources included in the response, the maximum limit on the number of levels in the resource tree (starting from the target resource) that the Hosting CSE shall perform the discovery request upon and an offset for specifying the number of discovered resources the Hosting CSE shall skip over and not include within the response. Table 8.1.2-2 describes the Filter Criteria parameter.

A match shall happen when a resource matches the configured filter criteria conditions and the Originator has a Discover access right on the resource. A successful response contains a list for the matched resources addressable in any of the forms expressed in clause 9.3.1 if matches are found. If no matches are found, a successful response returns no matched resources. If Discovery Result Type parameter is specified in a discovery request, the Hosting CSE shall choose the addressing form specified by the Discovery Result Type parameter.

The discovery results may be modified by the Hosting CSE to restrict the scope of discoverable resources according to the Originator's access control policy or M2M service subscription.

The Hosting CSE may also implement a configured upper limit on the size of the answer. In such a case when the Originator and the Hosting CSE have different upper limits, the smaller of the two shall apply.

This procedure shall be used for the discovery of resources under <CSEBase> that match the provided Filter Criteria parameter. The discovery result shall be returned to the Originator using a successful Response message.

Table 10.2.6.1-1: Discovery procedure via Retrieve Operation

| Discovery <resource> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: For the allowed Result Content parameter options for Discovery related RETRIEVE see clause 8.1.2. To: Address of the root of where the discovery begins. Filter Criteria: Filter criteria for searching and expected returned result. The filterUsage parameter shall be set in this case. Discovery Result Type: optional, format of discovery results returned (see clause 8.1.2 for options applicable to Discovery, and how results shall be displayed). |
| Processing at Originator before sending Request | According to clause 10.1.3 with the following: Setup the RETRIEVE operation in the Request. Include the conditions in the filter criterion to limit the scope of the discovery results. Specify the desired format of returned discovery results. |
| Processing at Receiver | According to clause 10.1.3 with the following specific processing: Checks the validity of the Request (e.g. format of Filter Criteria). May change the filter criteria according to local policies. Searches matched resources as per the DISCOVER privileges from the addressed resource hierarchy. Any resources whose status is marked as “INACTIVE” are not searched, as well as any child resources of these “INACTIVE” resources. Limits the discovery result according to the upper limit on the size of the answer. The Hosting CSE shall use the appropriate addressing (see clause 9.3.1) form for each element included in the list in accordance with the incoming request. If Filter Criteria is provided in the request, the Hosting CSE uses it identifying the resources whose attributes match the Filter Criteria. The Hosting CSE shall respond to the Originator with the appropriate list of discovered resources in the Hosting CSE. If the Filter Criteria includes filterUsage element set to "IPEOnDemandDiscovery", the target is the <AE> resource and the Hosting CSE has no match from the discovery of existing resources, then the Hosting CSE shall send a NOTIFY request containing the Filter Criteria to the AE(i.e. pointOfAccess of the <AE> resource) and the Originator ID of this discovery request. When the CSE gets the successful NOTIFY response with the resource address(es) which are created under the <AE> resource, then the CSE shall check the DISCOVER privilege and return the address(es) to the Originator. When the CSE gets the unsuccessful NOTIFY response, then the CSE shall send the Response Status Code in the NOTIFY response to the Originator. The Hosting CSE may modify the Filter Criteria including upper limit provided by the Originator or the discovery results based on the local policies. If the size of the result list is bigger than the upper limit or the scope of discoverable resources, according to the Originator's access control policy or service subscription has been modified by the Hosting CSE, the full list is not returned. Instead, an incomplete list is returned and an indication is added in the response for warning the requestor. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: Contains the address list of discovered resources expressed in any of the methods depicted in clause 9.3.1. The address list may be empty if no result matching the filter criterion is discovered. Contains an incomplete list warning if the full list is not returned. |
| Processing at Originator after receiving Response | According to clause 10.1.3. |
| Exceptions | According to clause 10.1.3, with the following: The request contains invalid parameters. The on-demand discovery was rejected by the requested M2M Application. |

- 10.2.6.2	Discovery with Result Content parameter

When a Retrieve request contains the Result Content parameter set to “attributes+child-resource-references

” or “child-resource-references”, the Hosting CSE returns child resource references which are child resource identifiers. Similar to the ordinary resource discovery (see 10.2.6.1), the filterUsage parameter shall be set and the Filter Criteria parameter allows for for filtering child/descendant resources. Unlike ordinary resource discovery, the Hosting CSE checks RETRIEVE privilege of the Originator to determine whether the matching resource identifier can be returned.

- 10.2.6.3	Discovery-based Operations

Discovery-based operations are all operations where the FilterUsage parameter is ‘discoveryBasedOperation', as the use of Filter Criteria parameter results in identification of one or more resources used as target of the operation. The resource discovery procedures detailed in  10.2.6.1 and 10.2.6.2 are examples of discovery-based operations for Retrieve requests, therefore this clause addresses mainly Create, Update and Delete operations.

Discovery-based operations allow the execution of an operation on multiple resources residing on a CSE identified by applying the Filter Criteria parameter rules. The Filter Criteria parameter also provides means for limiting the scope of the filtering results for determination of the target resources for the operation.

The identification of the target resources shall be accomplished the same way as in the Discovery procedure described in 10.2.6.1. The matching resource set shall be identified based on resources meeting the matching filter criteria conditions. Target resources shall be identified after the filter handling conditions have been applied to the matching resource set, as detailed in clause 8.1.2. In addition, the Hosting CSE shall restrict the set of target resources according to the Originator's access control policy or M2M service subscription for the Create/Update/Retrieve operation indicated in the request.

The Hosting CSE may implement a configured upper limit on the size of the set in a discovery-based operation, based on local policy. In such a case when the Originator and the Hosting CSE have different upper limits, the smaller of the two shall apply.

A successful response contains a list of targeted resources resulting from the discovery operation, with the components based on the Result Content parameter (clause 10.2.6.2). If no matches are found, a successful response returns no matched resources. If Discovery Result Type parameter is specified in a discovery request, the Hosting CSE shall choose the addressing form specified by the Discovery Result Type parameter.

Procedural information for each of the discovery-based operations (Create, Update, Delete) is further provided in tables 10.2.6.3-1, 10.2.6.3-2 and 10.2.6.3-3, respectively.

Table 10.2.6.3-1: Discovery-based CREATE Operation

| Discovery-based <resource> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: For the allowed Result Content parameter options see clause 8.1.2. To: Address of the root of where the discovery begins. Filter Criteria: Filter criteria for searching. The filterUsage parameter shall be set to ‘discoveryBasedOperation' in this case. |
| Processing at Originator before sending Request | According to clause 10.1.2 with the following: Setup the CREATE operation in the Request. Include the conditions in the filter criterion to limit the scope of the discovery results. Specify the desired format of returned operation results. |
| Processing at Receiver | According to clause 10.1.2 with the following specific processing: Checks the validity of the Request (e.g. Content, format of Filter Criteria). May change the filter criteria according to local policies. Searches matched resources as per the CREATE privileges from the addressed resource hierarchy. Limits the discovery result according to the upper limit on the size of the answer. The Hosting CSE uses the matching Filter Criteria and the filter handling conditions provided in the request for identifying the target resources as described in clause 8.1.2. The target resource set may be restricted by applying upper limits based on the local policies, in addition to those provided by the Originator. The Hosting CSE may also modify the target resource set according to the Originator's access control policy or service subscription. The target resource set may be empty if no resources are discovered based on the matching Filter Criteria and the filter handling conditions. The Hosting CSE performs the CREATE operation on all the resources included in the target resource set. If errors are encountered in the resource creation process at the various targets identified in the discovery process, the Hosting CSE tracks them and continues the creation process until the resource target list is exhausted. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for Content as follows: Contains the address list of all the resources created. The addressing form (see clause 9.3.1) shall be based on the incoming request for all elements included. The list may be empty if no resources are identified as targets. The General Exceptions in clause 10.1.2 apply as follows: - If the Originator does not have CREATE privileges for any resource from the target set, the Receiver responds with an error. There is no corresponding error provided in the final response if there are no privileges for only some target resources. - If the resource with the specified name (if provided) already exists at all the resource from the target set, the Receiver responds with an error. There is no corresponding error provided in the final response if the resource with the specified name already exists at only some of the resources from the target set, - If the provided information in Content or Filter Criteria is not accepted by the Receiver (e.g. missing mandatory parameter), the Receiver responds with an error. Contains an incomplete list warning if the full list is not returned. |
| Processing at Originator after receiving Response | According to clause 10.1.2. |
| Exceptions | According to clause 10.1.2, with the following: The request contains invalid parameters. |

Table 10.2.6.3-2: Discovery-based UPDATE Operation

| Discovery-based <resource> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: For the allowed Result Content parameter options see clause 8.1.2. To: Address of the root of where the discovery begins. Filter Criteria: Filter criteria for searching. The filterUsage parameter shall be set to ‘discoveryBasedOperation' in this case. |
| Processing at Originator before sending Request | According to clause 10.1.4 with the following: Setup the UPDATE operation in the Request. Include the conditions in the filter criterion to limit the scope of the discovery results. Specify the desired format of returned operation results. |
| Processing at Receiver | According to clause 10.1.4 with the following specific processing: Checks the validity of the Request (e.g. Content, format of Filter Criteria). May change the filter criteria according to local policies. Searches matched resources as per the UPDATE privileges from the addressed resource hierarchy. Limits the discovery result according to the upper limit on the size of the answer. The Hosting CSE uses the matching Filter Criteria and the filter handling conditions provided in the request for identifying the target resources as described in clause 8.1.2. The target resource set may be restricted by applying upper limits based on the local policies, in addition to those provided by the Originator. The Hosting CSE may also modify the target resource set according to the Originator's access control policy or service subscription. The target resource set may be empty if no resources are discovered based on the matching Filter Criteria and the filter handling conditions. The Hosting CSE performs the UPDATE operation on all the resources included in the target resource set. If errors are encountered in the resource update process at the various targets identified in the discovery process, the Hosting CSE tracks them and continues the creation process until the resource target list is exhausted. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for Content as follows: Contains the address list of all the resources updated. The addressing form (see clause 9.3.1) shall be based on the incoming request for all elements included. The list may be empty if no resources are identified as targets. The General Exceptions in clause 10.1.4 apply as follows: - If the resource in To parameter does not exist, the Receiver responds with an error. - If the Originator does not have UPDATE privileges on any resource from the target set, the Receiver responds with an error. There is no corresponding error provided in the final response if there are no privileges for only some target resources. - If the provided information in Content or Filter Criteria is not accepted by the Receiver (e.g. missing mandatory parameter), the Receiver responds with an error. |
| Processing at Originator after receiving Response | According to clause 10.1.4. |

Table 10.2.6.3-3: Discovery-based DELETE Operation

| Discovery-based <resource> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: For the allowed Result Content parameter options see clause 8.1.2. To: Address of the root of where the discovery begins. Filter Criteria: Filter criteria for searching. The filterUsage parameter shall be set to ‘discoveryBasedOperation' in this case. |
| Processing at Originator before sending Request | According to clause 10.1.5 with the following: Setup the DELETE operation in the Request. Include the conditions in the filter criterion to limit the scope of the discovery results. Specify the desired format of returned operation results. |
| Processing at Receiver | According to clause 10.1.5 with the following specific processing: Checks the validity of the Request (e.g. format of Filter Criteria). May change the filter criteria according to local policies. Searches matched resources as per the DELETE privileges from the addressed resource hierarchy. Limits the discovery result according to the upper limit on the size of the answer. The Hosting CSE uses the matching Filter Criteria and the filter handling conditions provided in the request for identifying the target resources as described in clause 8.1.2. The target resource set may be restricted by applying upper limits based on the local policies, in addition to those provided by the Originator. The Hosting CSE may also modify the target resource set according to the Originator's access control policy or service subscription. The target resource set may be empty if no resources are discovered based on the matching Filter Criteria and the filter handling conditions. The Hosting CSE performs the DELETE operation on all the resources included in the target resource set. If errors are encountered in the resource deletion process at the various targets identified in the discovery process, the Hosting CSE tracks them and continues the creation process until the resource target list is exhausted. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for Content as follows: Contains the address list of all the resources deleted. The addressing form (see clause 9.3.1) shall be based on the incoming request for all elements included. The list may be empty if no resources are identified as targets. The General Exceptions in clause 10.1.5 apply as follows: - If the resource in To parameter does not exist, the Receiver responds with an error. - If the Originator does not have DELETE privileges on any resource from the target set, the Receiver responds with an error. There is no corresponding error provided in the final response if there are no privileges for only some target resources. |
| Processing at Originator after receiving Response | According to clause 10.1.5. |

### ********************** End of Change 2******************************************