| Input Contribution |  |
| --- | --- |
| Meeting ID* | ARC 38 |
| Title:* | Identifying User that Originates Request |
| Source:* | Dale Seed, Convida, Seed.Dale@convidawireless.com |
| Date:* | 2018-12-01 |
| Input related to* | WI-0083 - oneM2M Service Subscribers and Users (TR-0054) |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) |  |
| Decision requested or recommendation:* | Approval for insertion into TR-0054 |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution proposes to add an alternative method for indentifying the M2M Service User or Subscriber that orginates a oneM2M Request.  Rather than appending the M2M Service User identifier to the From request parameter, this contribution defines a new oneM2M request parameter called User.   Defining a new oneM2M request parameter may be a better option from a backwards compatability perspective.

### -----------------------Start of change 1-------------------------------------------

### Specifying a new User request parameter

Rather than appending a M2M-User-ID (or M2M-Subscriber-ID) to the From request parameter, which could cause backwards compatability problems, a new oneM2M request parameter can be defined (e.g. User).  This new User request parameter enables a oneM2M request to include the M2M-User-ID (or M2M-Subscriber-ID) of the M2M Service User (or M2M Service Subscriber) originating the request.  For example, a User parameter can be configured with an SP-Relative-M2M-User-ID such as /bob123 or an Absolute-M2M-User-ID such as //myoperator.com/bob123.

-----------------------End of change 1-------------------------------------------