| Input Contribution |  |
| --- | --- |
| Meeting ID* | ARC#38 |
| Title:* | TS-0001 Rel-3 remanining editor’s notes clean-up |
| Source:* | SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr |
| Date:* | 2018-12-02 |
| Input related to* | TS-0001 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) | n/a |
| Decision requested or recommendation:* | Incorporate the changes |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This CR proposes introductory texts that replaces editor’s notes as placeholders in clause 10.2.5 Request message handling.

Questions for further clarification are

- do we have a concept of non-blocking notifications for

  - subscription/notification

  - end-to-end communication, and etc.

*********************** Start of Change 1 ***********************

### 10.2.5	Request message handling

#### 10.2.5.1	Introduction

A request can be processed as blocking, non-blocking synchronous or non-blocking asynchronous mode as illustrated in clause 8.2.1 and 8.2.2. Sometimes, more than one requests can be batched up at a CSE and delivered to another CSE while implicitly creating <delivery> resources. When a request cannot be directly forwarded to an AE or CSE, the AE or CSE can do long polling on incoming the request to it.

To support end-to-end secure communications, a request can be contained in a notify message assuring tamper proof of its content.

#### 10.2.5.2	Non-blocking communication management

Unlikely to blocking requests, non-blocking requests handling involves in implicit resource manipulations (i.e. <request> resource). The following clauses illustrates <request> resource handling procedures.

#### 10.2.5.3	Create <request>

#### 10.2.5.4	Retrieve <request>

*********************** End of Change 1 ***********************

*********************** Start of Change 2 ***********************

#### 10.2.5.20	End-to-end secure communication

End-to-end as Originator to Receiver communication is supported by encapsulating a request in a notify message. Depending on the Receiver entity, a notification target can be <AE> or <CSEBase> resource.

#### 10.2.5.21	End-to-AE communication

This procedure shall be used for sending a Notify request to an <AE> resource in case that the AE represented as the <AE> resource is the final target of the request contained in the notification. When a notification is received at the <AE> resource Hosting CSE, the Hosting CSE shall perform a notify re-targeting (Clause 10.2.5.23).In this description, the Receiver is the CSE hosting the <AE> resource.

Table 10.2.5.21-1: <AE> NOTIFY

| <AE> NOTIFY |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 apply |
| Processing at Originator before sending Request | According to clause 10.1.6 |
| Processing at Receiver | The Hosting CSE shall re-target the Notify request to the AE according to clause 10.2.5.23. |
| Information in Response message | According to clause 10.1.6 |
| Processing at Originator after receiving Response | According to clause 10.1.6 |
| Exceptions | According to clause 10.1.6 |

#### 10.2.5.22	End-to-CSE communication

This procedure shall be used for sending a Notify request to a <CSEBase> resource in case that the CSE represented as the <CSEBase> resource is the final target of the request contained in the notification. In this description, the Receiver is the Hosting CSE of the <CSEBase> resource.

Table 10.2.5.22-1: <CSEBase> NOTIFY

| <CSEBase> NOTIFY |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 apply |
| Processing at Originator before sending Request | According to clause 10.1.6 |
| Processing at Receiver | According to clause 10.1.6. |
| Information in Response message | According to clause 10.1.6 |
| Processing at Originator after receiving Response | According to clause 10.1.6 |
| Exceptions | According to clause 10.1.6 |

*********************** End of Change 2 ***********************