| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | ARC#38 |
| Source:* | NTT |
| Date:* | 2018-12-03 |
| Contact:* | Hiroyuki Maeomichi(maeomichi.hiroyuki@lab.ntt.co.jp), NTT, Kei Harada(harada.kei@lab.ntt.co.jp), NTT |
| Reason for Change/s:* | To enable hierarchical resource content retrieval with discovery |
| CR against: Release* | R4 |
| CR against: WI* | Active < Work Item number(optional)> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No STE Small Technical Enhancements / < Work Item number(optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 V3.12.0 |
| Clauses * | 10.2.6.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | none |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

In use cases like data analysis, bulky data retrieval happens. With current specification, AE needs to issue discovery request to get URI list of matched resource, then issue multiple retrieve requests for each URI. This multiple requests and responses causes execution performance problem and synchronization problem, in which resource discovered by the discovery may deleted when AE tryies to retrieve.

This CR try to enable discovery to have a request with set of matched resource contents with a single operation.

## -----------------------Start of change 1-------------------------------------------

#### 10.2.6.2	Discovery with Result Content parameter

When a Retrieve request contains the Result Content parameter set to “attributes+child-resource-references

” or “child-resource-references”, the Hosting CSE returns child resource references which are child resource identifiers. Similar to the ordinary resource discovery (see 10.2.6.1), this Retrieve request with the Result Content parameter also can contain Filter Criteria parameter for filtering child/descendant resources. Unlike ordinary resource discovery, the Hosting CSE checks RETRIEVE privilege of the Originator to determine whether the matching resource identifier can be returned.

When a Retrieve request contains the Result Content parameter set to “attributes-and-child-resources”, the Hosting CSE returns child resources which are matched with the specified Filter Criteria and their parent resources. Unlike ordinary resource discovery, The Hosting CSE checks RETRIEVE privilege of the Origintator to determine whether the matching resource identifier can be returned.

## -----------------------End of change 1------------------------------------------