| Input Contribution |  |
| --- | --- |
| Meeting ID:* | MAS#31.1 |
| Title:* | modification_of_deviceInfo_access_mode |
| Source:* | Kei Harada(harada.kei@lab.ntt.co.jp), NTT, Hiroyuki Maeomichi(maeomichi.hiroyuki@lab.ntt.co.jp), NTT, |
| Date:* | 2017-10-20 |
| Input related to* | TS-0001 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) | TS-0001 |
| Decision requested or recommendation:* | Request to agree the proposed modifications based on MAS/ARC joint session at TP#31 and ARC#31.1. |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

## Introduction

In current TS-0001, most of [deviceInfo] attributes are RO. In case of using Device Management , Management Agent can create [deviceInfo] locally in CSE so there may not be any problems. However, in case of interworking, IPE cannot write some of [deviceInfo] attributes. This is the problem of current TS-0001.

I consulted ARC and MAS members about this proplem with INC(ARC-2017-0391) at TP31, then we discussed how to fix them. We decided to change [deviceInfo] attributes into WO as the first step.This CR will realize this decision.

I’ve submitted the CR which has almost same contents with this INC to ARC and discussed in ARC#31.1 telcon. ARC members agreed to almost of this change, but I need MAS members’ opinions because [deviceInfo] relates to MAS area.

### -----------------------Start of change 1-------------------------------------------

# D.8	Resource deviceInfo

The [deviceInfo] resource is used to share information regarding the device. The [deviceInfo] resource is a specialization of the <mgmtObj> resource.

Figure D.8-1: Structure of [deviceInfo] resource

The [deviceInfo] resource shall contain the child resources specified in table D.8-1.

Table D.8-1: Child resources of [deviceInfo] resource

| Child Resources of [deviceInfo] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 |

The [deviceInfo] resource shall contain the attributes specified in table D.8-2.

Table D.8-2: Attributes of [deviceInfo] resource

| Attributes of [deviceInfo] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1(L) | RW | See clause 9.6.1.3. |
| mgmtDefinition | 1 | WO | See clause 9.6.15. This attribute shall have the fixed value "deviceInfo". |
| objectIDs | 0..1 (L) | WO | See clause 9.6.15. |
| objectPaths | 0..1 (L) | WO | See clause 9.6.15. |
| description | 0..1 | RW | See clause 9.6.15. |
| deviceLabel | 1 | WO | Unique device label assigned by the manufacturer. This attribute is a specialization of [objectAttribute] attribute. The value of the attribute typically exposes the device’s serial number that is specific to a manufacturer and possibly further restricted within the manufacturer by a deviceType or model. This attribute shall be formatted as either single value-only string or a string format that contains a list of key-value pairs. When this attribute contains a list of key-value pairs, the list of key-value pairs is identified by a “¦”(vertical line) character as the first character in the string. Within the list of key-value pairs, each key and value shall be separated by “:”(colon) and each pairs shall be separated by “ (SPACE(U+0020))”. An example for the key-value string about OMA DWAPI is “¦systemID:0123 serviceID:xyz”. When using reserved characters (e.g., SPACE, “:”, “%”, or “¦”) within a key or value element, the reserved characters are escaped by identifying the ascii value of the character with a percent escape character preceeding the ascii value. For example if the previous examples systemID key’s value included a SPACE character the string is represented as “¦systemID:01%2023 serviceID:xyz”. It is also possible to use a list of URNs.. |
| manufacturer | 1 | WO | The name/identifier of the device manufacturer. This attribute is a specialization of [objectAttribute] attribute. |
| manufacturerDetailsLink | 0..1 | WO | URL to manufacturer's website. This attribute is a specialization of [objectAttribute] attribute. |
| manufacturingDate | 0..1 | WO | Manufacturing date of device. This attribute is a specialization of [objectAttribute] attribute. |
| model | 1 | WO | The name/identifier of the device mode assigned by the manufacturer. This attribute is a specialization of [objectAttribute] attribute. |
| subModel | 0..1 | WO | Device sub-model name. This attribute is a specialization of [objectAttribute] attribute. |
| deviceType | 1 | WO | The type (e.g. cell phone, photo frame, smart meter) or product class (e.g. X-series) of the device. This attribute is a specialization of [objectAttribute] attribute. |
| deviceName | 0..1 | WO | Device name. This attribute is a specialization of [objectAttribute] attribute. |
| fwVersion | 0..1 | WO | The firmware version of the device (see note). |
| swVersion | 0..1 | RW | The software version of the device. This attribute is a specialization of [objectAttribute] attribute. |
| hwVersion | 0..1 | RW | The hardware version of the device. This attribute is a specialization of [objectAttribute] attribute. |
| osVersion | 0..1 | WO | Version of the operating system (defined by manufacturer). This attribute is a specialization of [objectAttribute] attribute. |
| country | 0..1 | WO | Country code of the device. It could be manufacturing country, deployment country or procurement country. This attribute is a specialization of [objectAttribute] attribute. |
| location | 0..1 | RW | Location where the device is installed. It may be configured via the user interface provided by the ‘presentationURL' property or any other means. This attribute is a specialization of [objectAttribute] attribute. |
| systemTime | 0..1 | RW | Reference time for the device. This attribute is a specialization of [objectAttribute] attribute. |
| supportURL | 0..1 | WO | URL that points to product support information of the device. This attribute is a specialization of [objectAttribute] attribute. |
| presentationURL | 0..1 | WO | To quote UPnP: "the control point can retrieve a page from this URL, load the page into a web browser, and depending on the capabilities of the page, allow a user to control the device and/or view device status. The degree to which each of these can be accomplished depends on the specific capabilities of the presentation page and device". This attribute is a specialization of [objectAttribute] attribute. |
| protocol | 0..1(L) | WO | A list of MIME types for all supported communication protocol(s) of the device. Example: application/x-alljoin;version=1.0 application/x-echonet-lite;version=1.0 indicates the device supports both AllJoyn v1.0 and Echonet Lite v1.0. This attribute is a specialization of [objectAttribute] attribute. |
| NOTE: If the device only supports one kind of Software this is identical to swVersion. This attribute is a specialization of [objectAttribute] attribute. |  |  |  |

### -----------------------End of change 1-------------------------------------------