| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | MAS 34 |
| Source:* | Wolfgang Granzow, Qualcomm, wgranzow@qti.qualcomm.com Andreas Kraft, Deutsche Telekom, a.kraft@telekom.de |
| Date:* | 2018-03-10 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | Release 3 |
| CR against: WI* | Active WI-0017 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional) Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0023v3_7_0 |
| Clauses * | 6.2.2, Annex A |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR corrects text about the nodeLink attribute. This attribute has been specified in TS-0001 and TS-0004 as an optional attribute which is common to all <flexContainer> specializations. It is not a specific [customAttribute] as the current text in clause 6.2.2 and A.2 implies.

Thereby some other issues, mostly editorials, which have been noted in clauses 6.2.2 and Annexes A.2, A.3, A.4, are fixed with this CR.

Also, another recently introduced attribute, contentSize, is added to the figure and the table in Annex A.

R01 changes:

- Changes agreed at presentation: updated text on non-applicability of nodeLink attribute for ModuleClass and Action specializations

- Correction of further incorrect cross-references in Annex A

- Adding missing stateTag attribute to resource types in Annex A (note that this is not specific to flexContainers)

- Moving creator attribute up in the list above those which are flexContainer specific

R02 changes:

- Adding missing dynamicAuthorizationConsultationIDs attribute to resource types in Annex A (this attribute is not specific to flexContainers)

### -----------------------Start of change 1-------------------------------------------

### 6.2.2	Resource mapping for Device model

When the AE exposes a controlling interface for a home domain device which is specified as an information model in clause Error: Reference source not found, a specialization of the <flexContainer> resource shall be created as the mapping of the model following conversion rules:

- Rule 1-1: Each Device model defined in clause Error: Reference source not found shall be mapped to a specialization of <flexContainer> resource with associated 'DeviceClass ID' (e.g. "org.onem2m.home.device.tv") on containerDefinition attributes.

- Rule 1-2: Each entry of 'Module' table shall be mapped to child resource(s) which is mapped as a specialised <flexContainer> following the rule in clause Error: Reference source not found 'Resource mapping for ModuleClass'.

- Rule 1-3: Each ‘Property’ of a Device model shall be mapped to a specialized [objectAttribute] of a [deviceInfo] resource that is defined in Annex D.8 of TS-0001[3]. The specialized <flexContainer> resource of the Device model shall contain the nodeLink attribute. As defined in TS-0001 [3] and in TS-0004 [4] nodeLink is an optional attribute common to all <flexContainer> specializations. The nodelink attribute of a <flexContainer> specialization which represents a Device model links to a <node> resource that is hosted on the same hosting CSE as the <flexContainer>. That <node> resource shall contain as child-resource the [deviceInfo] resource of the Device model instance.  The <node> resource contains all the management information as specialized <mgmtObj> resources (e.g. [firmware]) about the Device model instance for different management purposes.

- Rule 1-4: XSD file for each Device model shall be named according to the following naming convention:
'HD-<name of Device model>-v<version of TS>.xsd'
For example, XSD file for ‘deviceAirConditioner' as defined in TS-0023v3.7.0 is named as ‘HD-deviceAirConditioner-v3_7_0.xsd'.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

  - Example for Device model ‘deviceAirConditioner'

The present clause explains the creation process for the device typed 'deviceAirConditioner' (see clause Error: Reference source not found for device model definition of ‘deviceAirConditioner').

Using the definition, 'deviceAirConditioner' model is mapped to [deviceAirConditioner] resource which is a specialization of <flexContainer> resource (See ).

Editor’s Note: Figure A.2-1 above needs to be updated but the format is not editable. The creator attribute should be removed (since only flexContainer-specific attributes are shown), angle brackets around nodeLink needs to be removed, new common contentSize attribute needs to be added to [deviceAirConditioner], rename contDefinition to containerDefinition.

Figure A.2 -1: Structure of [deviceAirConditioner] resource

The AE creates the [deviceAirConditioner] specialization of <flexContainer> resource for the Device model [deviceAirConditioner] resource.

The [deviceAirConditioner] resource contains the child resource specified inTable A.2-1 Table A.2-1.

Table A.2-1: Child resources of [deviceAirConditioner] resource

| Child Resources of [deviceAirConditioner] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <flexContainer> as defined in the specialization [binarySwitch] | 0..1 | This resource is used to map 'binarySwith' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [runState] | 0..1 | This resource is used to map 'runState' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airConJobMode] | 0..1 | This resource is used to map ‘airConJobMode’ ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airConOperationMode] | 0..1 | This resource is used to map ‘airConOperationMode’ ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airCleanOperationMode] | 0..1 | This resource is used to map ‘airCleanOperationMode’ ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [temperature] | 0..1 | This resource is used to map ‘temperature’ ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [timer] | 0..1 | This resource is used to map 'timer' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [sleepTimer] | 0..1 | This resource is used to map 'sleepTimer' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [turbo] | 0..1 | This resource is used to map 'turbo' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airFlow] | 0..1 | This resource is used to map 'airFlow' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [powerSave] | 0..1 | This resource is used to map 'powerSave' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airQualitySensor] | 0..1 | This resource is used to map 'airQualitySensor' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [filterInfo] | 0..1 | This resource is used to map 'filterInfo' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <subscription> | 0..n | See clause 9.6.8 in oneM2M TS-0001 [Error: Reference source not found] |

The [deviceAirConditioner] resource contains the attributes specified in Table A.2-2Table A.2-2.

Table A.2-2-: Attributes of [deviceAirConditioner] resource

| Attributes of [deviceAirConditioner] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| expirationTime | 1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| labels | 0..1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| stateTag | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| creator | 0..1 (L) | RW | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found] |
| containerDefinition | 1 | WO | The value is "org.onem2m.home.device.airconditioner" |
|  | 0..1 (L) |  | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found]. |
| ontologyRef | 0..1 (L) | RW | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found]. |
| contentSize | 1 | RO | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found]. |
| nodeLink | 0..1 | RW | nodeLink attribute links to a <node> resource that is hosted on the same hosting CSE as the <flexContainer>. See clause and Error: Reference source not found for more details. |

  - Example of ModuleClass 'binarySwitch'

The [binarySwitch] resource is used to share information regarding the modeled binary switch module as a ModuleClass. The [binarySwitch] resource is a specialization of the <flexContainer> resource.

Figure A.3-1-: Structure of [binarySwitch] resource

Editor’s Note: Figure A.3-1 above needs to be updated but the format is not editable. Add new common contentSize and nodeLink attributes, rename contDefinition to containerDefinition.

The [binarySwitch] resource contains the child resource specified in Table A.3-1Table A.3-1.

Table A.3-1-: Child resources of [binarySwitch] resource

| Child Resources of [binarySwitch] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <flexContainer> as defined in the specialization [toggle] | 0..1 | This resource is used to map 'toggle' Action defined in Clause Error: Reference source not found. |
| [variable] | <subscription> | 0..n | See clause 9.6.8 in oneM2M TS-0001 [Error: Reference source not found] |

The [binarySwitch] resource contains the attributes specified inTable A.3-2 Table A.3-2.

Table A.3-2-: Attributes of [binarySwitch] resource

| Attributes of [binarySwitch] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| expirationTime | 1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| labels | 0..1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| stateTag | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| creator | 0..1 (L) | RW | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found] |
| containerDefinition | 1 | WO | The value is "org.onem2m.home.moduleclass.binaryswitch" |
|  | 0..1 (L) |  | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found] |
| ontologyRef | 0..1 (L) | RW | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found] |
| contentSize | 1 | RO | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found]. |
| nodeLink | 0..1 | RW | Not applicable to a ModuleClass specialization. This attribute is not present in an instantiation of this resource. |
| powerState | 1 | RW | See clause Error: Reference source not found |

  - Example of Action 'toggle'

The [toggle] resource is used to share information regarding the modelled toggle as an Action. The [toggle] resource is a specialization of the <flexContainer> resource.

Figure A.4-1-: Structure of [toggle] resource

Editor’s Note: Figure A.4-1 above needs to be updated but the format is not editable. Add new common contentSize and nodeLink attributes, rename contDefinition to containerDefinition.

The [toggle] resource contains the child resource specified in Table A.4-1Table A.4-1.

Table A.4-1-: Child resources of [toggle] resource

| Child Resources of [toggle] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 in oneM2M TS-0001 [Error: Reference source not found] |

The [toggle] resource contains the attributes specified in A.4-2Table A.4-2.

Table A.4-2-: Attributes of [toggle] resource

| Attributes of [toggle] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| expirationTime | 1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| labels | 0..1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| stateTag | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| creator | 0..1 (L) | RW | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found] |
| containerDefinition | 1 | WO | The value is "org.onem2m.home.moduleclass.binaryswitch.toggle" |
|  |  |  | Error: Reference source not found |
| ontologyRef | 0..1 (L) | RW | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found] |
| contentSize | 1 | RO | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found]. |
| nodeLink | 0..1 | RW | Not applicable to an Action specialization. This attribute is not present in an instantiation of this resource. |

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?