| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | MAS#34 |
| Source:* | Wolfgang Granzow, Qualcomm, wgranzow@qti.qualcomm.com |
| Date:* | 2018-03-15 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | Release 3 |
| CR against: WI* | Active WI MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional) Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0006v3_6_0 |
| Clauses * | 7.15, 7.16, 7.17 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR provides corrections to the Field Device Configuration (FDC) specific resource types defined in TS-0022. This aligns TS-0006 with the latest version of TS-0022 and TS-0005.

Changes to TS-0022 with no impact on TS-0006 (it needs to be confirmed that these changes do not impact the html and xml files of the TR069 model):

[registration]:

- Multiplicity of originatorID attribute   1  0..1

[authenticationProfile]:

- Datatype of TLSCiphersuites is a list

- symmKeyValue data type changed from xs:base64Binary  xs:hexBinary

Revision R01:  improved wording of the proposed text in clause 7.17.2 (as suggested by Tim Carey)

### -----------------------Start of change 1-------------------------------------------

## 7.15	Resource [registration]

The Resource [registration] represents the configuration information needed to register a AE or CSE with a Registrar CSE.

The Resource [registration] is a multi-instance object where the key of the object is the originatorID (i.e., AE-ID, CSE-ID). The following rules are used to determine the object instance based on the originatorID:

- When the originatorID resource is for a CSE-ID, the TR-069 object instance Device.X_oneM2M_org_CSE.{i} shall be used for the specified CSE-ID.

- When the originatorID resource is for an AE-ID, the TR-069 object instance Device.X_oneM2M_org_AE.{i} shall be used for the specified AE-ID.

The information shall be created using the Add Object RPC of TR-069 [Error: Reference source not found].

The information shall be deleted using the Delete Object RPC of TR-069 [Error: Reference source not found].

The information shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The capabilities shall be enabled and disabled using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.15-1: Resource [registration] for CSE

| Attribute Name of [registration] | BBF TR-181 [6] Parameter (X_oneM2M_org_CSE) |
| --- | --- |
| originatorID | ID – See description of the type of object to instantiate. |
| poA | PointOfAccess |
| externalID | ExternalID |
| triggerRecipientID | TriggerRecipientID |
| mgmtLink [authenticationProfile] | AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table) |

Table 7.15-2: Resource [registration] for AE

| Attribute Name of [registration] | BBF TR-181 [6] Parameter (X_oneM2M_org_AE) |  |  |
| --- | --- | --- | --- |
| originatorID | ID – See description of the type of object to instantiate. |  |  |
| poA | PointOfAccess |  |  |
| appID | ApplicationID |  |  |
| mgmtLink [authenticationProfile] |  | AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table) |  |

## 7.16	Resource [dataCollection]

The Resource [dataCollection] represents data collection (measurement) and transmittal (reporting) properties for an AE.

The information shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The capabilities shall be enabled and disabled using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.16-1: Resource [dataCollection]

| Attribute Name of [dataCollection] | BBF TR-181 [6] Parameter |
| --- | --- |
| containerPath | ContainerPath |
| reportingSchedule | ReportingSchedule |
| measurementSchedule | CollectionSchedule |
| mgmtLink [authenticationProfile] | AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table) |

## 7.17	Security Solutions

### 7.17.1	Introduction

This clauses in the section of the this present document contains information on how to map the security specific management resources from TS-0022 [8] to managed objects and parameters as defined in the TR-181 [Error: Reference source not found] data model or the Remote Procedure Calls (RPCs) in TR-069 [Error: Reference source not found].

### 7.17.2	Resource [authenticationProfile]

The Resource [authenticationProfile] represents configuration information regarding establishing mutually-authenticated secure communications. The security principal using this configuration information can be a CSE or AE or the Managed ADN/ASN/MN acting as security principal on behalf of AEs on the Node, see clause 7.1.4 of TS-0022 [8].

The Resource [authenticationProfile] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile.{i} object.

The AuthenticationRule instance shall be created using the Add Object RPC of TR-069 [Error: Reference source not found].

The AuthenticationRule instance shall be deleted using the Delete Object RPC of TR-069 [Error: Reference source not found].

The information of a AuthenticationProfile instance shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The information of a AuthenticationProfile instance shall be updated using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.17.2-1: Resource [authenticationProfile]

| Attribute Name of [authenticationProfile] | Parameters of Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile.{i} |
| --- | --- |
| SUID | SUID |
| TLSCiphersuites | TLSCiphersuites |
| symmKeyID | SymmetricKeyID |
| symmKeyValue | SymmetricKeyValue |
| MAFKeyRegLabels | MAFKeyRegLabels |
| MAFKeyRegDuration | MAFKeyRegDuration |
| mycertFingerprint | MyCert (reference) |
| rawPubKeyID | RawPubKeyID |
| mgmtLink [trustAnchorCred] or [MAFClientRegCfg] | TrustAnchorCredentials or MAFClientRegistration (list of references) |

The parameter MyCert is a TR-069 reference parameter that references a row in the Device.Security.Certificate table where the value of the mycertFingerprint attribute matches the value of a Device.Security.Certficate.{i}.X_oneM2M_org_Fingerprint parameter. The X_oneM2M_org_Fingerprint parameter shall be a unique key for the Device.Security.Certificate table.

Depending on the setting of the SUID attribute (see clause 7.1.4 of oneM2M TS-0022 [8]), the TrustAnchorCredentials or MAFClientRegistration parameter is used. The TrustAnchorCredentials is a list of references to a row in the Device.X_oneM2M_org_SecuritySolution.TrustAnchorCredential table. The MAFClientRegistration parameter is a reference to a row in the Device.X_oneM2M_org_SecuritySolution.MAFClientRegistration table.

### 7.17.3	Resource [trustAnchorCred]

The Resource [trustAnchorCred] represents configuration information regarding certificates provided by certificate authorities used be managed entities to authenticate peer endpoints, see clause 7.1.6 of TS-0022 [8].

The Resource [trustAnchorCred] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_SecuritySolution.TrustAnchorCredential.{i} object.

The TrustAnchorCredential instance shall be created using the Add Object RPC of TR-069 [Error: Reference source not found].

The TrustAnchorCredential instance shall be deleted using the Delete Object RPC of TR-069 [Error: Reference source not found].

The information of a TrustAnchorCredential instance shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The information of a TrustAnchorCredential instance shall be updated using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.17.3-1: Resource [trustAnchorCred]

| Attribute Name of [trustAnchorCred] | Parameters of Device.X_oneM2M_org_SecuritySolution.TrustAnchorCredential.{i} |
| --- | --- |
| certFingerprint | Fingerprint |
| URI | RemoteTrustStore |

### 7.17.4	Resource [myCertFileCred]

The Resource [myCertFileCred] represents configuration information regarding certificates presented by the managed entity to remote entities for the establishment of secure communications, see clause 7.1.5 of TS-0022 [8].

The Resource [myCertFileCred] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.Security.Certificate.{i} object.

The Certificate instance shall be created either using the Download RPC of TR-069 [4] or via an out-of-band mechanism.

The Certificate instance shall be deleted using using the Download RPC of TR-069 [4] or via an out-of-band mechanism.

The information of a Certificate instance shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The information of a Certificate instance shall be updated using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.17.4-1: Resource [myCertFileCred]

| Attribute Name of [myCertFileCred] | Parameters of Device.Security.Credential.{i} |
| --- | --- |
| SUIDs | X_oneM2M_org_SUIDs |
| myCertFileFormat | X_oneM2M_org_Format |
| myCertFileContent | The certificate is downloaded as part of the Download RPC of TR-069 |

The parameter AuthenticationProfile is a TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table where the value of the mycertFingerprint attribute matches the value of a Device.Security.Certficate.{i}.X_oneM2M_org_Fingerprint parameter. The X_oneM2M_org_Fingerprint parameter shall be a unique key for the Device.Security.Certificate table

### 7.17.5	Resource [MAFClientRegCfg]

The Resource [MAFClientRegCfg] represents configuration information that permits a MAF client to register with a MAF, see clause 7.1.7 of TS-0022 [8].

The Resource [MAFClientRegCfg] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_SecuritySolution.MAFClientRegistration.{i} object.

The MAFClientRegistration instance shall be created using the Add Object RPC of TR-069 [Error: Reference source not found].

The MAFClientRegistration instance shall be deleted using the Delete Object RPC of TR-069 [Error: Reference source not found].

The information of a MAFClientRegistration instance shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The information of a MAFClientRegistration instance shall be updated using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.17.5-1: Resource [MAFClientRegCfg]

| Attribute Name of [MAFClientRegCfg] | Parameters of Device.X_oneM2M_org_SecuritySolution.MAFClientRegistration.{i} |
| --- | --- |
| mgmtLink [authenticationProfile\ | AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table) |
| Fqdn | FQDN |
| adminFQDN | AdminFQDN |
| httpPort | HTTPPort |
| coapPort | CoAPPort |
| websocketPort | WebsocketPort |

### 7.17.6	Resource [MEFClientRegCfg]

The Resource [MEFClientRegCfg] represents configuration information that permits a MEF client to register with a MEF, see clause 7.1.8 of TS-0022 [8].

The Resource [MEFClientRegCfg] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_SecuritySolution.MEFClientRegistration.{i} object.

The MEFClientRegistration instance shall be created using the Add Object RPC of TR-069 [Error: Reference source not found].

The MEFClientRegistration instance shall be deleted using the Delete Object RPC of TR-069 [Error: Reference source not found].

The information of a MEFClientRegistration instance shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The information of a MEFClientRegistration instance shall be updated using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.17.6-1: Resource [MEFClientRegCfg]

| Attribute Name of [MEFClientRegCfg] | Parameters of Device.X_oneM2M_org_SecuritySolution.MEFClientRegistration.{i} |
| --- | --- |
| mgmtLink [authenticationProfile\ | AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table) |
| Fqdn | FQDN |
| adminFQDN | AdminFQDN |
| httpPort | HTTPPort |
| coapPort | CoAPPort |
| websocketPort | WebsocketPort |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?