| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | MAS #36 |
| Source:* | Hongna Chang, HUAWEI, changhongna@huawei.com Yongjing Zhang, HUAWEI, zhangyongjing@huawei.com |
| Date:* | 2018-7-3 |
| Reason for Change/s:* | Adding the information model of water gas meter devices for the smart city domain. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0084 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0023 V4.0.0 |
| Clauses * | 5.3, 5.5, 5.6 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | NA |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes to add some module classes and  a device model in order to manage the gasmeter device and unify data modle of gasmeter for smart city.

### -----------------------Start of change 1-------------------------------------------

### 5.5.2.1	deviceSmartGasMeter

A smart gas meter is a metering device that is used to measure consumption data for gas.

Table 5.5.xx-1: Modules of deviceSmartGasMeter Device model

| Module Instance Name | Module Class Name | Optional | Description |
| --- | --- | --- | --- |
| battery | battery | true | See clause Error: Reference source not found1.10. |
| connectivity | connectivity | true | See clause Error: Reference source not found1.xx. |
| periodicalReportConfig | periodicalReportConfig | true | See clause Error: Reference source not found1.xx. |
| gasMeterReportInfo | gasMeterReportInfo | false | See clause Error: Reference source not found2.xx. |
| gasMeterControl | gasMeterControl | false | See clause Error: Reference source not found2.xx. |
| gasMeterAlarm | gasMeterAlarm | false | See clause Error: Reference source not found2.xx. |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 5.3.1.xx connectivity

This ModuleClass provides capabilities to monitor network connectivity.

Table 5.3.xx-2: DataPoints of connectivity ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| rsrp | xs:integer | R | false | dBm | Reference signal received power (RSRP) is a measurement of the received power level in an LTE cell network. |
| rsrq | xs:integer | R | true | dB | Reference signal received quality (RSRQ) indicates the quality of the received reference signal. RSRQ is defined as the ratio N×RSRP/(E-UTRA carrier RSSI), where N is the number of RB’s of the E-UTRA carrier RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks. |
| cellID | xs:integer | R | true |  | Serving Cell ID in case Network Bearer Resource is a Cellular Network. |
| rssi | xs:integer | R | true | dBm | In telecommunications, received signal strength indicator (RSSI) is a measurement of the power present in a received radio signal. |
| signalECL | xs:integer | R | true |  | Based on measurements of the reference signal’s received power, the UE will select an entry coverage enhancement level (ECL) to camp into the cell. The coverage level will determine the Narrowband Physical Random Access Channel (NPRACH) resources used by the device and will inform the eNB of the device receiver sensitivity conditions. |
| sinr | xs:integer | R | true | dB | signal to interference plus noise ratio (SINR) is commonly used in wireless communication as a way to measure the quality of wireless connections. |
| pci | xs:string | R | true |  | Physical Cell ID is an indentification of a cell at physical layer. |
| dailyActivityTime | xs:integer | R | true | second | Daily communication time |
| commFreqValue | xs:integer | R | true | MHz | Communication frequency value (commFreqValue) is the transmission frequency of the wireless signal. |

### 5.3.1.xx periodicalReportConfig

This ModuleClass provides capabilities to set parameters of periodic report.

Table 5.3.xx-3: DataPoints of periodicalReportConfig ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| backoffTime | xs:integer | RW | false | second | The number of seconds to wait before connecting to network. |
| period | xs:integer | RW | true | second | Reporting period. |
| retryTimes | xs:integer | RW | true |  | The number of times the message shal be re-sent in the case of report failure. |
| retryInterval | xs:integer | RW | true | second | Time interval between each message transmission retry. |

### 5.3.2.xx gasMeterReportInfo

This ModuleClass provides information of measurements of a gas meter.

Table 5.3.2.xx-4: DataPoints of gasMeterReportInfo ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| totalUseValue | xs:integer | R | false | Litre | The total consumption of gas. |
| valveStatus | xs: valveStatus | R | true |  | The status of gasmeter valve. |

### 5.3.2.xx gasMeterControl

This ModuleClass provides capabilities to control the gas meter valve and recharge.

Table 5.3.2.xx-5: DataPoints of gasMeterControl ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| valveControl | xs:boolean | RW | false |  | Used by the server to remotely set the gas meter valve status. |
| billNo | xs:integer | RW | true |  | The bill number of remote recharge. |
| rechargeValue | xs:integer | RW | true | local currency | Amount of recharge money. |
| surplusValue | xs:integer | R | true | local currency | Amount of surplus money. |
| rechargeGas | xs:integer | RW | true | Cubic meter | Amount of recharge gas. |
| surplusGas | xs:integer | R | true | Cubic meter | Amount of surplus gas. |

### 5.3.2.xx gasMeterAlarm

This ModuleClass provides capabilities to set service parameters of a gas meter.

Table 5.3.2.xx-6: DataPoints of gasMeterAlarm ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| lowGasAlarm | xs:boolean | R | true |  | The alarm of insufficient amount of gas. |
| lowValueAlarm | xs:boolean p | R | true |  | The alarm of insufficient amount of money. |
| leakageAlarm | xs:boolean | R | true |  | The alarm of gas leakage. |
| lockedDownAlarm | xs:boolean | R | true |  | The alarm of the gasmeter being locked. When the gas meter is in use, the lower wheel does not work, called the "dead meter". |
| largeFlowAlarm | xs:boolean | R | true |  | The alarm of large flow occurring. The instantaneous flow passed exceeds the setting maximum flow of the gasmeter. |
| magneticDisturb | xs:boolean | R | true |  | The alarm of magnetic interference occurring. |
| singleCountAlarm | xs:boolean | R | true |  | The alarm of single reed pipe counting. The normal condition is that the double reed pipes are worked. If only single reed pipe is worked, it means that the metering has a problem, so it is called single reed pipe counting. |
| lossOfPowerAlarm | xs:boolean | R | true |  | The alarm of the battery is out of pressure. |
| communicationAlarm | xs:boolean | R | true |  | The alarm of loss communication. |

-----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?