| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | MAS #36 |
| Source:* | Hongna Chang, HUAWEI, changhongna@huawei.com Yongjing Zhang, HUAWEI, zhangyongjing@huawei.com |
| Date:* | 2018-7-3 |
| Reason for Change/s:* | Adding the information model of water meter devices for the smart city domain. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0084 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0023 V4.0.0 |
| Clauses * | 5.3, 5.5 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | NA |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes to add some module classes and  a device model of the water meter device for smart city.

### -----------------------Start of change 1-------------------------------------------

### 5.5.2.1	deviceSmartWaterMeter

A smart water meter is a metering device that is used to measure consumption data for water.

Table 5.5.xx-1: Modules of deviceSmartWaterMeter Device model

| Module Instance Name | Module Class Name | Optional | Description |
| --- | --- | --- | --- |
| battery | battery | true | See clause Error: Reference source not found1.10. |
| connectivity | connectivity | true | See clause Error: Reference source not found1.xx. |
| periodicalReportConfig | periodicalReportConfig | true | See clause Error: Reference source not found1.xx. |
| waterMeterSetting | waterMeterSetting | false | See clause Error: Reference source not found2.xx. |
| waterMeterReportInfo | waterMeterReportInfo | false | See clause Error: Reference source not found2.xx. |
| waterMeterControl | binarySwitch | true | See clause Error: Reference source not found2.xx. |
| waterMeterAlarm | waterMeterAlarm | false | See clause Error: Reference source not found2.xx. |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 5.3.2.xx waterMeterReportInfo

This ModuleClass provides information of measurements of the watermeter.

Table 5.3.2.xx-2: DataPoints of waterMeterReportInfo ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| dailyUseWaterTime | xs:integer | R | false | s | The total time of water consumption daily. |
| cumulativeFlow | xs:float | R | false | m3 | The cumulative consumption of water daily. |
| positiveCumulativeFlow | xs:float | R | true | m3 | The positive cumulative consumption of water daily. |
| negativeCumulativeFlow | xs:float | R | true | m3 | The negative cumulative consumption of water daily. |
| peakFlowRate | xs:float | R | true | m3/h | Daily maximum instantaneous water flow rate. |
| peakFlowRateTime | m2m:timestamp | R | true |  | The timestamp of the daily highest instantaneous water flow rate. |
| lowestFlowRate | xs:float | R | true | m3/h | Daily lowest instantaneous water flow rate. |
| lowestFlowRateTime | m2m:timestamp | R | true |  | The timestamp of daily lowest instantaneous water flow rate. |
| peakReverseFlowRate | xs:float | R | true | m3/h | Daily reverse maximum instantaneous water flow rate. |
| peakReverseFlowRateTime | m2m:timestamp | R | true |  | The timestamp of daily reverse highest instantaneous water flow rate. |
| lowestReverseFlowRate | xs:float | R | true | m3/h | Daily reverse lowest instantaneous water flow rate. |
| lowestReverseFlowRateTime | m2m:timestamp | R | true |  | The timestamp of daily reverse lowest instantaneous water flow rate. |
| intervalFlow | list of xs:float | R | true | m3 | Water consumption records measured at the interval of “flowInterval” describerd in 5.3.2.xx per day. |
| reverseIntervalFlow | list of xs:float | R | true | m3 | Water reverse consumption records measured at the interval of “reverseFlowIntervral” describerd in 5.3.2.xx per day. |
| waterIntervalTemperature | list of xs:float | R | true | ℃ | Water temperature records measured at the interval of “waterTemperatureInterval” describerd in 5.3.2.xx per day. |
| waterIntervalPressure | list of xs:float | R | true | bar | Water pressure records measured at the interval of “waterPressInteveral” describerd in 5.3.2.xx per day. |

### 5.3.2.xx waterMeterSetting

This ModuleClass provides capabilities to set service parameters for data sampling and reporting.

Table 5.3.2.xx-3: DataPoints of waterMeterSetting ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| flowInterval | xs:integer | RW | false | s | The measurement interval of water comsumption. |
| reverseFlowIntervral | xs:integer | RW | false | s | The measurement interval of reverse water comsumption. |
| waterTemperatureInterval | xs:integer | RW | true | s | The measurement interval of water temperature. |
| waterPressInteveral | xs:integer | RW | true | s | The measurement interval of reverse water pressure. |
| intensiveSampleInterval | xs:integer | RW | true | s | The time interval of intensive data sampling. |
| intensiveReportInterval | xs:integer | RW | true | s | The time interval of intensive data report. |
| intensiveReportStartTime | m2m:timestamp | RW | true |  | The start time of data intensive report. |

### 5.3.2.xx waterMeterAlarm

This ModuleClass provides capabilities to provide alarm information of watermeter, such as the alarm of low water flow. Once an alarm is fired, a notification should be sent out from the device and no historical alarm is stored locally, therefore no need to associate timestamp with the alarms.

Table 5.3.2.xx-4: DataPoints of waterMeterAlarm ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| highFlowAlarmThreshold | xs:float | RW | false | m3 | The threshold of continuous high flow alarm. |
| highFlowDuration | xs:integer | RW | true | s | The duration of high water flow. |
| highFlowAlarm | xs:boolean | R | false |  | Continuous high water flow alarm |
| lowFlowAlarmThreshold | xs:float | RW | true | m3 | The threshold of continuous low flow alarm. |
| lowFlowDuration | xs:integer | RW | true | s | The duration of low water flow. |
| lowFlowAlarm | xs:boolean | R | true |  | Continuous low water flow alarm |
| reverseFlowAlarmThreshold | xs:float | RW | true | m3 | The threshold of continuous reverse flow alarm. |
| reverseFlowDuration | xs:integer | RW | true | s | The duration of reverse water flow. |
| reverseFlowAlarm | xs:boolean | R | false |  | Reverse flow alarm |
| highPressureAlarmThreshold | xs:float | RW | false | bar | The threshold of high water pressure alarm. |
| highPressureAlarm | xs:boolean | R | false |  | High water pressure alarm |
| lowPressureAlarmThreshold | xs:float | RW | false | bar | The threshold of low water pressure alarm. |
| lowPressureAlarm | xs:boolean | R | false |  | Low water pressure alarm |
| highTemperatureAlarmThreshold | xs:float | RW | false | ℃ | The threshold of maximum water temperature alarm. |
| highTemperatureAlarm | xs:boolean | R | false |  | High water temperature alarm |
| lowTemperatureAlarmThreshold | xs:float | RW | false | ℃ | The threshold of minimum water temperature alarm. |
| lowTemperatureAlarm | xs:boolean | R | false |  | Low water temperature alarm |
| highTemperatureInnerlAlarmThreshold | xs:float | RW | true | ℃ | The threshold of high temperature alarm inside water meter. |
| innerhHighInternalTemperatureAlarm | xs:boolean | R | true |  | Internal high temperature alarm |
| innerErrorAlarm | xs:boolean | R | true |  | Internal error alarm |
| innerTempratureSensorFault | xs:boolean | R | true |  | Internal temperature sensor failure |
| tamperAlarm | xs:boolean | R | true |  | Data was tampered alarm |
| waterTempratureSensorFault | xs:boolean | R | false |  | Water temperature sensor failure |
| pressureSensorFault | xs:boolean | R | true |  | Pressure sensor failure |
| communicationAlarm | xs:boolean | R | true |  | Communication abnormality alarm |
| magneticInterference | xs:boolean | R | true |  | Magnetic interference warning |
| storageFault | xs:boolean | R | true |  | Storage failure alarm |
| urgencyButtonPush | xs:boolean | R | true |  | Indicate the event of the urgency button being pushed. Pushing the urgency button may give a user a temporary right to still use the water meter (e.g. for 3 days) after an unpaid bill. |
| buttonFault | xs:boolean | R | true |  | Button error flag |
| demolitionAlarm | xs:boolean | R | true |  | Demolition sign |
| impulseFault | xs:boolean | R | true |  | Pulse anomaly flag |
| vibrationSensorFault | xs:boolean | R | true |  | Vibration sensor failure |

-----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?