| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | MAS 36 |
| Source:* | Maciej Goluch, Orange, Maciej.Goluch@orange.com Paweł Strzemecki, Orange, Pawel.Strzemecki2@orange.com |
| Date:* | 2018-07-17 |
| Reason for Change/s:* | See the introduction below |
| CR against: Release* | TS-0023 |
| CR against: WI* | Active WI-0081 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0023 v. 3.7.1 |
| Clauses * | 5.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR adapts the TS-0023 document to reccommedations included in MAS-2018-0074R01 	Improvements for data point tables CR in terms of the structure of Data Points tables.

R01: Unit column values changed to shortcuts in the whole document

### -----------------------Start of change 1-------------------------------------------

## ModuleClasses

### Common Domain

#### 3Dprinter

This ModuleClass provides capabilities for a 3D printer.

Table 5.3.1-1: Actions of 3Dprinter

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | start3Dprint | none | true | Start 3D printing. |
| none | stop3Dprint | none | true | Stop 3D printing. |

Table 5.3.1-2: DataPoints of 3Dprinter ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| printType | hd:enum3DprinterTechnology | R | false |  | The type of printing technology (see clause Error: Reference source not found). |
| printSizeX | xs:float | R | false | mm | This data pointrepresents the maximum size of a printing object in the direction of X-axis. |
| printSizeY | xs:float | R | false | mm | This data poin represents the maximum size of printing object in the direction of Y-axis. |
| printSizeZ | xs:float | R | false | mm | This data point represents the maximum size of printing object in the direction of Z-axis. |
| network | xs:boolean | R | false |  | This value indicates the Wide Area Network (WAN) connectivity of the 3D printer, such as Internet or GSM. ”False” indicates that the printer does not have network connectivity to a WAN. “True” indicates that the printer has WAN network connectivity. |
| memorySize | xs:float | R | false | MB | This value represents the total memory size of the printer. The unit of measure is. |

#### acousticSensor

This ModuleClass provides capabilities for an acoustic sensor.

Table 5.3.2-3: DataPoints of acousticSensor ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| loudness | xs:float | R | false | dB | . |
| acousticStatus | xs:integer | R | true |  | The acousticStatus is expressed in percent, whereas a value of 0 means “no sound” and a value of 100 means “most noisy”. |

#### airConJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of an air conditioner.

Table 5.3.3-4: DataPoints of airConJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd:enumAirConJobMode | RW | false |  | Currently active job mode (see clause Error: Reference source not found). |
| currentJobModeName | xs:string | R | true |  | Name of the current job mode as a string. This can be used when currentJobMode is vendor-specific. |
| jobModes | list of hd:enumAirConJobMode | R | false |  | List of possible job states the device supports (see clause Error: Reference source not found). |

#### airFlow

This ModuleClass provides capabilities for controlling the air flow of a device.

Table 5.3.4-5: DataPoints of airFlow ModuleClass

| Name | Type | Readable | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| speed | xs:integer | RW | false |  | The current speed level in the range of the [minSpeed, maxSpeed] data poins. |
| minSpeed | xs:integer | R | true |  | The minimum value for the speed level. If not present, the default is 0. |
| maxSpeed | xs:integer | R | true |  | The maximum value for the speed level. If not present, the default is 100. |
| verticalDirection | hd:enumVerticalDirection | RW | true |  | The vertical direction of the air flow (see clause Error: Reference source not found). |
| supportedVerticalDirection | List of hd:enumVerticalDirection | R | true |  | List of supported vertical directions. |
| horizontalDirection | hd:enumHorizontalDirection | RW | true |  | The horizontal direction of the air flow (see clause Error: Reference source not found). |
| supportedHorizontalDirection | List of hd:enumHorizontalDirection | R | true |  | List of supported horizontal directions. |
| automode | xs:Boolean | RW | true |  | Status of the automode feature. “True” indicates that the speed is set by the device, “False” indicates that the device is not controlling the speed. |

#### airPurifierJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of an airPurifier.

Table 5.3.5-6: DataPoints of airPurifierJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd:enumAirPurifierJobMode | RW | false |  | Currently active job mode (see clause Error: Reference source not found). |
| currentJobModeName | xs:string | R | true |  | Name of the current job mode as a string. This can be used when currentJobMode is vendor-specific. |
| jobModes | list of hd:enumAirPurifierJobMode | R | false |  | List of possible job states the device supports (see clause Error: Reference source not found). |

#### airQualitySensor

This ModuleClass provides capabilities for a monitoring sensor that measures the air quality.

Table 5.3.6-7: DataPoints of airQualitySense ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| sensorPM1 | xs:integer | R | true |  | The concentration of particle matter under 1㎛.. The minimum value is 0, and the maximum value is 1000. |
| sensorPM2 | xs:integer | R | true |  | The concentration of particle matter under 2.5㎛. The minimum value is 0, and the maximum value is 1000. |
| sensorPM10 | xs:integer | R | true |  | The concentration of particle matter under 10㎛. The minimum value is 0, and the maximum value is 1000. |
| sensorOdor | xs:integer | R | true |  | The concentration of odor that reflects air pollution. The minimum value is 0, and the maximum valu is 1000. |
| sensorHumidity | xs:integer | R | true |  | The measured humidity. The minimum value is 0, and the maximum value is 100. |
| monitoringEnabled | xs:boolean | RW | true |  | The current status of monitoring. "True" indicates enabled, and "False" indicates not enabled. |
| co2 | xs:integer | R | true | ppm | This value indicates the CO2 level. |
| co | xs:integer | R | true | ppm | This value indicates the CO level. |
| ch2o | xs:integer | R | true | ppm | This value indicates the CH2O level. |
| voc | xs:integer | R | true | ppm | This value indicates the VOC (Volatile Organic Compounds). |

#### alarmSpeaker

This ModuleClass provides the capabilites to initiate and monitor an alarm.

Table 5.3.7-8: DataPoints of alarmSpeaker ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| tone | hd:enumTone | RW | true |  | Representing the tones of the alarm (see clause Error: Reference source not found). |
| light | hd:enumAlertColourCode | R | true |  | Representing the lighting mode of the alarm (see clause Error: Reference source not found). |
| alarmStatus | xs:boolean | R | false |  | "True" indicates the alarm start while "False" indicates the alarm stop. |

#### audioVolume

This ModuleClass provides capabilities to control and monitor volume

Table 5.3.8-9: Actions of audioVolume

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | upVolume | none | true | Increase the volume by the amount of the stepValue up to the maxValue. |
| none | downVolume | none | true | Decrease the volume by the amount of the stepValue down to 0. |

Table 5.3.8-10: DataPoints of audioVolume

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| volumePercentage | xs:integer | RW | false |  | The rounded percentage of the current volume in the range of [0, maxValue]. 0 percentage shall mean no sound produced. |
| stepValue | xs:integer | R | true |  | Step value used by the “UpVolume” and “DownVolume” actions. |
| maxValue | xs:integer | R | true |  | Maximum value allowed for Volume. maxValue is 100 by default if “maxValue” is not provided. |
| muteEnabled | xs:boolean | RW | false |  | The current status of the mute enablement. "True" indicates enabled (that is, no sound), and "False" indicates not enabled (that is, sound is played). |

#### autoDocumentFeeder

This ModuleClasses provides capabilities to monitor the state of autoDocumentFeeder (ADF). ADF is a feature which takes several pages and feeds the paper one page at a time into a scanner or printer, allowing the user to scan, print or fax, multiple-page documents without having to manually replace each page.

Table 5.3.9-11: DataPoints of autoDocumentFeeder ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentAdfState | hd:enumAdfState | R | false |  | Current state of the ADF. |
| adfStates | list of hd:enumAdfState | R | false |  | List of possible ADF states that are supported by the device (see clause Error: Reference source not found). |

#### battery

This ModuleClass provides capabilities to indicate the detection of low battery and gives an alarm if the triggering criterion is met. The level data point in the ModuleClass represents the current battery charge level.

Table 5.3.10-12: DataPoints of battery ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| level | xs:integer | R | false |  | The rounded percentage of the current charging level of a battery in the range of [0, 100]. 0 percentage shall mean that no charge remains. |
| capacity | xs:integer | R | true |  | The total capacity of battery in mAh. |
| charging | xs:boolean | R | true |  | The status of charging. "True" indicates enabled, and "False" indicates not enabled. |
| discharging | xs:boolean | R | true |  | The status of discharging. "True" indicates charging, and "False" indicates not charging. |
| lowBattery | xs:boolean | R | true |  | To indicate that the battery is on a low charge level. |
| batteryThreshold | xs:integer | RW | true |  | When a battery’s “level” is less than “batteryThreshold” then “lowBattery” is set to “True”. This datapoint can be used to raise an alarm, depending on the implementation. |
| electricEnergy | xs:integer | R | true | A | Rated electric energy |
| voltage | xs:integer | R | true | V | Rated voltage. |
| material | xs:string | R | true |  | The material of the cell (for example lithium ion, nickel and lead. |

#### binaryObject

This ModuleClass describes the handling of a binary object (blob).

Table 5.3.11-13: DataPoints of binaryObject ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| object | xs:string | RW | false |  | This data point contains the base64 encoded binary object. |
| objectType | xs:string | RW | false |  | This data point contains the type and subtype of the binary object as a MIME type. |
| size | xs:integer | RW | true |  | The size of the decoded binary object. |
| hash | xs:string | RW | true |  | The hash code of the blob. If present, it is used to check the decoded content of the “object” data point for integrity. The algorithm used for generating the hash value is SHA-2 [15]. The data point contains the hash as a hex encoded value. |

#### binarySwitch

This ModuleClass provides capabilities to control and monitor the state of power.

Table 5.3.12-14: Actions of binarySwitch ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | toggle | none | true | Toggle the switch. |

Table 5.3.12-15: DataPoints of binarySwitch ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| powerState | xs:boolean | RW | false |  | The current status of the binarySwitch. "True" indicates turned-on, and "False" indicates turned-off. |

#### bioElectricalImpedanceAnalysis

This ModuleClass provides the analysis of human body tissue based on impedance measurement.

Table 5.3.13-16: DataPoints of bioElectricalImpedanceAnalysis ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| water | xs:float | R | false | pct | The water content measurement from the BIA. |
| fat | xs:float | R | false | pct | The fat content measurement from the BIA. |
| muscle | xs:float | R | false | pct | The muscle content measurement from the BIA. |
| bone | xs:float | R | false | pct | The bone content measurement from the BIA. |
| visceraFat | xs:float | R | false | pct | The viscera fat content measurement from the BIA. |
| kcal | xs:float | R | false | kcal | The kcal (kilocalories) measurement from the BIA. |
| resistance | xs:float | R | false | ohm | The resistance of human body. |

#### bodyCompositionAnalyser

This ModuleClass provides the capability to report the measurement of body composition analyser characteristics.

Table 5.3.14-17: DataPoints of body composition analyser ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| bodyLength | xs:float | R | true | cm | The measurement of body length by Weight scale and Body composition analyser. |
| Bmi | xs:float | R | true | kg/m2 | The measurement of Body Mass Index (BMI) by a weight scale and a body composition analyser. |
| fatFreeMass | xs:float | R | true | kg | The measurement of fat free mass by a weight scale and a body composition analyser. |
| softLeanMass | xs:float | R | true | kg | The measurement of soft lean mass by a weight scale and a body composition analyser. |
| muscleMass | xs:float | R | true | kg | The measurement of muscle mass by a weight scale and a body composition analyser. |
| basalMetabolism | xs:float | R | true | kcal | The measurement of basal metabolism by a weight scale and a body composition analyser. |
| impedance | xs:float | R | true | ohm | The measurement of impedance by a weight scale and a body composition analyser. |

#### boiler

This ModuleClass provides capabilities to control the status of the boiling functionality for water heaters.

Table 5.3.15-18: DataPoints of boiler ModuleClass

| Name | Type | R-W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| status | xs:boolean | RW | false |  | The status of boiling. “True” indicates boiling, “False” indicates not boiling. |

#### brewing

This ModuleClass provides capabilities to control and monitor a brewing process. It is intended to be part of devices that prepare hot drinks such as a coffee or a tea.

Table 5.3.16-19: DataPoints of brewing ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| cupsNumber | xs:integer | RW | false |  | The current number of the cups requested to brew. |
| strength | hd:enumTasteStrength | RW | true |  | The current strength of the drink taste (see clause Error: Reference source not found). A higher value indicates a stronger taste. |

#### brightness

This ModuleClass provides capabilities to control and monitor the brightness of a light for example from a lamp. Brightness is scaled as a percentage. A lamp or a monitor can be adjusted to a level of light between very dim (0% is the minimum brightness) and very bright (100% is the maximum brightness).

Table 5.3.17-20: DataPoints of brightness ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| brightness | xs:integer | RW | false | pct | The status of brightness level. |

#### clock

This ModuleClass provides capabilities to control and monitor time and date information.

Table 5.3.18-21: DataPoints of clock ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentTime | xs:time | RW | false |  | Information of the current time |
| currentDate | xs:date | RW | false |  | Information of the current date |
| currentTimeZone | xs:string | RW | true |  | Name of current time zone according to the IANA Timezone data format (TZ) [17], for example, “America/New York”. |

#### clothesDryerJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of a clothes dryer.

Table 5.3.19-22: DataPoints of clothesDryerJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd:enumClothesDryerJobMode | RW | false |  | Currently active job mode (see clause Error: Reference source not found). |
| currentJobModeName | xs:string | R | true |  | The name of current job mode as a string. This can be used when “currentJobMode” is vendor-specific. |
| jobModes | list of hd:enumClothesDryerJobMode | R | false |  | List of possible job states the device supports. |

#### clothesWasherJobMode

This ModuleClasses provides capabilities to control and monitor the job mode of a washer.

Table 5.3.20-23: DataPoints of clothesWasherJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd: enumClothesWasherJobMode | RW | false |  | Currently active job mode (see clause Error: Reference source not found). |
| currentJobModeName | xs:string | RW | true |  | The name of the current job mode as a string. This can be used when the currentJobMode is vendor-specific. |
| jobModes | list of hd:enumClothesWasherJobMode | R | false |  | List of possible job states that the device supports (see clause Error: Reference source not found). |

#### clothesWasherDryerJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of clothesWasherDryer.

Table 5.3.21-24: DataPoints of clothesWasherDryerJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd: enumClothesWasherDryerJobMode | RW | false |  | Currently active job (see clause Error: Reference source not found). |
| currentJobModeName | xs:string | RW | true |  | The name of the current job mode as a string. This can be used when currentJobMode is vendor-specific. |
| jobModes | list of hd:enumClothesWasherDryerJobMode | R | false |  | List of possible job states the device supports (see clause Error: Reference source not found). |

#### clothesWasherJobModeOption

This ModuleClasses provides capabilities to control and monitor the washing job mode options of a washer.

Table 5.3.22-25: DataPoints of clothesWasherJobModeOption ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| washTemp | hd:enumGeneralTemperature | RW | true |  | This data point represents the water temperature level (see clause Error: Reference source not found). |
| soilLevel | hd:enumGeneralLevel | RW | true |  | This data point represents the washing level (see clause Error: Reference source not found). |
| spinSpeed | hd:enumGeneralSpeed | RW | true |  | This data point represents the spin-dry speed level (see clause Error: Reference source not found). |
| preWash | xs:boolean | RW | true |  | This data point indicates pre-wash. “True” indicates enabled, “False” indicates disabled. |
| speedWash | xs:boolean | RW | true |  | This data point indicates speed wash. “True” indicates enabled, “False” indicates disabled. |
| steamTreat | xs:boolean | RW | true |  | This data point indicates steam treat. “True” indicates enabled, “False” indicates disabled. |
| coldWash | xs:boolean | RW | true |  | This data point indicates cold wash. “True” indicates enabled, “False” indicates disabled. |
| extraRinse | xs:boolean | RW | true |  | This data point indicates extra rinse. “True” indicates enabled, “False” indicates disabled. |

#### colour

This ModuleClass provides the capabilities to set the value of the Red, Green, and Blue colour channels for a colour device.

Table 5.3.23-26: DataPoints of colour ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| red | xs:integer | RW | false |  | The value of the Red colour channel of RGB. The range is [0,255]. |
| green | xs:integer | RW | false |  | The value of the Green colour channel of RGB. The range is [0,255]. |
| blue | xs:integer | RW | false |  | The value of the Blue colour channel of RGB. The range is [0,255]. |

#### colourSaturation

This ModuleClass provides cababilities to control and monitor a colour saturation value.

Table 5.3.24-27: DataPoints of colourSaturation ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| colourSaturation | xs:integer | RW | false |  | The status of colour saturation level. “colourSaturation” has a range of [0,100]. A “colourSaturation” value of 0 means that a device displays or produces black and white images. A “colourSaturation” value of 50 means that a device displays or produces normal colour images. A “colourSaturation” value of 100 means that a device displays or produces very colourfull images. |

#### cookerHoodJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of a cookerHood.

Table 5.3.25-28: DataPoints of cookerHoodJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd: enumCookerHoodJobMode | RW | false |  | The currently active job mode. |
| currentJobModeName | xs:string | R | true |  | The name of the current job mode as a string. This can be used when currentJobMode is vendor-specific. |
| jobModes | list of hd: enumCookerHoodJobMode | R | false |  | List of possible job states the device supports (see clause Error: Reference source not found). |

#### credentials

This ModuleClass provides the capability to manage user credentials which allows a user to authenticate on an appliance or a server that is associated with the appliance. The authentication depends on a user login and password, or on a token. An example appliance which may include this ModuleClass is a camera.

Table 5.3.26-29: DataPoints of credentials ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| loginName | xs:string | W | true |  | The user’s login name. |
| password | xs:string | W | true |  | The user’s password. |
| token | xs:string | W | true |  | An authentication token, for example an OAuth token. |

#### dehumidifierJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of a dehumidifier device.

Table 5.3.27-30: DataPoints of dehumidifierJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd:enumDehumidifierJobMode | RW | false |  | The currently active job mode (see clause Error: Reference source not found). |
| currentJobModeName | xs:string | R | true |  | The name of the current job mode as a string. This can be used when currentJobMode is vendor-specific. |
| jobModes | list of hd:enumDehumidifierJobMode | R | false |  | List of possible job states the device supports. |

#### dishWasherJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of a dishWasher.

Table 5.3.28-31: DataPoints: DataPoints of dishWasherJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd: enumDishWasherJobMode | RW | false |  | The currently active job mode. |
| currentJobModeName | xs:string | R | true |  | The name of the current job mode as a string. This can be used when the currentJobMode is vendor-specific. |
| jobModes | list of hd: enumDishWasherJobMode | R | false |  | List of possible job states the device supports (see clause Error: Reference source not found) |

#### doorStatus

This ModuleClass provides the status of a door. It is intended to be part of a device such as a refrigerator and an oven that might have multiple doors.

Table 5.3.29-32: DataPoints of doorStatus ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| doorState | hd:enumDoorState | R | false |  | Current state of the door (see clause Error: Reference source not found). |
| openDuration | m2m:timestamp | R | true |  | The time duration the door has been open. |
| openAlarm | xs:boolean | RW | true |  | The state of the door open alarm. "True" indicates that the open alarm is active. "False" indicates that the open alarm is not active. |

#### electricVehicleConnector

This ModuleClass provides information about charging/discharging devices for electric vehicles.

Table 5.3.30-33: DataPoints of electricVehicleConnector ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| Status | xs:boolean | R | false |  | The status of connection. “True” means connected, “False” means not connected. |
| chargingCapacity | xs:integer | R | true | Ah | Rated charging capacity. |
| dischargingCapacity | xs:integer | R | true | Ah | Rated discharging capacity. |

#### energyConsumption

This ModuleClass describes the measured energy consumed by the device since power up. One particular use case for the energyConsumption ModuleClass is a smart meter.

Table 5.3.31-34: DataPoints of energyConsumption ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| power | xs:float | R | false | W | The power of the device. |
| absoluteEnergyConsumption | xs:float | R | true | Wh | The absolute energy consumption, reflecting the real measurement of accumulative energy. |
| roundingEnergyConsumption | xs: integer | R | true |  | This energy consumption data is calculated by multiplying significantDigits with multiplyingFactors, and rounding down the result. |
| significantDigits | xs:integer | R | true |  | The number of effective digits for data. |
| multiplyingFactors | xs:float | R | true |  | The unit for data multiplying factors, for example 1 kWh, 0,1 kWh, 0,01 kWh etc. |
| voltage | xs:float | R | true | V | The voltage of the device. |
| current | xs:float | R | true | A | The current of the device. |
| frequency | xs:float | R | true | Hz | The frequency of the device. |
| measuringScope | xs:string | RW | true |  | The measuring scope of the meter, for example the whole house, a room, or a device. |

#### energyGeneration

This ModuleClass provides information about generation data on electric generator devices such as a photo voltaic power system, fuel cells, or microgeneration.

Table 5.3.32-35: DataPoints of energyGenerationModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| powerGenerationData | xs:float | R | true | W | Amount of instaneous generation data. |
| roundingEnergyGeneration | xs:integer | R | true |  | This energy consumption data is calculated by multiplying significantDigits with multiplyingFactors, and rounding down the result. |
| significantDigits | xs:integer | R | true |  | The number of effective digits for data. |
| multiplyingFactors | xs:floatr | R | true |  | The unit for data multiplying factors, for example 1 kWh, 0,1 kWh, 0,01 kWh etc. |
| generationSource | xs:string | RW | false |  | The type of generating source. |

#### faultDetection

This ModuleClass provides information about whether a fault has occurred in a device.

Table 5.3.33-36: DataPoints of faultDetection ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| status | xs:boolean | R | false |  | The status of fault detection. |
| code | xs:integer | R | true |  | The numeric representation of the fault. |
| description | xs:string | R | true |  | The message representation of the fault. |

#### filterInfo

This ModuleClass is for monitoring filter information of a device.

Table 5.3.34-37: DataPoints of filterInfo ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| usedTime | xs:integer | R | false |  | The cumulative used time in seconds of a filter |
| needsReplacement | xs:boolean | R | true |  | This value indicates that the filter needs to be replaced. |
| filterLifetime | xs:integer | R | true |  | Percentage life time remaining for the water filter. |

#### foaming

This ModuleClass provides capabilities to control and monitor desired parameters of foam e.g. for foaming milk. It is initially intended to be part of a device that prepare drinks with milk (for example a coffee machine or hot chocolate machine).

Table 5.3.35-38: DataPoints of foaming ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| foamingStrength | hd:enumFoamStrength | RW | false |  | The current strength of foamed milk. A higher value indicates more foamed milk (see clause Error: Reference source not found) |

#### galleryMode

This ModuleClass provides information about the mode of display. The galleryMode includes the display orientation, display interval and display order.

Table 5.3.18-1: DataPoints of galleryMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| displayOrientation | hd:displayOrientation | RW | false |  | The orientation of display |
| displayInterval | xs:integer | RW | true | s | The interval of changing display content. |
| displayOrder | hd:displayOrder | RW | true |  | The sequence of the displaying content |

#### geoLocation

This ModuleClass provides the capability to get or set geo-location information.

Table 5.3.36-39: DataPoints of geoLocation ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| latitude | xs:float | RW | false | deg | The current latitude part of a geo-location. |
| longitude | xs:float | RW | false | deg | The current longitude part of a geo-location. |
| altitude | xs:float | RW | true | m | The optional current altitude part of a geo-location. |
| heading | xs:float | RW | true |  | The azimuth of a device measured in degrees to true north. North is 0.0 degrees, east is 90.0 degrees, south is 180.0 degrees, west is 270.0 degrees. A negative value indicates an unknown heading. |
| horizontalAccuracy | xs:float | R | true |  | The optional current horizontal accuracy of the geo-location. The unit of measures is meters and describes a radius around the latitude/longitude coordinate. |
| verticalAccuracy | xs:float | R | true | m | The optional current vertical accuracy of the altitude. |
| headingAccuracy | xs:float | R | true | deg | The optional current maximum deviation between the heading and the true geomagnetic heading. |
| targetLatitude | xs:float | RW | true | deg | The optional target latitude part of a geo-location. This can be used to move a device to a new location. |
| targetLongitude | xs:float | RW | true | deg | The optional target longitude part of a geo-location. This can be used to move a device to a new location. |
| targetAltitude | xs:float | RW | true | m | The optional target altitude part of a geo-location. This can be used to move a device to a new altitude. |

#### glucometer

This ModuleClass provides the capability to report the measurement of glucose characteristics.

Table 5.3.37-40: DataPoints of glucometer ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| concentration | xs:float | R | false | mg/dl | The measurement of concentration by Glucometer. |
| hba1c | xs:float | R | true | pct | The measurement of HbA1c by Glucometer. |
| contextExercise | xs:float | R | true | pct | The measurement of context exercise by Glucometer. |
| contextMedication | xs:float | R | true | mg/dl | The measurement of context medication by Glucometer. |
| contextCarbohydratesAmount | xs:float | R | true | g | The measurement of context carbohydrates by Glucometer. |
| contextCarbohydratesSource | xs:string | R | true |  | The timing of meals (for example “breakfast carbohydrates”). |
| contextMeal | xs:string | R | true |  | The style of meals (for examople “casual”). |
| contextLocation | xs:string | R | true |  | The body location where the Glucometer is worn (for example “finger”). |
| contextTester | xs:string | R | true |  | The test style (for example “self”). |
| contextHealth | xs:string | R | true |  | The severity of symptoms (for example “minor”), |

#### grinder

This ModuleClass is for controlling a grinder, for example in a coffee machine.

Table 5.3.38-41: DataPoints of grinder ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| useGrinder | xs:boolean | RW | false |  | The current status of the grinder enablement. "True" indicates enabled, and "False" indicates disabled. |
| coarseness | hd:enumGrindCoarseness | RW | true |  | The wished coarseness of the solid supplies after grinding, for example for coffee beans (see clause Error: Reference source not found). |
| grainsRemaining | hd:enumGrainsLevel | R | true |  | The level of remaining grains in a machine having a grinder, for example for remaining coffee beans in the coffee machine grinder (see clause Error: Reference source not found). |

#### heatingZone

This ModuleClass provides the capabilities to monitor the status of the heating zone, for example for a cooktop.

Table 5.3.39-42: DataPoints of heatingZone ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| heatingLevel | xs:integer | R | false |  | The current heating level of the zone. The value range is from 0 (indicating that the zone is not heating) up to the maxHeatingLevel. |
| maxHeatingLevel | xs:integer | R | false |  | The maximum value allowed for the heating level of the zone. |

#### height

This ModuleClass provides the capability to report the measurement of height.

Table 5.3.40-43: DataPoints of height ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| height | xs:float | R | false | cm | The height measurement. |

#### hotWaterSupply

This ModuleClass provides information about the status of supplying hot water into tanks or bath tubs.

Table 5.3.41-44: DataPoints of hotWaterSupply ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| bath | xs:boolean | RW | true |  | The status of whether a bath tub is filled. |

#### impactSensor

This ModuleClass describes the capabilities on an impact sensor. The impact is a high force or shock over a short time period and the impactSensor detects this.

Table 5.3.42-45: DataPoints of impactSensor ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| impactStatus | xs:boolean | R | false |  | The “impactStatus” indicates as follows: “True” means that a physical impact is detected, “False” means indicates a normal status (no impact detected). |
| impactLevel | xs:float | R | true |  | The “impactLevel” provides the level of impact. The unit of measure is “g” (G-force). |
| impactDirectionHorizontal | xs:float | R | true |  | The “impactDirection” indicates the horizontal direction where the impact comes from. The value is 0° to 360°. 0 is the front of the sensor and with clockwise increment. |
| impactDirectionVertical | xs:float | R | true |  | The “impactDirection” indicates the vertical direction where the impact comes from. The value is 0° to 360°. 0 is the front of the sensor and with upward increment. |

#### keepWarm

This module allows to control the ‘keep warm’ feature in devices like coffe machines, kettles etc. It allows to keep water warm for a desired time. This ModuleClass inherits from binarySwitch (see clause 5.3.1.12) to store setting for the ‘keep warm’ feature. If the “powerState” data point in a keepWarmSwitch is “True” then the ‘keep warm’ function will be performed just after boiling (or heating) process is finished (otherwise this function will not be applied).

Table 5.3.43-46: DataPoints of keepWarm ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| time | xs:integer | RW | true |  | The desired duration of ‘keep water warm’ function. It indicates how long water shall be kept warm, for example after the boiling in a kettle. The value indicates a time expressed in minutes. |

#### keypad

This ModuleClass provides the capability to perform a user defined service through the key-in number. For example, a user can define key 1 as "perform a takeout from a restaurant with combo meal 1". The IoT service provider or user can define the services.

Table 5.3.44-47: DataPoints of keypad ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| keyNumber | xs:integer | R | false |  | The number of the pressed key. |

#### liquidLevel

This ModuleClass provides the desired level of water (or other liquid)  for an appliance, for example the desired level of milk for a cup of coffee from a coffee machine.

Table 5.3.45-48: DataPoints of liquidLevel ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| liquidLevel | hd:enumLiquidLevel | RW | false |  | The desired level of water or other liquid, for example the desired level of milk in a cup of coffee (see clause Error: Reference source not found). |

#### liquidRemaining

This ModuleClass provides the status of water level (or other liquid) for an appliance, for example the level of remaining milk in a coffee machine.

Table 5.3.46-49: DataPoints of liquidRemaining ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| liquidRemaining | hd:enumLiquidLevel | R | false |  | The remaining level of water or other liquid in an appliance (see clause Error: Reference source not found). |

#### lock

This ModuleClass provides the function to lock and unlock an object.

Table 5.3.47-50: DataPoints of lock ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| lock | xs:boolean | RW | false |  | "True" indicates the object is locked, while "False" indicates the object is not locked. |

#### mediaSelect

This ModuleClass provides capabilities to control and monitor media input and output of device such as TV or SetTopBox.

Table 5.3.48-51: DataPoints of mediaSelect ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| mediaID | xs:integer | RW | false |  | The numeric index of the activated media in the supported media sources list “supportedMediaSources”. |
| supportedMediaSources | list of hd:enumSupportedMediaSource | R | false |  | List of supported input or output media for the given device (see clause Error: Reference source not found). |
| mediaName | xs:string | R | true |  | Specifies a pre-defined media input or output. |
| status | xs:boolean | R | true |  | Specifies whether the specific media instance is selected (“True”) or not (“False”). |
| mediaType | hd:enumSupportedMediaSource | R | false |  | Specifies the type of the media (see clause Error: Reference source not found). |

#### motionSensor

This ModuleClass provides the capabilities to indicate the occurrence of motion and raising of an alarm if the triggering criterion is met.

Table 5.3.49-52: DataPoints of motionSensor ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| alarm | xs:boolean | R | false |  | The detection of the motion occurrence. |
| silentTime | xs:integer | RW | true | s | The time that a motionSensor restrains from sending an alarm in case continous motions are detected after one alarm is produced. This data point can be used to avoid repeated alarm reports. |
| sensitivity | xs:integer | RW | true |  | The level of the detection accuracy of the motion sensor. This data point can be used to control the number of the report. |

#### numberValue

This ModuleClass provides the capabilities to represent a number. It also has capabilities for controlled increment and decrement a counter. It can be used to present a number-related functionality in a technology where there is only a weak semantic specification of that functionality.

Table 5.3.50-53: Actions of numberValue ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | decrementNumberValue | none | true | Decrement the “numberValue” by the value of “step”, down to the value of “minimum”. |
| none | incrementNumberValue | none | true | Increment the "numberValue" by the value of "stepValue", up to the value of "maxValue". |
| none | resetNumberValue | none | true | Reset the “numberValue” to its “defaultValue”. |

Table 5.3.50-54: DataPoints of numberValue ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| numberValue | xs:float | RW | false |  | The actual value of the number. |
| minValue | xs:float | RW | true |  | The optional minimum value of the number. The default is the system-specific minimum value for a float value. |
| maxValue | xs:float | RW | true |  | The optional maximum value of the number. The default is the system-specific maximum value for a float value. |
| defaultValue | xs:float | RW | true |  | The optional default value for the number. The default is 0.0 . |
| step | xs:float | RW | true |  | The optional step size for controlled increment and decrement. The default is 1.0 , even when this data point is not implemeneted. |

#### openLevel

This ModuleClass provides the capabilities to control and monitor the open status of an entity, for example a curtain.

Table 5.3.51-55: Actions of openLevel ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | open | None | true | Increase the open level by the amount of the “stepValue” up to the “maxLevel”. |
| none | close | None | true | Decrease the open level by the amount of the “stepValue” down to the “minLevel”. |

Table 5.3.51-56: DataPoints of openLevel ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| openLevel | xs:integer | RW | false | pct | The rounded percentage of the current open level of entity in the range of [0, 100]. 0 percentage shall mean the entity is closed. |
| stepValue | xs:integer | RW | true |  | The step value used by the “open” and “close” actions. |
| minLevel | xs:integer | RW | true |  | The minimum value allowed for the “openLevel” status. The default value is 0, which means fully closed. |
| maxLevel | xs:integer | RW | true |  | The maximum value allowed for the “openLevel” status. The default value is 100, which means fully opened. |

#### operationMode

This ModuleClasses provides capabilities to control or monitor the operation mode of appliances.

Table 5.3.52-57: DataPoints of operationMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| startPause | xs:boolean | RW | false |  | A value of “True” triggers or starts an operation, and “False” pauses the operation. |

#### overcurrentSensor

This ModuleClass provides capabilities for an over-current sensor.

Table 5.3.53-58: DataPoints of overcurrentSensor ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| overcurrentStatus | xs:boolean | R | false |  | The overcurrentStatus indicates as follows: “True” indicates that an over-current is detected, and “False” indicates a normal status, this means that an over-current is not detected. |
| detectedTime | m2m:timestamp | R | true |  | The time when the over-current was detected. |
| duration | xs:float | R | true | ms | The duration of the detected over-current. |

#### oximeter

This ModuleClass provides the capability to report the measurement of blood oxygen characteristics.

Table 5.3.54-59: DataPoints of oximeter ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| oxygenSaturation | xs:integer | R | false | pct | The measurement of oxygensaturation by Oximeter. |

#### ozoneMeter

This ModuleClass provides capabilities for an ozone meter. The “ozoneValue…” attributes are optional, but one of them SHALL be provided.

Table 5.3.55-60: DataPoints of ozoneMeter ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| ozoneValuePPM | xs:float | R | true | ppm |  |
| ozoneValueMG | xs:float | R | true | mg/m |  |
| ozoneStatus | hd:enumOzoneStatus | R | true |  | The ozoneStatus indicates the level of ozone status. (see clause Error: Reference source not found). |
| maxValue | xs:float | R | true |  | The mazimum value shows the measurement range of the ozone meter (for example maxValue=5 means the range is 0 to 5 ppm). This attribute is only used that the ozoneMeter provides “ppm” value. |

#### phoneCall

This ModuleClass provides the capability get or set the caller and receipient IDs as well as to initate and terminate a call.

Table 5.3.56-61: Actions of phoneCall ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | call | none | true | Initiate an outgoing call. |
| none | answer | none | true | Answer (pickup) an incoming call. |
| none | hangup | none | true | Hangup an established call. |

Table 5.3.56-62: DataPoints of phoneCall ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| callerID | xs:string | RW | false |  | This data point represents the initiating caller identiﬁcation of a call. In case of an outgoing call this would be the local line ID. This data point is optional. When it is empty for an incomming call, then the caller ID is unknown. When it is empty for an outgoing call, then it is exepected that it is set by the PBX or the operator. The format of caller ID is not speciﬁed here. |
| recipientID | xs:string | RW | false |  | This data point represents the receiving caller identiﬁcation of a call. In case of an incoming call this would be the local line ID and optionally extension. The format of caller ID is not speciﬁed here. |
| callState | hd:enumCallState | R | true |  | This data point represents the current state of an associated phone device regarding calls. |

#### playerControl

This ModuleClass provides capabilities to control and monitor the operational modes of a media player functionality.

Table 5.3.57-63: Actions of playerControl ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | nextTrack | none | true | Go forward to a next chapter, section or similar marker in the media. |
| none | previousTrack | none | true | Go back to a previous chapter, section or similar marker in the media. |

Table 5.3.57-64: DataPoints of playerControl ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentPlayerMode | hd:enumPlayerMode | RW | false |  | The current mode of the player. |
| currentPlayerModeName | xs:string | R | true |  | Name of current player mode in string. This can be used when “currentPlayerMode” is vendor-specific. |
| supportedPlayerModes | list of hd:enumPlayerMode | R | false |  | List of supported modes for a player. |
| speedFactor | xs:float | RW | true |  | The optional factor of speeding up or slowing down playback, rewind or fast forward. |

#### powerSave

This ModuleClass provides capabilities to enable the power saving mode of a device and monitor the current status.

Table 5.3.58-65: DataPoints of powerSave ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| powerSaveEnabled | xs:boolean | RW | false |  | The current status of the power saving mode. "True" indicates enabled, and "False" indicates not enabled. |

#### printQueue

This ModuleClass provides the capabilities for monitoring printing list information.

Table 5.3.59-66: DataPoints of printQueue ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| uri | list of xs:uri | trueR | false |  | The URI of the printing file. The URI could be “file://www.example.com/file.extension” |
| printingState | list of hd:enumJobState | R | false |  | The printingState is indicating the status of the printing file. |

#### pulsemeter

This ModuleClass provides the capability to report the measurement of pulse characteristics.

Table 5.3.60-67: DataPoints of pulsemeter ModuleClass

| Name | Type | R-W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| pulseRate | xs:float | R | false | bpm | The measurement of pulserate by pulsemeter. |
| rr | xs:float | R | true | ms | The measurement of RR interval by pulsemeter. |
| energy | xs:float | R | true | kcal/h | The measurement of energy by pulsemeter. |
| modality | xs:string | R | true |  | The modality of a particular SpO2 measurement. |

#### pushButton

This ModuleClass provides the capability to indicate the operation of a push button style switch. A typical application can be an SOS button.

Table 5.3.61-68: DataPoints of pushButton ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| pushed | xs:boolean | R | false |  | This data point indicates the press of the button. |

#### recorder

This ModuleClass provides the capability to record video/audio for a defined duration.

Table 5.3.62-69: DataPoints of recorder ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| duration | xs:integer | RW | false | s | The duration for video/audio recording. Set to trigger the recorder. |

#### refrigeration

This ModuleClass provides capabilities for a refrigeration function.

Table 5.3.63-70: DataPoints of refrigeration ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| rapidFreeze | xs:boolean | RW | true |  | Controls the rapid freeze capability. “True” indicates active, “False” indicates inactive. |
| rapidCool | xs:boolean | RW | true |  | Controls the rapid cool capability. “True” indicates active, “False” indicates inactive. |
| defrost | xs:boolean | RW | true |  | Controls the defrost cycle. “True” indicates active, “False” indicates inactive. |

#### relativeHumidity

This ModuleClass provides the capability for a device to report the humidity based on a specified rule that is vendor dependent.

Table 5.3.64-71: DataPoints of relativeHumidity ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| relativeHumidity | xs:float | R | false | pct | The measurement of the relative humidity value |
| desiredHumidity | xs:float | RW | true |  | Desired value for humidity. This data point indicates the desired humidity. |

#### remoteControlEnable

This ModuleClasses provides capabilities to monitor the remote controllability of the appliance.

Table 5.3.65-72: DataPoints of remoteControlEnable ModuleClass

| Name | Type | R-W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| remoteControlEnabled | xs:boolean | R | false |  | This data point enables or disables remote controllability and is set by a user locally. “True” indicates enabled remote access, and “False” indicates disabled remote access. |

#### robotCleanerJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of a robotCleaner.

Table 5.3.66-73: DataPoints of robotCleanerJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd:enumRobotCleanerJobMode | RW | false |  | Currently active job mode (see clause Error: Reference source not found). |
| currentJobModeName | xs:string | R | true |  | Name of current job mode as a string. This can be used when “currentJobMode” is vendor-specific. |
| jobModes | list of hd:enumRobotCleanerJobMode | R | false |  | List of possible job states the device supports (see clause Error: Reference source not found). |

#### runState

This ModuleClasses provides capabilities to control and the monitor machine state of appliances.

Table 5.3.67-74: DataPoints of runState ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentMachineState | hd:enumMachineState | RW | false |  | The currently active machine state (see clause Error: Reference source not found). |
| machineStates | list of hd:enumMachineState | R | false |  | A list of possible machine states the device supports (see clause Error: Reference source not found). |
| currentJobState | hd:enumJobState | R | true |  | The currently active job state at the level of some transaction being executed by the device (see clause Error: Reference source not found). |
| jobStates | list of hd:enumJobState | R | true |  | The list of possible job states that the device supports (see clause Error: Reference source not found). |
| progressPercentage | xs:float | R | true |  | The indication of current job progress in percentage. |

#### securityMode

This ModuleClasses provides capabilities to control and monitor a security mode.

Table 5.3.68-75: DataPoints of securityMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentSecurityMode | hd:enumSecurityMode | RW | false |  | Current security mode (see clause Error: Reference source not found). |
| securityModes | list of hd:enumSecurityMode | R | false |  | List of possible security modes the device supports (see clause Error: Reference source not found). |

#### sessionDescription

This ModuleClass provides the capabilities for a sessionDescription containing a URL at twhich the specified media can be accessed and the definition of media using SDP.

Table 5.3.69-76: DataPoints of mediaType ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| url | xs:uri | R | true |  | A URL at which the specified media can be accessed. |
| sdp | xs:string | R | true |  | Media description using SDP. One or more comma separated multiple SDP lines (SDP media or attribute line) can be included using SDP description syntax as defined in the SDP specification in RFC4566 [16]. |

#### signalStrength

This ModuleClass provides the capability to monitor the strength of the signal.

Table 5.3.70-77: DataPoints of signalStrength ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| lqi | xs:integer | truR | false |  | The current value of link quality indicator, which reflects the scaling of rssi by dividing the received signal strength over reference signal strength. The common unit for lqi is percentage [0,100]. For the detailed definition, please see IEEE 802.15.4 [Error: Reference source not found], clause 6.7.8. |
| rssi | xs:float | R | true |  | The current value of received signal strength indicator, which reflects the raw signal level. |

#### smokeSensor

This ModuleClass provides the capabilities to indicate the detection of smoke and raising an alarm if the triggering criterion is met.

Table 5.3.71-78: DataPoints of smokeSensor ModuleClass

| Name | Type | R-W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| alarm | xs:boolean | R | false |  | The alarm is indicated as follows: “True” indicates that smoke has been detected, “False” indicates a normal status, that means that smoke is not detected. |
| detectedTime | m2m:timestamp | RW | true |  | The date and time the smoke is detected. |
| smokeThreshhold | xs:integer | RW | true | ppm | The threshhold to trigger the alarm. |
| currentValue | xs:integer | R | true |  | The current data value of the smoke sensor. |

#### sphygmomanometer

This ModuleClass provides the capability to report the measurement of blood pressure characteristics.

Table 5.3.72-79: DataPoints of sphygmomanometer ModuleClass

| Name | Type | R-W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| diastolicPressure | xs:float | R | false | mmHg | The measurement of diastolic pressure by sphygmomanometer. |
| systolicPressure | xs:float | R | false | mmHg | The measurement of systolic pressure by sphygmomanometer. |
| meanPressure | xs:float | R | false | mmHg | The measurement of mean arterial pressure by sphygmomanometer. |

#### spinLevel

This ModuleClass provides capabilities to control and monitor the level of spin. It is intended to be part of devices which use spinning function such as a washing machine and a dryer.

Table 5.3.73-80: DataPoints of spinLevel ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| spinLevelStrength | hd:enumSpinLevelStrength | RW | false |  | The value of spin-dry level (see clause Error: Reference source not found). A higher value indicates a higher spin level. |

#### steamClosetJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of steamCloset.

Table 5.3.74-81: DataPoints of steamClosetJobMode ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentJobMode | hd:enumSteamClosetJobMode | RW | false |  | Currently active job mode (see clause Error: Reference source not found). |
| currentJobModeName | xs:string | R | true |  | Name of current job mode as a string. This can be used when “currentJobMode” is vendor-specific. |
| jobModes | list of hd:enumSteamClosetJobMode | R | false |  | List of possible job states the device supports (see clause Error: Reference source not found). |

#### televisionChannel

This ModuleClass provides capabilities to set and get channels of a device that has a channel list.

Table 5.3.75-82: Actions of televisionChannel ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | upChannel | None | true | Change the current channel to the next channel in the stored list of available channels. If the current channel is the last one in the list, the new set channel may be the first one in the list. |
| none | downChannel | None | true | Change the current channel to the previous channel in the stored list of available channels. If the current channel is the first one in the list, the new set channel may be the last one in the list. |

Table 5.3.75-83: DataPoints of televisionChannel ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| channelNumber | xs:integer | RW | false |  | Current channel number. |
| availableChannels | list of xs:integer | R | true |  | The list of available channel numbers which may be build by automatic scan and/or manual selction. |
| previousChannel | xs:integer | R | true |  | The channel number which was selected previously. |
| channelName | xs:string | R | true |  | Current human-friendly channel name in string, for example ‘CNN’. |

#### temperature

This ModuleClass provides capabilities to represent the current temperature and target temperature of devices such as an air conditioner, refrigerator, oven etc.

Table 5.3.76-84: DataPoints of temperature ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| currentTemperature | xs:float | R | false |  | The current temperature. |
| targetTemperature | xs:float | RW | true |  | The desired temperature to reach. |
| unit | xs:string | R | true | C |  |
| minValue | xs:float | R | true |  | Minimum value of “targetTemperature”. |
| maxValue | xs:float | R | true |  | Maximum value of “targetTemperature”. |
| stepValue | xs:float | R | true |  | Step value allowed for “targetTemperature”. |

#### temperatureAlarm

This ModuleClass provides the capabilities to indicate the detection of abnormal temperatures and raises an alarm if the triggering criterion is met.

Table 5.3.77-85: DataPoints of temperatureAlarm ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| alarm | xs:boolean | R | false |  | This data point indicates the status of detection of an abnormal temperature. “True” indicates an abnormal temperature, “False” indicates a normal temperature. |
| temperature | xs:float | R | true | C | To report the value of the temperature. |
| temperatureThreshhold | xs:integer | RW | true |  | The threshhold to trigger the alarm. |

#### textMessage

This ModuleClass provides capabilities to set and get a text message.

Table 5.3.78-86: Actions of textMessage ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | resetTextMessage | none | true | Reset the receiver of the message to the “defaultValue”. |

Table 5.3.78-87: DataPoints of textMessage ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| textMessage | xs:string | RW | false |  | The current message value. |
| supportedMessageValues | list of xs:string | R | true |  | List of supported values for the message. Each of the values in this list must be URL-encoded. An encoded value must not contain white spaces. |
| minLength | xs:integer | R | true |  | The optional minimum length in characters of the message. The default is 0. |
| maxLength | xs:integer | R | true |  | The optional maximum length in characters of the message. The default is unlimited. |
| messageEncoding | xs:string | R | true |  | The optional expected method for character encoding of the message. The default is "UTF-8". |
| defaultValue | xs:string | RW | true |  | The optional default value for “textMessage”. The default is an empty string. |

#### timer

This ModuleClass provides capabilities to monitor and control the times when the appliance executes its operations, that means when it starts, when it ends etc.

Table 5.3.79-88: Actions of timer ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | activateClockTimer | None | true | Activate current clock timer. |
| none | deactivateClockTimer | None | true | Deactivate current clock timer. |

Table 5.3.79-89: DataPoints of timer ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| referenceTimer | xs:integer | R | true |  | A timer (for example. a time-based value, App Defined Epoch, Progressive) expressed in seconds. The value indicates a time counter to be used as reference for the other time-based data points of this ModuleClass. Usually it is the time since the last event of power-on of the producer (or more in detail the time since the boot of its connectivity node). |
| targetTimeToStart | xs:integer | RW | true |  | A time span (for example a time-based value, App Defined Epoch, Fixed) expressed in seconds. The value indicates the time when the appliance is expected to start its operation, starting counting from the last ”referenceTimer”. |
| targetTimeToStop | xs:integer | RW | true |  | A time span (for example a time-based value, App Defined Epoch, Fixed) expressed in seconds. The value indicates the time when the appliance is expected to stop its operation, starting counting from the last ”referenceTimer”. |
| estimatedTimeToEnd | xs:integer | R | true |  | A timer (for example a time-based value, App Defined Epoch, Progressive) expressed in seconds. The value indicates the time to the end of an appliance’s operations. It is calculated at runtime by the device itself during the execution of its operation. |
| runningTime | xs:integer | R | true |  | A timer (for example a time-based value, App Defined Epoch, Progressive) expressed in seconds. It indicates the time of the current operation. Usually its value is increasing one value per second. It starts counting from 0 when the operation starts and stops counting when the operation ends. |
| targetDuration | xs:integer | R | true |  | A time span (for a time-based value, App Defined Epoch, Fixed) expressed in seconds. The value indicates a time that represents the target duration of the operation as per user selection. |
| absoluteStartTime | m2m:timestamp | RW | true |  | An absolute time to specify the start time. |
| absoluteStopTime | m2m:timestamp | RW | true |  | An absolute time to specify the stop time. |

#### turbo

This ModuleClass provides capabilities to enable turbo mode and monitor the current status of the turbo function. It is intended to be part of devices which use turbo function such as an air conditioner, a washing machine etc.

Table 5.3.80-90: DataPoints of turbo ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| turboEnabled | xs:boolean | RW | false |  | The current status of the turbo mode. "True" indicates enabaled, and "False" indicates not enabled. |

#### uvSensor

This ModuleClass describes the capabilities of an ultraviolet sensor.

Table 5.3.81-91: DataPoints of uvSensor ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| uvValue | xs:float | R | false | mW/cm2 |  |
| uvStatus | hd:enumUvStatus | R | true |  | The “uvStatus” indicates the level of the UV radiation status (see clause Error: Reference source not found). |

#### waterFlow

This ModuleClass provides capabilities for controlling the water strength of a device.

Table 5.3.82-92: DataPoints of waterFlow ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| waterLevelStrength | hd:enumWaterFlowStrength | RW | false |  | The desired level of water flow (see clause Error: Reference source not found). A higher value indicates higher water flow. |

#### waterSensor

This ModuleClass provides the capabilities to indicate whether or not water has been sensed, and raising an alarm if the triggering criterion is met.

Table 5.3.83-93: DataPoints of waterSensor ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| alarm | xs:boolean | R | false |  | The detection of water. The alarm is indicated as follows: “True” indicates that water has been detected, “False” indicates a normal status, that means that water is not detected. |

#### weight

This ModuleClass provides the capability to report the measurement of weight.

Table 5.3.84-94: DataPoints of weight ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| weight | xs:float | R | false | kg | The weight measurement. |

### City Domain

### Health Domain

### Home Domain

### Industry Domain

### Vehicular Domain

### -----------------------Endo of change 1-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?