| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | MAS #37 |
| Source:* | Bareau Cyrille, Orange, cyrille.bareau@orange.com Bonnardel Gregory, Orange, gbonnardel.ext@orange.com Gharout Said, Orange, said.gharout@orange.com Goluch Maciej, Orange, maciej.goluch@orange.com Kraft Andreas, DT, A.Kraft@telekom.de Strzemecki Paweł, Orange, pawel.strzemecki2@orange.com |
| Date:* | 2018-09-13 |
| Reason for Change/s:* | See the introduction below |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0088 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0022 v. 4.0 |
| Clauses * | 7 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR brings new Management Object [wifiClient] which shall be used to configure WiFi connection on client device.

It also enhances the Data formats for device configuration section with enumeration and  complex data types sections and adds new data types used in [wifiClient] Management Object.

R01: Definitions of data type used in new [wifiClient] Management Object moved from the external CR against TS-0004 to this document. New fields toggleRadioStatus and radioStatus added.

### -----------------------Start of change 1-------------------------------------------

# 7.1.X	Resource wifiClient

This specialization of <mgmtObj> is used to store configuration of WiFi connection on the client device.

Figure 7.1.X-1: Structure of [wifiClient] resource

The [wifiClient] resource shall contain the child resources specified in table 7.1.X-1.

Table 7.1.X-1: Child resources of [wifiClient] resource

| Child Resources of [wifiClient] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 of oneM2M TS-0001 [2] |

The [wifiClient] resource shall contain the attributes specified in table 7.1.X.-2

Table 7.1.X-2: Attributes of [wifiClient] resource

| Attributes of [wifiClient] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| resourceID | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| resourceName | 1 | WO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| parentID | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| expirationTime | 1 | RW | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| creationTime | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| labels | 0..1(L) | RW | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| mgmtDefinition | 1 | WO | See clause 9.6.15 of oneM2M TS-0001 [2].. This attribute shall have the fixed value 1027(wifiClient). |
| objectIDs | 0..1 (L) | WO | See clause 9.6.15 of oneM2M TS-0001 [2]. |
| objectPaths | 0..1 (L) | WO | See clause 9.6.15 of oneM2M TS-0001 [2]. |
| description | 0..1 | RW | See clause 9.6.15 of oneM2M TS-0001 [2]. |
| ssid | 1 | RW | Indicates ssid of wifi network. This attribute is a specialization of [objectAttribute] attribute. |
| credentials | 0..1 | RW | Indicates encryption type and credentials used to authenticate connection. This attribute is a specialization of [objectAttribute] attribute. |
| macAddress | 0..1 | RW | Indicates MAC address of network’s access point. This attribute is a specialization of [objectAttribute] attribute. |
| channel | 0..1 | RO | Indicates WiFi channel. This attribute is a specialization of [objectAttribute] attribute |
| connectionStatus | 0..1 | RW | Indicates current status of the connection. This attribute is a specialization of [objectAttribute] attribute. |
| scan | 1 | RW | The action that allows to scan the environment to find available wifi networks. The action is triggered by assigning value "TRUE" to this attribute. This attribute is a specialization of [objectAttribute] attribute. This action will provide all the available ssids in scanResult. |
| scanResult | 1 | RW | Indicates list of networks detected by the device. This attribute is a specialization of [objectAttribute] attribute. |
| update | 1 | RW | The action is to trigger the device to update its Wifi configuration based on the provided values (e.g. ssid, credentials, …).The action is triggered by assigning value "TRUE" to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| updateStatus | 1 | RW | Indicates status of update operation. This attribute is a specialization of [objectAttribute] attribute. |
| toggleRadioStatus | 0..1 | RW | The action that allows to turn on and turn off the wifi sub-syste on wifi client device according to present radioStatus attribute value. The action is triggered by assigning value "TRUE" to this attribute. This attribute is a specialization of [objectAttribute] attribute. This action will switch radioStatus. |
| radioStatus | 0..1 | RW | Indicates whether the wifi sub-system on wifi client device is turned on or turned off. This attribute is a specialization of [objectAttribute] attribute. |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 7.2.X Resource [wifiClient]

### 7.2.X.1 Introduction

This specialization of <mgmtObj> is used to store configuration of WiFi connection on the client device.

Table 7.2.X.1-1: Data Type Definition of [wifiClient]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| wifiClient | DCFG-wifiClient-v4_0_0.xsd |  |

Table 7.2.X.1-2: Resource specific attributes of [wifiClient]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.15 of oneM2M TS-0004 [4]. | 1027 (wifi) |
| objectIDs | O | NP | See clause 7.4.15 of oneM2M TS-0004 [4]. |  |
| objectPaths | O | NP | See clause 7.4.15 of oneM2M TS-0004 [4]. |  |
| description | O | O | See clause 7.4.15 of oneM2M TS-0004 [4]. |  |
| ssid | M | O | dcfg:ssid |  |
| credentials | O | O | dcfg:wifiCredentials |  |
| macAddress | O | O | xs:string |  |
| channel | O | O | xs:integer |  |
| update | O | O | xs:boolean |  |
| scan | M | O | xs:boolean |  |
| toggleRadioStatus | O | O | xs:boolean |  |
| scanResult | O | O | dcfg:listOfSsids |  |
| updateStatus | O | O | m2m:actionStatus |  |
| connectionStatus | O | O | dcfg:wifiConnectionStatus |  |
| radioStatus | O | O | xs:boolean |  |

- 7.2.X.2 Resource specific procedure on CRUD operations

- When management is performed using technology specific protocols, the procedures defined in clause 7.4.15.2 on 473 oneM2M TS-0004,'<mgmtObj> specific procedures' shall be used. There are no change from the generic procedures in 474 clause 7.2.2 of oneM2M TS-0004 [4] for operations on this resource except retrieve operation which is described below.

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

## 7.3	Data formats for device configuration

### 7.3.1	Introduction

The clause 7.3 defines data formats of resource attributes and parameters used in present document.

Any data types of XML elements defined for use in present document shall be one of name spaces in table 7.3.1-1.

Table 7.3.1-1: Namespaces used in present document

| Name space | prefix | Name space definition |
| --- | --- | --- |
| oneM2M protocol CDT | m2m: | http://www.onem2m.org/xml/protocol |
| Device Configuration | dcfg: | http://www.onem2m.org/xml/deviceConfig |
| oneM2M Security | sec: | http://www.onem2m.org/xml/securityProtocols |

### 7.3.2	Simple oneM2M data types for device configuration

Table 7.3.2-1 describes simple data type definitions specific to security. The types in table 7.3.2-1 are either:

- Atomic data types derived from XML Schema data types by restrictions other than enumeration

- List data types constructed from other XML Schema or oneM2M-defined atomic data types.

Table 7.3.2-1: oneM2M simple data types for device configuration

| XSD type name | Used for | Examples | Description |
| --- | --- | --- | --- |
| dcfg:TLSCiphersuite | A TLS Ciphersuite identifier | C0A5 | Four hexadecimal characters representing a TLS Cipher suite identifier. The list of TLS cipher suites identifiers can be found at the IANA TLS Cipher Suite Registry [Error: Reference source not found] |
| dcfg:ListOfTLSCiphersuite | A list of TLS Ciphersuite identifiers |  | xs:list of elements of data type dcfg:TLSCiphersuite |
| dcfg:niURI | Identifying information with a hash value | ni:///sha-256;UyaQV... ni:///1;UyaQV... ("1" is a short identifier for sha-256) | An xs:anyURI conforming to the Named Information 'ni' URI scheme specified in IETF RFC 6920 [Error: Reference source not found], with no authority field. |
| dcfg:nihURI | Identifying information with a human speakable encoding of a hash value | nih:sha-256-32;53269057;b nih:sha-256-32;5326-9057;b nih:6;5326-9057 ("6" is a short identifier for sha-256-32) | An xs:anyURI conforming to the Human Speakable Named Information 'nih' URI scheme specified in IETF RFC 6920 [Error: Reference source not found], with no authority field. A checkdigit may be present. |
| dcfg:ssid | SSID of WiFi network |  | The ssid of wifi network |
| dcfg:listOfSsids | List of SSIDs |  | xs:list of elements of data type m2m:ssid. |

### 7.3.3	Enumerated oneM2M data types for device configuration

#### 7.3.3.1 dcfg:wifiConnectionStatus

Used to indicate  WiFi connection status

Table 7.3.3.1-3: Interpretation of dcfg:wifiConnectionStatus

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | Disconnected |  |
| 1 | Connected |  |
| 2 | Idle | Indicates the state when the device tries to establish the connection to the network. It expires after the set number of unsuccessful attempts or when the connection is established. |
| 3 | No SSID available |  |
| 4 | Scan completed |  |
| 5 | Failed |  |
| 6 | Lost |  |

#### 7.3.3.2 dcfg:wifiEncryptionType

Used for indicating which WiFi encryption protocol is used to provide security in wireless network

Table 7.3.3.2-1: Interpretation of dcfg:wifiEncyrptionType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | none |  |
| 2 | WEP |  |
| 3 | WPA Personal |  |
| 4 | WPA2 Personal |  |
| 5 | WPA3 Personal |  |
| 6 | WPA Enterprise |  |
| 7 | WPA2 Enterprise |  |
| 8 | WPA3 Enterprise |  |

### 7.3.4 Complex type definitions

#### 7.3.4.1 dcfg:wifiCredentials

Used for credentials  in [wifiClient] Management Object resource. Values contained in username, password, wep.key and wpa.psk attributes shall not be readable. Each Retrieve request shall be forwarded to device and from there the data shall be retrieved.

Table 7.3.4.1-1: Type Definition of dcfg:wifiCredentials

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| encryptionType | dcfg:wifiEncryptionType | 1 | Indicates type of encryption used to provide network security |
| username | xs:string | 0..1 | Indicates username used to authenticate user in WPA-Enterprise, WPA2-Enterprise or WPA3-Enterpise protocols Only valid if encryptionType=6,7 or 8 |
| password | xs:string | 0..1 | Indicates password used to authenticate user in WPA-Enterprise, WPA2-Enterprise or WPA3-Enterpise protocols Only valid if encryptionType=6,7 or 8 |
| wep.key | xs:string | 0..1 | Indicates key in case of WEP encryption. Only valid if field encryptionType =2 |
| wpa.psk | xs:string | 0..1 | Indicates WPA Pre Shared Key Only valid if field encryptionType = 3,4 or 5 |

### -----------------------End of change 3-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?